3GPP TSG-RAN4 Meeting #117	R4-2522032

# Dallas, United States, 17th - 21st November, 2025

Agenda item:		4.13.1

Source:				Keysight Technologies UK Ltd

Title:					On NB-IoT and NR NTN OTA Test Time Reduction

Document for:		Approval

# Introduction

This contribution is reviewing test time reduction for NB-IoT and NR NTN OTA testing.

# Discussion

CTIA informed 3GPP RAN4 about their recent decision to include informative NB-IoT NTN test cases in their upcoming Version 9 CTIA Certification OTA Test Plan [1]. In order to reduce test time, a relatively drastic test approach was adopted for an NB-IoT NTN test case where the UE location and the satellite location were co-located, i.e., the NTN configuration was changed to a TN configuration by setting Koffset (k-Offset) to 0ms.

To address excessive test time concerns, especially the sensitivity search for TIS, the approved OTA test procedures deviate from current 3GPP TS 36.521-4 test procedures as follows: - For both TRP and TIS, implementation of a co-located configuration was selected where the UE location is the same as the satellite location, and the kOffset is set to 0ms. - For TRP and TIS, P0(Nominal PUSCH) may be adjusted higher in order to ensure that the DUT transmits at maximum TX power - For TIS, the uplink is control only, i.e. no user plane PUSCH data shall be configured.

Some discussion was held in the last RAN4#116-bis meeting on whether RAN4 should adopt the same approach as CTIA but some concerns were raised whether all NB-IoT NTN UEs would be able to support a truly co-located configuration with Koffset=0ms and the unrealistic scenario of this approach, e.g., from the WF [2].

### Sub-topic 2-1 Test method for NTN Issue 2-1-1: Discussions on alternative test configurations for NB-IoT NTN suggested in LS by CTIA Agreements: - RAN4 further check the improved configurations in LS as alternative for NB-IoT NTN OTA testing to reduce testing time. - Including feasibility of setting k-Offset=0ms for all NB-IoT NTN UEs - RAN4 will decide the configurations next meeting. Reply LS will be sent to CTIA.

We do believe 3GPP specs support Koffset to be set to 0ms but acknowledge that rather unrealistic test condition/ scenario for any NTN OTA testing.

An alternate NB-IoT test time approach for RAN4 to consider is what RAN5 has already adopted for NB-IoTNTN test cases to save a significant amount of test time, e.g., as originally introduced for OOB in [3]

7. Optionally, for GSO only device to reduce the test time, preconfigure the UE by any means to deactivate ephemeris consistency check so that the UE accepts NGSO (LEO-600) ephemeris. 8a. In case the ephemeris consistency check on the UE can be deactivated as described in step 7, the test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 36.508 [12] Table 8.2.6.2.1-2 for NGSO (LEO-600). 8b. In case the ephemeris consistency check on the UE cannot be deactivated as described in step 7, the test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 36.508 [12] Table 8.2.6.2.1-1 for GSO if UE supports only GSO and Table 8.2.6.2.1-2 for NGSO (LEO-600) if UE supports only NGSO satellites or both GSO and NGSO satellites. Test system shall send same SIB31-NB information during the duration of the test as defined in TS 36.508 [12] clause 8.2.6.3.1.

This ephemeris consistency check was validated by Keysight internally with at least one chipset vendor and was found to reduce test time by ~1/5. The advantages of this approach is that the NTN nature of the tests are preserved as well as the traceability to RAN5.

Additionally, we would like to point out efforts in Rel-19, e.g., WI for NGSO time varying orbit[4], to define even more realistic NTN conditions for conformance testing (RF, RRM and Demodulation). The reduction of the NTN to a TN scenario for OTA testing would therefore be counterproductive.

Proposal 1: Consider the ephemeris consistency check introduced and adopted in RAN5 specs already as alternate test time reduction approach for Rel-17 NB-IoT NTN OTA testing.

For Rel-18 and beyond NB-IoT NTN testing, further test time optimization could be achieved by disabling HARQ feedback functionality added in the Rel-18 enhancements work item [5], e.g.,

4.1.1 IoT-NTN Performance Enhancements in Rel-18 to address remaining issues from Rel-17 This work considers Rel-17 IoT-NTN as baseline as well as Rel-17 NR-NTN outcome and the further IoT-NTN performance enhancements objectives are listed below: - Disabling of HARQ feedback to mitigate impact of HARQ stalling on UE data rates [RAN1,RAN2] - Study and specify needed improved GNSS operations for a new position fix for UE pre-compensation during long connection times and for reduced power consumption. Simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed. [RAN1, RAN2] - NOTE: The need for RAN4 Core requirements for this objective will be identified after the conclusion on the need for improvements.

Proposal 2: Consider disabling of the HARQ feedback as alternate test time reduction approach for Rel-18 and beyond NB-IoT NTN OTA testing.

For NR NTN OTA testing, it is our understanding that NGSO is the most prevalent orbit from a commercial perspective; however, 3GPP defined testing using GSO and/or NGSO in Clauses 6 and 7 of [6], e.g.,

All requirements in this section are applicable to devices supporting GSO and/or NGSO satellites.

NR NTN does not have the restrictions that Rel-17 NB-IoT has due to technology advancements, i.e., for NB-IoT DL transmissions, it is not possible to transmit new DL data until the previous one is ACK’ed by the UE. For NR NTN, this restriction has been lifted from the beginning by allowing multiple HARQ processes. It will be worthwhile to study whether the ephemeris consistency check is needed for GSO or not. For NGSO, the test times for NR NTN OTA should be acceptable as is regardless of release.

Observation 1: For NGSO, the test times for NR NTN OTA should be acceptable as is regardless of release

Proposal 3: Study whether the ephemeris consistency check for NR NTN OTA testing is needed for GSO and/or whether the test time without this consistency check is acceptable.

To address test time concerns for devices that support both NGSO and GSO, it is proposed that for these devices, the test equipment in the OTA test environment shall emulate the signal with constant doppler and constant delay (per RAN5 agreements) according to ephemeris for NGSO (which LEO is FFS). UE RF test cases are already evaluating the GSO configuration when the device supports both NGSO and GSO [7].

Proposal 4: For devices that support both NGSO and GSO, the test equipment in the OTA test environment for the NR NTN OTA TC shall emulate the signal with constant doppler and constant delay (per RAN5 agreeements) according to ephemeris for NGSO (which LEO is FFS)

# Conclusion

The following observations and conclusions were made in this contribution.

Observation 1: For NGSO, the test times for NR NTN OTA should be acceptable as is regardless of release

Proposal 1: Consider the ephemeris consistency check introduced and adopted in RAN5 specs already as alternate test time reduction approach for Rel-17 NB-IoT NTN OTA testing.

Proposal 2: Consider disabling of the HARQ feedback as alternate test time reduction approach for Rel-18 and beyond NB-IoT NTN OTA testing.

Proposal 3: Study whether the ephemeris consistency check for NR NTN OTA testing is needed for GSO and/or whether the test time without this consistency check is acceptable.

Proposal 4: For devices that support both NGSO and GSO, the test equipment in the OTA test environment for the NR NTN OTA TC shall emulate the signal with constant doppler and constant delay (per RAN5 agreeements) according to ephemeris for NGSO (which LEO is FFS)

# References

- R4-2511714, LS on NB-IoT NTN OTA Testing, CTIA OTA WG, RAN4#116

- R4-2514868, WF on TRP_TRS_MIMO_OTA_Ph3, vivo, 3GPP TSG-RAN4 Meeting #116bis, October 2025

- R5-247333, Update of test conditions for IoT-NTN OOB blocking, ROHDE & SCHWARZ, MediaTek Beijing Inc., 3GPP TSG-RAN5 Meeting #105, November 2024

- RP-252076, Enhanced requirements and conductive test methodology for NR NTN and IoT NTN, Samsung, Xiaomi, 3GPP TSG RAN Meeting #109, September 2025

- RP-241351, WID on IoT NTN enhancements, MediaTek Inc., 3GPP TSG RAN Meeting #104, June 2024

- TS 38.101-5, User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements

- TS 38.521-5: User Equipment (UE) conformance specification; Radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performances