3GPP TSG-RAN WG4 Meeting #117	R4-2522122

Dallas, US, November 17 – 21, 2025

Agenda Item:	4.15.1

Source:	MediaTek Inc.

Title:	Discussion on core part for FR2-1 SSB based L3 measurement delay reduction

Document for:	Discussion

# Introduction

The core part for R19 RRM phase 5 WI was closed in RAN4#116 meeting. In this paper we discuss the remaining issues in the maintenance for both part 1 (i.e., optimizing CSSF) and part 2 (I.e., optimizing Rx BSF) based on the WF [1][2].

# Discussion

Issue 1-1-1: Condition of re-entering FSB after exiting <Way Forward>: - Further discuss in next meeting considering the below options: - Option1: Define T2 for re-evaluation after the UE exits FBS mode. - Option2: Not to define T2 for re-evaluation after the UE exits FBS mode. - Option 2.1: Need to update core requirements. - Option 2.2: Not need to update core requirements.

Previously RAN4 defined the conditions to apply FBS (i.e., Fast Beam Sweeping using multi-Rx capability) for L3 measurement delay reduction. The condition states that when serving cell condition is worse than a threshold (including RSRP and RSRQ threshold) FBS will be applied and a timer T1 will be triggered for 90s. Then the question is what will happen after this timer expires? Shall the UE re-enter again to FBS mode?

The main motivation behind activating FBS when serving cell condition drops below a threshold is that UE might expect a handover to occur very soon, e.g., within 90s period (UE moving from one cell to another) and therefore it will be useful to expedite L3 measurement using FBS. However, if handover does not happen within 90s, it is more likely that the UE has moved slightly away from the cell center and stayed stationary there. In this case we don’t see the necessity to mandate the UE to keep applying FBS since the UE might stay stationary for very long time (days, months or even years). However, it should up to UE whether to re-enter FBS again as long as UE has applied FBS during the first 90s since the conditions are met.

Another scenario is that when the conditions are met and FBS is applied while T1 timer is running, then the conditions become unmet during T1. In this case UE will exit FBS and whenever the conditions are met again UE re-enter FBS and T1 timer will be triggered again. Therefore, the same requirements of the first entry apply for this re-entry (i.e., when meeting, not meeting, then meeting the conditions).

Therefore, for the two scenarios shown below we provide our view:

- When FBS conditions are always met and T1 timer expires, re-entry to FBS should be up to UE

- When FBS conditions become unmet during T1 timer, and then met again, re-entry to FBS can be performed again same as the first entry.

Observation 1: It is possible that FBS conditions are always met for some stationary UEs for very long time (days, months or even years). However, it should up to UE whether to re-enter FBS again as long as UE has applied FBS during the first 90s since the conditions are met.

Proposal 1: Not to mandate UEs of keep applying FBS repeatedly since FBS conditions might be always met for some stationary UEs for very long time.

Proposal 2: When FBS conditions are always met and T1 timer expires, re-entry to FBS should be up to UE.

- Not need to update core requirements.

Proposal 3: When FBS conditions are met, not met, then met again, re-entry to FBS can be performed same as the first entry.

- Not need to update core requirements.

Issue 1-1-4: Applicability of FBS when the target and serving carrier frequency are on different <Way Forward>: - Further discuss the options: - Option 1: When the target and serving carrier frequency are different, FBS applies to target carrier only. - Option 2: When the target and serving carrier frequency are different, FBS applies to the serving carrier and target carrier. - Option 3: Introduce UE capability indicating UE can support FBS on both target and serving carrier frequency.

For issue 1-1-4, it is not clear the intention of this issue or the impact on the specs. The issue depends on the scenario for which FBS is applied. For example, when FBS is activated to perform handover, there is only target carrier in this case for which FBS can be applied. On the other hand, for intra-frequency measurements and inter-frequency measurements with measurement gap, the requirements of FBS applies to both independently, thus we don’t see any update needed in the spec.

Proposal 4: Regarding applicability of FBS when the target and serving carrier frequency are different:

- If FBS is activated based on handover operation, there is only target carrier in this case for which FBS can be applied.

- If FBS is activated based on triggering conditions, the requirements of FBS applies independently to both intra-frequency measurements and inter-frequency measurements with measurement gap. Thus, no update in the spec is needed.

# Summary

In this contribution, the following observations and proposals are concluded:

Observation 1: It is possible that FBS conditions are always met for some stationary UEs for very long time (days, months or even years). However, it should up to UE whether to re-enter FBS again as long as UE has applied FBS during the first 90s since the conditions are met.

Proposal 1: Not to mandate UEs of keep applying FBS repeatedly since FBS conditions might be always met for some stationary UEs for very long time.

Proposal 2: When FBS conditions are always met and T1 timer expires, re-entry to FBS should be up to UE.

- Not need to update core requirements.

Proposal 3: When FBS conditions are met, not met, then met again, re-entry to FBS can be performed same as the first entry.

- Not need to update core requirements.

Proposal 4: Regarding applicability of FBS when the target and serving carrier frequency are different:

- If FBS is activated based on handover operation, there is only target carrier in this case for which FBS can be applied.

- If FBS is activated based on triggering conditions, the requirements of FBS applies independently to both intra-frequency measurements and inter-frequency measurements with measurement gap. Thus, no update in the spec is needed.

# References

- R4-2514800, WF on RRM requirements for NR_RRM_Ph5_Part1, Ericsson.

- R4-2514801, WF on NR_RRM_Ph5_Part2, CATT.