3GPP TSG-RAN WG4 Meeting #117	R4-2522126

Dallas, US, November 17 – 21, 2025

Agenda Item:	7.4.3.2

Source:	MediaTek Inc.

Title:	Discussion on RRM requirements for (e)RedCap UE enhancement

Document for:	Discussion

# Introduction

In the last meeting, RAN4 started the discussion on R20 RRM phase 6 WI according to the agreed WID shown below. The approved WF is captured in [1]. In this paper, we discuss and provide our views on the suggested RRM requirements for €RedCap UE.

- Specify the following requirements for (e)RedCap Ues [RAN4] - Relaxed RLM/BFD RRM requirements - Use R17 RLM/BFD relaxation for legacy UE as a baseline - Support of less than 5 MHz channel bandwidth (15 PRB transmission bandwidth configuration and 12 PRB SSB) - RF requirements, considering example bands: n100, n106, n26, n28 and n85 - RRM requirements including relaxed RLM/BFD RRM requirements - Note 1: Use R18 3 MHz requirements for non-RedCap UE as a baseline - Note 2: Consider both 1Rx and 2Rx €RedCap Ues and HD-FDD

# Discussion

## 2.1 Relaxed RLM/BFD RRM requirements

In NR, existing Rel-17 RLM/BFD relaxation for power saving has been specified only for non-RedCap UEs in FR1 and FR2, and in this release it is agreed to discuss this relaxation for (e)RedCap UEs. In our view, the relaxed RLM/BFD requirements for (e)RedCap can discussed taking into account the following aspects

- 2Rx (e)RedCap UE

- 1Rx (e)RedCap UE

In the last meeting, RAN4 agreed to re-use the existing (non-RedCap) R17 RLM/BFD relaxation requirements for both FR1 and FR2, and for full-duplex FDD and TDD, to 2Rx (e)RedCap UE as shown below [1].

Issue 2-2-2: Relaxed RLM/BFD for 2Rx (e)RedCap UE Agreement: - Reuse the existing R17 RLM/BFD relaxation requirements for both FR1 and FR2, and for full-duplex FDD and TDD.

The difference between 1Rx and 2Rx RedCap UE from the legacy RLM/BFD requirements is that the evaluation requirement TEvaluate_out_SSB,RedCap for 1Rx RedCap UE is extended (doubled) compared to 2Rx RedCap UE (as shown below), compensating for the diversity gain. However, in the context of relaxing these RLM/BFD requirements in this release, we don’t see the need to have different relaxation requirements/factors for 2Rx and 1Rx RedCap UE. In our view, RAN4 can further discuss whether to reuse the R17 RLM/BFD relaxation requirements for 1Rx RedCap UE as well.

Observation 1: The RLM/BFD evaluation requirement TEvaluate_out_SSB,RedCap for 1Rx RedCap UE is extended (doubled) compared to 2Rx RedCap UE

Proposal 1: RAN4 to discuss whether to reuse R17 RLM/BFD relaxation requirements for 1Rx RedCap UE same as 2Rx RedCap UE.

Regarding the relaxation criteria, in R17 RLM/BFD relaxation two criteria were defined:

- Serving cell quality criteria: UE estimated SINR should be larger than a threshold

- Low mobility criteria: UE estimated SINR or RSRP should not drop more than a threshold within a given time period

In our view the same criteria can be applied for RedCap UE supporting RLM/BFD relaxation.

Proposal 2: Re-use R17 RLM/BFD relaxation criteria (serving cell quality criteria and Low mobility criteria) for RedCap UE supporting RLM/BFD relaxation.

## 2.2 Support of less than 5 MHz (LT5) channel bandwidth

In the last meeting, RAN4 made the following agreement regarding LT5 [1].

Issue 2-1-2: LT5 for 2Rx (e)RedCap UE Agreement: - The existing LT5 requirements for non-RedCap UE can be directly reused for 2Rx (e)RedCap UE with full-duplex FDD, TDD modes. - Handover - RLM/BFD - Intra-frequency/Inter-frequency measurement - Further check for HD-FDD

Since there are the following cases for RedCap UE, the existing LT5 requirements for non-RedCap UE can only apply for 2Rx RedCap full duplex (FDD, TDD)

- 2Rx RedCap full duplex (FDD, TDD)  reuse the existing LT5 requirements

- 2Rx RedCap HD-FDD   ?

- 1Rx RedCap full duplex (FDD, TDD)  ?

- 1Rx RedCap HD-FDD  ?

For HD-FDD 2Rx RedCap UE, in our view, the existing LT5 requirements for non-RedCap can also apply to 2Rx RedCap HD-FDD with adding similar condition used in legacy for HD-FDD RedCap requirements (e.g., for HD-FDD handover, HD-FDD RLM/BFD).

Proposal 3: The existing LT5 requirements for non-RedCap can also apply to 2Rx RedCap HD-FDD with adding similar condition used in legacy HD-FDD RedCap requirements.

On the other hand, for 1Rx RedCap UE, RAN4 need further discussion on how define LT5 requirements for such case taking into account the PDCCH hypothetical parameters. RAN4 also need to discuss any consequences from targeting coverage equivalence between 1Rx and 2Rx RedCap UEs.

Proposal 4: For LT5 requirements for 1Rx RedCap UE, RAN4 to discuss any consequences from targeting coverage equivalence between 1Rx and 2Rx RedCap UEs.

# Summary

In this contribution, the following observations and proposals are concluded:

Observation 1: The RLM/BFD evaluation requirement TEvaluate_out_SSB,RedCap for 1Rx RedCap UE is extended (doubled) compared to 2Rx RedCap UE

Proposal 1: RAN4 to discuss whether to reuse R17 RLM/BFD relaxation requirements for 1Rx RedCap UE same as 2Rx RedCap UE.

Proposal 2: Re-use R17 RLM/BFD relaxation criteria (serving cell quality criteria and Low mobility criteria) for RedCap UE supporting RLM/BFD relaxation.

Proposal 3: The existing LT5 requirements for non-RedCap can also apply to 2Rx RedCap HD-FDD with adding similar condition used in legacy HD-FDD RedCap requirements.

Proposal 4: For LT5 requirements for 1Rx RedCap UE, RAN4 to discuss any consequences from targeting coverage equivalence between 1Rx and 2Rx RedCap UEs.

# References

- R4-2514821, WF on NR_RRM_Ph6, Ericsson.