3GPP TSG-RAN WG4 Meeting #117    	  											R4-2522195

Dallas, USA, 17th – 21st of November, 2025

Title: 	Confirmation of demodulation performance requirements values for NTN NB-IoT TDD

Agenda Item:	6.21.3 – UE and SAN demodulation performance requirements

Release:	Rel-19

Source: 	THALES, Iridium Satellite LLC, CCL, Nordic Semiconductor

Document for:	Discussion

# WID information

A new WID Introduction of IoT-NTN TDD mode (RP-242415, New WID on introduction of IoT-NTN TDD mode, 3GPP TSG RAN Meeting #105, Melbourne, Australia, September 9-12, 2024) was approved in RAN#105. That WID has been revised in RAN#106, as the study phase had been completed. Then, a work plan for Revised WID on Introduction of IoT-NTN TDD mode (RP-243293, Revised WID on introduction of IoT-NTN TDD mode, 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9-12, 2024) is proposed for RAN 1, 2, and 4.

## Core part WI

The work item aims to specify enhancements for NB-IoT NTN to enable NTN operation with a NB-IoT TDD mode leveraging commonalities with half-duplex NB-IoT FDD NTN, by defining a new NB-IoT TDD mode for NTN based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures for the NB-IoT operation in the targeted unpaired MSS allocated band (TN deployment is not expected in this band). The feature is not intended to be applicable to existing 3GPP bands.

The work objectives assume the following:

- LEO @600 km and @1200 km orbit respectively, with set-1 satellite parameters as reference scenarios (See 3GPP TR 36.763)

- Target the 1616-1626.5 MHz MSS allocated band

- Standalone deployment with anchor and non-anchor carriers (i.e. operating in carrier(s) used only for NB-IoT)

- Operate with Earth fixed Tracking area, with either Earth fixed cells or Earth moving cells for NGSO

- The new NB-IoT NTN TDD mode allows configuring the usage of radio resources in the targeted MSS allocated band with a periodic subset of the UL and DL subframes in N radio frames. The periodic pattern should consist of non-overlapping set of usable contiguous UL subframes (U) and set of usable contiguous DL subframes (D), and guard periods, which is periodic every N radio frames, with N=9 for the target MSS allocated band.

This work item includes the following objectives:

- Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, including:

  - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern, necessary adaptation (if needed) and associated UE procedures [RAN1, RAN2]

      - Support a pattern with a period of 9 radio frames for the target MSS allocated band, where D=U=8 with a fixed guard period. RAN1 to consider whether there is a need for a mechanism to achieve an adjustable guard period for the purpose of allowing deployment with the TDD frame structure of the legacy system operating in the target MSS allocated band.

  - Other necessary impacts on higher layers [RAN2]

  - RRM and RF core requirements [RAN4]

- Specify a new NB-IoT TDD operating NTN band for the MSS allocation spanning 1616-1626.5 MHz for DL and UL, based on the outcome of the study, to be used as example band for this WI [RAN4].

  - Specify band numbering

  - Specify SAN and UE RF characteristics

  - Specify DL and UL channelization.

  - Specify channel bandwidth as 200 kHz

  - Note1: No NTN-NTN coexistence study needed.

  - Note2: Leverage existing work as much as possible for TN-NTN coexistence of adjacent bands

Note3: Reuse the existing RAN4 requirements as much as possible (no intention to have any relaxation of the RF emissions requirements)

## Performance part WI

Specify the following:

- RRM performance requirements, as needed [RAN4]

- Necessary SAN and UE demodulation requirements [RAN4]

- SAN conformance testing requirements [RAN4]

## Remaining work from current Work Plan

The following updated & adapted WP from R4-2502291/RP-252935 is:

- 3GPPRAN4#116-bis (2025-10-13 - 2025-10-17)

  - Start discussion on demodulation performance and specific configuration related to the new TDD pattern;

  - Start discussion of conformance testing for core part, endorse CRs if any.

- 3GPPRAN4#117 (2025-11-17 - 2025-11-21)

  - Continue discussion and drafting CRs for demodulation performance;

  - Start drafting performance requirements CRs (TS 36.102, TS 36.108, TS 36.133), endorse CRs if any;

  - Start discussion of conformance testing for demodulation performance part (TS 36.181), endorse CRs if any.

- 3GPPRAN4#118 (2026-02-09 - 2026-02-13)

  - Finish demodulation performance CR;

  - Finish conformance testing CRs (if any left).

For the conformance testing aspects, please also recall:

Issue 1-5-2: Work Split for NB-IoT TDD NTN in TS 36.181

Agreement:

| Clause/Requirement | Company (other companies are encouraged to contribute) |
| --- | --- |
| Definitions/Abbreviations | Iridium |
| Conducted/Radiated REFSENS update | THALES |
| Conducted/Radiated Dynamic range update | Iridium |
| Conducted ON/OFF requirements | Iridium |
| ACS requirement (double-check) | Iridium |
| EVM Annex?, Other.. |  |

Note: Other identified Clauses are not precluded

For further information please also check:

- R4-2502292, Way Forward for [114][318] IoT_NTN_TDD_UE_SAN_RF, THALES;

- R4-2504719, Way Forward for [114bis][316] IoT_NTN_TDD_UE_SAN_RF, THALES;

- R4-2508781, Way Forward for [115][316] IoT_NTN_TDD_UE_SAN_RF, THALES;

- R4-2512666, Way Forward for [116][316] IoT_NTN_TDD_UE_SAN_RF, THALES.

Please also check the approved list of documents at RAN#109:

| T-Doc | Title | Company | Type | Decision |
| --- | --- | --- | --- | --- |
| RP-252329 | Status Report - WID on introduction of IoT NTN TDD mode | Iridium Satellite LLC | WI status report | Noted |
| RP-252624 | Corrections to Rel-19 IoT-NTN TDD mode (36.211/213) | RAN1 | CR pack | Approved |
| RP-252795 | Introduction of IoT-NTN TDD mode | RAN2 | CR pack | Approved |
| RP-252407 | RAN4 CRs to IoT_NTN_TDD | RAN4 | CR pack | Approved |
| RP-252935 | Revised WID for introduction of IoT-NTN TDD mode | Iridium Satellite LLC | WID revised | Approved |
| RP-252872 | WI summary for WI Core part: Introduction of IoT-NTN TDD mode | Iridium | WI summary | Approved |

Moreover, at RAN4#116bis, R4-2515042 presented the simulation results collection for IoT_NTN_TDD (Thales, Nokia, Iridium Satellite LLC, CCL, Nordic Semiconductor), and the Way Forward was included in Way Forward for [116bis][312] Rel-19 Demodulation_Part2 (R4-2515160, Nokia) with the following work split:

Specification – TS 36.102

| Big CR | Nordic Semiconductor |
| --- | --- |
| NPDSCH (inc Test Applicability) | Nordic Semiconductor |

Specification – TS 36.108

| Big CR | THALES |
| --- | --- |
| NPUSCH Format 0 and Format 1 Requirements (inc Test Applicability if required) | THALES |
| NPRACH Requirements (inc Test Applicability if required) | THALES |

Specification – TS 36.181

| Big CR | Nokia |
| --- | --- |
| NPUSCH Format 0 and Format 1 Requirements (inc Test Applicability if required) | Nokia |
| NPRACH Requirements (inc Test Applicability if required) | Nokia |

This contribution is therefore to confirm the values from R4-2515042.

# Confirmation of demodulation performance values

## NPRACH demodulation performance values (SAN side)

R4-2515042 indicates the following NPRACH simulation results (Format 0) with impairments:

|  |  |  |  |  | Source | Thales (dB) | Iridium, CCL (dB) | Nordic (dB) | Span (dB) | STD (dB) | Average (dB) | Implementation Margin and Impairement Margin (or higher) (dB) | Requirements SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency Offset | Tx Duration | Repetition number | Channel model | Antenna Configuration | Tdoc | R4-2514422 | R4-2513825 | R4-2514247 |  |  |  |  |  |
| 200 Hz | 8 ms | 8 | NTN-TDLA100-1 | 1T1R |  | 8,9 | 5,8 |  | 3,1 | 2,2 | 7,4 | 2,5 | 9,9 |
|  | 16 ms | 16 |  | 1T1R |  | 4,62 | 1,5 |  | 3,1 | 2,2 | 3,1 | 2,5 | 5,6 |
| 200 Hz | 8 ms | 8 | NTN-TDLC5-1 | 1T1R |  | 5,48 | 2,2 |  | 3,3 | 2,3 | 3,8 | 2,5 | 6,3 |
|  | 16 ms | 16 |  | 1T1R |  | 2,27 | -0,5 |  | 2,8 | 2,0 | 0,9 | 2,5 | 3,4 |

R4-2515042 indicates the following NPRACH simulation results (Format 1) with impairments:

|  |  |  |  |  | Source | Thales (dB) | Iridium, CCL (dB) | Nordic (dB) | Span (dB) | STD (dB) | Average (dB) | Implementation Margin and Impairement Margin (or higher) (dB) | Requirements SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency Offset | Tx Duration | Repetition number | Channel model | Antenna Configuration | Tdoc | R4-2514422 | R4-2513825 | R4-2514247 |  |  |  |  |  |
| 200 Hz | 8 ms | 8 | NTN-TDLA100-1 | 1T1R |  | 9,22 | 6 |  | 3,2 | 2,3 | 7,6 | 2,5 | 10,1 |
| 200 Hz | 8 ms | 8 | NTN-TDLC5-1 | 1T1R |  | 5,51 | 2,4 |  | 3,1 | 2,2 | 4,0 | 2,5 | 6,5 |

However, the 16 repetitions for NPRACH format 1 could still be used, as represented below:

|  |  |  |  |  | Source | Thales (dB) | Iridium, CCL (dB) | Nordic (dB) | Span (dB) | STD (dB) | Average (dB) | Implementation Margin and Impairement Margin (or higher) (dB) | Requirements SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency Offset | Tx Duration | Repetition number | Channel model | Antenna Configuration | Tdoc | R4-2514422 | R4-2513825 | R4-2514247 |  |  |  |  |  |
| 200 Hz | 16 ms | 16 | NTN-TDLA100-1 | 1T1R |  | 4,62 | 2,1 |  | 2,5 | 1,8 | 3,4 | 2,5 | 5,9 |
| 200 Hz | 16 ms | 16 | NTN-TDLC5-1 | 1T1R |  | 2,1 | -0,3 |  | 2,4 | 1,7 | 0,9 | 2,5 | 3,4 |

## NPUSCH demodulation performance values (SAN side)

R4-2515042 indicates the following NPUSCH simulation results (Format 1) with impairments:

|  |  |  |  |  |  |  |  | Source | Thales (dB) | Iridium, CCL (dB) | Nordic (dB) | Span (dB) | STD (dB) | Average (dB) | Implementation Margin and Impairement Margin (or higher) (dB) | Requirements SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BW/Subcarrier Spacing | Allocated Subcarriers | Frequency Offset | Channel model | Antenna Configuration | Tx Duration | Repetition Number | % of TP | Tdoc | R4-2514422 | R4-2513825 | R4-2514247 |  |  |  |  |  |
| 200 kHz/15 kHz | 12 | 8 Hz | NTN-TDLA100-1 | 1T1R | 4 ms | 4 | 70 |  | 7 | 11,3 |  | 4,3 | 3,0 | 9,2 | 2,5 | 11,7 (A7-2) |
| 200 kHz/15 kHz | 12 | 8 Hz | NTN-TDLC5-1 | 1T1R | 4 ms | 4 | 70 |  | 4 | 7,9 |  | 3,9 | 2,8 | 6,0 | 2,5 | 8,5 (A7-2) |

However, other configurations with 1 subcarrier/1 tone allocation could be still included, as represented below:

|  |  |  |  |  |  |  |  | Source | Thales (dB) | Iridium, CCL (dB) | Nordic (dB) | Span (dB) | STD (dB) | Average (dB) | Implementation Margin and Impairement Margin (or higher) (dB) | Requirements SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BW/Subcarrier Spacing | Allocated Subcarriers | Frequency Offset | Channel model | Antenna Configuration | Tx Duration | Repetition Number | % of TP | Tdoc | R4-2514422 | R4-2513825 | R4-2514247 |  |  |  |  |  |
| 200 kHz/15 kHz | 1 | 8 Hz | NTN-TDLA100-1 | 1T1R | 16 ms | 2 | 70 |  |  | -1,1 |  | 0,0 | #DIV/0! | -1,1 | 2,5 | 1,4 (A14-3) |
| 200 kHz/15 kHz | 1 | 8 Hz | NTN-TDLC5-1 | 1T1R | 16 ms | 2 | 70 |  |  | -4,1 |  | 0,0 | #DIV/0! | -4,1 | 2,5 | -1,6 (A14-3) |
|  | 1 |  | NTN-TDLC5-1 |  | 8 ms | 1 |  |  | -2,5 | -1,2 |  | 1,3 | 0,9 | -1,9 | 2,5 | 0,7 (A14-3) |

R4-2515042 indicates the following NPUSCH simulation results (Format 2) with impairments:

|  |  |  |  |  |  |  |  | Source | Thales (dB) | Iridium, CCL (dB) | Nordic (dB) | Span (dB) | STD (dB) | Average (dB) | Implementation Margin and Impairement Margin (or higher) (dB) | Requirements SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BW/Subcarrier Spacing | Allocated Subcarriers | Frequency Offset | Channel model | Antenna Configuration | Tx Duration | Repetition Number | % of TP | Tdoc | R4-2514422 | R4-2513825 | R4-2514247 |  |  |  |  |  |
| 200 kHz/15 kHz | 1 | 16 | NTN-TDLA100-1 | 1T1R | 16 ms | 8 | 70 |  |  | -2,1 |  | 0,0 | #DIV/0! | -2,1 | 2,5 | 0,4 |

## NPDSCH demodulation performance values (UE side)

R4-2515042 indicates the following NPDSCH simulation results with impairments:

|  |  |  |  |  |  |  | Source | Thales (dB) | Iridium, CCL (dB) | Nordic (dB) | Span (dB) | STD (dB) | Average (dB) | Implementation Margin and Impairement Margin (or higher) (dB) | Requirements (Average with IM) SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth | Carrier Type | Repetition number | Channel model | Number of Ports | Antenna Configuration | % of TP | Tdoc | R4-2514422 | R4-2513825 | R4-2514247 |  |  |  |  |  |
| 200 kHz | Anchor | 4 | NTN-TDLC5-200 | 1 | 1T1R | 70 |  | -1,3 | -1,3 | -2 | 0,7 | 0,4 | -1,5 | 2,5 | 1,0 |
|  | Non-Anchor | 4 | NTN-TDLA100-10 | 1 | 1T1R | 70 |  |  | -1,1 |  | 0,0 | #DIV/0! | -1,1 | 2,5 | 1,4 |

# Conclusions

Proposal 1: RAN4 to confirm NPRACH Format 0 demodulation requirement of 9,9 dB with (Frequency Offset, Tx Duration, Repetition number, Channel model, Antenna Configuration) configuration of (200 Hz, 8, 8, NTN-TDLA100-1, 1T1R).

Proposal 2: RAN4 to confirm NPRACH Format 0 demodulation requirement of 5,6 dB with (Frequency Offset, Tx Duration, Repetition number, Channel model, Antenna Configuration) configuration of (200 Hz, 16, 16, NTN-TDLA100-1, 1T1R).

Proposal 3: RAN4 to confirm NPRACH Format 0 demodulation requirement of 6,3 dB with (Frequency Offset, Tx Duration, Repetition number, Channel model, Antenna Configuration) configuration of (200 Hz, 8, 8, NTN-TDLC5-1, 1T1R).

Proposal 4: RAN4 to confirm NPRACH Format 0 demodulation requirement of 3,4 dB with (Frequency Offset, Tx Duration, Repetition number, Channel model, Antenna Configuration) configuration of (200 Hz, 16, 16, NTN-TDLC5-1, 1T1R).

Proposal 5: RAN4 to confirm NPRACH Format 1 demodulation requirement of 10,1 dB with (Frequency Offset, Tx Duration, Repetition number, Channel model, Antenna Configuration) configuration of (200 Hz, 8, 8, NTN-TDLA100-1, 1T1R).

Proposal 6: RAN4 to confirm NPRACH Format 1 demodulation requirement of 6,5 dB with (Frequency Offset, Tx Duration, Repetition number, Channel model, Antenna Configuration) configuration of (200 Hz, 8, 8, NTN-TDLC5-1, 1T1R).

Proposal 7: RAN4 to consider NPRACH Format 1 demodulation requirement of 5,9 dB with (Frequency Offset, Tx Duration, Repetition number, Channel model, Antenna Configuration) configuration of (200 Hz, 16, 16, NTN-TDLA100-1, 1T1R).

Proposal 8: RAN4 to consider NPRACH Format 1 demodulation requirement of 3,4 dB with (Frequency Offset, Tx Duration, Repetition number, Channel model, Antenna Configuration) configuration of (200 Hz, 16, 16, NTN-TDLC5-1, 1T1R).

Proposal 9: RAN4 to confirm NPUSCH Format 1 demodulation requirement of 11,7 dB with (BW/Subcarrier Spacing, Allocated Subcarriers, Frequency Offset, Channel model, Antenna Configuration, Tx Duration, Repetition Number) configuration of (200 kHz/15 kHz, 12, 8 Hz, NTN-TDLA100-1, 1T1R, 4, 4).

Proposal 10: RAN4 to confirm NPUSCH Format 1 demodulation requirement of 8,5 dB with (BW/Subcarrier Spacing, Allocated Subcarriers, Frequency Offset, Channel model, Antenna Configuration, Tx Duration, Repetition Number) configuration of (200 kHz/15 kHz, 12, 8 Hz, NTN-TDLC5-1, 1T1R, 4, 4).

Proposal 11: RAN4 to consider NPUSCH Format 1 demodulation requirement of 1,4 dB with (BW/Subcarrier Spacing, Allocated Subcarriers, Frequency Offset, Channel model, Antenna Configuration, Tx Duration, Repetition Number) configuration of (200 kHz/15 kHz, 1, 8 Hz, NTN-TDLA100-1, 1T1R, 16, 2).

Proposal 12: RAN4 to consider NPUSCH Format 1 demodulation requirement of -1,6 dB with (BW/Subcarrier Spacing, Allocated Subcarriers, Frequency Offset, Channel model, Antenna Configuration, Tx Duration, Repetition Number) configuration of (200 kHz/15 kHz, 1, 8 Hz, NTN-TDLC5-1, 1T1R, 16, 2).

Proposal 13: RAN4 to consider NPUSCH Format 1 demodulation requirement of 0.7 dB with (BW/Subcarrier Spacing, Allocated Subcarriers, Frequency Offset, Channel model, Antenna Configuration, Tx Duration, Repetition Number) configuration of (200 kHz/15 kHz, 1, 8 Hz, NTN-TDLC5-1, 1T1R, 8, 1).

Proposal 14: RAN4 to confirm NPUSCH Format 2 demodulation requirement of 0,4 dB with (BW/Subcarrier Spacing, Allocated Subcarriers, Frequency Offset, Channel model, Antenna Configuration, Tx Duration, Repetition Number) configuration of (200 kHz/15 kHz, 1, 16 Hz, NTN-TDLA100-1, 1T1R, 16, 8).

Proposal 15: RAN4 to confirm NPDSCH demodulation requirement of 1,0 dB with (Bandwidth, Carrier Type, Repetition number, Channel model, Number of Ports, Antenna Configuration) configuration of (200 kHz, Anchor, 4, NTN-TDLC5-200, 1, 1T1R).

Proposal 16: RAN4 to confirm NPDSCH demodulation requirement of 1,4 dB with (Bandwidth, Carrier Type, Repetition number, Channel model, Number of Ports, Antenna Configuration) configuration of (200 kHz, Non-Anchor, 4, NTN-TDLA100-10, 1, 1T1R).

# ANNEX with simulation results from R4-2514422 - Demodulation performance requirements for NTN NB-IoT TDD (without tolerance and implementation margin) – RAN4#116bis

## NPRACH simulation results and proposals (SAN side)

The results from this section are to be integrated as part of TS 36.108.

[RAN4#116bis] Proposal 1: RAN4 to consider the following simulation results for TDD NB-IoT NPRACH with NTN-TDLA channel model

[RAN4#116bis] Proposal 2: RAN4 to consider the following simulation results for TDD NB-IoT NPRACH with NTN-TDLC channel model

[RAN4#116bis] Proposal 3: Try to align as much as possible with RAN1 assumption and consider NTN-TDLC as well for NPRACH.

[RAN4#116bis] Proposal 4: RAN4 to consider the integration of the following specifications:

The probability of detection shall be equal to or exceed 99% for the SNR levels listed in table 8.5.3.2.1-1 and 8.5.3.2.1-2.

Table 8.5.3.2.1-1: NPRACH missed detection requirements for FDD

| Number of TX antennas | Number of RX antennas | TxDuration | Repetition number | Propagation conditions and correlation matrix (Annex D) | Frequency offset | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Preamble format 0 | Preamble format 1 |
| 1 | 1 | 8 | 8 | NTN-TDLA100-1 | 200 Hz | 13.3 | 13.7 |
| 1 | 1 | 16 | 16 | NTN-TDLA100-1 | 200 Hz | 9.8 | 9.6 |
| 1 | 2 | 8 | 8 | NTN-TDLA100-1 | 200 Hz | 6.0 | 6.1 |
| 1 | 2 | 16 | 16 | NTN-TDLA100-1 | 200 Hz | 1.6 | 1.7 |

Table 8.5.3.2.1-2: NPRACH missed detection requirements for TDD

| Number of TX antennas | Number of RX antennas | TxDuration | Repetition number | Propagation conditions and correlation matrix (Annex D) | Frequency offset | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Preamble format 0 | Preamble format 1 |
| 1 | 1 | 8 | 8 | NTN-TDLA100-1 | 200 Hz | 8.9 | 9.22 |
| 1 | 1 | 16 | 16 | NTN-TDLA100-1 | 200 Hz | 4.62 | 4.62 |
| 1 | 1 | 8 | 8 | NTN-TDLC5-1 | 200 Hz | 5.48 | 5.51 |
| 1 | 1 | 16 | 16 | NTN-TDLC5-1 | 200 Hz | 2.27 | 2.1 |

And where (see TS 36.108):

The propagation conditions used for the performance measurements in multi-path fading environment are indicated as a combination of a channel model name and a maximum Doppler frequency, i.e., NTN-TDLA<DS>-<Doppler> or NTN-TDLC<DS>-<Doppler> where '<DS>' indicates the desired delay spread and '<Doppler>' indicates the maximum Doppler frequency (Hz).

Table D.2.2-1 show the propagation conditions that are used for the performance measurements in multi-path fading environment.

Table D.2.2-1: Channel model parameters

| Combination name | Tapped delay line model | Maximum Doppler frequency |
| --- | --- | --- |
| NTN-TDLA100-5 | NTN-TDLA100 | 5 Hz |
| NTN-TDLC5-5 | NTN-TDLC5 | 5 Hz |

## NPUSCH simulation results and proposals (SAN side)

The results from this section are to be integrated as part of TS 36.108.

[RAN4#116bis] Proposal 5: RAN4 to consider the following simulation results for TDD NB-IoT NPUSCH format 1 with NTN-TDLA100 channel model with TBS 136 bits (12 subcarriers and QPSK, A7-2):

Table P5. NPUSCH format 1 with NTN-TDLA100, TBS 136 bits, 12 subcarriers, QPSK (A7-2)

| SNR | BLER | Comment |
| --- | --- | --- |
| -10 | 1 |  |
| -9 | 1 |  |
| -8 | 1 |  |
| -7 | 1 |  |
| -6 | 1 |  |
| -5 | 1 |  |
| -4 | 1 |  |
| -3 | 0,990099 |  |
| -2 | 0,952381 |  |
| -1 | 0,925926 |  |
| 0 | 0,884956 |  |
| 1 | 0,826446 |  |
| 2 | 0,746269 |  |
| 3 | 0,662252 |  |
| 4 | 0,502513 |  |
| 5 | 0,414938 |  |
| 6 | 0,355872 |  |
| 7 | 0,294118 | Target SNR value selected: 7dB (at 70% throughput or 30% BLER) |
| 8 | 0,220264 |  |
| 9 | 0,19084 |  |
| 10 | 0,169205 |  |

[RAN4#116bis] Proposal 6: RAN4 to consider the following simulation results for TDD NB-IoT NPUSCH format 1 with NTN-TDLC channel model with TBS 16 bits (1 subcarrier and π/2 BPSK, A14-3):

Table P5. NPUSCH format 1 with NTN-TDLC, TBS 16 bits, 1 subcarrier, π/2 BPSK (A14-3)

| SNR | BLER | Comment |
| --- | --- | --- |
| -10 | 0,980392 |  |
| -9 | 0,970874 |  |
| -8 | 0,892857 |  |
| -7 | 0,806452 |  |
| -6 | 0,704225 |  |
| -5 | 0,636943 |  |
| -4 | 0,485437 |  |
| -3 | 0,357143 | Target SNR value selected: -2.5dB (at 70% throughput or 30% BLER) |
| -2 | 0,242131 | Target SNR value selected: -2.5dB (at 70% throughput or 30% BLER) |
| -1 | 0,190476 |  |
| 0 | 0,13624 |  |
| 1 | 0,0816327 |  |
| 2 | 0,0515996 |  |
| 3 | 0,03125 |  |
| 4 | 0,0167308 |  |
| 5 | 0,009 |  |

Some more results were also provided below with 4 NPUSCH repetitions instead of 1 repetition from previous figure, but we are not proposing any demodulation performance test for this configuration.

| SNR | BLER | Comment |
| --- | --- | --- |
| -10 | 0,574713 |  |
| -9 | 0,408163 |  |
| -8 | 0,275482 | Target SNR value selected: -8dB (at 70% throughput or 30% BLER) |
| -7 | 0,177305 |  |
| -6 | 0,108108 |  |
| -5 | 0,0586166 |  |
| -4 | 0,0334672 |  |
| -3 | 0,0176835 |  |
| -2 | 0,009 |  |

[RAN4#116bis] Proposal 7: RAN4 shall not consider 4 repetitions for NPUSCH demodulation performance with A14-3 – 1 tone with 16 bits payload (and if possible 2 repetitions should be excluded as well for 1 tone transmission).

[RAN4#116bis] Proposal 8: RAN4 to consider the following simulation results for TDD NB-IoT NPUSCH format 1 with NTN-TDLC channel model with TBS 136 bits (12 subcarriers and QPSK, A7-2):

| SNR | BLER | Comment |
| --- | --- | --- |
| -10 | 1 |  |
| -9 | 1 |  |
| -8 | 1 |  |
| -7 | 1 |  |
| -6 | 1 |  |
| -5 | 1 |  |
| -4 | 1 |  |
| -3 | 1 |  |
| -2 | 0,990099 |  |
| -1 | 0,970874 |  |
| 0 | 0,862069 |  |
| 1 | 0,70922 |  |
| 2 | 0,534759 |  |
| 3 | 0,431034 |  |
| 4 | 0,294118 | Target SNR value selected: 4dB (at 70% throughput or 30% BLER) |
| 5 | 0,203252 |  |
| 6 | 0,158983 |  |
| 7 | 0,0855432 |  |
| 8 | 0,0522193 |  |
| 9 | 0,0311333 |  |
| 10 | 0,016518 |  |

[RAN4#116bis] Proposal 9: RAN4 to consider the integration of the following specifications:

Table 8.5.1.1.1-3: Minimum requirements for NPUSCH format 1, 200KHz Channel Bandwidth, 15KHz subcarrier spacing, multiple subcarriers, 1Tx, for TDD

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions and correlation matrix (Annex D) | FRC (Annex A) | Frequency offset | Tx-Duration | Repetition number | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 15KHz | 12 | NTN TDLA100-1 | A7-2 | 8 Hz | 4 | 4 | 70% | 7 |
| 1 | 1 | 15KHz | 12 | NTN TDLC5-1 | A7-2 | 8 Hz | 4 | 4 | 70% | 4 |
| 1 | 1 | 15KHz | 1 | NTN TDLA100-1 | A14-3 for TDD operation | 8 Hz | 16 TBD | 16 TBD | 70% | TBD |
| 1 | 1 | 15KHz | 1 | NTN TDLC5-1 | A14-3 for TDD operation | 8 Hz | 8 | 1 | 70% | -2.5 |

Note: A7-2 considers 12 subcarriers.

Where:

- Option 1/ 12 subcarriers with 15 kHz & 8 Hz frequency offset (for less than 8ms duration) corresponds to existent Table 8.5.1.1.1-2 configuration:

Table 8.5.1.1.1-1: Minimum requirements for NPUSCH format 1, 200KHz Channel Bandwidth, 3.75KHz subcarrier spacing, 1Tx

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions and correlation matrix (Annex D)) | FRC (Annex A) | Frequency offset | TxDuration | Repetition number | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 3.75KHz | 1 | NTN TDLA100-1 | A7-1 | 128 Hz | 256 | 4 | 70% | -2.0 |
| 1 | 1 | 3.75KHz | 1 | NTN TDLC5-1 | A7-1 | 128 Hz | 256 | 4 | 70% | -3.6 |
| 1 | 2 | 3.75KHz | 1 | NTN TDLA100-1 | A7-1 | 128 Hz | 256 | 4 | 70% | -5.4 |
| 1 | 2 | 3.75KHz | 1 | NTN TDLC5-1 | A7-1 | 128 Hz | 256 | 4 | 70% | -6.5 |

Table 8.5.1.1.1-2: Minimum requirements for NPUSCH format 1, 200KHz Channel Bandwidth, 15KHz subcarrier spacing, multiple subcarriers, 1Tx

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions and correlation matrix (Annex D) | FRC (Annex A) | Frequency offset | Tx-Duration | Repetition number | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 15KHz | 12 | NTN TDLA100-1 | A7-2 | 8 Hz | 16 | 16 | 70% | -3.2 |
| 1 | 1 | 15KHz | 12 | NTN TDLC5-1 | A7-2 | 8 Hz | 16 | 16 | 70% | -4.0 |
| 1 | 2 | 15KHz | 12 | NTN TDLA100-1 | A7-2 | 8 Hz | 16 | 16 | 70% | -7.1 |
| 1 | 2 | 15KHz | 12 | NTN TDLC5-1 | A7-2 | 8 Hz | 16 | 16 | 70% | -7.8 |

With A7-2 configuration:

Table A.7.1-1: FRC parameters for NB-IoT NPUSCH format 1

| Reference channel | A7-1 | A7-2 |
| --- | --- | --- |
| Subcarrier spacing (kHz) | 3.75 | 15 |
| Number of allocated subcarriers | 1 | 12 |
| Diversity | No | No |
| Modulation | BPSK | QPSK |
| ITBS / IRU | 0 / 1 | 9 / 0 |
| Payload size (bits) | 32 | 136 |
| Allocated resource unit | 2 | 1 |
| Code rate (target) | 1/3 | 2/3 |
| Code rate (effective) | 0.29 | 0.56 |
| Transport block CRC (bits) | 24 | 24 |
| Code block CRC size (bits) | 0 | 0 |
| Number of code blocks - C | 1 | 1 |
| Total number of bits per resource unit | 96 | 288 |
| Total symbols per resource unit | 96 | 144 |
| Channel estimation length (ms) Note 1 | 16 | 2 (when repetition = 2) 4 (when repetition > 2) |

- Option 2/ 1 subcarrier – 8ms pi/2 BPSK with FRC A14-3 TDD:

Table A.14-1 FRC parameters for reference sensitivity

| Reference channel | A14-1 for FDD operation | A14-2 for FDD operation | A14-3 for TDD operation |
| --- | --- | --- | --- |
| Sub-carrier spacing (kHz) | 15 | 3.75 | 15 |
| Number of tone | 1 | 1 | 1 |
| Diversity | No | No | No |
| Modulation | π/2 BPSK | π/2 BPSK | π/2 BPSK |
| Frequency offset | 0 | 0 | 0 |
| Channel estimation length (ms) (NOTE) | 4 | 16 | 4 |
| Number of NPUSCH repetition | 1 | 1 | 1 |
| IMCS / TBS | 0 / 0 | 0 / 0 | 0 / 0 |
| Payload size (bits) | 32 | 32 | 16 |
| Allocated resource unit | 2 | 2 | 1 |
| Code rate (target) | 1/3 | 1/3 | 1/3 |
| Code rate (effective) | 0.29 | 0.29 | 0.42 |
| Transport block CRC (bits) | 24 | 24 | 24 |
| Code block CRC size (bits) | 0 | 0 | 0 |
| Number of code blocks - C | 1 | 1 | 1 |
| Total number of bits per resource unit | 96 | 96 | 96 |
| Total symbols per resource unit | 96 | 96 | 96 |
| Tx time (ms) | 16 | 64 | 8 |
| NOTE: Channel estimation lengths are included in the table for information only. |  |  |  |

And where (see TS 36.108):

The propagation conditions used for the performance measurements in multi-path fading environment are indicated as a combination of a channel model name and a maximum Doppler frequency, i.e., NTN-TDLA<DS>-<Doppler> or NTN-TDLC<DS>-<Doppler> where '<DS>' indicates the desired delay spread and '<Doppler>' indicates the maximum Doppler frequency (Hz).

Table D.2.2-1 show the propagation conditions that are used for the performance measurements in multi-path fading environment.

Table D.2.2-1: Channel model parameters

| Combination name | Tapped delay line model | Maximum Doppler frequency |
| --- | --- | --- |
| NTN-TDLA100-5 | NTN-TDLA100 | 5 Hz |
| NTN-TDLC5-5 | NTN-TDLC5 | 5 Hz |

## NPDSCH simulation results and proposals (UE side)

The results from this section are to be integrated as part of TS 36.102. The BLER NPDSCH has been simulated with:

- TBS 616 bits

- NRep 4

- NSF 4

- QPSK

- NTN-TDLC5-200

- Frequency offset: 200 Hz

[RAN4#116bis] Proposal 10: RAN4 to assume 4 NPDSCH repetitions, with an SNR target around -1.3dB, Rmax=4.

[RAN4#116bis] Proposal 11: RAN4 to consider the following simulation results for TDD NB-IoT NPDSCH with NTN-TDLC channel model

[RAN4#116bis] Proposal 12: RAN4 to consider the integration of the following specifications (with 1 single test number):

Table 8.3.2.1.1.1-2: Minimum performance for NPDSCH under Standalone with 1 NRS port

| Test number | Bandwidth | Carrier Type | Reference Channel | Repetition number | Propagation condition | Number of NRS ports | Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 200kHz | Anchor | currently R.NB.6 FDD, could be reused for TDD | 4 | NTN-TDLC5-200 | 1 | 1x1 | 70% | -1.3 | NB1, NB2 |
| 2 | 200kHz | Non-anchor | TBD (currently R.NB.6-1 TDD) | TBD | NTN-TDLC5-200 | 1 | 1x1 | 70% | TBD | NB1, NB2 |

And where (see TS 36.102):

The propagation conditions used for the performance measurements in multi-path fading environment are indicated as a combination of a channel model name and a maximum Doppler frequency, i.e., NTN-TDLA<DS>-<Doppler>, or NTN-TDLC<DS>-<Doppler> where '<DS>' indicates the desired delay spread and '<Doppler>' indicates the maximum Doppler frequency (Hz).

Table D.1.2-1 show the propagation conditions that are used for the performance measurements in multi-path fading environment for NLOS and LOS propagation conditions.

Table D.1.2-1: Channel model parameters for NTN

| Combination name | Model | Maximum Doppler frequency |
| --- | --- | --- |
| NTN-TDLA100-10 | NTN-TDLA100 | 10 Hz |
| NTN-TDLA100-200 | NTN-TDLA100 | 200 Hz |
| NTN-TDLC5-30 | NTN-TDLC5 | 30 Hz |
| NTN-TDLC5-200 | NTN-TDLC5 | 200 Hz |