3GPP TSG-RAN4 Meeting #117	R4-2520636

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-5 | CR | - | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft Big CR on Demodulation Requirements for NTN Phase 3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ph3-Perf |  |  |  |  |  | Date: |  |  | 2025-11-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduce UE demodulation requirements for NTN Phase 3 |  |  |  |  |  |  |  |  |
| Summary of change: |  | R4-2523081: Add test applicability rules for PDSCH demodulation requirements for RedCap and eRedCap over NR NTN in the specification R4-2523082: Introduce a new clause to add new PDSCH requirements for RedCap for NTN support R4-2520023: Add new PDSCH requirements for NTN eRedCap R4-2521353: Adding FRC for RedCap UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE demodulation requirements for NTN RedCap and eRedCap will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.1 8.2.1.2.1 8.2.1.2.2.1 A.3.2.1.1 A.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-5 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

#### 8.2.1.1.3	Applicability of requirements for RedCap

The performance requirements in Table 8.2.1.1.3-1 shall apply for UEs which support optional feature nonTerrestrialNetwork-r17 and supportOfRedCap.

Table 8.2.1.1.3-1: Requirements applicability for RedCap UEs

| UE capability | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| RedCap with 1RX | FR1 FDD and HD-FDD (Note 1) | PDSCH | All tests in Clause 8.2.1.2.1.1.1 |  |
| RedCap with 2RX | FR1 FDD and HD-FDD (Note 1) | PDSCH | All tests in Clause 8.2.1.2.2.1.2 |  |
| Note 1: If UE supports only HD-FDD in a FDD band, this UE is tested with HD-FDD mode otherwise UE is tested with full-duplex FDD mode |  |  |  |  |

#### 8.2.1.1.4	Applicability of requirements for eRedCap

The performance requirements in Table 8.2.1.1.4-1 shall apply for UEs which support optional feature nonTerrestrialNetwork-r17 and supportOfERedCap-r18.

Table 8.2.1.1.4-1:  Requirements applicability for eRedCap UEs

| UE capability | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| eRedCap with 1RX | FR1 FDD and HD-FDD (Note 1) | PDSCH | All tests in Clause 8.2.1.2.1.1.2 |  |
| eRedCap with 2RX | FR1 FDD and HD-FDD (Note 1) | PDSCH | All tests in Clause 8.2.1.2.2.1.3 |  |
| Note 1: If UE supports only HD-FDD in a FDD band, this UE is tested with HD-FDD mode otherwise UE is tested with full-duplex FDD mode |  |  |  |  |

==============Second change==============

##### 8.2.1.2.1	1RX requirements

#### 8.2.1.2.1.1	FDD

##### 8.2.1.2.1.1.1	Minimum requirements for RedCap

The performance requirements are specified in Table 8.2.1.2.1.1.1-3, with the addition of test parameters in Table 8.2.1.2.1.1.1-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 8.2.1.2.1.1.1-1.

Table 8.2.1.2.1.1.1-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH mapping Type A normal performance under 1 receive antenna conditions and with different channel models and MCSs for RedCap | 1-1, 1-2, 1-3, 1-4 |

Table 8.2.1.2.1.1.1-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI-RS for tracking | CSI-RS periodicity | Slots | 20 for CSI-RS resource 1,2,3,4. |
|  | CSI-RS offset | Slots | 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4. |
| Number of HARQ Processes |  |  | 16 for Test 1-1, Test 1-3 32 for Test 1-2 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 4 |

Table 8.2.1.2.1.1.1-3: Minimum performance for Rank 1

| Test num. | Reference channel (Note 1) | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.TBD. FDD R.PDSCH. TBD. HD-FDD | 10 / 15 | QPSK, 0.30 | NTN-TDLA100-200 | 1x1 Low | 70 | [4.0] |
| 1-2 | R R.PDSCH.TBD. FDD R.PDSCH. TBD. HD-FDD | 10 / 15 | QPSK, 0.30 | NTN-TDLC5-200 | 1x1 Low | 70 | [2.5] |
| 1-3 | R.PDSCH.1-2.1 FDD R.PDSCH. TBD HD-FDD | 10 / 15 | 16QAM, 0.48 | NTN-TDLC5-200 | 1x1 Low | 70 | [10.5] |

##### 8.2.1.2.1.1.2	Minimum requirements for eRedCap

The performance requirements are specified in Table 8.2.1.2.1.1.2-3, with the addition of test parameters in Table 8.2.1.2.1.1.2-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 8.2.1.2.1.1.2-1.

Table 8.2.1.2.1.1.2-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH mapping Type A normal performance under 1 receive antenna conditions and with different channel models and MCSs for eRedCap UE with reduced baseband bandwidth | 1-1, 1-2 |
| Verify the PDSCH mapping Type A normal performance under 1 receive antenna conditions and with different channel models and MCSs for eRedCap without reduced baseband bandwidth | 2-1, 2-2 |

Table 8.2.1.2.1.1.2-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD / HD-FDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI-RS for tracking | CSI-RS periodicity | Slots | 20 for CSI-RS resource 1,2,3,4. |
|  | CSI-RS offset | Slots | 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4. |
| Number of HARQ Processes |  |  | 16 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | FDD: 10, HD-FDD: 12 if mod(i,5) = 0 11 if mod(i,5) = 1 10 if mod(i,5) = 2 14 if mod(i,5) = 3 |
| cellSpecificKoffset-r17 (Note 1) |  | Slots/15kHz | 8 |
| Maximum number of HARQ transmission |  |  | 4 |
| Note 1: The number of slots between PDSCH and corresponding HARQ-ACK information contains the number of slots specified by cellSpecificKoffset-r17. Note 2: i is the slot index per frame; i = {0,…,9}. |  |  |  |

Table 8.2.1.2.1.1.2-3: Minimum performance for Rank 1 with reduced baseband bandwidth

| Test num. | Reference channel (Note 1) | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | [TBD for FDD] [TBD for HD-FDD] | 10 / 15 | QPSK, 0.30 | TDLA100-200 | 1x1 | 70 | TBD |
| 1-2 | R.PDSCH.1-25.1 FDD [TBD for HD-FDD] | 10 / 15 | 16QAM, 0.48 | TDLC5-200 | 1x1 | 70 | TBD |
| Note 1: Applied reference channel depends on the supported operation mode: FDD or HD-FDD. |  |  |  |  |  |  |  |

Table 8.2.1.2.1.1.2-4: Minimum performance for Rank 1 without reduced baseband bandwidth

| Test num. | Reference channel (Note 1) | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | [TBD for FDD] [TBD for HD-FDD] | 10 / 15 | QPSK, 0.30 | TDLA100-200 | 1x1 | 70 | TBD |
| 2-2 | R.PDSCH.1-12.3 FDD [TBD for HD-FDD] | 10 / 15 | 16QAM, 0.48 | TDLC5-200 | 1x1 | 70 | TBD |
| Note 1: Applied reference channel depends on the supported operation mode: FDD or HD-FDD. |  |  |  |  |  |  |  |

==============Third change==============

##### 8.2.1.2.2.1.2	Minimum requirements for eRedCap

The performance requirements are specified in Table 8.2.1.2.2.1.2-3 with the addition of test parameters in Table 8.2.1.2.2.1.2-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 8.2.1.2.2.1.2-1.

Table 8.2.1.2.2.1.2-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH mapping Type A normal performance under 2 receive antennas conditions and with different channel models and MCSs for eRedCap UE with reduced baseband bandwidth | 1-1, 1-2 |
| Verify the PDSCH mapping Type A normal performance under 2 receive antennas conditions and with different channel models and MCSs for eRedCap without reduced baseband bandwidth | 2-1, 2-2 |

Table 8.2.1.2.2.1.2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD / HD-FDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI-RS for tracking | CSI-RS periodicity | Slots | 20 for CSI-RS resource 1,2,3,4. |
|  | CSI-RS offset | Slots | 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4. |
| Number of HARQ Processes |  |  | 16 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | FDD: 10, HD-FDD: 12 if mod(i,5) = 0 11 if mod(i,5) = 1 10 if mod(i,5) = 2 14 if mod(i,5) = 3 |
| cellSpecificKoffset-r17 (Note 1) |  | Slots/15kHz | 8 |
| Maximum number of HARQ transmission |  |  | 4 |
| Note 1: The number of slots between PDSCH and corresponding HARQ-ACK information contains the number of slots specified by cellSpecificKoffset-r17. Note 2: i is the slot index per frame; i = {0,…,9}. |  |  |  |

Table 8.2.1.2.2.1.2-3: Minimum performance for Rank 1 with reduced baseband bandwidth

| Test num. | Reference channel (Note 1) | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | [TBD for FDD] [TBD for HD-FDD] | 10 / 15 | QPSK, 0.30 | TDLA100-200 | 1x2 | 70 | TBD |
| 1-2 | R.PDSCH.1-25.1 FDD [TBD for HD-FDD] | 10 / 15 | 16QAM, 0.48 | TDLC5-200 | 1x2 | 70 | TBD |
| Note 1: Applied reference channel depends on the supported operation mode: FDD or HD-FDD. |  |  |  |  |  |  |  |

Table 8.2.1.2.2.1.2-4: Minimum performance for Rank 1 without reduced baseband bandwidth

| Test num. | Reference channel (Note 1) | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | [TBD for FDD] [TBD for HD-FDD] | 10 / 15 | QPSK, 0.30 | TDLA100-200 | 1x2 | 70 | TBD |
| 2-2 | R.PDSCH.1-12.3 FDD [TBD for HD-FDD] | 10 / 15 | 16QAM, 0.48 | TDLC5-200 | 1x2 | 70 | TBD |
| Note 1: Applied reference channel depends on the supported operation mode: FDD or HD-FDD. |  |  |  |  |  |  |  |

==============Fourth change==============

### A.3.2.1	FDD

#### A.3.2.1.1	Reference measurement channels for SCS 15 kHz FR1

Table A.3.2.1.1-1: PDSCH Reference Channel for FDD (QPSK)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-1.1 FDD | R.PDSCH.1-1.2 FDD |  |  |  |
| Channel bandwidth | MHz | 10 | 10 |  |  |  |
| Subcarrier spacing | kHz | 15 | 15 |  |  |  |
| Number of allocated resource blocks | PRBs | 52 | 25 |  |  |  |
| Number of consecutive PDSCH symbols |  | 12 | 12 |  |  |  |
| Allocated slots per 2 frames | Slots | 19 | 19 |  |  |  |
| MCS table |  | 64QAM | 64QAM |  |  |  |
| MCS index |  | 4 | 4 |  |  |  |
| Modulation |  | QPSK | QPSK |  |  |  |
| Target Coding Rate |  | 0.30 | 0.30 |  |  |  |
| Number of MIMO layers |  | 1 | 1 |  |  |  |
| Number of DMRS REs |  | 12 | 12 |  |  |  |
| Overhead for TBS determination |  | 0 | 0 |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A |  |  |  |
| For Slots i = 1,…, 19 | Bits | 4096 | 2024 |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A |  |  |  |
| For Slots i = 1,…, 19 | Bits | 24 | 16 |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i = 0 | CBs | N/A | N/A |  |  |  |
| For Slots i = 1,…, 19 | CBs | 1 | 1 |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A |  |  |  |
| For Slots i = 10, 11 | Bits | 13104 | 6300 |  |  |  |
| For Slots i =1,…, 9, 12, …, 19 | Bits | 13728 | 6600 |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 3.891 | 1.923 |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

Table A.3.2.1.1-2: PDSCH Reference Channel for FDD (16QAM)

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-2.1 FDD | R.PDSCH.1-2.2 FDD | R.PDSCH.1-2.3 FDD |  |  |  |
| Channel bandwidth | MHz | 10 | 10 | 10 |  |  |  |
| Subcarrier spacing | kHz | 15 | 15 | 15 |  |  |  |
| Number of allocated resource blocks | PRBs | 52 | 40 | 25 |  |  |  |
| Number of consecutive PDSCH symbols |  | 12 | 12 | 12 |  |  |  |
| Allocated slots per 2 frames | Slots | 19 | 19 | 19 |  |  |  |
| MCS table |  | 64QAM | 64QAM | 64QAM |  |  |  |
| MCS index |  | 13 | 13 | 13 |  |  |  |
| Modulation |  | 16QAM | 16QAM | 16QAM |  |  |  |
| Target Coding Rate |  | 0.48 | 0.48 | 0.48 |  |  |  |
| Number of MIMO layers |  | 1 | 1 | 1 |  |  |  |
| Number of DMRS REs |  | 12 | 12 | 12 |  |  |  |
| Overhead for TBS determination |  | 0 | 0 | 0 |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A |  |  |  |
| For Slots i = 1,…, 19 | Bits | 13064 | 9992 | 6272 |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A |  |  |  |
| For Slots i = 1,…, 19 | Bits | 24 | 24 | 24 |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |
| For Slot i = 0 | CBs | N/A | N/A | N/A |  |  |  |
| For Slots i = 1,…, 19 | CBs | 2 | 2 | 1 |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A |  |  |  |
| For Slots i = 10, 11 | Bits | 26208 | 20160 | 12600 |  |  |  |
| For Slots i = 1,…, 9, 12, …, 19 | Bits | 27456 | 21120 | 13200 |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 12.411 | 9.492 | 5.958 |  |  |  |
| NOTE 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms NOTE 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |  |

==============Fifth change==============

### A.3.2.2	HD-FDD

#### A.3.2.2.1	Reference measurement channels for SCS 15 kHz FR1

Table A.3.2.2.1-1: PDSCH Reference Channel for HD-FDD (QPSK)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-1.1 HD-FDD | R.PDSCH.1-1.2 HD-FDD |  |  |  |
| Channel bandwidth | MHz | 10 | 10 |  |  |  |
| Subcarrier spacing | kHz | 15 | 15 |  |  |  |
| Number of allocated resource blocks | PRBs | 52 | 25 |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} |  | 8 | 8 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} |  | 12 | 12 |  |  |  |
| Allocated slots per 2 frames | Slots | 15 | 15 |  |  |  |
| MCS table |  | 64QAM | 64QAM |  |  |  |
| MCS index |  | 4 | 4 |  |  |  |
| Modulation |  | QPSK | QPSK |  |  |  |
| Target Coding Rate |  | 0.30 | 0.30 |  |  |  |
| Number of MIMO layers |  | 1 | 1 |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} |  | 12 | 12 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} |  | 12 | 12 |  |  |  |
| Overhead for TBS determination |  | 0 | 0 |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} | Bits | 2664 | 1256 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} | Bits | 4096 | 2024 |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} | Bits | 16 | 16 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} | Bits | 24 | 16 |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i = 0 | CBs | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} | CBs | 1 | 1 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} | CBs | 1 | 1 |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A |  |  |  |
| For Slots i = 10, 11 | Bits | 13104 | 6300 |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} | Bits | 8736 | 4200 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,9,12,…,19} | Bits | 13728 | 6600 |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 2.786 | 1.364 |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

Table A.3.2.1.1-2: PDSCH Reference Channel for HD-FDD (16QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-2.1 HD-FDD | R.PDSCH.1-2.2 HD-FDD | R.PDSCH.1-2.3 HD-FDD |  |  |
| Channel bandwidth | MHz | 10 | 10 | 10 |  |  |
| Subcarrier spacing | kHz | 15 | 15 | 15 |  |  |
| Number of allocated resource blocks | PRBs | 52 | 40 | 25 |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} |  | 8 | 8 | 8 |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} |  | 12 | 12 | 12 |  |  |
| Allocated slots per 2 frames | Slots | 15 | 15 | 15 |  |  |
| MCS table |  | 64QAM | 64QAM | 64QAM |  |  |
| MCS index |  | 13 | 13 | 13 |  |  |
| Modulation |  | 16QAM | 16QAM | 16QAM |  |  |
| Target Coding Rate |  | 0.48 | 0.48 | 0.48 |  |  |
| Number of MIMO layers |  | 1 | 1 | 1 |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} |  | 12 | 12 | 12 |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} |  | 12 | 12 | 12 |  |  |
| Overhead for TBS determination |  | 0 | 0 | 0 |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} | Bits | 8456 | 6400 | 3968 |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} | Bits | 13064 | 9992 | 6272 |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} | Bits | 24 | 24 | 24 |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} | Bits | 24 | 24 | 24 |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i = 0 | CBs | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} | CBs | 2 | 1 | 1 |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} | CBs | 2 | 2 | 1 |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A |  |  |
| For Slots i = 10, 11 | Bits | 26208 | 20160 | 12600 |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} | Bits | 17472 | 13440 | 8400 |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,9,12,…,19} | Bits | 27456 | 21120 | 13200 |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 8.876 | 6.776 | 4.243 |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

==============End of change==============