# 3GPP TSG-RAN WG4 Meeting #117	R4-2522643

# Dallas, US, 17th – 21st Nov, 2025

Agenda item:			6.15.1

Source:	Moderator (Ericsson)

Title:	Ad-hoc minutes for Netw_Energy_NR_enh_RRM

Document for:	Approval

# Introduction

This document is the Ad-hoc minutes for Netw_Energy_NR_enh with the following topics covered.

Note 1: The following terminologies are used.

- Case #1: No always-on SSB on the cell

- Case #2: Always-on SSB is periodically transmitted on the cell

  - Alt Time-C1: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern is supported (the interval between SSB bursts is even and supported in legacy specification) when the center frequency locations of always-on SSB and on-demand SSB are same.

  - Alt Time-C2: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern is supported when the center frequency locations of always-on SSB and on-demand SSB are same.

  - Alt Scenario 3: When the center frequency locations of always-on SSB and on-demand SSB are different and AO-SSB and OD-SSB are located in the same BWP.

- Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321)

- Scenario #2A refers to “When UE receives SCell activation command (e.g., as defined in TS 38.321)”

- Scenario #3A refers to “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed”

- Scenario #3B refers to “When SCell activation is completed and SCell is activated” or “After SCell activation is completed and SCell is activated”

Note 2: The following abbreviations are used.

- AO-SSB: Always-on SSB

- OD-SSB: On-demand SSB

- FMW: Fast Measurement Window

- EMP: Effective Measurement Periodicity

# Topic #1: Maintenance (AI 4.21.1)

Issue 1-1: OD-SSB interruption

- Background

##### 8.2.3.2.x Interruptions at OD-SSB activation/deactivation The requirements in this clause shall apply for the UE configured with E-UTRA PSCell and one SCell. When OD-SSB on one deactivated SCell in MCG is activated or deactivated the UE is allowed an interruption on any serving cell in MCG, provided that the OD-SSB activation/deactivation command and SCell activation/deactivation command are not received at the same time: - of up to X2 slots, if the active serving cell is not in the same band as any of the SCells on which OD-SSB is being activated or deactivated, or - of up to Y2 slots + TSMTC_duration if the active serving cells are in the same band as any of the SCells on which OD-SSB is being activated or deactivated, provided the cell specific reference signals from the active serving cells and the SCells on which OD-SSB is being activated or deactivated are available in the same slot. 8.3.x1 OD-SSB based SCell Activation Delay Requirement for Deactivated SCell .. The starting point of an interruption window on SpCell or any activated SCell, as specified in clause 8.2, shall not occur before slot n+1+THARQNR slot length and not occur after slot slot n+1+THARQ+3ms+TXNR slot length, where NR slot length is with respect to the numerology used in the SCell being activated, and TX is: - 0, if Tactivation_time is 3 ms; - TFirst, OD-SSB, for any scenario where Tactivation_time includes TFirst, OD-SSB; - Tuncertainty_MAC + TFineTiming, OD-SSB, for any scenario where Tactivation_time includes only TFineTiming, OD-SSB and no TFirst, OD-SSB.

- Proposals

  - Option 1: Huawei

    - The interruption length is the longest SMTC duration/OD-SSB duration among all active serving cells in the same band.

  - Option 2: Ericsson, LG

    - RAN4 to align the interruption length of OD-SSB based deactivated SCell measurement with SCell activation as the RF retuning time without the SMTC duration.

- Recommended WF

  - Further discussion.

[Discussion]

HW: Option 1. AGC accuracy may be impacted if only following OD-SSB. Compromise to OD-SSB duration.

QC: We should follow the legacy condition

OPPO: OD-SSB activation we don’t need to consider the condition; for deactivated SCell measurement, we need to consider.

Agreement

If the active serving cells are in the same band as any of the SCells on which OD-SSB is being activated or deactivated, the interruption length is RF retuning time plus the time of transmitted one OD-SSB burst.

FFS the following conditions

- The cell specific reference signals from the active serving cells and the SCells on which OD-SSB is being activated or deactivated are available in the same slot in deactivated OD-SSB SCell measurements

- Otherwise, the condition is not needed.

Issue 1-2: Maintenance CR

| No | T-doc number | Company | Note | Recommendation |
| --- | --- | --- | --- | --- |
| 1 | R4-2520813 | Nokia, Nokia Shanghai Bell | Add transition requirement for BFD | Revised |
| 2 | R4-2520925 | Huawei, HiSilicon | Signalling update | Agree |
| 3 | R4-2521244 | Nokia, Nokia Shanghai Bell | Re-submit CR from RAN4 #116bis | Withdrawn |
| 4 | R4-2521780 | Ericsson | Change SFN offset Interruption change | Revised |
| 5 | R4-2521819 | Ericsson, Huawei | Endorsed Big CR in RAN4 #116bis | Agree |
| 6 | R4-2521961 | ZTE Corporation, Sanechips | SSB-less cell as timing reference | Postpone |

# Topic #2: On-demand SSB(OD-SSB) performance part (AI 6.15.2.1)

Issue 2-1: Performance CR part 1

RAN4 to discuss the following OD-SSB test cases.

| T-doc number | Company | Note | Recommendation |
| --- | --- | --- | --- |
| R4-2520110 | CATT | TC10 | Endorse |
| R4-2520479 | Xiaomi | TC5 | Endorse |
| R4-2520814 | Nokia, Nokia Shanghai Bell | TC1 | Endorse |
| R4-2520908 | LG Electronics Inc. | TC9 | Endorse |
| R4-2520927 | Huawei, HiSilicon | TC2 | Endorse |
| R4-2520993 | CMCC | TC6 | Endorse |
| R4-2521100 | vivo | TC8 | Endorse |
| R4-2521328 | OPPO | TC7 | Endorse |
| R4-2521683 | Samsung | TC4 | Endorse |
| R4-2521782 | Ericsson | TC3 | Endorse |
| R4-2521957 | ZTE Corporation, Sanechips | TC11 | Endorse |

# Topic #3: Adaptation of common signal/channel transmission (AI 6.15.2.2)

Issue 3-1: Performance CR

Background

| T-doc number | Company | Note | Recommendation |
| --- | --- | --- | --- |
| R4-2520929 | Huawei, HiSilicon, China Telecom | L1-SINR | Endorse |
| R4-2521684 | Samsung | L1-RSRP | Endorse |
| R4-2521956 | ZTE Corporation, Sanechips | L3-RSRP | Endorse |

# Topic #4: OD-SIB1 (AI 6.15.2.3)

Issue 4-1: Performance CR

| T-doc number | Company | Note | Recommendation |
| --- | --- | --- | --- |
| R4-2521246 | Nokia, Nokia Shanghai Bell | Cell reselection FR2 to FR2 target Cell | Revised |
| R4-2521102 | vivo | Cell reselection FR1 to FR1 target Cell | Revised |