3GPP TSG-RAN WG4 Meeting # 117	R4-2522746

# Dallas, USA, 07th – 11th April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | <CR#> | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for Rel-19 ATG TC on SCell Activation and deactivation of unknown SCell in FR1 in non-DRX |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG_enh_RRM-Perf |  |  |  |  |  | Date: |  |  | 2025-11-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To introduce new Test Cases for SCell Activation and deactivation of unknown SCell in FR1 in non-DRX (together with interruption requirement verification). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the following test cases: - SCell Activation and deactivation of unknown SCell in FR1 in non-DRX - Interruption requirement verification |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Without this test case, UE behavior for unknown SCell activation and deactivation in ATG FR1 non-DRX operation cannot be verified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.19.4.7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This Test is based on A.19.4.7.1 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R4-2521823 |  |  |  |  |  |  |  |  |

### <<Start of Change 1>>

#### A.19.4.7.3	SCell Activation and deactivation of unknown SCell in FR1 in non-DRX

##### A.19.4.7.3.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in clause 8.3D, when the SCell in FR1 is unknown by the UE at the time of activation. and both the PCell and SCell are co-located in an ATG NR SA configuration. The test also verifies that any PCell interruption occurring due to SCell activation or deactivation remains within the limits defined in clause 8.2D.

The supported test configurations are shown in table A.6.5.3.1.1-1 and table A.6.5.3.1.1-1A. The test parameters are given in table A.6.5.3.1.1-2 and cell-specific parameters in table A.6.5.3.1.1-3, except the parameters that are defined in A.19.4.7.3.1-1. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are two NR carriers, each with one cell. Both cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1, but is not aware of Cell 2. The UE is only monitoring the PCC. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell (Cell 2) becomes configured on radio channel 2. The UE now starts monitoring the SCC. The test equipment sends a MAC message for activation of the SCell.

The point in time at which the MAC message is received at the UE antenna connector, in slot # denoted n, defines the start of time period T2. The UE shall be able to report valid CSI in PCell for the activated SCell at latest in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length, as defined in clause 8.3D. The UE shall start reporting CSI in PCell after at least one CSI-RS transmission occasion for channel measurement plus nCSI_ref  slots, as defined in 5.2.2.5 in [26], and reporting after slot n+THARQ+3 msNR slot length and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCell shall occur in the slot n+1+THARQNR slot length to n+1+THARQ+3 ms+TXNR slot length+Ninterruption, as defined in clause 8.3D, where Ninterruption is the interruption length given in clause 8.2D.

Time period T3 starts when a MAC message for deactivation of SCell, sent from the test equipment to the UE in a slot # denoted m, is received at the UE antenna connector. The UE shall carry out deactivation of the SCell in a slot m+THARQ+3msNR slot length, as defined in clause 8.3D, and The starting point of any PCell interruption due to the deactivation shall occur in the slot m+1+THARQNR slot length to m+1+THARQ+3 msNR slot length, as defined in clause 8.3D.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCell, respectively.

The test equipment verifies the activation time by counting the slots from the time when the SCell activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the slots from the time when the SCell deactivation command is sent until CQI reporting for SCell is discontinued.

Table A.19.4.7.3.1-1: General test parameters for unknown FR1 SCell activation case, 160 ms SCell measurement cycle

| Parameter |  | Unit | Value |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| T1 |  | ms | 100 |  |  | During this time the PCell shall be known and the SCell configured, but not detected. |
| Propagation Condition | Config 1,2,1A,2A |  |  |  | AWGN + 220Hz |  |
|  | Config 3,3A |  |  |  | AWGN +500Hz |  |

##### A.19.4.7.3.2	Test Requirements

The test requirements defined in clause A.6.5.3.3.2 shall apply to this test case, except Tactivation_time will be replaced with the value TFirstSSB_MAX + TSMTC_MAX + 2*Trs + 5 ms as defined in clause 8.3D.

### <<End of Change 1>>