3GPP TSG-RAN WG4 Meeting #117		   				 		  								 	   R4-2523066

Dallas, USA, 17th – 21th Nov., 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | draft | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of test case of RRC Re-establishment and Random Access for NB-IoT in TDD mode with Satellite Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_Ph3-Perf |  |  |  |  |  | Date: |  |  | 2025-11-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of test case of RRC Re-establishment and Random Access for NB-IoT in TDD mode with Satellite Access |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add test cases of RRC Re-establishment and Random Access for NB-IoT in TDD mode with Satellite Access |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No test case of RRC Re-establishment and Random Access for NB-IoT in TDD mode with Satellite Access |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new) A.15.3.1.1, A.15.3.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | NA |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES 1>>>

#### A.13.3.1.1	HD-FDD and TDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under normal coverage

##### A.13.3.1.1.1	Test Purpose and Environment

The purpose is to verify that the NB-IoT HD-FDD and TDD intra-frequency RRC re-establishment delay is within the specified limits. These tests will verify the requirements for Cat-NB1 UE in clause 6.5A.

The test parameters are given in table A.13.3.1.1.1-1 and table A.13.3.1.1.1-2 below. nCell1 and nCell2 are NB-IoT cells with different physical cell ID on the same frequency carrier. The test consists of 3 successive time periods, with time duration of T1, T2 and T3 respectively. At the start of time period T2, cell 1, which is the active cell, is deactivated. The time period T3 starts after the occurrence of the radio link failure.

The UE shall be provided with the valid information about the SAN serving cells before the test.

Table A.13.3.1.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GEO/GSO, HD-FDD duplex mode |
| 2 | NGSO, HD-FDD duplex mode |
| 3 | NGSO, TDD duplex mode |
| Note 1: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. GEO configuration only applies for Rel-17 UEs. GSO configuration is applicable for Rel-18 and onward UEs, when SIB33 is provided to the UE. Note 2: Config 2 and Config 3 are applicable when SIB33 is provided to the UE. |  |

Table A.13.3.1.1.1-2: General test parameters for HD-FDD and TDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under normal coverage

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NB-IOT operational mode |  |  | Standalone |  |
| Initial condition | Active cell |  | nCell1 |  |
|  | Neighbour cells |  | nCell2 |  |
| Final condition | Active cell |  | nCell2 |  |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| NPDCCH repetition level |  |  | 16 | NPDCCH Rmax |
| N310 |  | - | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  | - | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  | Ms | 0 | Radio link failure timer; T310 is disabled |
| T311 |  | Ms | 15000 | RRC re-establishment timer |
| DRX |  |  | OFF |  |
| T1 |  | S | 5 |  |
| T2 |  | Ms | 400 |  |
| T3 |  | S | 15 |  |

Table A.13.3.1.1.1-3: nCell 1, nCell 2 specific test parameters for HD-FDD and TDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under normal coverage

| Parameter |  | Unit | nCell 1 |  |  | nCell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| BWchannel |  | kHz | 200 |  |  | 200 |  |  |
| NPDSCH parameters | Config. 1, 2 |  | R.18 HD-FDD |  |  | R.18 HD-FDD |  |  |
|  | Config. 3 |  | R.X TDD |  |  | R.X TDD |  |  |
| NPDCCH parameters |  |  | R.30 HD-FDD |  |  | R.30 HD-FDD |  |  |
| NOCNG Patterns | Config. 1, 2 |  | NOP.3 FDD |  |  | NOP.3 FDD |  |  |
|  | Config. 3 |  | NOP.X TDD |  |  | NOP.X TDD |  |  |
| NPBCH_RA |  | dB | 0 |  |  | 0 |  |  |
| NPBCH_RB |  | dB |  |  |  |  |  |  |
| NPSS_RA |  | dB |  |  |  |  |  |  |
| NSSS_RA |  | dB |  |  |  |  |  |  |
| NPDCCH_RA |  | dB |  |  |  |  |  |  |
| NPDCCH_RB |  | dB |  |  |  |  |  |  |
| NPDSCH_RA |  | dB |  |  |  |  |  |  |
| NPDSCH_RB |  | dB |  |  |  |  |  |  |
| NOCNG_RANote 1 |  | dB |  |  |  |  |  |  |
| NOCNG_RBNote 1 |  | dB |  |  |  |  |  |  |
|  |  | dBm/15 kHz | -98 |  |  |  |  |  |
|  |  | dB | 7 | -Infinity | -Infinity | -Infinity | 4 | 4 |
| Note2 |  | dB | 7 | -Infinity | -Infinity | -Infinity | 4 | 4 |
| NRSRP Note2 |  | dBm/15 kHz | -91 | -Infinity | -Infinity | -Infinity | -94 | -94 |
| Propagation Condition |  |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  |  | 1x1 |  |  | 1x1 |  |  |
| Timing offset to nCell 1 |  | ms | - |  |  | 3 |  |  |
| Note 1: NOCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Es/Iot and NRSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

##### A.13.3.1.1.2	Test Requirements

The RRC re-establishment delay is defined as the time from the start of time period T3, to the moment when the UE starts to send NPRACH preambles to cell 2 for sending the RRCConnectionReestablishmentRequest message to cell 2.

The RRC re-establishment delay to an unknown NB-IoT HD-FDD and TDD intra frequency cell shall be less than 10.6  s.

The rate of correct RRC re-establishments observed during repeated tests shall be at least 90%.

NOTE:	The RRC re-establishment delay in the test is derived from the following expression:

Tre-establish_delay= TUL_grant + TUE-re-establish_delay_NB-IoT.

Where:

-	TUL_grant = It is the time required to acquire and process uplink grant from the target cell. The NPRACH reception at the system simulator is used as a trigger for the completion of the test; hence TUL_grant is not used.

-	TUE-re-establish_delay_NB-IoT = 100 ms + NNB-Iot-freq*Tsearch_NB-IoT + TSI_NB-IoT + TPRACH_NB-IoT

-	NNB-Iot-freq = 1

-	Tsearch_NB-IoT = 1400 ms

-	TSI_NB-IoT = 8320 ms; it is the time required for receiving all the relevant system information as defined in TS 36.331 for the target NB-IoT cell.

-	TPRACH_NB-IoT = 80 ms; it is the additional delay caused by the random access procedure.

## <<< END OF CHANGES 1>>>

## <<< START OF CHANGES 2>>>

#### A.13.3.2.1	Contention Based Random Access Test for UE category NB1 UEs in Satellite Access - Standalone mode in normal coverage

##### A.13.3.2.1.1	Test Purpose and Environment

The purpose of this test is to verify whether the behavior of the random access procedure of a category NB1 UE in Normal Coverage is according to the requirements when connected to a NTN NB-IoT cell, whether the NPRACH power settings and timing are within specified limits, and whether the UE determines properly the enhanced coverage level based on the NRSRP measurement and the configured criterion in NRSRP-ThresholdsPrach [2]. This test will verify the requirements in Clause 6.6A.2, Clause 6.6A.3 and Clause 7.20A.2 in an AWGN model.

For this test a single NB-IoT cell is used. The test parameters are given in tables A.13.3.2.1.1-1, A.13.3.2.1.1-2 and A.13.3.2.1.1-3. The UE shall perform timing pre-compensation before the initial NPRACH transmission using AT command-based test approach.

Table A.13.3.2.1.1-1: nCell specific test parameters for HD-FDD and TDD contention based random access test for UE category NB1 Standalone mode in Normal Coverage

| Parameter |  | Unit | Value | Comments |
| --- | --- | --- | --- | --- |
| NB-IOT operational mode |  |  | Standalone |  |
| BWchannel |  | kHz | 200 |  |
| NPDSCH parameters Note 2 | Config. 1, 2 |  | R.18 HD-FDD | As defined in A.3.1.5.3 |
|  | Config. 3 |  | R.X TDD |  |
| NPDCCH parameters Note 2 |  |  | R.30 HD-FDD | As defined in A.3.1.6.3 |
| NPBCH_RA |  | dB | 0 |  |
| NPBCH_RB |  | dB | 0 |  |
| NPSS_RA |  | dB |  |  |
| NSSS_RA |  | dB |  |  |
| NPDCCH_RA |  | dB |  |  |
| NPDCCH_RB |  | dB |  |  |
| NPDSCH_RA |  | dB |  |  |
| NPDSCH_RB |  | dB |  |  |
| NOCNG_RA Note 1 |  | dB |  |  |
| NOCNG_RB Note 1 |  | dB |  |  |
| DRX |  |  | OFF |  |
|  |  | dBm/15 kHz | -98 |  |
|  |  | dB | 3 |  |
| Note 3 |  | dB | 3 |  |
| NRSRP Note 3 |  | dBm/15 kHz | -95 |  |
| Io Note 3 |  | dBm/180 KHz | -82.45 |  |
| Propagation Condition |  | - | AWGN |  |
| Antenna Configuration |  |  | 1x1 |  |
| Note 1: NOCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: The NPDSCH and NPDCCH reference measurement channels are used in the test only when a downlink transmission dedicated to the UE under test is required. Note 3: Es/Iot, NRSRP and Io level has been derived from other parameters for information purpose. They are not settable parameters themselves. |  |  |  |  |

Table A.13.3.2.1.1-2: NTN specific test parameters for HD-FDD and TDD contention based random access test for UE category NB1 Standalone mode in Normal Coverage

| Parameter | Value | Comment |
| --- | --- | --- |
| Configuration 1 | SSC.1 | GSO Test Configuration, HD-FDD duplex mode |
| Configuration 2 | SSC.2 | NGSO Test Configuration, HD-FDD duplex mode |
| Configuration 2 | SSC.2 | NGSO Test Configuration, TDD duplex mode |

Table A.13.3.2.1.1-3: NPRACH-Configuration parameters for HD-FDD and TDD contention based random access test for UE category NB1 Standalone mode in Normal Coverage

| Field |  | Value |  |  |  | Comment |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Parameters not per NPRACH resource |  |  |  |  |  |  |  |
| RSRP-ThresholdsNPRACH-InfoList |  | {39, 24} |  |  |  | Corresponding to {-101, -116} dBm as defined in Table 9.1.4-1 |  |
| nprach-CP-Length |  | us66dot7 |  |  |  |  |  |
| nrs-Power |  | -5 dBm/15 kHz |  |  |  | As defined in clause 6.7.3 in TS 36.331. |  |
| Backoff Parameter Index |  | 1 |  |  |  | As defined in table 7.2-2 in TS 36.321 |  |
| Configured UE transmitted power () |  | 23 dBm for power class 3, 20 dBm for power class 5 |  |  |  | As defined in clause 6.2B.4 in TS 36.102 |  |
| powerRampingStep |  | dB2 |  |  |  |  |  |
| preambleInitialReceivedTargetPower |  | dBm-112 |  |  |  |  |  |
| preambleTransMax-CE |  | n6 |  |  |  |  |  |
| Parameters per NPRACH Resource |  |  |  |  |  |  |  |
| NPRACH Resource |  | Level 0 | Level 1 | Level 2 |  |  |
| nprach-Periodicity | Config. 1, 2 | ms40 | ms240 | ms1280 |  |  |
|  | Config. 3 | ms640 | ms1280 | ms2560 |  |  |
| nprach-StartTime |  | ms8 | ms64 | ms512 |  |  |
| nprach-SubcarrierOffset |  | n0 | n0 | n0 |  |  |
| nprach-NumSubcarriers |  | n12 | n12 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart |  | zero | zero | zero |  |  |
| maxNumPreambleAttemptCE |  | n3 | n6 | n10 |  |  |
| numRepetitionsPerPreambleAttempt | Config. 1, 2 | n2 | n8 | n64 |  |  |
|  | Config. 3 | n4 | n8 | n16 |  |  |
| npdcch-NumRepetitions-RA | Config. 1, 2 | r4 | r16 | r128 |  |  |
|  | Config. 3 | r4 | r8 | r16 |  |  |
| npdcch-StartSF-CSS-RA | Config. 1, 2 | v1dot5 | v1dot5 | v1dot5 |  |  |
|  | Config. 3 | v16 | v48 | v64 |  |  |
| npdcch-Offset-RA |  | zero | zero | Zero |  |  |
| nprach-NumCBRA-StartSubcarriers |  | n8 | n8 | n8 |  |  |
| ra-ResponseWindowSize (per NPRACH Resource) |  | pp2 | pp2 | pp2 |  |  |
| mac-ContentionResolutionTimer (per NPRACH Resource) |  | pp8 | pp8 | pp8 |  |  |
| Note 1: See Clause 6.7.3 in TS 36.331 for further information on the parameters in this table. |  |  |  |  |  |  |  |

##### A.13.3.2.1.2	Test Requirements

Contention based random access is triggered by not explicitly assigning a random access preamble via dedicated signalling in the downlink.

A.13.3.2.1.2.1	Random Access Response Reception

To test the UE behavior specified in Subclause 6.6A.2, the System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble after 3 preamble transmission attempts (the preamble may be transmitted multiple times in each attempt) have been received by the System Simulator. In response to the first 2 preamble transmission attempts, the System Simulator shall transmit a Random Access Response not corresponding to the transmitted Random Access Preamble.

The UE may stop monitoring for Random Access Response(s) and shall transmit the msg3 if the Random Access Response contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble.

The UE shall re-select a preamble and transmit with the calculated NPRACH transmission power when the backoff time expires if all received Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

In addition, the power applied to all preambles shall be in accordance with what is specified in Subclause 6.6A.2. The power of the first preamble shall be -25 dBm with an accuracy specified in clause 6.3B.4 of TS 36.102 [60]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3B.4 of TS 36.102 [60].

The transmit timing of all NPRACH transmissions shall be within the accuracy specified in Subclause 7.20A.2.

A.13.3.2.1.2.2	No Random Access Response Reception

To test the UE behavior specified in subclause 6.6A.2.2, the System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble after 3 preamble transmission attempts have been received by the System Simulator. The System Simulator shall not respond to the first 2 preamble transmission attempts.

The UE shall re-select a preamble and transmit with the calculated NPRACH transmission power when the backoff time expires if no Random Access Response is received within the RA Response window. The RA response window shall be started at the point in time indicated by clause 5.1.4 in TS 36.321[17].

In addition, the power applied to all preambles shall be in accordance with what is specified in Subclause 6.6A.2. The power of the first preamble shall be [-25] dBm with an accuracy specified in clause 6.3B.4 of TS 36.102 [60].The relative power applied to additional preambles shall have an accuracy specified in clause 6.3B.4 of TS 36.102 [60].

The transmit timing of all NPRACH transmissions shall be within the accuracy specified in Subclause 7.20A.2.

A.13.3.2.1.2.3	Receiving a NACK on msg3

To test the UE behavior specified in subclause 6.6A.2.3, the System Simulator shall NACK all UE msg3 following a successful Random Access Response.

The UE shall re-transmit the msg3 upon the reception of a NACK on msg3 until the maximum number of re-transmissions defined by maxNumPreambleAttemptCE in the table A.13.3.2.1.1-3 is reached.

A.13.3.2.1.2.4	Reception of an Incorrect Message over Temporary C-RNTI

To test the UE behavior specified in Subclause 6.6A.2.4, the System Simulator shall send a message addressed to the temporary C-RNTI with a UE Contention Resolution Identity included in the MAC control element not matching the CCCH SDU transmitted in msg3 uplink message.

The UE shall re-select a preamble and transmit with the calculated NPRACH transmission power when the backoff time expires unless the received message includes a UE Contention Resolution Identity MAC control element and the UE Contention Resolution Identity included in the MAC control element matches the CCCH SDU transmitted in the uplink message.

A.13.3.2.1.2.5	Reception of a Correct Message over Temporary C-RNTI

To test the UE behavior specified in Subclause 6.6A.2.4, the System Simulator shall send a message addressed to the temporary C-RNTI with a UE Contention Resolution Identity included in the MAC control element matching the CCCH SDU transmitted in the msg3 uplink message.

The UE shall send ACK if the Contention Resolution is successful.

A.13.3.2.1.2.6	Contention Resolution Timer expiry

To test the UE behavior specified in Subclause 6.6A.2.5, the System Simulator shall not send a response to a msg3.

The UE shall re-select a preamble and transmit with the calculated NPRACH transmission power when the backoff time expires if the Contention Resolution Timer expires.

A.13.3.2.1.2.7	NPRACH Resource Selection

The UE shall select NPRACH resources and transmits or re- transmits NPRACH preambles using the NPRACH resources and NPRACH configuration corresponding to the coverage enhancement level 0. The rate of correct coverage enhancement level selection during repeated tests shall be at least 90%.

Note:	Correct coverage enhancement level selection is a prerequisite for testing the other NPRACH requirements.

## <<< END OF CHANGES 2>>>