# 3GPP TSG-RAN WG4 Meeting #118	R4-2600545

# Gothenburg, Sweden, Feb 9 – 13, 2026

Agenda item:			8.6

Source:	Feature lead (Apple)

Title:	Running summary of 6G RRM

Document for:	Information

# Introduction

In RAN #110 meeting, the revised SID of study on 6G Radio was agreed [RP-253876], and the RAN4 RRM related scopes were summarized as follows:

(5) 6G RRM core and performance requirements d) RRM aspects for 6GR [RAN4, RAN1, RAN2] - RRM requirement and procedure aspects aiming at improvements and/or simplification compared to 5G NR - Study how to improve 6G requirement specification, including structure and drafting principles Others aspects that have RRM impact: (2) Physical Layer structure for 6GR h) Initial access [RAN1, RAN2, RAN4] i) 6GR spectrum utilization and aggregation. [RAN1, RAN2, RAN4] (4) Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3] (7) Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR: - 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3] - Study if any additional migration option(s) is needed (other than standalone, MRSS, and inter-RAT mobility between NR-6G). [RAN] [RAN2, RAN1, RAN3, RAN4] RAN plenary starts this study in March 2026 and will make a decision by September 2026 whether to expand WG SI scope to cover additional migration option(s). - Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4] (8) AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4]

In this running summary, the following information is captured for 6G RRM study:

- The key RAN4 agreements and tdocs from last RAN4 meeting, including topic summary, WF and AdHoc minutes.

- RRM related agreements summary from RAN1

- RRM related agreements summary from RAN2

The information of feature lead for 6G RRM is attached below.

| Feature Lead | Affiliation | Email |
| --- | --- | --- |
| Jie Cui (Jerry) | Apple | jie_cui@apple.com |
| Jin-Yup Hwang | LGE | jinyup.hwang@lge.com |

# Status update

FL note: to be added when RRM scopes are concluded.

# RAN4 - 6G RRM agreements

## RAN4 #117 meeting (Nov-2025)

### Key tdocs

| R4-2522270 | Topic Summary for [117][105] 6G RRM |
| --- | --- |
| R4-2522443 | AH minutes on [117][105] 6G RRM |
| R4-2522442 | WF on [117][105] 6G RRM |

### Key agreements

Topic 2-1: MG related scope

[Agreement]

RAN4 RRM to first study the following 6G MG related sub-topics:

- Sub-topic 1: Gap-less measurement and its side conditions

- Sub-topic 3: Adaptive MG operation and UE assisted MG configuration

- Sub-topic x: MG pattern/configuration design in 6G (FFS if considering the scopes from subtopic 2 and 4 in FL summary, and details can be decided in next meeting)

- Sub-topic y: Granularity of MG applicability, e.g., per-UE, per-FR, per-CC, per-CC group, or per-band group

- FFS in next meeting: Sub-topic 5: Multi-carrier measurements in MG (to decide where to place this topic in next meeting, e.g., in MG or in RRM framework)

Detailed scope for above sub-topics can be decided in Feb meeting, and other MG related sub-topics are open for discussion in a contribution-driven way from Q3 2026.

Topic 2-2: interruption related scope

Agreement:

Whether or when RAN4 to start the study of this interruption-free or interruption reduction based RRM can be discussed from Q2 2026 if progress in RAN4 and other WGs is sufficient to support this study. This discussion can be contribution driven.

Topic 3: RRM framework: Measurement capability/delay/overhead/accuracy/unified measurement

[Agreement]:

      - For Sub-topic 1: Unified measurements

        - Define the exact study scope with priorities in RAN4#118 for this sub-topic, including

          - The study on the use case and scenarios for the study scope, including the definition of the unified measurement

          - The study on the benefits and drawbacks to define framework of the measurement requirement unification and the potential impact on the measurement requirements and the behaviors

          - The study on whether and how the identified RRM measurement requirements/behaviors can be unified,

          - Others are not precluded

[Agreement]:

- For Sub-topic 2: UE group for RRM

    - Define the exact study scope with priorities in RAN4#118 for this sub-topic, including

      - Study on the potential scenario and use cases for UE group

      - Study on the benefit and drawback the UE group for RRM

      - Study on the following aspects:

        - potential gain and drawback compared with the RRM measurement without UE group, e.g., existing power saving schemes

        - impact to UE and network

        - impact to other WGs

      - Study on the RAN4-relevant assumptions for UE grouping

[Agreement]

- Sub-topic 3: Identification/measurement/tracking/delay reduction

    - Define the exact study scope with priorities in RAN4#118 for this sub-topic, including

        - RRM measurement requirements

          - It will be decided in RAN4#118 if this study should be pending on the progress of the unified measurement study.

        - Hold the following work until sufficient progress made in RAN1

          - Study searcher number

          - Study measurement capability for number of cells, beams and frequency layers

          - Rx beam sweeping factor

          - Others proposals are not precluded.

## RAN4 #116bis meeting (Oct-2025)

### Key tdocs

| R4-2514512 | Topic Summary for [116bis][105] 6G RRM |
| --- | --- |
| R4-2514644 | WF on [116bis][105] 6G RRM |
| R4-2514625 | AdHoc meeting minutes on 6G RRM |

### Key agreements

Issue 3: General RRM scope

Agreement:

The scope of this issue is merged to Issue 5: RRM framework. This category is removed.

Issue 4-1: MG related scope

Agreement in main session (Tue):

Measurement gap, including gap-less measurement, is agreed as part of RAN4 RRM 6G study. The detailed scope will be further decided.

- Interruption enhancement due to gap-less measurement will be part of the MG discussion.

Issue 5: RRM framework: Measurement capability/delay/overhead/accuracy

Agreement:

RRM framework: measurement capability/delay/overhead/accuracy/quantities/unified measurement, is agreed as part of RAN4 RRM 6G study. The detailed scope will be further decided.

Note: the title of the main topic can be revised after selection of the sub-topics.

# RAN1 - RRM related agreements summary

## Overview of 6GR air interface

### RAN1 #123 (Nov-2025)

Agreement

If the minimum spectrum allocation is 3MHz with 15kHz SCS for 6GR,

- Opt1: Design of the common signals/channels (at least for SSB) for initial access by assuming bandwidth larger than 3MHz, which is applicable to any spectrum allocations with adjustment, if applicable

- Opt2: A single design of the common signals/channels (at least for SSB) for initial access by assuming minimum spectrum allocation as target bandwidth 3MHz, which is applicable to any spectrum allocations

Agreement

Skeleton for TR 38.760-1 “Study on 6G Radio RAN1 aspects” v0.0.3 in R1-2509569 is endorsed.

Agreement

- For scalable 6GR design for diverse device types, RAN1 can at least consider the following, targeting applicable to all 6G device types,

  - Basic initial access procedures from RAN1 perspective

  - Other PHY features after initial access procedure, e.g., Other DL/UL control, scheduling/HARQ

  - Coverage features to meet the identified coverage target

  - Energy saving both at BS and UE sides

  - MRSS

  - Note: whether these features are supported, mandatory or optional is separate discussion

Agreement

- For the RAN1 study of “Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band”,

  - The link budget template candidates 1 and 2 are used to calculate the metric(s) as starting point to compare existing 5G mid-band and 6G deployments in at least around 7 GHz, with potential future update.

  - During initial access/random access

    - Coverage target is referring the bottleneck channel (i.e. Rel-15 NR Msg3) during initial access/random access for existing 5G mid-band

  - FFS target value(s) of data rate for data channels relative to 5G mid-band

  - Following deployment scenarios are considered

    - Urban macro (both O2I and outdoor)

    - Sub-urban macro (both O2I and outdoor)

  - Following carrier frequencies are considered to calculate the metric(s)

    - [4 GHz] as the existing 5G mid-band

    - 7 GHz as 6G deployment

Template in R1-2509615 is to be used for collecting inputs on the values from companies.

Agreement

- For the smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework, from physical layer perspective, RAN1 to consider at least

  - Overall device complexity

  - Overall system performance impact

  - Energy efficiency for both BS and UE

  - Aim at a single common signals/channels design in idle mode and initial access for diverse device types, as well as meeting mobile broadband service requirements as high priority

### RAN1 #122bis (Oct-2025)

Agreement

- High-level aspects to consider for the 6GR sync signal structure include, but not limited to

  - Sync raster design

  - Spectrum allocation

  - smallest maximum supported RF and BB UE BW without spectrum aggregation

  - mobile broadband service requirements as high priority

  - Energy efficiency for both BS and UE

  - Detection/tracking performance, latency, and complexity

    - Including initial cell search

  - Coverage target

  - Common design for diverse device types

  - Consideration of the supported deployment

  - Consideration on whether the single sync signal structure is sufficient

  - Note: Aspects impacting on the periodicity is to be discussed under AI11.5

Agreement

- The aspects to consider for supporting NTN include, but not limited to

  - Initial access, including cell search and SSB periodicity

  - Coverage

  - Duplexing

  - Capacity

  - Signalling overhead

  - GNSS-less/resilient/based operation

  - Large/varying doppler and propagation delay

  - Beamforming / beam management / beam hopping

Note:

- High-level aspects to consider to enable lower CAPEX/OPEX with respect to current networks include, but not limited to

  - UE/NW implementation complexity

  - UE/NW energy efficiency

  - MRSS

  - Spectrum efficiency

### RAN1 #122 (Aug-2025)

Agreement

- For the study of RAN1 6GR design, consider the minimum spectrum allocation in which 6G can operate, subject to further discussion and confirmation in RAN.

  - Note: RAN4 involvement is necessary.

Agreement

- On enhanced overall coverage, identify coverage target(s) considering diverse use cases and device types

Agreement

- Identify the high-level aspects which impact on the 6GR sync signal structure and associated periodicity.

Agreement

- Identify the high-level aspects which impact on the NR-6GR MRSS support

  - Including the lessons learned from LTE-NR DSS

Agreement

- Study and identify the lessons learned from NR BWP framework

Agreement

- Study and identify the lessons learned from NR spectrum utilization and aggregation framework

  - DC is subject to RANP decision in June 2026

  - Note: MRSS aspects are separate discussion

Agreement

- Study the following smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, subject to further discussion and confirmation in RAN

  - Opt1: 3MHz

  - Opt2: 5MHz

  - Opt3: 10MHz

  - Opt4: 20MHz

  - FFS: the UL bandwidth may be different to the DL bandwidth

  - FFS: the bandwidth value may be different for different SCS, duplex modes, and bands.

  - FFS: whether RF and BB UE BW are same or different

Agreement

- Study and identify the lessons learned from NR duplex modes

- On 6GR duplexing study, RAN1 considers at least following duplex types

  - FD-FDD

  - Semi-static TDD

  - gNB semi-static SBFD

  - HD-FDD on UE side

  - Dynamic TDD

- Study whether to consider following duplexing types

  - gNB dynamic SBFD

  - UE SBFD

  - gNB FD

  - Note: Other duplex modes are not precluded

## Waveform and frame structure for 6GR air interface

### RAN1 #123 (Nov-2025)

#### Frame structure

Agreement

- RAN1 assumes maximum channel bandwidth 800MHz or 400MHz at network side for FR2-1

- 800MHz or 400MHz, to be down-selected in the future

- FFS: 800MHz or 400MHz at UE side.

Agreement

For how to enable UE to support 400MHz bandwidth when a network supports 400 MHz Channel Bandwidth (CBW), the following options 1/2/3/4/5 are considered from RAN1 understanding for studying

  - Option 5: Variance of Option 3 by assuming single FFT and 2 RF chain.

  - FFS which aspects of the BB processor in option 3 and 4 should be separated/parallelled.

  - Note: DL and UL design options may be considered independently.

  - To provide potential specification impact of each option.

  - To provide investigations on performance/energy efficiency/cost/complexity for the above options.

  - Inform RAN4 about the above information.

Agreement

Final LS R1-2509578 is endorsed.

### RAN1 #122bis (Oct-2025)

#### Frame structure

Agreement

For communication, 6GR considers NR frame structure used as a starting point for the study item,

  - Resource defined by one subcarrier and one symbol is called as resource element (RE).

  - Resource block (RB) is defined where the number of consecutive subcarriers per RB is the same for all numerologies and the number of subcarriers per RB is 12

  - Radio Frame length is 10ms

  - Each radio frame is split into 10 subframes, each with a duration of 1 ms

  - For given SCS and for given symbol, the symbol duration, normal CP length and boundary is same as NR design.

  - A slot is defined as supporting 14 consecutive symbols for normal CP case and all subcarrier spacings.

Agreement

6GR study assumes same SCS between 6GR Sync signals and other channels/signals (except PRACH) for a given band.

- FFS: same/different SCS between 6GR sync signal and other channels/signals (except PRACH) for FR2-1.

- Note: ISAC is separately discussed in ISAC session.

Agreement

- RAN1 assumes 400MHz maximum channel bandwidth at network side and 30kHz SCS around 7GHz

- Study whether and how to enable UE to support 400MHz bandwidth

### RAN1 #122 (Aug-2025)

#### Frame structure

Agreement

- 6GR takes the following SCS as start point for discussion for all the signals/channels except PRACH.

  - For sub 6GHz

    - The following subcarrier spacing is at least supported

      - 15kHz SCS for FDD, 30kHz SCS for TDD

    - FFS: 30kHz SCS for FDD for around e.g., 1-2.5GHz

    - FFS: 7.5kHz SCS for sub1GHz (FDD)

    - Whether to discuss the FFS will be subject to RANP decision.

  - For around 7GHz

    - The following subcarrier spacing options can be studied

      - 30kHz, 60kHz

  - FFS: For around 15GHz

    - The following subcarrier spacing options can be studied

      - 30kHz, 60kHz, 120kHz

    - Whether to discuss it will be subject to RANP decision

  - For between 24.25GHz - 52.6GHz

    - Subcarrier spacing 120kHz is supported

  - FFS whether to allow using additional subcarrier spacing for SSB

- FFS subcarrier spacing for PRACH and up to initial access discussion.

Conclusion

- Numerologies for sensing is up to sensing agenda discussion.

Agreement

- 6GR supports normal cyclic prefix, i.e., same as the normal CP defined in NR.

  - FFS potential need for other CP

## Energy efficiency

### RAN1 #123 (Nov-2025)

Agreement

Add the following as one of reference configurations for BS power consumption

|  | Set 4 around 7** GHz |
| --- | --- |
| Duplex | TDD |
| System BW | 400 MHz |
| SCS | 30 kHz |
| Number of TRP | 1 |
| Total number of DL TX Rus | 256 |
| Total DL power level | [62] dBm |
| Total number of UL Rx Rus | 256 |
| **Pending agreement in 11.2 whether to evaluate 15 GHz. |  |

Agreement

Include the following non-sleep states as 6G UE power consumption model.

- Other power state(s) is not precluded

- FFS: Configuration and relative power value(s) for EE processing in FR2 (including 24.25 GHz – 52.6 GHz)

- Note: Pending agreement in 11.2 whether to evaluate 15 GHz

| Power State | Characteristics | Relative Power in FR1 (including around 7GHz) | Relative Power in FR2 (including 24.25 GHz – 52.6 GHz) |
| --- | --- | --- | --- |
| EE Processing | Processing DL WUS of OFDM-based sequence(s) for at least wake-up indication and, if applicable, other 6GR signal(s) of OFDM-based sequence(s) for synchronization and/or measurement, in an energy efficient manner, based on the following configuration: FR1 (including around 7GHz) - 5 MHz BW, 1-RX / 2-RX for reception; FFS: Power values for 2-RX, i.e., X1, X2, X3 - Residue CFO up to [5] ppm and residue timing offset up to [2] us - Noise Figure (NF) = 6GR UE NF - Reception time up to one slot - Note: Whether/how to scale power values for different reception time setting within one slot to be further discussed/decided FR2 (including 24.25 GHz – 52.6 GHz) - FFS Note: No implication on which configuration(s) to be supported by 6GR EE-processing can only be performed during a sleep state with additional relative power value added w.r.t. the sleep state and without triggering UE transition out of the sleep state. Additional energy overhead [15], in unit of (relative power x ms), is included for each time entering or leaving EE processing during ultra-deep/deep sleep. Ramp-up or ramp-down time for EE processing is [2] ms. | 10 / X1 (during micro sleep) 12 / X2 (during light sleep) 15 / X3 (during other sleep) | FFS |
| PDCCH-only | No PDSCH and same-slot scheduling; this includes time for PDCCH decoding and any micro-sleep within the slot. | 100 | 175 |
| SSB or CSI-RS proc. | SSB can be used for fine time-frequency sync. and RSRP measurement of the serving/camping cell. TRS is the considered CSI-RS for sync. FFS the power scaling for processing other configurations of CSI-RS. | 100 | 175 |
| PDCCH + PDSCH | PDCCH + PDSCH. ACK/NACK in long PUCCH is modeled by UL power state. | 300 | 350 |
| UL | Long PUCCH or PUSCH or PRACH FFS: Power values for more Tx power levels, i.e., X4, X5, X6, X7 | 250 (0 dBm) X4 (10 dBm) X5 (15 dBm) X6 (20 dBm) 700 (23 dBm) X7 (26 dBm) | 350 (FFS Tx power level) |

Agreement

Add the following as one of reference configurations for BS power consumption

|  | Set 4 around 7** GHz |
| --- | --- |
| Duplex | TDD |
| System BW | 400 MHz |
| SCS | 30 kHz |
| Number of TRP | 1 |
| Total number of DL TX Rus | 256 |
| Total DL power level | [62] dBm |
| Total number of UL Rx Rus | 256 |
| **Pending agreement in 11.2 whether to evaluate 15 GHz. |  |

Agreement

Include the following non-sleep states as 6G UE power consumption model.

- Other power state(s) is not precluded

- FFS: Configuration and relative power value(s) for EE processing in FR2 (including 24.25 GHz – 52.6 GHz)

- Note: Pending agreement in 11.2 whether to evaluate 15 GHz

| Power State | Characteristics | Relative Power in FR1 (including around 7GHz) | Relative Power in FR2 (including 24.25 GHz – 52.6 GHz) |
| --- | --- | --- | --- |
| EE Processing | Processing DL WUS of OFDM-based sequence(s) for at least wake-up indication and, if applicable, other 6GR signal(s) of OFDM-based sequence(s) for synchronization and/or measurement, in an energy efficient manner, based on the following configuration: FR1 (including around 7GHz) - 5 MHz BW, 1-RX / 2-RX for reception; FFS: Power values for 2-RX, i.e., X1, X2, X3 - Residue CFO up to [5] ppm and residue timing offset up to [2] us - Noise Figure (NF) = 6GR UE NF - Reception time up to one slot - Note: Whether/how to scale power values for different reception time setting within one slot to be further discussed/decided FR2 (including 24.25 GHz – 52.6 GHz) - FFS Note: No implication on which configuration(s) to be supported by 6GR EE-processing can only be performed during a sleep state with additional relative power value added w.r.t. the sleep state and without triggering UE transition out of the sleep state. Additional energy overhead [15], in unit of (relative power x ms), is included for each time entering or leaving EE processing during ultra-deep/deep sleep. Ramp-up or ramp-down time for EE processing is [2] ms. | 10 / X1 (during micro sleep) 12 / X2 (during light sleep) 15 / X3 (during other sleep) | FFS |
| PDCCH-only | No PDSCH and same-slot scheduling; this includes time for PDCCH decoding and any micro-sleep within the slot. | 100 | 175 |
| SSB or CSI-RS proc. | SSB can be used for fine time-frequency sync. and RSRP measurement of the serving/camping cell. TRS is the considered CSI-RS for sync. FFS the power scaling for processing other configurations of CSI-RS. | 100 | 175 |
| PDCCH + PDSCH | PDCCH + PDSCH. ACK/NACK in long PUCCH is modeled by UL power state. | 300 | 350 |
| UL | Long PUCCH or PUSCH or PRACH FFS: Power values for more Tx power levels, i.e., X4, X5, X6, X7 | 250 (0 dBm) X4 (10 dBm) X5 (15 dBm) X6 (20 dBm) 700 (23 dBm) X7 (26 dBm) | 350 (FFS Tx power level) |

Agreement

Include the following sleep states for 6G UE power consumption model.

Note: Ultra Deep Sleep is not intended for connected mode

| Power State | Characteristics | Relative Power |
| --- | --- | --- |
| Ultra Deep Sleep | Time interval for the sleep should be larger than the total transition time entering and leaving this state. Accurate timing or frequency may not always be maintained. | 0.05 + Y* |
| Deep Sleep | Time interval for the sleep should be larger than the total transition time entering and leaving this state. Accurate timing may not always be maintained. | 1 + Y* (Optional: 1) |
| Light Sleep | Time interval for the sleep should be larger than the total transition time entering and leaving this state. | 20 |
| Micro sleep | Immediate transition is assumed for power saving study purpose from or to a non-sleep state | 45 |
| * Y value equals to 0.1, if EE processing is assumed for the evaluation; zero, otherwise. |  |  |

| Sleep type | Additional transition energy: (Relative power x ms) | Total transition time** |
| --- | --- | --- |
| Ultra deep sleep | [40000] | [1600] ms*** |
| Deep sleep | 450 | 20 ms*** |
| Light sleep | 100 | 6 ms |
| Micro sleep | 0 | 0 ms* |
| * Immediate transition is assumed for power saving study purpose from or to a non-sleep state ** Ramp-up time is no less than half of the total transition time *** Time for sync/re-sync is not included |  |  |

Agreement

For evaluation purposes and relative comparison over different candidate energy saving schemes for 6GR, define the following baseline power saving configurations for UE for evaluation purpose for FR1 (including around 7GHz):

- 5G NR I-DRX (1.28s cycle) for idle mode

  - Group paging rate (for a PO): TBD

- 5G NR C-DRX settings of (cycle, on-duration timer, inactivity timer) are assumed for the following 6GR traffic models for connected mode:

  - VoIP: (40 ms, 8 ms, 10 ms)

  - FTP3: (160 ms, 8 ms, 100 ms)

  - Instant message: (320 ms, 8 ms, 100 ms)

  - XR: (16 ms, 10 ms, 4 ms)

- Companies can evaluate and report other traffic(s) and/or configuration(s) with justification

Note: The corresponding evaluation is not intended for energy efficiency comparison with 5G/NR.

Agreement

Total DL power level for Set 4 is 56 dBm.

The following relative power levels applies to Set 4 for a CAT 1 BS and a CAT 2 BS:

| Power state | Set 4 |  |
| --- | --- | --- |
|  | CAT 1 BS | CAT 2 BS |
| Deep sleep | 1 | 1 |
| Light sleep | 31 | 3.1 |
| Micro sleep | 85 | 7.3 |
| Active DL | 530 | 67 |
| Active UL | 175 | 10 |

Agreement

For the UE power model, assume:

- ΓB: The ratio of active bandwidth to the bandwidth of reference configuration (100 MHz)

- ΓTput: The ratio of the maximum schedulable PDSCH throughput w.r.t. that of current active bandwidth, ΓTput∈{1, 1/2, 1/4}

  - FFS: How the maximum schedulable PDSCH throughput ratio ΓTput is ensured by NW

  - Companies to report the assumed technique to achieve the reduction in the maximum schedulable PDSCH throughput. Current techniques include at least: reduced scheduled maximum bandwidth, reduced scheduled maximum rank, and reduced scheduled duty cycle.

  - No antenna adaptation is assumed simultaneously

- For evaluation purpose,

  - Company to report Tmin value for the evaluation

  - T>Tmin is another relaxed adaptation delay. Company to report T value for the evaluation.

  - The adaptation delay is applied when adapting ΓB and/or ΓTput

- Companies to report SMaxBW from one of the two options:

  - Option 1: SMaxBW=0 / Z1 / Z2 for Maximum BW (MaxBW) assumed for the evaluation ∈{100 MHz,200 MHz, 400 MHz}

  - Option 2: SMaxBW=0

- Note: No implication on which configuration(s) and what adaptation(s) to be supported by 6GR

Include the following scaling for DL bandwidth adaptation for 6GR UE power consumption model:

- For PDCCH+PDSCH:

| ΓB | ΓTput | Scaling factor when adaptation delay T is assumed | Scaling factor when adaptation delay Tmin is assumed |
| --- | --- | --- | --- |
| 5% | 1 | X01 | X11 +SMaxBW |
| 20% | 1 | X02 | X12 +SMaxBW |
| 100% | {1, 1/2} | 1 | 1+SMaxBW |
| 200% | {1, 1/2} | {X03, Y03} | {X13, Y13} + SMaxBW |
| 400% | {1, 1/2, 1/4} | {X04, Y04, Z04} |  |

- For PDCCH-only:

| ΓB | ΓTput | Scaling factor when adaptation delay T is assumed | Scaling factor when adaptation delay Tmin is assumed |
| --- | --- | --- | --- |
| 5% | 1 | X21 | X31 +SMaxBW |
| 20% | 1 | X22 | X32 +SMaxBW |
| 100% | 1 | 1 | 1+SMaxBW |
| 200% | {1, 1/2} | {X23, Y23} | {X33, Y33} + SMaxBW |
| 400% | {1, 1/2, 1/4} | {X24, Y24, Z24} |  |

- Other DL signal/channel processing:

| ΓB | Scaling factor when adaptation delay T is assumed | Scaling factor when adaptation delay Tmin is assumed |
| --- | --- | --- |
| 5% | X41 | X51 +SMaxBW |
| 20% | X42 | X52 +SMaxBW |
| 100% | 1 | 1+SMaxBW |
| 200% | X43 | X53 + SMaxBW |
| 400% | X44 |  |

- Not applicable to sleep states and EE processing state

- Further consolidate the table for cases of a same scaling factor

- FFS: Scaling factor for other value(s) of ΓB<100%

For UL bandwidth adaptation,

- No scaling for FR1 and around 7GHz

- In case scaling is needed for FR2 (including 24.25 GHz – 52.6 GHz), companies can report the assumed scaling factor

Note: The term “bandwidth” used here is reusing the definition as 5GNR, which will be updated according to 6GR discussion

Note: All columns and rows of the above tables will be further checked, and corresponding values will be checked and confirmed/extended.

### RAN1 #122bis (Oct-2025)

Agreement

At least the following NR metrics,

- Network energy saving gain relative to baseline for BS

- UE energy saving gain relative to baseline for UE

- Impact to UPT (User-Perceived Throughput), if applicable,

as well as the metrics

- Impact to latency, if applicable

- Impact to QoS/delay budget satisfaction rate, if applicable

are used for 6G energy efficiency evaluation.

Agreement

Apply the following evaluation methodology framework for Quantitative analysis,

- For NW unloaded/empty load case or UE idle/inactive mode:

- For energy saving: analytical calculation

- For performance impact: analytical calculation, LLS

- For loaded cases and connected-mode UEs

- For energy saving: SLS

- For performance impact: LLS, SLS

Agreement

For evaluation purposes, expand the existing BS power model reference configuration with a set for ~7 GHz operation with the following parameters:

| Property | Configuration for Set 4 around 7 GHz |
| --- | --- |
| Duplex | TDD |
| BW | [100, 200, 400] MHz |
| SCS | [30 kHz, 60 kHz] |
| Number of TRP | 1 |
| Total number of DL TX RUs | [128, 256] |
| Total DL power level | [56] dBm |
| Total number of UL Rx RUs | [128, 256] |

Note: Bracketed values to be confirmed. Other values are not precluded.

The above configuration has no implication on supported BW, SCS for 6GR.

Agreement

Study whether/how to further update the BS model considering the following aspects, e.g.,

- Whether to downselect between Cat.1 and Cat. 2,

- Updates of parameter values (including defining a new Cat),

- Updates of power scaling, power states (including additional PSs)

- Etc.

Note: The defined BS power models does not preclude use case-specific enhancements regarding, e.g., multi-TRP, SBFD, multi-carrier etc

Agreement

Study and evaluate NW energy savings and the impact on UE performance and user experience with respect to 20ms and longer periodicities of sync signal(s) at least for initial access with the following consideration, but not limited to:

BS assumptions:

- Cell-common signaling (e.g., sync signal(s), broadcast PDCCH, SIB-1, SIB, paging, PRACH), e.g.,

  - Clustered provisioning of different cell-common signaling,

  - On-demand provisioning of different cell-common signaling,

- UE-specific signaling (for low, light, medium loads), e.g.,

  - Clustered provisioning with cell-common signaling,

  - Unclustered provisioning with cell-common signaling,

UE impact:

- Cell search complexity and latency, including frequency search latency,

- UE Power consumption,

- Sync signal detection, coverage and tracking performance,

- RRM, mobility,

- Beam management,

- Other properties are not precluded,

- Improvements to address identified impact, e.g.,

  - Additional sync signal needs,

  - Adaptation of sync signal transmission periodicity,

  - Sparser synch raster.

Agreement

Study and evaluate on-demand and/or periodic SIB-1 transmission with respect to

- NW energy savings potential and UE power consumption impact,

- SIB-1 acquisition delay,

- NW and UE complexity,

- Coverage,

- Applicable deployment scenarios, e.g.,:

  - Standalone cell/carrier,

  - Multiple TRPs/cells/carriers.

Agreement

For 6GR energy efficiency evaluation purposes, reuse the existing UE power consumption model FR1 and FR2 reference configurations in TR 38.840 for operation up to around 7GHz and within 24.25 GHz – 52.6 GHz, respectively.

- Scaling rules can be updated, including additional rule(s) for scaling UE power consumption, and including around 7GHz specific update

  - FFS: details.

- Power value and transition time update, if necessary, including around 7GHz specific update

- No implication on supported BW, SCS, modulation and antenna setting for 6GR

- Revisit if SCS for around 7GHz is different with respect to the reference configuration

Agreement

Study and evaluate DL WUS of OFDM based sequence and corresponding mechanisms for 6GR EE improvement, regarding at least the following aspects:

- Coverage target for DL WUS (e.g., same as PDCCH, common sync signal, or other)

- Measurements and/or synchronization.

- System overhead and network energy consumption/UE energy saving for UE operation with the DL WUS.

- RRC states

- Other functionalities

Agreement

For evaluation purposes, study extending NR UE power consumption scaling w.r.t. at least BW and/or antenna setting, considering at least the different characteristics in RF/BB power consumption and static/dynamic power consumption.

Agreement

For evaluation purposes, study extending NR UE power consumption model for UE operation with DL WUS of OFDM-based sequence, regarding the following aspects:

- Power state(s), sleep and non-sleep, and corresponding characteristics and power value(s)

- Transition energy and time for each of sleep state(s)

- Companies to report the assumption(s) for achieving the proposed power value(s), e.g., time/frequency domain detection, noise figure assumption(s), synchronization assumption(s), BW/antenna assumption(s), etc.

Agreement

Study and evaluate on-demand sync signal(s) mechanisms for 6GR energy efficiency, considering, e.g.,:

- On-demand sync signal(s) for single cell/carrier, multi-carrier/cell, multi-TRP,

- Network-triggered and UE-triggered on-demand sync signal(s),

- Idle and/or connected modes,

- Other mechanisms/aspects/signals/channels are not precluded.

Agreement

Study and evaluate multi-carrier/cells/TRPs mechanisms for 6GR NES, considering, e.g.,:

- Sync signal-less carriers/cells/TRPs for at least intra-band and collocated inter-band multi-carrier/cell/TRPs, including potential extensions to additional deployments and scenarios,

- RRC states,

- UE energy consumption and complexity,

- Other mechanisms/aspects/signals/channels are not precluded.

### RAN1 #122 (Aug-2025)

Agreement

Study how to reuse and update reference configurations in TR 38.864 for 6G BS.

Agreement

Study how/whether to reuse or update the power model in TR 38.864 for evaluating BS power consumption for 6G BS.

Agreement

- Study metric(s) for UE energy efficiency.

- Study metric(s) for BS energy efficiency.

Agreement

Study reference configurations and power consumption model for 6G UE, considering but not restricted to the following:

- TR 38.840 (UEPS), TR38.875 (RedCap), TR38.865 (eRedCap), and TR38.869 (LP-WUS/WUR) for reference configurations

- TR 38.840 (UEPS), TR38.875 (RedCap), and TR38.869 (LP-WUS/WUR) for power consumption models

Agreement

- Study baseline BS setting(s) for evaluating 6G BS EE improvement/impact, considering NR features and 6G BS reference configuration(s)

- Study baseline UE setting(s) for evaluating 6G UE EE improvement/impact, considering NR features and 6G UE reference configuration(s)

# RAN2 - RRM related agreements summary

## Radio protocol architecture

### RAN2 #132 (Nov-2025)

#### TN/NTN integration

Agreements

- 1	The basic assumption with 5G NTN deployment scenarios (LEO, MEO, GSO) and payload type can be adopted in 6G NTN.   FFS whether we will study with and/or without GNSS, depending on RANP and other WG discussion first.

2	For 6G NTN, RAN2 will consider at least the following features/procedures to treated together with the TN design (e.g. common features/procedures):

-	HARQ

-	L2 timer handling, e.g., the timer value range should cover both TN and NTN, adaptive UP timer start time based on RTT;

-	Measurement framework (including measurement gap)

-	Mobility framework (including TN and NTN mobility)

-	TAC. e.g., support multiple TACs per PLMN in one cell;

-	SIB acquisition/design

-	PWS (Public Warning System) services over an intended area;

-	Initial access framework (including network type indiciation)

3	At least the following NTN features will be further studied in RAN2 (maybe treated in NTN specific AI).  Focus will be on RAN2 impacts.

-	Uplink time and frequency pre-compensation;

-	TA reporting;

-	Satellite switch with re-sync;

-	Beam hopping.

-	Discontinuous coverage

NOTE:  Common design can be considered if something can be applicable to TN as well later

- 4	The following features can be discussed in RANP

-	MBS broadcast and multicast via NTN;

-	Dual connection;

-	Carrier Aggregation.

-	VLEO

#### User plane

Framework for service awareness:

Agreements

1	Initial study on application/service awareness in RAN and UE should focus on identifying traffic characteristics/requirements common among the target 6G use cases and exploring whether/how it can be made available to RAN and UE in a generalized framework (as much as possible).  This can be done in coordination with SA2 when needed.

2	RAN2 to discuss how the characteristics and information of services can influence L2 protocols for both UL/DL

#### Control plane

RRC Modelling and connection management

Agreements

1	Inactive mode or sub-state: In addition to what was agreed last meeting, it can also support fast transition to data exchange and network controlled transitions into state/sub-state.

2	For fast transition to data exchange, we can consider small data transmission in the new state/sub-state and/or transition to RRC connected.

3	Revisit the modelling of state or sub-state after some further progress is made on functionalities and understanding how things would work in these states.

4	Study single paging mechanism and understand the pain points and impacts of each paging mechanism.

RRC Structure and (re)configuration

Agreements

- Delta configuration is still useful in 6G to reduce signalling overhead.

At least the following issues have been identified

- Issue identified: The need code introducing additional restraints (e.g., Need S) and conditions (e.g. conditional presence) are the main causes of implementation complexity and compatibility issues in delta configuration

- Issue identified - The ambiguity "Functionally mandatory UE configuration parameters can be absent in over-the-air RRC messages for initial configuration of a feature/functionality" is very common in 5G RRC signalling.

- Issue identified - The ambiguity "UE configuration parameters which shouldn't be modified after initial configuration of a feature/functionality can be sent in subsequent over-the-air RRC messages with new values" is relatively common in 5G RRC signalling.

- Goal is to address the issues

Agreements

- Issue identified: UE can only apply a part of RRC reconfiguration.  NOTE this is not related to IODT issue.

- Study how to solve the issue

Initial and System Access and Others

Agreements on system information

- Assume on demand SI is supported and study further.   Study further on demand SIB1 from RAN2 perspective.

- Assume area specific SI is supported.  Further study what information can be common.   FFS area specific SIB1

Study SI scheduling mechanism to enable more flexible scheduling and clustering/bundling of transmission(s) with other common signalling.  Study should also take into account UE and NW complexity.   Aim to have one mechanism if possible.

Before discussing SIB1 splitting, understand SIB1 design, content and size.  Then we can revisit how and if splitting is needed taking into account benefits and UE and NW impacts, and taking access latency into account.

Agreements on multicarrier

- RAN2 to study the need for multicarrier operation in IDLE/INACTIVE (if supported) (carrier(s) for paging and/or system information and/or random access) by taking into account e.g. latency,  device categories/features, capacity, coverage requirements, UE power consumption/complexity, NW energy saving.  Close coordination with RAN1/RAN4 is required.

- Discussion on modelling of cell is FFS and also depends on RAN1/RAN4 progress.   Discussions on this will not happen next meeting.

#### Common User plane and Control plane

Energy efficiency

- Cell DTX/DRX

- Agreements

- Study further how to achieve sleeping opportunities (e.g. CELL DTX/DRX) for connected mode.

- There are benefits from NW perspective to aligning the sleeping opportunities (e.g. CELL_DTX/DRX) across different states.

- For non-connected state wait for further progress on the design common signal/SI/paging/RA, etc design from RAN2 and other WGs.

C-DRX

Agreements

- As a starting point for UE sleeping opportunities, assume C-DRX like mechanism is supported.  Study further details of C-DRX like mechanism.

- Study DL WUS.   Study whether and how it interacts/work together with C-DRX like scheme.  Take RAN1 progress into account with respect to L1 power saving features.   Aim to avoid duplicate functionalities.

- Study interaction with NW sleeping opportunities (e.g. CELL DTX/DRX)

### RAN2 #131bis (Oct-2025)

#### User plane

Agreements on UP

3.	6G user plane should aim to reduce the radio and end-to-end latency for general services (including eMBB)

4.	Study potential benefits and standardized mechanisms for applications/service-awareness in the RAN (e.g. immersive communications, AI mobile traffic).  Understand the issues and shortcoming with the current NR QoS.   Aim to look at a general framework.

5.	Support at least the following scheduling schemes: dynamic grant and configured grant.  Further study configured grant.

#### Control plane

Agreements

Continue study

-	Idle mode –

-	Connected mode –

-	Inactive mode or sub-state: Should support at least on these functions: UE based mobility (e.g. cell selection/reselection), UE context and identification and energy efficient for both UE and NW.  Further study fast transitions, data transfer, further improvements to UE identification, and RNA management and paging/wake-up simplifications/enhancements.   We can consider the pros/cons of different modelling, after having some discussions on the functionality.

Agreements on Spectrum aggregation

- 1	Study spectrum aggregation of multiple pieces of spectrum and understand the issues that need to be addressed.

- 2	Study how to use UL and DL spectrum more efficiently (e.g. decoupling of UL and DL, etc).  Understand the deployment scenarios and problems to address

Agreements on System Information

- -	System information design should consider energy efficiency and low complexity for both network and UE.

-	System information design should ensure that the public warning requirements are met.

-	Study RAN2 aspects of system information designs (e.g. on-demand SIB and SSB, periodicity of SSBs)

-	Study how to improve flexibility and extensibility of scheduling SI design

-	Study mechanisms to improve SI update mechanism

-	Study aspects related to SIB1 size.  Understand the issues and desired content/size from RAN2 point of view.

-	Study areas specific SIB design.  Understand 5G pain points and what improvements can be considered.

NOTE: this is a starting list

Agreements

Paging design should consider energy efficiency and simplicity for both UE and Network.

#### Common User plane and Control plane

Agreements 2. Study further requirements of other non-AI/ML use cases/services and understand if we can have more common requirements across different use cases.

Network and UE Energy Efficiency

Design goal

1. 	Strive to develop solutions that consider network energy efficiency and UE power saving

## Mobility

### RAN2 #132 (Nov-2025)

Mobility framework/targets:

Agreements on mobility

1.	Study at least the following mobility schemes

- NW configures and triggers the mobility without pre-configuration, and UE performs it immediately

- Pre-configured solutions and early UE configuration processing

- UE triggered mobility based on some pre-configuration

- CFRA, RACH-less, early UL/DL sync, early CSI.

Schemes should consider the lower interruption, robustness requirements and throughput degradation while considering UE and NW complexity and resource efficiency.

As first step, understand shortcomings/problems from 5G and how the schemes/solutions proposed can address the issues/requirements for 6G.   Aim to reduce the number of schemes required for 6G and consider how to unify after some initial discussions on the solutions

2.	DAPS as in NR is not considered in this study item

IDLE/INACTIVE mobility:

- Agreements

- NR cell (re) selection framework (e.g best cell principle, cell ranking, inter-freq/RAT prioritization, etc)  is the baseline of 6G cell reselection framework.

- Understand the issues we want to address in 6G.   Study further how other 6G related aspects with energy saving, spectrum aggregation, service awareness, features, deployment type (e.g. TN, NTN), slicing, speed dependent etc impact cell selection/reselection, etc.

- These discussions will resume once we have some progress/understanding from other WGs on cells, reference signals, measurement framework/requirements etc.

Inter-RAT mobility:

Agreements

1	For inter-RAT mobility (5G and 6G), the following is supported as a starting point:

­	For RRC_CONNECTED state, both handover and re-direction can be supported for inter-RAT mobility

­	Inter-RAT cell reselection information can be provided to UE.

2	Wait for further mobility discussions on the intra-RAT solutions, before coming back to look at the need for further enhancements.  Consider MRSS.

### RAN2 #131bis (Oct-2025)

Agreements

1	Study mobility with the following requirements in 6G mobility design:

-	Minimize interruption time and ensure service continuity (i.e. minimize throughput degradation during mobility). Consider complexity and gains when discussing solutions.

-	Robustness of mobility procedures

-	Energy efficiency for both UE and NW

2	Study aspects related to mobility (e.g. early DL/UL synchronization, UE configuration processing, pre-configurations, conditional handover, early CSI acquisition)