3GPP TR 36.811 V1.2.0 (2011-02)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

Evolved Universal Terrestrial Radio Access (E-UTRA); 
Add 2 GHz band LTE for ATC of MSS in North America Work Item Technical Report

(Release 10)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

LTE, radio

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2010, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	6

2	References	6

3	Definitions, symbols and abbreviations	6

3.1	Definitions	6

3.2	Symbols	6

3.3	Abbreviations	6

4	Background	7

4.1	Task description	7

4.2	Regulatory background	7

4.3	MSS/ATC Bandwidth Deployment Scenarios	10

5	Study of E-UTRA requirements	12

5.1	Operating bands and channel arrangement	12

5.1.1	Frequency bands	12

5.1.2	Channel bandwidths per operating band	13

5.1.3	Carrier frequency and EARFCN	13

5.1.4	TX–RX frequency separation	13

5.2	Specific UE RF requirements	14

5.2.1	Dual duplex spacing	14

5.2.1.1	Signaling support of variable duplex spacing	14

5.2.1.2	RF architecture	15

5.2.1.3	Impacts to radio transmission and reception requirements	15

5.2.1.3.1	Self interference and desense	16

5.2.1.3.2	Receiver requirements	16

5.2.1.3.3	Transmitter requirements	16

5.2.1.3.4	Specification differences between one band and two bands approach	17

5.2.2	UE Maximum Output Power	17

5.2.3	UE Maximum Output Power with additional requirements	18

5.2.3.1 A-MPR Study for frequency range of 2000 – 2010 MHz	18

5.2.4	Output RF spectrum emissions	23

5.2.4.1	Additional Spectrum Emission Mask	25

5.2.4.1.1	Minimum requirement (network signalled value "NS_03")	25

5.2.4.2	Spurious Emissions	26

5.2.4.2.1	 Spurious emission band UE co-existence	26

5.2.5	Receiver characteristics	26

5.2.5.1	 Reference sensitivity power level	27

5.2.5.1.1	 Minimum requirements (QPSK)	27

5.2.5.2	 Requirement for large transmission configurations	27

5.2.6	 Blocking characteristics	27

5.2.6.1	 In-band blocking	27

5.2.6.1.1	 Minimum requirements	27

5.2.6.2	 Out-of-band blocking	28

5.2.6.2.1	 Minimum requirements	28

5.3	Specific BS RF requirements	29

5.3.1	Operating band unwanted emissions	29

5.3.2	Additional spurious emissions requirements	31

5.3.3	Co-location with other base stations	32

5.3.4	General blocking requirement	32

5.3.5	Blocking requirement for co-location with other base stations	32

6	Summary of required changes to E-UTRA specifications	33

6.1	 Required changes to TS 36.101	33

6.2	 Required changes to TS 36.104	33

6.3	 Required changes to TS 36.113	34

6.4	 Required changes to TS 36.124	34

6.5	 Required changes to TS 36.133	34

6.6	 Required changes to TS 36.141	35

6.7	 Required changes to TS 25.461	36

6.8	 Required changes to TS 25.466	36

6.9	 Required changes to TS 36.307 Version 8.1.0	36

6.10	 Required changes to TS 36.307 Version 9.1.1	36

6.11	 Required changes to TS 25.101	37

6.12	 Required changes to TS 25.104	37

6.13	 Required changes to TS 25.141	37

6.14	 Required changes to TS 37.104	38

6.15	 Required changes to TS 37.141	38

7	Project plan	39

8	Open issues	39

Annex A (Informative): Change history	39

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document is a technical report for Adding 2 GHz LTE for ATC of MSS in North America work item, which was approved at 3GPP TSG RAN#47 [2]. The objective of this work item is to add the 2 GHz band LTE for ATC of MSS in North America to the appropriate 3GPP core specifications for LTE FDD networks.  In addition to the schedule and status of the work items, the report includes a description of the motivation, requirements, study results and specification recommendations.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]      3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	     RP-100376 “Work Item proposal: Adding 2 GHz band LTE for ATC of MSS in North America”, RAN#47

# 3	Definitions, symbols and abbreviations

Delete from the above heading those words which are not applicable.

Clause numbering depends on applicability and should be renumbered accordingly.

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [x] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [x].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

UE	User Equipment

BS	Base station

UL	Uplink (Reverse Link)

DL	Downlink (Forward Link)

# 4	Background

This work item, targeted for the Rel-10 timeframe, proposes adding the 2 GHz band LTE for ATC of MSS in North America to the 3GPP specifications for the above purposes.  The effort involves developing the minimum RF characteristics and performance requirements for terrestrial FDD E-UTRA (LTE) networks.

In North America (US and Canada), the FCC and Industry Canada have studied and approved rules granting authority to operator(s) to integrate Ancillary Terrestrial Component (ATC) into their Mobile Satellite Services (MSS) network(s).

## 4.1	Task description

The purpose of this band class standardization is to enable the operators to offer enhanced mobile services through the  terrestrial component with the spectrum resources allocated and authorized for MSS and ATC in this band.

The FCC has authorized two operators to deploy ATC networks in the S-band (Terrestar and DBSD North America, formerly ICO Global Communications) per the following authorizations:

- FCC  DA 10-60 Terrestar : 2000 – 2010 MHz Uplink, 2190-2200 MHz Downlink

- FCC  DA 09-38 DBSD: 2010 – 2020 MHz Uplink, 2180-2190 MHz Downlink

Based on the band allocation, there are two Tx-Rx frequency separation scenarios envisioned for this band:  170 and 190 MHz.

The ATC equipment will adhere to all 3GPP and FCC emission masks to protect adjacent band services.  It is expected that during the work item progress the impacts will be studied.

## 4.2	Regulatory background

In 2003, the FCC and Industry Canada have approved rules permitting Mobile Satellite Services (MSS) and Ancillary Terrestrial Component (ATC) operations in assigned 2 GHz frequency bands.  In the US, these rules have been legislated in Part 25 of the 47 Code of Federal Regulations, allowing ATC network operations to coexist with neighboring bands and services.  The arrangement of the band is shown below:

Two operators have been authorized in the band (Terrestar and DBSD North America), 10+10 MHz each:

  - Terrestar : 2000 – 2010 MHz Uplink, 2190-2200 MHz Downlink (Authorization provided by FCC Docket  DA 10-60)

  - DBSD: 2010 – 2020 MHz Uplink, 2180-2190 MHz Downlink (Authorization provided by FCC Docket DA 09-38)

As illustrated the allocation for the two operators demands two different duplex spacing of 170 MHz and 190 MHz. It is proposed that this would be only defined as one band and not split into two bands in order to keep the total number of bands to a minimum and because there is such a high degree of commonality and synergy between the two.  It is anticipated that in defining the performance requirements, the two duplex spacings will not yield separate specification values due to the large separation.  But in the case that they do, the proposed methodology is to specify the worst case requirement between the two as a single requirement for the band.  Such findings will be captured in the technical report.

The out of band emission (OOBE) requirements for this band are illustrated as follows.

Node B OOBE

ATC base stations authorized herein may transmit with power spectral density levels up to 32 dBW/MHz EIRP.  ATC base stations authorized herein must attenuate out-of-band emissions by a factor of (43 + 10*log(P) dB) below the transmitter power (P) on frequencies outside the 2180-2200 MHz band.

There also exists an Operator-to-Operator Agreement between S-band operator and United States Federal Government Agencies Operating Earth Stations in the 2200-2290 MHz Band, and the EIRP spectral density of out-of-band emissions from any base station authorized herein that is situated within 133 kilometers of an existing Federal Government earth station operating in the 2200-2290 MHz band shall not exceed -100.6 dBW/4kHz in any portion of  that band.

ATC base stations also need to prescribe limits on the radiated power and power spectral density of emissions in the 1559-1610 MHz band from 2 GHz ATC base stations and mobile terminals.

|  | Frequency | dBW/MHz EIRP | dBW EIRP of discrete emissions of less than 700 Hz bandwidth |
| --- | --- | --- | --- |
| Base Stations | 1559-1610 MHz | -100 | -110 |

For a wideband carrier, the EIRP level is to be measured using a root mean square (RMS) detector function with a resolution bandwidth of 1 MHz or equivalent and the video bandwidth is not less than the resolution bandwidth.  For a narrowband carrier, the EIRP level is to be measured using an RMS detector function with a resolution bandwidth of 1 kHz or equivalent.  The measurements are to be made over a 20 millisecond averaging period when the base station is transmitting.

The eNB OOBE requires -13 dBm/MHz at the edge of the band, hence it will be using the specifications  defined in 3GPP TS 36.104.   This OOBE is illustrated in the figure below.  The additional requirement for eNB for protection of the Federal Government Earth stations will be met through an external filter and will not be specified as part of the 3GPP specifications.

The measurement instruments with a resolution bandwidth of one megahertz or more shall be used to verify compliance with the emission limits Compliance with this rule is based on the use of measurement instrumentation employing a resolution bandwidth of 1 megahertz or greater.  However, in the 1 megahertz bands immediately outside and adjacent to the frequency block a resolution bandwidth of at least one percent of the emission bandwidth of the fundamental emission of the transmitter may be employed.  A narrower resolution bandwidth is permitted in all cases to improve measurement accuracy provided the measured power is integrated over the full required measurement bandwidth (i.e., 1 megahertz or 1 percent of emission bandwidth, as specified).  The emission bandwidth is defined as the width of the signal between two points, one below the carrier center frequency and one above the carrier center frequency, outside of which all emissions are attenuated at least 26 dB below the transmitter power.

UE OOBE

The EIRP spectral density of ATC terminal transmissions in transmission bandwidths less than 1.23 megahertz shall not exceed -60 dBW/Hz.

Emissions on frequencies lower than 1995 MHz shall be attenuated by at least 70 + 10 log P.  Emissions in 1995-2000 MHz shall be attenuated by at least a value as determined by linear interpolation from 70 + 10 log P at 1995 MHz, to 43 + 10 log P dB at the nearest MSS band edge at 2000 MHz or 2020 MHz respectively.

ATC terminals also need to prescribe limits on the radiated power and power spectral density of emissions in the 1559-1610 MHz band from 2 GHz ATC base stations and mobile terminals.

|  | Frequency | dBW/MHz EIRP | dBW EIRP of discrete emissions of less than 700 Hz bandwidth |
| --- | --- | --- | --- |
| Mobile Terminals | 1559-1610 MHz | -95 | -105 |

The figure below illustrates the UE OOBE.

The UE OOBE requires -13 dBm/MHz at the edge of the band, hence it will be specified similarly to  section 6.6.2 of TS 36.101 at the edge of the band to meet this requirement.  There is an additional -40 dBm/MHz protection at 5 MHz away, only at the lower part of the band which needs to be met to protect PCS operations.  Detailed requirements to comply with this mask will be included in the technical report.

S-Band UE will meet the 3GPP spurious emission requirements for UE-to-UE co-existence per 3GPP TS 36.101. Spurious emission requirements from the addition of this new band are expected to be the same as those required from the addition of other new US bands in 3GPP.

The measurement instruments with a resolution bandwidth of one megahertz or more shall be used to verify compliance with the emission limits Compliance with this rule is based on the use of measurement instrumentation employing a resolution bandwidth of 1 megahertz or greater.  However, in the 1 megahertz bands immediately outside and adjacent to the frequency block a resolution bandwidth of at least one percent of the emission bandwidth of the fundamental emission of the transmitter may be employed.  A narrower resolution bandwidth is permitted in all cases to improve measurement accuracy provided the measured power is integrated over the full required measurement bandwidth (i.e., 1 megahertz or 1 percent of emission bandwidth, as specified).  The emission bandwidth is defined as the width of the signal between two points, one below the carrier center frequency and one above the carrier center frequency, outside of which all emissions are attenuated at least 26 dB below the transmitter power.

## 4.3	MSS/ATC Bandwidth Deployment Scenarios

The following diagram illustrates the S-band Mobile Satellite Services with an Ancillary Terrestrial Component (MSS/ATC) spectrum:

Two operators have been authorized in the band (Terrestar and DBSD North America), 10+10 MHz each:

  - Terrestar : 2000 – 2010 MHz Uplink, 2190-2200 MHz Downlink

  - DBSD: 2010 – 2020 MHz Uplink, 2180-2190 MHz Downlink

MSS and ATC services are meant to coexist in this band and could be deployed differently in each market.  The services also need to be offered with respect to regulatory requirements and therefore carrier placements are going to be such to reduce potential interference.  The following scenarios are envisioned for these services.  Please note that even though the figures show a specific 10 MHz spectral range (2180 -2190 MHz), it is for illustration purposes and the same deployment could be in 2190-2200 MHz. Also the notations of forward and reverse links in these figures signify that the downlink and uplink will be deployed in a symmetrical manner of bandwidth allocation.

Scenario 1: Maximize Terrestrial Capacity: One 10 MHz LTE Carrier

This case doesn’t require illustration.

Scenario 2: Split Capacity

One 5 MHz LTE Carrier, 5 MHz Satellite Carrier Allocation

One 5 MHz LTE Carrier, One 3 MHz LTE Carrier, 2 MHZ Satellite Allocation

Scenario 3: Maximize Satellite Capacity

One 1.4 MHz LTE Carrier, 8.6 MHz Satellite Allocation

Scenario 4: Interference mitigation scenario

Reuse of three 3 MHz LTE Carriers, 1 MHz Satellite Allocation

# 5	Study of E-UTRA requirements

## 5.1	Operating bands and channel arrangement

<Text to be added>

### Two operators, namely TerreStar and DBSD North America, have been authorized by the FCC to deploy networks in this band. Each of them is assigned a 10 MHz pair:	13

### •	Terrestar block  : 2000 – 2010 MHz Uplink, 2190-2200 MHz Downlink (Authorization provided by FCC Docket  DA 10-60)	13

### •	DBSD block: 2010 – 2020 MHz Uplink, 2180-2190 MHz Downlink (Authorization provided by FCC Docket DA 09-38)	13

### The two company’s spectrum blocks are adjacent to each other forming an “inner block” and an “outer block,” but have different duplex spacings: 190 MHz for the Terrestar block and 170 MHz for the DBSD block.  Thus, defining this as a single band necessitates that two duplex spacings be supported.	13

### 5.1.1	Frequency bands

The S-band is being specified as a single operating band supported by a single duplexer, a default duplex spacing of 180 MHz which allows a variability of ±10 MHz. This would cover the two operator allocations defined above and is defined in Table 5.2-1:

Table 5.2-1: E-UTRA frequency bands

| E-UTRA Operating Band |  | Uplink (UL) operating band BS receive UE transmit |  |  |  | Downlink (DL) operating band BS transmit UE receive |  |  |  | Duplex Mode |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FUL_low – FUL_high |  |  |  | FDL_low – FDL_high |  |  |  |  |  |
| [23] |  | 2000 MHz | – | 2020 MHz |  | 2180 MHz | – | 2200 MHz |  | V-FDD (Note 1) |  |
| Note 1: The duplex spacing is defined in Tables 5.2-4 and 5.2-5 |  |  |  |  |  |  |  |  |  |  |  |

### 5.1.2	Channel bandwidths per operating band

The transmission bandwidth configuration in Table 5.2-2 shall be supported for each of the specified channel bandwidths.

Table 5.2-2: E-UTRA channel bandwidth

| E-UTRA band / channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| [23] | Yes | Yes | Yes | Yes |  |  |

### 5.1.3	Carrier frequency and EARFCN

The relation between EARFCN and the carrier frequency in MHz for the downlink is given by the following equation, where FDL_low and NOffs-DL are given in Table 5.2-3 and NDL is the downlink EARFCN.

FDL = FDL_low + 0.1(NDL – NOffs-DL)

The relation between EARFCN and the carrier frequency in MHz for the uplink is given by the following equation where FUL_low and NOffs-UL are given in Table 5.2-3 and NUL is the uplink EARFCN.

FUL = FUL_low + 0.1(NUL – NOffs-UL)

Table 5.2-3 E-UTRA channel numbers

| E-UTRA Operating Band | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | FDL_low (MHz) | NOffs-DL | Range of NDL | FUL_low (MHz) | NOffs-UL | Range of NUL |
| [23] | 2180 | 7500 | 7500 – 7699 | 2000 | 25500 | 25500 – 25699 |

### 5.1.4	TX–RX frequency separation

The default E-UTRA TX channel (carrier centre frequency) to RX channel (carrier centre frequency) separation is specified in Table 5.2-4 for the TX and RX channel bandwidths defined in Table 5.2-2.

Table 5.2-4: Default UE TX-RX frequency separation

| E-UTRA Operating Band | TX - RX carrier centre frequency separation |
| --- | --- |
| [23] | 180 MHz |

The range E-UTRA TX channel (carrier centre frequency) to RX channel (carrier centre frequency) separations for operating bands supporting variable duplex FDD is specified in Table 5.2-5

Table 5.2-5: TX-RX frequency separation for operating bands supporting variable duplex FDD

| E-UTRA Operating Band | TX - RX carrier centre frequency separation |  |
| --- | --- | --- |
|  | Allowed offset | Separation |
| [23] | -10 MHz | 170 MHz |
|  | +10 MHz | 190 MHz |

## 5.2	Specific UE RF requirements

<Text to be added>

### 5.2.1	Dual duplex spacing

Two operators, namely TerreStar and DBSD North America, have been authorized by the FCC to deploy networks in this band. Each of them is assigned a 10 MHz pair:

- Terrestar block  : 2000 – 2010 MHz Uplink, 2190-2200 MHz Downlink (Authorization provided by FCC Docket  DA 10-60)

- DBSD block: 2010 – 2020 MHz Uplink, 2180-2190 MHz Downlink (Authorization provided by FCC Docket DA 09-38)

The two company’s spectrum blocks are adjacent to each other forming an “inner block” and an “outer block,” but have different duplex spacings: 190 MHz for the Terrestar block and 170 MHz for the DBSD block.  Thus, defining this as a single band necessitates that two duplex spacings be supported.

#### 5.2.1.1	Signaling support of variable duplex spacing

The signaling support for variable duplex spacing is already available in the current Release 8 LTE specifications.  In TS 36.331, as shown below, the SystemInformationBlockType2  (SIB2) includes the ul-CarrierFreq  field.  For FDD systems, if this field is absent, the value determined from the default TX-RX frequency separation defined in TS 36.101 applies.  To enable variable duplex spacing in a band,  the UL carrier frequency can be explicitly defined.

SystemInformationBlockType2 information element

-- ASN1START

SystemInformationBlockType2 ::=      SEQUENCE {

ac-BarringInfo                   SEQUENCE {

ac-BarringForEmergency               BOOLEAN,

ac-BarringForMO-Signalling           AC-BarringConfig             OPTIONAL,  -- Need OP

ac-BarringForMO-Data             AC-BarringConfig             OPTIONAL   -- Need OP

}                                                                 OPTIONAL,  -- Need OP

radioResourceConfigCommon        RadioResourceConfigCommonSIB,

ue-TimersAndConstants            UE-TimersAndConstants,

freqInfo                         SEQUENCE {

ul-CarrierFreq                   ARFCN-ValueEUTRA             OPTIONAL,  -- Need OP

ul-Bandwidth                     ENUMERATED {n6, n15, n25, n50, n75, n100}

OPTIONAL,  -- Need OP

additionalSpectrumEmission           AdditionalSpectrumEmission

},

mbsfn-SubframeConfigList         MBSFN-SubframeConfigList         OPTIONAL, -- Need OR

timeAlignmentTimerCommon         TimeAlignmentTimer,

...

}

AC-BarringConfig ::=             SEQUENCE {

ac-BarringFactor                 ENUMERATED {

p00, p05, p10, p15, p20, p25, p30, p40,

p50, p60, p70, p75, p80, p85, p90, p95},

ac-BarringTime                   ENUMERATED {s4, s8, s16, s32, s64, s128, s256, s512},

ac-BarringForSpecialAC               BIT STRING (SIZE(5))

}

MBSFN-SubframeConfigList ::=     SEQUENCE (SIZE (1..maxMBSFN-Allocations)) OF MBSFN-SubframeConfig

MBSFN-SubframeConfig ::=         SEQUENCE {

radioframeAllocationPeriod           ENUMERATED {n1, n2, n4, n8, n16, n32},

radioframeAllocationOffset           INTEGER (0..7),

subframeAllocation               CHOICE {

oneFrame                         BIT STRING (SIZE(6)),

fourFrames                       BIT STRING (SIZE(24))

}

}

-- ASN1STOP

#### 5.2.1.2	RF architecture

Although the proposed 2GHz band has two duplex spacings, a terminal implementation is not necessarily required to use two duplexers to support this band.   As an example, a possible UE radio block implementation as shown in Figure 1 below where the same antenna,  duplexer, transceiver, PA, and baseband sections are used regardless of the duplex or frequency of operation within the band. Here, it is assumed that the RF components are sufficiently wideband to support the aggregate bandwidth of the two blocks.  For example, the duplexer is required to have a passband of at least 20MHz and a duplex spacing of 180 MHz.  Operating with different duplex spacings can be managed in software to provide tuning control for the Rx and Tx LO’s.  Frequency tracking is always maintained on the uplink so that the UE is synchronized with the basestation no matter the duplex spacing.  The limitation with this architecture is that only a single duplex spacing can be supported at any given instant in time; however, a variable duplex band only operates with a single duplex spacing until it is reconfigured by SIB messaging.

Figure 1.  Example radio architecture supporting variable duplex spacing in US 2GHz band.

#### 5.2.1.3	Impacts to radio transmission and reception requirements

To help understand the impact to radio requirements of dual duplex spacing in this band, it may be convenient to compare this to the conventional case of single duplex spacing.  Consider as shown in Figure 2a, the proposed spectrum allocation of the 2GHz S-band with dual duplex spacing. The outer block (assigned to Terrestar) has 190 MHz duplex spacing while the inner block (assigned to DBSD) has 170 MHz duplex spacing. For comparison, also consider as shown in Figure 2b, a fictitious allocation of the same band with single 180 MHz duplex spacing.   In both of these two cases, the duplex spacing is at least 17 times the maximum channel bandwidth (10 MHz).

Figure 2.  Comparison of the dual duplex spacing S-band with a fictictious single duplex spacing band.

##### 5.2.1.3.1	Self interference and desense

On important distinction between treating the band with two duplex spacing vs. single duplex spacing is the self interference due to transmitter PA noise and intermodulation products extending into the receive band.  In the case of this 2GHz band, however, the spacing from Tx to Rx is very large so that the self-interference is expected to be negligible.  To evaluate the self  interference at such large duplex spacing, a measurement of PA noise was made using a commercially available Band 1 PA.  Band 1 is close in frequency to S-band and shares a similar large duplex spacing of 190 MHz; therefore, its performance should be similar to that expected of S-band.  With 28dBm output power, the Band 1 PA noise density reaches its floor and is flat at the receiver band with a measured power level of  -141 dBm/Hz. Factoring in the  45 dB attenuation from the duplexer, the expected self interference at the receiver input is -186 dBm/Hz, which is much lower than the receiver thermal noise floor. Therefore, for the 2 GHz S-band in interest, the impact from self interference is negligible.  Furthermore, because of the large separation between Tx and Rx, intermodulation terms from the transmitter are also expected to be negligible.

##### 5.2.1.3.2	Receiver requirements

We now consider the reception requirements (i.e., reference sensitivity, adjacent channel selectivity, blocking, intermodulation, etc).  Given the above conclusion that self-interference is negligible for this band, the reception requirements are mainly driven by the performance of receiver chain and interference from neighboring bands, which depends only on the location of the DL block. Comparing the diagrams of Figure 2, it can be seen that there is no difference between the receiver requirements for the block defined by the inner block (i.e., DBSD DL block) and a block X DL if the band were defined with single duplex.  Similarly, the outer block (i.e., Terrestar DL block) is identical to the block Y DL block in terms of receiver requirements.

##### 5.2.1.3.3	Transmitter requirements

The transmitter emissions (i.e., ACLR, out of band, spurious, etc) are mainly driven by requirements limiting interference to other bands and within its own band.  We have already established that self-interference is not a consideration for this 2 GHz S-band.  Interference to other bands is a function of the location of the UL block, the channel bandwidth, and the size of the uplink allocation.  Comparing the diagrams of Figure 2, it can be seen that there is no difference between the transmitter requirements for the block defined by the inner block (i.e., DBSD UL block) and a block Y UL if the band were defined with single duplex.  Similarly, the outer block (i.e., Terrestar UL block) is identical to the block X UL block in terms of transmitter requirements.

##### 5.2.1.3.4	Specification differences between one band and two bands approach

The analysis and reasoning in the previous section shows that, in general, there is no difference in the specification between defining the S-band as a single band with dual duplex spacing compared to defining it as a single band with fictitious single duplex spacing. .  The large duplex separation makes self interference negligible for this band so that the actual separation, whether it be 170 MHz or 190 MHz does not affect the performance requirements.  In the following table  all UE RF requirements in 36.101 that may potentially need changes for this 2 GHz S-band are listed.  A brief discussion is also provided to compare the difference between one-band with dual duplex spacing solution versus two-band solution. In general, there should be no difference in requirements between defining the band as a single band vs. defining the band as two bands.  Where there are possible differences, the worst case specification is proposed to be used when defining as a single band.  Detailed analysis as appropriate will be provided in future contributions to support any requirement changes needed.

| Clause | Description | Comments and Comparison to 2-band Solution |
| --- | --- | --- |
| 6.2.2 | UE Maximum Output Power | No changes are needed for a one-band vs. two-band solution. Due to the large duplex gap, maximum output power relaxation is not required at the band edges. |
| 6.2.4 | UE Maximum Output Power with additional requirements (A-MPR) | FFS. Will determine from emission analysis whether A-MPR is needed. For a two-band solution, the analysis is done separately on the inner block and the outer block. For one-band with dual duplex spacing solution, the higher A-MPR number from the two blocks will be used. |
| 6.6.2.2 | Additional Spectrum Emission Mask | FFS. Will determine if any additional spectrum emission mask is needed. For a two-band solution, the analysis is done separately on the inner block and the outer block. For one-band with dual duplex spacing solution, the tighter emission limit from the two blocks will be used. |
| 6.6.3.2 | Spurious emission band UE co-existence | FFS. Will determine later if any changes needed. For the two-band solution, the analysis is done separately for the Terrestar band and DBSD band. For one-band with dual duplex spacing solution, the worst case number from the two bands will be used. |
| 7.3.1 | Reference sensitivity requirement | No differences are expected between a one-band vs. two-band solution. The self interference is negligible for this band due to the large duplex spacing. The location of the uplink RB’s is to be allocated as close as possible to the downlink operating band within the channel bandwidth and in accordance with the duplex spacing as indicated in the SIB message. |
| 7.5, 7.6, 7.7, and 7.8 | ACS, blocking, spurious, and intermod | FFS. ACS, blocking, intermod, and other receiver requirements are defined as a function of the channel bandwidth and are not a function of the duplex separation. As with reference sensitivity, the uplink RB’s are to be allocated as close as possible to the downlink operating band within the channel bandwidth and in accordance with the duplex spacing as indicated in the SIB message. No differences are expected between a one-band vs. two band solution. |

Table 1 UE RF requirements that may require changes for the 2GHz S-Band.

### 5.2.2	UE Maximum Output Power

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth. The period of measurement shall be at least one sub frame (1ms).

# Table 5.2.2-1: UE Power Class

| EUTRA band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [23] |  |  |  |  | 23 | ±2 |  |  |

### 5.2.3	UE Maximum Output Power with additional requirements

A-MPR is needed to meet additional FCC emission requirements and possible UE coexistence requirements.  Detailed number will be determined in the future study.

Table 5.2.3-1: Additional Maximum Power Reduction (A-MPR) / Spectrum Emission requirements

| Network Signalling value | Requirements (sub-clause) | E-UTRA Band | Channel bandwidth (MHz) | Resources Blocks | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_03 | 6.6.2.2.1 | 2, 4, 10, [23], 35, 36 | 3 | >5 | ≤ 1 |
|  |  |  | 5 | >6 | ≤ 1 |
|  |  |  | 10 | >6 | ≤ 1 |
|  |  |  | 15 | >8 | ≤1 |
|  |  |  | 20 | >10 | ≤1 |
| [NS_11] | TBD | [23]1 | 1.4, 3, 5, 10 | Table 5.2.3-2 | Table 5.2.3-2 |
| Note 1 Applies to the lower block of Band [23], i.e. a carrier placed in the 2000-2010 MHz region. |  |  |  |  |  |

Table 5.2.3-2: A-MPR for NS_11

| Channel Bandwidth | Parameters |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 3 | Fc (MHz) | <2004 | ≥2004 |  |  |
|  | L_CRB (RBs) | 1-15 | >5 |  |  |
|  | A-MPR | ≤5 | ≤ 1 |  |  |
| 5 | Fc (MHz) | <2004 | 2004 ≤ Fc <2007 |  | ≥2007 |
|  | L_CRB (RBs) | 1-25 | 1-6 & 15-25 | 8-12 | >6 |
|  | A-MPR | ≤7 | ≤ 4 | 0 | ≤ 1 |
| 10 | Fc (MHz) | 2005 |  |  |  |
|  | RB_start (RBs) | 0-49 |  |  |  |
|  | L_CRB (RBs) | 1-50 |  |  |  |
|  | A-MPR | ≤ 12 |  |  |  |

### 5.2.3.1 A-MPR Study for frequency range of 2000 – 2010 MHz

Based on the assumptions shown below this chapter presents results of a study which aims to define appropriate A-MPR for frequency range of 2000 – 2010 MHz belonging to Band 23. The final MPR values are subject to change based on RAN4 determining the coexistence values for both band [23] and band 2.

Figure 5.2.3.1-1 UL frequency arrangement of Band 23

Emissions requirements and simulation assumptions

Following emission requirements have been taken into account in this contribution.

- NS_03 Mask

- H-Block requirement as specified in [1]

- G-Block – 40 dBm / 1 MHz (This emission value was not agreed during the time the study was made)

- Band 2 (PCS) – 50 dBm / 1 MHz (This emission value was not agreed during the time the study was made)

Following simulation assumptions have been used in this contribution

- PA Operating point UTRAACLR1 = 33 dBc

- Carrier leakage = -25 dBc

- IQ Image = - 25 dBc

- Counter-IM3 = - 60 dBc

A-MPR needed for 1.4 MHz bandwidth

Simulations showed that when 1.4 MHz carrier is placed on worst case position Fc=2000.7 MHz the margin with full allocation to H-block requirement is 2 dB. Keeping in mind the fact that PA and modulator were driven with minimum requirements this margin could be satisfactory hence no A-MPR is required.

Figure 5.2.3.1-2 Spectrum plots from 1.4 MHz BW

A-MPR needed for 3 MHz bandwidth

Following table presents required A-MPR needed for 3 MHz carrier to meet the emission requirement listed previously in this chapter.

RB_Start for all cases is 0.

Table 5.2.3.1-1 Required A-MPR for 3 MHz carrier in lower 10 MHz frequency block of band 23

|  | #RB |  |  |  |
| --- | --- | --- | --- | --- |
| Fc | 15 | 10 | 5 | 1 |
| 2001.5 | 4.71 | 2.77 | 1.37 | 3.56 |
| 2002 | 3.22 | 1 | 1 | 1.84 |
| 2002.5 | 2.53 |  |  | 1.86 |
| 2003 | 1.32 |  |  | 1 |
| 2003.5 | 1 |  |  |  |

Following table captures the A-MPR requirement based on studies presented above in similar format that has been used in 36.101. Last column represents the NS_03 requirement.

Table 5.2.3.1- 2 A-MPR table for 3 MHz signal

A-MPR needed for 5 MHz bandwidth

Following table presents required A-MPR needed for 5 MHz carrier to meet the emission requirement listed previously in this chapter.

RB_Start for all cases is 0.

Table 5.2.3.1-3 Required A-MPR for 5 MHz carrier in lower 10 MHz frequency block of band 23

Table 1 is divided into four sections by colouring those with different colours. These sections map to A-MPR table presented below so that one section corresponds one A-MPR value. A-MPR value for Fc >=2007 is the NS_03 requirement and hence not derived from table 1.

Following table captures the A-MPR requirement based on studies presented above in similar format that has been used in 36.101. Last column represents the NS_03 requirement.

Table 5.2.3.1-4 A-MPR table for 5 MHz signal

A-MPR needed for 10 MHz bandwidth

Following table presents required A-MPR needed for 10 MHz carrier to meet the emission requirement listed previously in this chapter.

Fc for all cases is 2005 MHz.

Table 5.2.3.1-5 Required A-MPR for 5 MHz carrier in lower 10 MHz frequency block of band 23

The simulated A-MPR values can presented as a table 5.2.3.1- 6 shown below. Table 5 is divided into three sections by colouring those with different colours. These sections map to A-MPR table presented below so that one section corresponds one RB_Start region.

Table 5.2.3.1- 6 A-MPR table for 10 MHz carrier

Summary

Next we combine results from previous chapters into a single A-MPR table see below.

Table5.2.3.1- 7 NS_12 A-MPR table for band 23

In figure 5.2.3.1-3 below we have illustrated some possible carrier arrangements that could be done based on the proposed A-MPR table above.

Figure 5.2.3.1-3 Posible channel arrengements for lower block of band 23

To summarize following observations can be made from A-MPR table 7 and figure 3.

- For 1.4 MHz no A-MPR is needed hence can be used freely but because of large control signal overhead this is not attractive bandwidth but could be used in combination with other channel bandwidths

- If frequency range is filled up with 3 MHz carriers then only the lowest one required A-MPR also there will be room for 1 MHz guard band against H-block. Higher block requires the A-MPR indicated by the NS_03 requirement.

- If 5 MHz carriers are used then the lower carrier requires A-MPR but higher one does not require any additional A-MPR in addition to NS_03 requirement.

- If 10 MHz carrier is used a over provisioned PUCCH must be used like with band 13. Then RB’s locatred at RB_start =15 – 35 do not require A-MPR but the PUCCH carrier must be located into this region which reduced the amount of contiguous RB’s for PUSCH. Maximum contiguous allocation for PUCCH is approximately 15.

- If a combination of 1.4, 3 and 5 MHz carriers is used those can be placed such way that no A-MPR is required to any of the carriers in addition to the NS_03 A-MPR. In addition there will be room for 600 kHz guard band against G-Block.

- If satellite carriers are also placed into the band there will be several possibilities to have feasible LTE channel arrangements.

### 5.2.4	Output RF spectrum emissions

The following figures overlay the FCC-S-Band emission requirements with LTE OOBE additional emission mask for NS_03.  It is always assumed in these figures that the LTE channel is located at the edge of the Lower Block or Upper Block.  The unit of emission is dBm/MHz. It should be noted that the FCC requirement within the first 1 MHz outside of the channel edge is not drawn in the figures. According to the FCC ruling on S-band, within the the first 1 MHz immediately outside of channel edge, the maximum emission power level is -13 dBm and the measurement bandwidth used can be 1% of the emission bandwidth. This conforms with definition of general OOBE emission mask for the first 1 MHz.

From the figures below, it was found that the FCC has more stringent emission requirement than that of LTE OOBE  in the frequency range from 1995 to 2000 MHz.  We prefer to address this emission requirement difference in UE Co-Existence requirements instead of adding a new OOBE mask for S-band due to the following reasons. First, the strict FCC emission requirement of -40dBm/MHz at 1995 MHz frequency range is intended to protect G-block so this is an UE coexistence issue.  Second,  LTE OOBE is defined with respect to the LTE channel instead of operation band, while the FCC emission requirements for the 1995-2000 MHz range is the requirement for the S-band.  If this FCC requirement is reflected in a new OOBE mask, it has to assume the worst case that the  LTE channel edge is located at 2000 MHz, ie.lower boundary of the S-band, as shown in the Figure 2 a).  The OOBE has to drop to the level of -40 dBm/MHz just 5 MHz away from the channel edge.  This requirement is unnecessarily stringent for a LTE channel allocated not on the lower edge of S-band. For example, for a LTE channel located in the upper half of the S-band, as shown in the Figure 1 a),  as far as the OOBE drops to -40dBm/MHz after 15 MHz,  the FCC requirement is still satisfied.  This requirement difference for different LTE channel locations can not be reflected in the OOBE mask.

- OOBE Requirements for an S-band channel with its Lower Edge at 2010 MHz

- OOBE Requirements for an S-band Channel with its Upper Edge at 2020 MHz

Figure 1. Comparison of  S-Band Upper block FCC requirements and LTE OOBE Mask (NS_03).

- OOBE Requirements for an S-Band Channel with its Lower Edge at 2000 MHz

- OOBE Requirements for an S-band Channel with its Upper Edge at 2010 MHz

Figure 2. Comparison of  S-Band Lower block FCC requirements and LTE OOBE Mask (NS_03)

#### 5.2.4.1	Additional Spectrum Emission Mask

This requirement is specified in terms of an "additional spectrum emission" requirement.

##### 5.2.4.1.1	Minimum requirement (network signalled value "NS_03" and “NS_11”)

S-band  uses the additional spectrum emission mask of NS_03 and NS_11.

Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

When "NS_03" or “NS_11” is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.2.2.1-1.

Table 6.6.2.2.1-1: Additional requirements

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -10 | -13 | -15 | -18 | -20 | -21 | 30 kHz |
| 1-2.5 | -13 | -13 | -13 | -13 | -13 | -13 | 1 MHz |
| 2.5-5 | -25 | -13 | -13 | -13 | -13 | -13 | 1 MHz |
| 5-6 |  | -25 | -13 | -13 | -13 | -13 | 1 MHz |
| 6-10 |  |  | -25 | -13 | -13 | -13 | 1 MHz |
| 10-15 |  |  |  | -25 | -13 | -13 | 1 MHz |
| 15-20 |  |  |  |  | -25 | -13 | 1 MHz |
| 20-25 |  |  |  |  |  | -25 | 1 MHz |

#### 5.2.4.2	Spurious Emissions

##### 5.2.4.2.1		Spurious emission band UE co-existence

This clause specifies the requirements for the specified E-UTRA band, for coexistence with protected bands

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

# Table 6.6.3.2-1: Requirements

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | Measurement Bandwidth (MHz) | Comment |
| 2 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, [23] | FDL_low | - | FDL_high | -50 | 1 |  |
| 4 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, [23] | FDL_low | - | FDL_high | -50 | 1 |  |
| 5 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, [23] | FDL_low | - | FDL_high | -50 | 1 |  |
| 10 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, [23] | FDL_low | - | FDL_high | -50 | 1 |  |
| 12 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, [23] | FDL_low | - | FDL_high | -50 | 1 |  |
| 13 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, [23] | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 763 | - | 775 | -35 | 0.00625 |  |
| 14 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, [23] | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 763 | - | 775 | -35 | 0.00625 |  |
| 17 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, [23] | FDL_low | - | FDL_high | -50 | 1 |  |
| [23] | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, [23] | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | Frequency range | 1998 | - | 1999 | -21 | 1 | Note 13 |  |
|  | Frequency range | 1997 | - | 1998 | -27 | 1 |  |  |
|  | Frequency range | 1996 | - | 1997 | -32 | 1 |  |  |
|  | Frequency range | 1995 | - | 1996 | -37 | 1 |  |  |
| Note 13 To meet this requirement NS_11 value shall be signalled when operating in 2000-2010 MHz |  |  |  |  |  |  |  |  |

### 5.2.5	Receiver characteristics

The frequency range of S-band (UL 2000 to 2020 MHz, DL 2180 to 2200 MHz) is very similar to the Band 1 (UL 1920 to 1980 MHz,  DL 2110 to 2170 MHz). Both bands have very large duplex spacing so the self interference is insignificant. It is also expected that UE coexistence emissions requirements will be addressed by A-MPR rather than by imposing additional requirements on the Tx filtering. Therefore, it is expected the receiver characteristics of S-band should be the same as those of Band 1.

#### 5.2.5.1		Reference sensitivity power level

##### 5.2.5.1.1		Minimum requirements (QPSK)

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 5.2.5.1.1-1 and table 5.2.5.1.1-2

# Table 5.2.5.1.1-1: Reference sensitivity QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| [23] | -104.7 | -101.7 | -100 | -97 |  |  | FDD |

Table 5.2.5.1.1-2specifies the minimum number of allocated uplink resource blocks for which the reference receive sensitivity requirement must be met.

Table 5.2.5.1.1-2: Minimum uplink configuration for reference sensitivity

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| [23] | 6 | 15 | 25 | 50 |  |  | FDD |

#### 5.2.5.2		Requirement for large transmission configurations

For some combinations of bandwidths and operating bands, a certain relaxation of the UE performance is allowed when the transmission configuration is larger than that in Table 7.3.1-2. Table 7.3.2-1 specifies the allowed maximum sensitivity degradation (MSD) when the UL resource block allocation is the maximum supported transmission bandwidth configuration NRB (Table 5.6-1). Unless given by Table 7.3.1-3, the MSD shall be verified with the network signalling value NS_01 (Table 6.2.4-1) configured.

# Table 7.3.2-1: Maximum Sensitivity Degradation

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dB) | 3 MHz (dB) | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | Duplex Mode |
| [23] | n/a | n/a | n/a | n/a |  |  | FDD |

### 5.2.6		Blocking characteristics

#### 5.2.6.1		In-band blocking

In-band blocking is defined for an unwanted interfering signal falling into the UE receive band or into the first 15 MHz below or above the UE receive band at which the relative throughput shall meet or exceed the minimum requirement for the specified measurement channels.

##### 5.2.6.1.1		Minimum requirements

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Tables  7.6.1.1-1 and 7.6.1.1-2.

Table 7.6.1.1-1: In band blocking parameters

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Wanted signal mean power | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |  |
|  |  | 6 | 6 | 6 | 6 | 7 | 9 |
| BWInterferer | MHz | 1.4 | 3 | 5 | 5 | 5 | 5 |
| FIoffset, case 1 | MHz | 2.1+0.0125 | 4.5+0.0075 | 7.5+0.0125 | 7.5+0.0025 | 7.5+0.0075 | 7.5+0.0125 |
| FIoffset, case 2 | MHz | 3.5+0.0075 | 7.5+0.0075 | 12.5+0.0075 | 12.5+0.0125 | 12.5+0.0025 | 12.5+0.0075 |
| Note 1: The transmitter shall be set to 4dB below PUMAX at the minimum uplink configuration specified in Table 7.3.1-2. Note 2: The interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1 |  |  |  |  |  |  |  |

Table 7.6.1.1-2: In-band blocking

| E-UTRA band | Parameter | Units | Case 1 | Case 2 | Case 3 |
| --- | --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -56 | -44 | -30 |
|  | FInterferer (Offset) | MHz | =-BW/2 - FIoffset, case 1 & =+BW/2 + FIoffset, case 1 | -BW/2- FIoffset, case 2 & +BW/2 + FIoffset, case 2 | -BW/2 – 9 MHz & -BW/2 – 15 MHz |
| 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11,12, 13, 18, 19, 20, 21, [23] 33,34,35,36,37,38,39,40 | FInterferer | MHz | (Note 2) | FDL_low -15 to FDL_high +15 |  |
| 17 | FInterferer | MHz | (Note 2) | FDL_low -9.0 to FDL_high +15 | FDL_low -15 and FDL_low -9.0 (Note 3) |
| Note 1 For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band. 2 For each carrier frequency the requirement is valid for two frequencies: - the carrier frequency -BW/2 -FIoffset, case 1 and - the carrier frequency + BW/2 + FIoffset, case 1. 3 Finterferer range values for unwanted modulated interfering signal are interferer center frequencies. 4 Case 3 only applies to assigned UE channel bandwidth of 5 MHz. |  |  |  |  |  |

#### 5.2.6.2		Out-of-band blocking

Out-of-band band blocking is defined for an unwanted CW interfering signal falling more than 15 MHz below or above the UE receive band.  For the first 15 MHz below or above the UE receive band the appropriate in-band blocking or adjacent channel selectivity in sub-clause 7.5.1 and sub-clause 7.6.1 shall be applied.

##### 5.2.6.2.1		Minimum requirements

. The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Tables  7.6.2.1-1 and 7.6.2.1-2.

For Table 7.6.2.1-2 in frequency range 1, 2 and 3, up to exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a 1MHz step size, where  is the number of resource blocks in the downlink transmission bandwidth configuration (see Figure 5.4.2-1). For these exceptions the requirements of clause 7.7 Spurious response are applicable.

For Table 7.6.2.1-2 in frequency range 4, up to exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a 1MHz step size, where  is the number of resource blocks in the downlink transmission bandwidth configurations (see Figure 5.4.2-1) and  is the number of resource blocks allocated in the uplink. For these exceptions the requirements of clause 7.7 spurious response are applicable.

Table 7.6.2.1-1: Out-of-band blocking parameters

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Wanted signal mean power | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |  |
|  |  | 6 | 6 | 6 | 6 | 7 | 9 |
| Note 1: The transmitter shall be set to 4dB below PUMAX at the minimum uplink configuration specified in Table 7.3.1-2. Note 2: Reference measurement channel is specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2. |  |  |  |  |  |  |  |

Table 7.6.2.1-2: Out of band blocking

| E-UTRA band | Parameter | Units | Frequency |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | range 1 | range 2 | range 3 | range 4 |
|  | PInterferer | dBm | -44 | -30 | -15 | -15 |
| 1, 2, 3, 4, 5 6, 7, 8, 9, 10,11,12, 13, 17, 18, 19, 20, 21, [23], 33,34, 35, 36, 37, 38, 39, 40 | FInterferer (CW) | MHz | FDL_low -15 to FDL_low -60 | FDL_low -60 to FDL_low -85 | FDL_low -85 to 1 MHz | - |
|  |  |  | FDL_high +15 to FDL_high + 60 | FDL_high +60 to FDL_high +85 | FDL_high +85 to +12750 MHz | - |
| 2, 5, 12, 17 | FInterferer | MHz | - | - | - | FUL_low - FUL_high |
| Note: For the UE which supports both Band 11 and Band 21 the out of blocking is FFS. |  |  |  |  |  |  |

## 5.3	Specific BS RF requirements

### 5.3.1	Operating band unwanted emissions

The operating band unwanted emission limits are defined from 10 MHz below the lowest frequency of the downlink operating band up to 10 MHz above the highest frequency of the downlink operating band. The requirements shall apply whatever the type of transmitter considered (single carrier or multi-carrier) and for all transmission modes foreseen by the manufacturer's specification.

As discussed in Section 4, the BS out-of-band-emission limits mandated by the FCC for this band are the same as those of the PCS (E-UTRA Band 2) and AWS-1 (E-UTRA Band 4), i.e. attenuate out-of-band emissions by a factor of (43 + 10*log(P) dB) below the transmitter power (P) for a resolution bandwidth of 1 MHz or greater. Therefore, the general limits for Category A (as specified in Tables 6.6.3.1-4 to 6.6.3.1-6 of TS 36.104) and the additional limits for E-UTRA bands >1GHz as specified in Table 6.6.3.3-2 of TS 36.104 for E-UTRA Bands 2 and 4 shall also be applied for this band. When the E-UTRA channel edge is aligned with a licensee’s band edge, the additional limits outside the E-UTRA channel in Table 6.6.3.3-2 are identical to the FCC emission limits outside the licensee’s band. The general (Category A) limits for E-UTRA bands >1GHz and the additional (FCC) limits for this band are shown in Figure 5.3-1 below for the 1.4MHz, 3MHz, 5MHz and 10MHz channel bandwidth options. Note that the general limits for Category B are not applicable in North America, and thus need not be specified for this band.

(1a)	1.4MHz channel bandwidth

(1b)	3MHz channel bandwidth

(1c)	5MHz and 10 MHz channel bandwidth

Figure 5.3-1: The general (Category A) limits for E-UTRA bands >1GHz and the additional (FCC) limits

### 5.3.2	Additional spurious emissions requirements

These requirements may be applied for the protection of system operating in frequency ranges other than the E-UTRA BS downlink operating band. The limits may apply as an optional protection of such systems that are deployed in the same geographical area as the E-UTRA BS, or they may be set by local or regional regulation as a mandatory requirement for an E-UTRA operating band.

The E-UTRA BS spurious emissions limits currently specified for co-existence with systems operating in other frequency bands are more stringent than the out-of-band emissions limits by the FCC. To ensure sufficient protection to the co-existing systems, the same limits as the other E-UTRA bands BS shall be specified for this band in Clause 6.6.4.3 of TS 36.104 and Clause 6.6.4.5.4 of TS 36.141. And to ensure sufficient protection from the co-existing systems, the same limits as the other E-UTRA bands BS shall be specified for coexistence with the BS in this band in Clause 6.6.4.3 of TS 36.104 and Clause 6.6.4.5.4 of TS 36.141. This is shown in Table 5.3-1 below:

Table 5.3-1: BS Spurious emissions limits for E-UTRA BS for co-existence with systems operating in other frequency bands:

| System type for E-UTRA to co-exist with | Frequency range for co-existence requirement | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- | --- |
| E-UTRA Band [23] | 2180 - 2200 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band [23]. |
|  | 2000 - 2020 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band [23], since it is already covered by the requirement in subclause 6.6.4.2. The requirement for E-UTRA BS operating in Band 2 and [25] are defined separately |
| E-UTRA Band [23] | 2000 – 2020 MHz | TBD | TBD | This requirement does not apply to E-UTRA BS operating in band [23]. This requirement only applies to E-UTRA BS operating in Band 2. |
| E-UTRA Band [23] | 2000 – 2020 MHz | TBD | TBD | This requirement does not apply to E-UTRA BS operating in band [23]. This requirement only applies to E-UTRA BS operating in Band [25]. |

Moreover, as discussed in Section 4, the BS in this band shall meet the following limits on emissions in the 1559-1610 MHz band as requested in FCC DA 09-38 and FCC DA 10-60 as shown in Table 5.3-2 below:

Table 5.3-2: FCC emission limits in the 1559-1610 MHz band:

|  | Frequency | dBW/MHz EIRP (Measurement bandwidth = 1 MHz) | dBW EIRP of discrete emissions of less than 700 Hz bandwidth (Measurement bandwidth = 1 kHz) |
| --- | --- | --- | --- |
| Base Stations | 1559-1610 MHz | -100 | -110 |

Therefore, these requirements shall be specified as additional spurious emissions requirements for this band in Clause 6.6.4.3 of TS 36.104 and Clause 6.6.4.5.4 of TS 36.141. Note that the limits by the FCC in the 1559-1610 MHz band (as shown in Table 5.3-2) are defined in terms of EIRP (effective isotropic radiated power), which is dependent on both the BS emissions at the antenna connector and the deployment (including antenna gain and feeder loss). The EIRP level is calculated using: EIRP = PTx + Gant where PTx denotes the BS transmitted power at the antenna connector, Gant equals the BS antenna gain minus feeder loss.

### 5.3.3	Co-location with other base stations

[These requirements may be applied for the protection of other BS receivers when GSM900, DCS1800, PCS1900, GSM850, UTRA FDD, UTRA TDD and/or E-UTRA BS are co-located with an E-UTRA BS. The requirements assume a 30 dB coupling loss between transmitter and receiver and are based on co-location with base stations of the same class.

The E-UTRA BS spurious emissions limits currently specified for co-location with another BS are more stringent than the out-of-band emissions limits by the FCC. To ensure sufficient protection to the co-located BS, the same limits as the other E-UTRA bands BS shall be specified for this band in Clause 6.6.4.4 of TS 36.104 and Clause 6.6.4.5.5 of TS 36.141. And to ensure sufficient protection from the co-located BS, the same limits as the other E-UTRA bands BS shall be specified for co-location with the BS in this band in Clause 6.6.4.4 of TS 36.104 and Clause 6.6.4.5.5 of TS 36.141. ]

### 5.3.4	General blocking requirement

[The blocking characteristics is a measure of the receiver ability to receive a wanted signal at its assigned channel in the presence of an unwanted interferer, which are either a 1.4MHz, 3MHz or 5MHz E-UTRA signal for in-band blocking or a CW signal for out-of-band blocking.

The general blocking requirements currently specified for E-UTRA BS apply the in-band blocking signal within the +/-20 MHz of the UL frequency range of a band, except when the DL frequency range of the same band falls within such range. Since the duplex distance of this band is larger than 20 MHz and thus no exception is needed, the same general blocking requirements as specified for E-UTRA Bands 2 and 5 BS shall be specified for this band in Clause 7.6.1 of TS 36.104 and Clause 7.6.5.1 of TS 36.141 to ensure sufficient BS receiver blocking ability.]

### 5.3.5	Blocking requirement for co-location with other base stations

[This additional blocking requirement may be applied for the protection of E-UTRA BS receivers when GSM, UTRA or E-UTRA BS operating in a different frequency band are co-located with an E-UTRA BS. The requirements assume a 30 dB coupling loss between interfering transmitter and E-UTRA BS receiver and are based on co-location with base stations of the same class.

The blocking requirements currently specified for E-UTRA BS when co-located with BS in other frequency bands do not apply when the interfering signal falls within the +/-10 MHz of the UL frequency range of a band, except for Band 13 in order to allow co-location with Band 14 BS. The blocking requirements shall be specified for this band in Clause 7.6.2 of TS 36.104 and Clause 7.6.5.2 of TS 36.141 using the same interfering signal as the other E-UTRA bands BS, but do not apply when the interfering signal falls within -10 MHz and +10 MHz of the UL frequency range of this band (i.e. 1990 – 2030 MHz).]

<Text to be added>

# 6	Summary of required changes to E-UTRA specifications

## 6.1		Required changes to TS 36.101

Required changes in TS36.101are shown in Table 6.1-1.

Table 6.1-1: Required Changes for TS36.101

| Clause | Description | Description of change |
| --- | --- | --- |
| 5.5 | Operating bands | Add a new row for the band into Table 5.5-1. |
| 5.6.1 | Channel bandwidths per operating band | Add channel bandwidths 1.4 – 10 MHz for the band into Table 5.6.1-1. |
| 5.7.3 | Carrier frequency and EARFCN | Add a new channel numbering for the band into Table 5.7.3-1. |
| 5.7.4 | TX–RX frequency separation | Add a new row for the band into Table 5.7.4-1. |
| 6.2.2 | UE Maximum Output Power | Add a new row for the band into Table 6.2.2-1. |
| 6.2.4 | UE Maximum Output Power with additional requirements | (FFS) Add a new row for the band into Table 6.2.4-1. |
| 6.6.2.2 | Additional Spectrum Emission Mask | Add new requirements for the band. |
| 6.6.3.2 | Spurious emission band UE co-existence | Add the band into Table 6.6.3.2-1 for mutual protection with E-UTRA Band 2, 4, 5, 10, 12, 13, 14 and 17. |
| 6.6.3.3 | Additional spurious emissions | (FFS) Add new requirements for the band. |
| 7.3 | Reference sensitivity power level | Add a new row for the band into Tables 7.3.1-1, 7.3.1-2, 7.3.2-1 and, when deems necessary, 7.3.1-3. |
| 7.6.1 | In-band blocking | Add the band into Table 7.6.1.1-2. |
| 7.6.2 | Out-of-band blocking | Add the band into Table 7.6.2.1-2. |

## 6.2		Required changes to TS 36.104

Required changes in TS36.104 are shown in Table 6.2-1.

Table 6.2-1: Required Changes for TS36.104

| Clause | Description | Description of change |
| --- | --- | --- |
| 5.5 | Operating bands | Add a new row for the band into Table 5.5-1. |
| 5.7.3 | Carrier frequency and EARFCN | Add new channel numbers for the band into Table 5.7.3-1. |
| 6.6.3 | Operating band unwanted emissions | Add the band to Category A. |
| 6.6.4.3 | Additional spurious emissions requirements | Add a new row for the band into Tables 6.6.4.3.1-1 and 6.6.4.3-1x. |
| 6.6.4.4 | Co-location with other base stations | Add a new row for the band into Tables 6.6.4.4.1-1 and 6.6.4.4.1-2. |
| 7.6.1 | General blocking requirement | Add the band into Tables 7.6.1.1-1, 7.6.1.1-1a, and 7.6.1.1-1b. |
| 7.6.2 | Co-location with other base stations | Add a new row for the band into Tables 7.6.2.1-1 and 7.6.2.1-2. |

## 6.3		Required changes to TS 36.113

Required changes in TS36.113 are shown in Table 6.3-1.

Table 6.3-1: Required changes for TS36.113

| Clause | Description | Description of change |
| --- | --- | --- |
| 4.5.2 | Receiver exclusion band | Add a new row for the band. |

## 6.4		Required changes to TS 36.124

Required changes in TS36.124 are shown in Table 6.4-1.

Table 6.4-1: Required changes for TS36.124

| Clause | Description | Description of change |
| --- | --- | --- |
| 4.4 | Receiver exclusion band | Add a new row for the band. |

## 6.5		Required changes to TS 36.133

Required changes in TS36.133 are shown in Table 6.5-1.

Table 6.5-1: Required changes for TS36.133

| Clause | Description | Description of change |
| --- | --- | --- |
| 4.2.2.3 | Measurements of intra-frequency E-UTRAN cells | Add the band to the band list for RSRP and SCH_RP. |
| 4.2.2.4 | Measurements of inter-frequency E-UTRAN cells | Add the band to the band list for RSRP and SCH_RP. |
| 8.1.2.2 | E-UTRAN intra frequency measurements | Add the band to the band list for SCH_RP. |
| 8.1.2.3 | E-UTRAN inter frequency measurements | Add the band to the band list for RSRP and SCH_RP. |
| 8.1.2.5 | E-UTRAN OTDOA Intra-Frequency RSTD Measurements | Add the band to the band list for PRP. |
| 8.1.2.6 | E-UTRAN Inter-Frequency OTDOA Measurements | Add the band to the band list for PRP. |
| 9.1.2 | Intra-frequency RSRP Accuracy Requirements | Add the band to the band list for RSRP. |
| 9.1.3 | Inter-frequency RSRP Accuracy Requirements | Add the band to the band list for RSRP. |
| 9.1.5 | Intra-frequency RSRQ Accuracy Requirements | Add the band to the band list for RSRP. |
| 9.1.6 | Inter-frequency RSRQ Accuracy Requirements | Add the band to the band list for RSRP. |
| 9.1.9 | UE Rx – Tx time difference | Add the band to the band list for RSRP. |
| 9.1.10 | Reference Signal Time Difference (RSTD) | Add the band to the band list for PRP. |
| A.9.1.1 | (RSRP) FDD Intra frequency case | Add the band to the band list for Noc, RSRP, and Io. |
| A.9.1.3 | (RSRP) FDD—FDD Inter frequency case | Add the band to the band list for Noc, RSRP, and Io. |
| A.9.2.1 | (RSRQ) FDD Intra frequency case | Add the band to the band list for Noc, RSRP, RSRQ, and Io. |
| A.9.2.3 | (RSRQ) FDD—FDD Inter frequency case | Add the band to the band list for Noc, RSRP, RSRQ, and Io. |

## 6.6		Required changes to TS 36.141

Required changes in TS36.141 are shown in Table 6.6-1.

Table 6.6-1: Required Changes for TS36.141

| Clause | Description | Description of change |
| --- | --- | --- |
| 5.5 | Operating bands | Add a new row for the band into Table 5.5-1. |
| 5.7.3 | Carrier frequency and EARFCN | Add new channel numbers for the band into Table 5.7.3-1. |
| 6.6.3 | Operating band unwanted emissions | Add the band to Category A. |
| 6.6.4.5.4 | Co-existence with other systems in the same geographical area | Add a new row for the band into Tables 6.6.4.5.4.1-1 and 6.6.4.5.4-1x. |
| 6.6.4.5.5 | Co-location with other base stations | Add a new row for the band into Tables 6.6.4.5.5-1 and 6.6.4.5.5-2. |
| 7.6.5.1 | (Blocking) General requirement | Add the band into Tables 7.6-1, 7.6-1a and 7.6-1x. |
| 7.6.5.2 | Co-location with other base stations | Add a new row for the band into Table 7.6-3 and 7.6-4.. |

## 6.7		Required changes to TS 25.461

Required changes in TS25.461 are shown in Table 6.7-1.

Table 6.7-1: Required changes for TS25.461

| Clause | Description | Description of change |
| --- | --- | --- |
| 4.3.7 | Operating bands | Add a new row for the band in Table 4.3.7.1. |

## 6.8		Required changes to TS 25.466

Required changes in TS25.466 are shown in Table 6.8-1.

Table 6.8-1: Required changes for TS25.466

| Clause | Description | Description of change |
| --- | --- | --- |
| Annex B | Assigned fields for additional data | Add an entry for the band in Table B.2-1. |

## 6.9		Required changes to TS 36.307 Version 8.1.0

Required change in TS36.307 Version 8.1.0 are shown in Table 6.9-1.

Table 6.9-1: Required changes for TS36.307 Version 8.1.0

| Clause | Description | Description of change |
| --- | --- | --- |
| X | Added Band | Add new section X and its tables X.1.1-1 and X.1.2-1 for the band to define as release-independent. |

## 6.10		Required changes to TS 36.307 Version 9.1.1

Required change in TS36.307 Version 9.1.1 are shown in Table 6.9-1.

Table 6.9-1: Required changes for TS36.307 Version 9.1.1

| Clause | Description | Description of change |
| --- | --- | --- |
| X | Added Band | Add new section X and its tables X.1.1-1 and X.1.2-1 for the band to define as release-independent. |

## 6.11		Required changes to TS 25.101

Required changes in TS25.101are shown in Table 6.11-1.

Table 6.11-1: Required Changes for TS25.101

| Clause | Description | Description of change |
| --- | --- | --- |
| 6.6.3 | Spurious emissions | Add the band DL frequency range into Tables 6.13 and 6.13A for UTRA Band II, IV, V, X, XII, XIII and XIV. |

## 6.12		Required changes to TS 25.104

Required changes in TS25.104 are shown in Table 6.12-1.

Table 6.12-1: Required Changes for TS25.104

| Clause | Description | Description of change |
| --- | --- | --- |
| 6.6.3.3 | Co-existence with other systems in the same geographical area | Add a new row for the band into Table 6.11. |
| 6.6.3.4 | Co-existence with co-located and co-sited base stations | Add a new row for the band into Tables 6.12 and 6.14. |
| 6.6.3.9 | Co-existence with Home BS operating in other bands | Add a new row for the band into Tables 6.20. |
| 7.5.2 | Minimum Requirement - Co-location with GSM900, DCS 1800, PCS1900, GSM850, UTRA FDD and/or E-UTRA FDD | Add a new row for the band into Tables 7.5C and 7.5E. |

## 6.13		Required changes to TS 25.141

Required changes in TS25.141 are shown in Table 6.13-1.

Table 6.13-1: Required Changes for TS25.141

| Clause | Description | Description of change |
| --- | --- | --- |
| 6.5.3.7.4 | Co-existence with other systems in the same geographical area | Add a new row for the band into Table 6.38. |
| 6.5.3.7.5 | Co-existence with co-located and co-sited base stations | Add a new row for the band into Tables 6.39 and 6.41. |
| 6.5.3.7.10 | Co-existence with Home BS operating in other bands | Add a new row for the band into Tables 6.47. |
| 7.5.5 | Blocking characteristics - Test Requirements | Add a new row for the band into Tables 7.4N and 7.4Q. |

## 6.14		Required changes to TS 37.104

Required changes in TS37.104 are shown in Table 6.14-1.

Table 6.14-1: Required Changes for TS37.104

| Clause | Description | Description of change |
| --- | --- | --- |
| 6.6.1.3 | Additional spurious emissions requirements | Add a new row for the band into Table 6.6.1.3.1-1. |
| 6.6.1.4 | Co-existence with co-located and co-sited base stations | Add a new row for the band into Table 6.6.1.4.1-1. |
| 7.5.2 | Co-location minimum requirement | Add a new row for the band into Table 7.5.2-1. |

## 6.15		Required changes to TS 37.141

Required changes in TS37.141 are shown in Table 6.15-1.

Table 6.15-1: Required Changes for TS37.141

| Clause | Description | Description of change |
| --- | --- | --- |
| 6.6.1.5.5 | Additional spurious emission requirements | Add a new row for the band into Table 6.6.1.5.5-1. |
| 6.6.1.5.6 | Co-location with other Base Stations | Add a new row for the band into Table 6.6.1.5.6-1. |
| 7.5.5.2 | Co-location test requirements | Add a new row for the band into Table 7.5.5.2-1. |

# 7	Project plan

The schedule and work task status for this work item are summarized in Table 7.1-1 below.

Table 7.1-1 Schedule and work task status

| RAN4 Meeting and Date | Work Tasks | Status |
| --- | --- | --- |
| RAN4#55 (April 2010) | TR Skeleton | Approved |
|  | Task description and regulatory background | Approved |
|  | Summary of required changes to E-UTRA specifications | Approved |
|  | Band and channel arrangement | Discussed |
| RAN4 Ad-hoc#3 (June 2010) | Work plan | Agreed |
|  | Continue discussion on regulatory background | Agreed |
|  | Bandwidth deployment scenarios | Agreed |
|  | Discussion on specific BS RF requirements | Discussed |
|  | Discussion on specific UE RF requirements (maximum output power with additional requirements, spurious emissions, reference sensitivity power level, blocking characteristics) | Discussed |
|  | Agreement on band and channel arrangement | Agreed |
| RAN4#56 (August 2010) | Continue discussion on specific BS RF requirements | Partially agreed |
|  | Continue discussion on specific UE RF requirements | Partially agreed |
| RAN4 Ad-hoc#4 (October 2010) | Agreement on specific UE RF requirements |  |
|  | Agreement on specific BS RF requirements |  |
|  | Updated Work Plan |  |
|  | Draft CRs for review and comment |  |
| RAN4#57 (November 2010) | Final TR |  |
|  | Final CRs |  |

# 8	Open issues

# Annex A (Informative): Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 2010-05 | RAN4#55 | R4-102131 |  |  | TS skeleton created from 3GPP TS template. |  | 0.0.1 |
| 2010-6 | R4 AH 10-03 | R4-102653 |  |  | Update based on approved text from the Work item | 0.0.1 | 0.1.0 |
| 2010-08 | RAN4#56 | R4-103299 R4-103427 |  |  | Update based on approved R4 AH 10-03 input documents Update based on approved R4#56 input documents | 0.1.0 0.2.0 | 0.2.0 0.3.0 |
| 2010-09 | RAN#49 | RP-100804 |  |  | Update presented for information (60% complete) | 0.3.0 | 1.0.0 |
| 2010-11 | R4#57 | R4-104120 |  |  | Update numbering, content unchanged, presented for information | 1.0.0 | 1.0.1 |
| 2011-01 | RAN4#57AH | R4-110462 |  |  | Update based on approved R4#57 documents R4-103565 “Work plan for Adding 2 GHz band LTE for ATC of MSS in North America R4-104070 “Updated Reference Sensitivity Requirements for 1.4 bandwidth for Band 23” R4-104247 “Updates for required changes for Adding 2GHz band LTE of MSs in North America” R4-104320 “Add 2 GHz band LTE for ATC of MSS in North America to TS36.133” R4-104321 “Add 2 GHz band LTE for ATC of MSS in North America to TS36.307” Rel-8 R4-104322 “Add 2 GHz band LTE for ATC of MSS in North America to TS36.307” Rel-9 R4-104323 “Add 2 GHz band LTE for ATC of MSS in North America to TS25.466” R4-104804 “Band 23 A-MPR” | 1.0.1 | 1.1.0 |
| 2011-02 | RAN4#58 | R4-111325 |  |  | Update based on approved R4#57AH documents: R4-110500 “Band 23 UE Requirements” | 1.1.0 | 1.2.0 |