3GPP TSG-RAN WG4 Meeting #63	R4-122244

Prague, 21 – 25 May, 2012

Source:	China Telecom

Title:	TP for TR ab.cde (inter-band CA) on IMD study of B1 + B7 CA

Agenda item:	6.11.1

Document for:	Approval

Introduction

In this contribution, the IMD products for 1UL/2DL are re-calculated for B1 + B7 CA, and proposed to be updated into the latest TR ab.cde.

References

[1]	R4-120123 “TP for TR ab.cde (inter-band CA): 1UL/2DL 2UL/2DL IMD study for B3 + B7 CA”, TeliaSonera AB, 3GPP TSG-RAN WG4 Meeting #62, Dresden, Germany, 6 – 10 February, 2012

[2]	R4-120246 “Harmonics and Intermodulation Products caused by LTE Advanced Carrier Aggregation of Band Combination (1 + 7)”, Alcatel-Lucent, 3GPP TSG-RAN WG4 Meeting #62, Dresden, Germany, 6 – 10 February, 2012

Text proposal for inter-band carrier aggregation TR ab.cde V0.1.0 (Release 11)

----- Unchanged sections omitted -----

----- Start of TP -----

6.3		Class A3. Low-low or high-high band combinations

This chapter shall include all inter band CA combinations which can be included in the group low-low or high-high combinations without transmitter harmonics.

6.3.3	LTE-Advanced Carrier Aggregation of Band 1 and Band 7 (1 UL)

Table 6.3.3-1: Inter-band non-contiguous CA

| E-UTRA CA Band | E-UTRA operating Band | Uplink (UL) band |  |  |  | Downlink (DL) band |  |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | UE transmit / BS receive |  |  | Channel BW MHz | UE receive / BS transmit |  |  | Channel BW MHz |  |
|  |  | FUL_low (MHz) – FUL_high (MHz) |  |  |  | FDL_low (MHz) – FDL_high (MHz) |  |  |  |  |
| CA_1-7 | 1 | 1920 | – | 1980 | 15 | 2110 | – | 2170 | 15 | FDD |
|  | 7 | 2500 | – | 2570 | 15, 20 | 2620 | – | 2690 | 15, 20 |  |

6.3.3.1		List of specific combination issues

6.3.3.1.1	Channel bandwidths per operating band for CA

Table 6.3.3.1.1-1: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| CA operating / channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| CA_1A-7A | 1 |  |  |  |  | Yes |  |
|  | 7 |  |  |  |  | Yes | Yes |

6.3.3.1.2	Co-existence studies for 1UL/2DL

Table 6.3.1.1.2-1 gives the intermodulation products for band 1 + band 7 CA with 2DLs. For the 3-tone IMD analysis the maximum transmission as defined in Table 6.1.2.1-1 is considered. Three-tone third order IMD products can fall into the band 7 receiver. However, these products will not fall into the BS own receive block if the frequency range as defined with the channel bandwidths given in Table 6.3.1.1.1-1 are used for the more detailed IMD calculation.

Considering bands in the same geographical area we observe that the BS distortion could fall into the BS receive bands of Band 3. With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters the IMDs generated within the band 3 receiver should be well below the receiver noise floor eliminating the possibility of receiver desensitization. Provided that the Bands 1 and 7 BS transmitters should not share the same antenna with Band 3 BS receiver.

Table 6.3.3.1.2-1: 2DLs B1 + B7 IMD products

| BS DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 2110 | 2170 | 2620 | 2690 |
| 2nd order harmonics frequency range (MHz) | 4220 to 4340 |  | 5240 to 5380 |  |
| 3rd order harmonics frequency range (MHz) | 6330 to 6510 |  | 7860 to 8070 |  |
| Two-tone 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency range (MHz) | 450 to 580 |  | 4730 to 4860 |  |
| Two-tone 3rd order IMD products | 2*f1_low – f2_high &(2*f1_low + f2_low) | 2*f1_high – f2_low & (2*f1_high + f2_high) | 2*f2_low – f1_high &(2*f2_low + f1_low) | 2* f2_high – f1_low& (2*f2_high + f1_high) |
| IMD frequency range (MHz) | 1530 to 1720 6840 to 7030 |  | 3070 to 3270 7350-7550 |  |
| Three-tone 3rd order IMD products | (f1_low – max BW f2) | (f1_high + max BW f2) | (f2_low – max BW f1) | (f2_high + max BW f1) |
| IMD frequency range (MHz) | 2090 to 2190 |  | 2605 to 2705 |  |

Table 6.3.1.1.2-2 gives the intermodulation products for band 1+ band 7 CA with 1UL. None of the intermodulation products fall into the own and any other receive bands.

Table 6.3.3.1.2-2: 1UL B1 + B7 harmonic products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 2500 | 2570 |
| 2nd order harmonics frequency range (MHz) | 3840 to 3960 |  | 5000 to 5140 |  |
| 3rd order harmonics frequency range (MHz) | 5760 to 5940 |  | 7500 to 7710 |  |

----- End of TP -----