3GPP TSG RAN WG4 Meeting #68	R4-134389

- Barcelona, Spain, 19th – 23th August, 2013

- Agenda Item:	8.23

- Source:	LG Electronics

- Title:	Passive IMD measurements levels of inter-band CA_1A-5A UE with 2ULs

Document for:	Discussion and Approval

- Introduction

In the last RAN4 #67 meetings, some interested companies provided the active/passive IMD analysis for 2ULs inter-band CA UE [1][2][3], which states that inter-modulation will not violate -50dBm/1MHz UE-to-UE co-existence emission requirement. But in some CA classes, 2ULs CA UE has impact on the IMD problems to meet the 1UL UE-to-UE co-existence emission limits. So WF was not agreed in the last RAN4 meeting.

In this contribution, we provided real measurements of IMD level for these passive components. And we propose that RAN4 need not make any additional UE-to-UE coexistence requirements for 2ULs inter-band CA class A1 UE since these passive & active IM3 levels are quite lower than -50dBm/MHz.

- Measurement method on the IMD level of passive components for CA_1A-5A UE with 2ULs

To analyze the IMD from passive components, we assume the typical values of RF components as follows

  - Aggregated maximum output power = 23dBm with 0dB MPR

    - PA gain = 27dB, PA IIP3 =40dB

  - Tx/Rx RF component IL = 3.5dB

    - 3.5dB (ANT SW: 0.5dB,  Diplexer :0.5dB, Duplexer: 2.5dB)

  - FE coupling loss = 40dB

  - Antenna loss =10dB

  - Diplexer Characteristics: Band 1- Band 5 isolation : 20dB

  - Duplexer Characteristics: Tx-Rx isolation: 50dB, Tx-Ant isolation : 35 dB (824-849MHz)

For the 2UL inter-band CA, the input signal level induced from the other operating Band Y into Band X through Duplexer/Diplexer/Switch/Quadplexer is quite high. That is derived in Eq. 2-1.

- The leakage signals level at RF Components of Band X                                  (Eq. 2-1)

S1 (Blue line) = PA output power of Band Y - Tx RF component loss of Band Y –Rx RF component loss of Band X

Figure1. Class A1: Examples of the signal level at passive/active components

In Figure 1, we denoted the point where passive IMD can be generated as point A, B and C to compare with active IMD at point P.

For the passive IMD analysis, we assume the input signal level from Band Y to Band X on these RF components at Duplexer, switch and Diplexer as follow

- Duplexer Passive IMD (point A): consider the 23dBm input signal from Band X and -1dBm input signal from Band Y

- Switch Passive IMD (point B): consider the 20.5dBm input signal from Band X and -0.5dBm input signal from Band Y

- Diplexer Passive IMD (point C): consider the 20dBm input signal from Band X and 20dBm input signal from Band Y

So, we measured the passive IMD level with aid of RF component vendors. Table 1-1 ~ 3-2 provides the measured data of PIM for Duplexer, Diplexer and Switch components.

Table 1-1. Band1 Duplexer Passive IMD level

Table 1-2. Band 5 Duplexer Passive IMD level

Table 2-1. Band 1 Switch Passive IMD level

| Component : Band1 Switch |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Category | Input F1 | Input F2 | Input F3 | F1 Power | F2 Power | F3 Power | Meas Frequency [MHz] | Comp 1 PIMD level [dBm] | Comp 2 PIMD level [dBm] | Comp 3 PIMD level [dBm] |
| 2-tone IM2 | 1950 | 836.5 |  | 24dBm | 0dBm |  | 2786.5 | <-90 | < -107 | < -96 |
|  | 1950 | 836.5 |  | 24dBm | 0dBm |  | 1113.5 | <-98 | <-101 | <-102 |
| 2-tone IM3 | 1950 | 836.5 |  | 24dBm | 0dBm |  | 4736.5 | < -98 | < -99 | < -99 |
|  | 1950 | 836.5 |  | 24dBm | 0dBm |  | 3063.5 | < -100 | < -103 | < -103 |
| 3-tone IM3 | 1980 | 839 | 849 | 24dBm | 0dBm | 0dBm | 1990 | No signal | No signal | No signal |
|  | 1920 | 824 | 834 | 24dBm | 0dBm | 0dBm | 1910 | No signal | No signal | No signal |

Table 2-2. Band 5 Switch Passive IMD level

| Component : Band5 Switch |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Category | Input F1 | Input F2 | Input F3 | F1 Power | F2 Power | F3 Power | Meas Frequency [MHz] | Comp 1 PIMD level [dBm] | Comp 2 PIMD level [dBm] | Comp 3 PIMD level [dBm] |
| 2-tone IM2 | 836.5 | 1950 |  | 24dBm | 0dBm |  | 2786.5 | < -97 | < -107 | < -107 |
|  | 836.5 | 1950 |  | 24dBm | 0dBm |  | 1113.5 | <-100 | < -100 | < -100 |
| 2-tone IM3 | 836.5 | 1950 |  | 24dBm | 0dBm |  | 3623 | < -105 | < -107 | < -107 |
|  | 836.5 | 1950 |  | 24dBm | 0dBm |  | 277 | < -102 | < -105 | < -105 |
| 3-tone IM3 | 849 | 1970 | 1980 | 24dBm | 0dBm | 0dBm | 859 | No signal | No signal | No signal |
|  | 824 | 1920 | 1930 | 24dBm | 0dBm | 0dBm | 814 | No signal | No signal | No signal |

Table 3-1. Band 1&5 Diplexer Passive IMD level

| Component : Band5 Diplexer |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Category | Input F1 | Input F2 | Input F3 | F1 Power | F2 Power | F3 Power | Meas Frequency [MHz] | Comp 1 PIMD level [dBm] | Comp 2 PIMD level [dBm] | Comp 3 PIMD level [dBm] |
| 2-tone IM2 | 836.5 | 1950 |  | 20dBm | 20dBm |  | 2786.5 | -123 | -135 | -136 |
|  | 836.5 | 1950 |  | 20dBm | 20dBm |  | 1113.5 | -122 | -125 | -131 |
| 2-tone IM3 | 836.5 | 1950 |  | 20dBm | 20dBm |  | 3623 | -136 | -136 | -135 |
|  | 836.5 | 1950 |  | 20dBm | 20dBm |  | 277 | -129 | -132 | -131 |
| 3-tone IM3 | 849 | 1970 | 1980 | 20dBm | 20dBm | 20dBm | 859 | No signal | No signal | No signal |
|  | 824 | 1920 | 1930 | 20dBm | 20dBm | 20dBm | 814 | No signal | No signal | No signal |

From the Table 1-1 ~ Table 3-1, the passive IMD level is seen to be quite low with the IMD2 and IMD3 levels less than - 85dBm/MHz.  Therefore we can see that the passive IMD products can have negligible impact on the adjacent band UE. From the previous IMD analysis [1] and the measurements results of the RF components, we propose the followings:

Observation: The measurement of Passive IMD levels for real RF components and analysis of Active IMD levels reveals quite lower than -50dBm/MHz, which is the conventional UE-to-UE coexistence requirements.

Proposal 1: RAN4 do not need to make any additional UE-to-UE coexistence requirements for 2ULs inter-band CA class A1 UE based on the related IMD measurements and analysis papers.

- Conclusions

In this contribution we measured the IMD level of these passive components such as Diplexer, duplexer and switch. The followings are the observations and proposals in this paper.

Observation: The measurement of Passive IMD levels for real RF components and analysis of Active IMD levels reveals quite lower than -50dBm/MHz, which is the conventional UE-to-UE coexistence requirements.

Proposal 1: RAN4 do not need to make any additional UE-to-UE coexistence requirements for 2ULs inter-band CA class A1 UE based on the related IMD measurements and analysis papers.

- Reference

- R4-133062, “Dual uplink CA intermodulation analysis,” Renesas Mobile

- R4-132510, “Consideration on the Passive IMD analysis for inter-band CA_1A-5A UE with 2ULs,” LG Electronics

- R4-132774, “2UL CA UE coexistence,” Qualcomm

- R4-131986, “WF on the protected Bands for inter-band CA with 2ULs,” LG U+, NII, KT, Nokia

- TR 36.860 v0.1.0

- R4-131416, “Protected Band for B1+B5 2 UP LINK CA,” LG U+

- TS 36.101 v11.5.0

Text proposal for TR 36.860

********************** Start of text proposal to TR 36.860 Chapter 5.2.3 ***********************

    - Dual Uplink CA Passive intermodulation measurements

- 5.2.3.1 Passive intermodulation measurement for CA class A1

To analyze the IMD from passive components, we assume the typical values of RF components as follows

  - Aggregated maximum output power = 23dBm with 0dB MPR

    - PA gain = 27dB, PA IIP3 =40dB

  - Tx/Rx RF component IL = 3.5dB

    - 3.5dB (ANT SW: 0.5dB,  Diplexer :0.5dB, Duplexer: 2.5dB)

  - FE coupling loss = 40dB

  - Antenna loss =10dB

  - Diplexer Characteristics: Band 1- Band 5 isolation : 20dB

  - Duplexer Characteristics: Tx-Rx isolation: 50dB, Tx-Ant isolation : 35 dB (824-849MHz)

For the 2UL inter-band CA, the input signal level induced from the other operating Band Y into Band X through Duplexer/Diplexer/Switch/Quadplexer is quite high. That is derived in Eq. 5-1.

- The leakage signals level at RF Components of Band X                                  (Eq. 5-1)

S1 (Blue line) = PA output power of Band Y - Tx RF component loss of Band Y –Rx RF component loss of Band X

Figure5.2.3.1-1. Class A1: Examples of the signal level at passive/active components

In Figure 5.2.3.1-1, we denoted the point where passive IMD can be generated as point A, B and C to compare with active IMD at point P.

For the passive IMD analysis, we assume the input signal level from Band Y to Band X on these RF components at Duplexer, switch and Diplexer as follow

- Duplexer Passive IMD (point A): consider the 23dBm input signal from Band X and -1dBm input signal from Band Y

- Switch Passive IMD (point B): consider the 20.5dBm input signal from Band X and -0.5dBm input signal from Band Y

- Diplexer Passive IMD (point C): consider the 20dBm input signal from Band X and 20dBm input signal from Band Y

So, we measured the passive IMD level with aid of RF component vendors. Table 5.2.3.1-1 ~ 5.2.3.1-5 provides the measured data of PIM for Duplexer, Diplexer and Switch components.

Table 5.2.3.1-1. Band1 Duplexer Passive IMD level

Table 5.2.3.1-2. Band 5 Duplexer Passive IMD level

Table 5.2.3.1-3. Band 1 Switch Passive IMD level

| Component : Band1 Switch |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Category | Input F1 | Input F2 | Input F3 | F1 Power | F2 Power | F3 Power | Meas Frequency [MHz] | Comp 1 PIMD level [dBm] | Comp 2 PIMD level [dBm] | Comp 3 PIMD level [dBm] |
| 2-tone IM2 | 1950 | 836.5 |  | 24dBm | 0dBm |  | 2786.5 | <-90 | < -107 | < -96 |
|  | 1950 | 836.5 |  | 24dBm | 0dBm |  | 1113.5 | <-98 | <-101 | <-102 |
| 2-tone IM3 | 1950 | 836.5 |  | 24dBm | 0dBm |  | 4736.5 | < -98 | < -99 | < -99 |
|  | 1950 | 836.5 |  | 24dBm | 0dBm |  | 3063.5 | < -100 | < -103 | < -103 |
| 3-tone IM3 | 1980 | 839 | 849 | 24dBm | 0dBm | 0dBm | 1990 | No signal | No signal | No signal |
|  | 1920 | 824 | 834 | 24dBm | 0dBm | 0dBm | 1910 | No signal | No signal | No signal |

Table 5.2.3.1-4. Band 5 Switch Passive IMD level

| - Component : Band5 Switch |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Category | Input F1 | Input F2 | Input F3 | F1 Power | F2 Power | F3 Power | Meas Frequency [MHz] | Comp 1 PIMD level [dBm] | Comp 2 PIMD level [dBm] | Comp 3 PIMD level [dBm] |
| 2-tone IM2 | 836.5 | 1950 |  | 24dBm | 0dBm |  | 2786.5 | < -97 | < -107 | < -107 |
|  | 836.5 | 1950 |  | 24dBm | 0dBm |  | 1113.5 | <-100 | < -100 | < -100 |
| 2-tone IM3 | 836.5 | 1950 |  | 24dBm | 0dBm |  | 3623 | < -105 | < -107 | < -107 |
|  | 836.5 | 1950 |  | 24dBm | 0dBm |  | 277 | < -102 | < -105 | < -105 |
| 3-tone IM3 | 849 | 1970 | 1980 | 24dBm | 0dBm | 0dBm | 859 | No signal | No signal | No signal |
|  | 824 | 1920 | 1930 | 24dBm | 0dBm | 0dBm | 814 | No signal | No signal | No signal |

Table 5.2.3.1-5. Band 1&5 Diplexer Passive IMD level

| Component : Band5 Diplexer |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Category | Input F1 | Input F2 | Input F3 | F1 Power | F2 Power | F3 Power | Meas Frequency [MHz] | Comp 1 PIMD level [dBm] | Comp 2 PIMD level [dBm] | Comp 3 PIMD level [dBm] |
| 2-tone IM2 | 836.5 | 1950 |  | 20dBm | 20dBm |  | 2786.5 | -123 | -135 | -136 |
|  | 836.5 | 1950 |  | 20dBm | 20dBm |  | 1113.5 | -122 | -125 | -131 |
| 2-tone IM3 | 836.5 | 1950 |  | 20dBm | 20dBm |  | 3623 | -136 | -136 | -135 |
|  | 836.5 | 1950 |  | 20dBm | 20dBm |  | 277 | -129 | -132 | -131 |
| 3-tone IM3 | 849 | 1970 | 1980 | 20dBm | 20dBm | 20dBm | 859 | No signal | No signal | No signal |
|  | 824 | 1920 | 1930 | 20dBm | 20dBm | 20dBm | 814 | No signal | No signal | No signal |

From the Table 5.2.3.1-1 ~ Table 5.2.3.1-5, the passive IMD level is seen to be quite low with the IMD2 and IMD3 levels less than - 85dBm/MHz.  Therefore we can see that the passive IMD products can have negligible impact on the adjacent band UE. From the previous IMD analysis [1] and the measurements results of the RF components, we propose the followings:

Observation: The measurement of Passive IMD levels for real RF components and analysis of Active IMD levels reveals quite lower than -50dBm/MHz, which is the conventional UE-to-UE coexistence requirements.

********************** End of text proposal to TR 36.860 chapter 5.2.3 ***********************