3GPP TSG RAN WG4#57	R4-134909

Riga, Latvia	Agenda Item 3

October 7th – 11th, 2013

Source: Renesas Mobile Europe Ltd, BlackBerry Ltd

# On EARFCN extension in GERAN

# Introduction

Following earlier work in RAN4 and RAN2, GERAN2 has discussed since GERAN#57 how to accommodate additional bands and carrier frequencies beyond what is currently allowed by a 16-bit EARFCN numbering space. A liaison statement was sent from GERAN2 to RAN4 and RAN2 in GP-130844 [1] namely asking for RAN4 feedback on the proposed alternatives.

This contribution discusses these alternatives and suggests a way forward.

# Discussion

RAN4 initially requested RAN2 in R4-124948 to “study the signalling to extend E-UTRA operating band number as well as related EARFCN numbering space.” with a recommendation to extend the operating band number to 128 (from 64) and to extend the EARFCN numbering space accordingly. RAN2 later decided to expand four-fold the initial EARFCN numbering space (resulting in a 2-bit extension).

The 16-bit EARFCN usage in GERAN has been specified as of Rel-8 (i.e. first E-UTRA release). The support of new LTE bands and associated carrier frequencies can be introduced in the GERAN/E-UTRAN UE using the GERAN Rel-8 signalling baseline.

Extending the EARFCN numbering space beyond 16 bits (Alternative 2 in [1]) would not only require a large amount of signalling changes in GERAN, it would also eat up highly valuable signalling capacity on the GERAN BCCH; the addition of a new System Information message, as has been proposed, would also have negative knock-on effect on BCCH acquisition time of all UEs in a GERAN cell given the need for scheduling this new message. Furthermore, no live network deployment would make use of the extended coding until strictly necessary, since the Rel-8 definition is otherwise sufficient.

Generally, the usual work practice in GERAN, given tight signalling capacity limitations, is that no new [BCCH] signalling is defined unless strictly necessary.

Alternative 3 in [1] assumes no EARFCN extension but relies on the UE transferring via an intermediate E-UTRA layer (that can be signalled with the 16-bit EARFCN). However, this intermediate E-UTRA layer might, in some deployments, simply not exist. Said otherwise, requiring an intermediate E-UTRA layer puts constraints on E-UTRA deployments that may not be acceptable. [Noting that using alternative 3 for selected bands / scenarios combined with approach 1 would reduce by just as much the risk of 16-bit EARFCN numbering space exhaustion]

Alternative 1 in [1] has been discussed since GERAN#57 in March 2013 (GP-130431). It proposes a local mapping of EARFCN values in the extended range (> 65535) to 16-bit encoded EARFCN code points in GERAN, based on the following observations:

- EARFCNs indicating Uplink FDD carriers (Range of NUL for FDD in 3GPP TS 36.104) are not used for interworking purpose in GERAN and would thus be used for indicating additional carriers (FDD or TDD), should the 16-bit based EARFCNs address space (Range of NDL in 3GPP TS 36.104) get depleted. It is suggested that available 16-bit EARFCN values above the currently specified TDD EARFCNs should be used for specifying either TDD or FDD new bands before EARFCNs in the extended range (> 65535) are introduced.

- EARFCNs corresponding to bands that are only deployed in regions not supporting GERAN networks would never be signalled in GERAN thus could be reused for future relevant DL FDD or TDD carriers in GERAN

This local EARFCN mapping in GERAN would be specified (in 3GPP TS 36.104) such that the values in the non-extended EARFCNs numbering space would be used to indicate future Downlink FDD and TDD E-UTRA carriers. This proposal would avoid any signalling change in GERAN, whilst allowing the future introduction of new EUTRA bands (and associated carriers) –Rel-8 signalling would thus be reused unchanged.

Alternative 1 has been discussed in GERAN#57, #58 and #59; no issues have been reported against the use of this approach which avoids the downsides associated with Alternative 2 and Alternative 3 described above. Alternative 1 allows increasing significantly the number of EARFCNs that can be signalled to the UE in GERAN. This proposal not altering the existing Rel-8 signalling baseline has a minimal impact to 3GPP specifications (namely 3GPP TS 36.104) and only those UEs and networks implementations supporting future EARFCNs that will be specified in the extended range (> 65535) will be subject to a straightforward adaptation (i.e. mapping).

# COnclusion

Given Alternative 1 allows to significantly increase the number of EARFCNs that can be signalled using the 16-bit EARFCNs range, while not requiring any signalling changes in GERAN, it is recommended that RAN4 endorses Alternative 1 [1] i.e. the definition of a local EARFCN mapping for GERAN in 3GPP TS 36.104 such that:

- EARFCNs indicating Uplink FDD carriers (Range of NUL for FDD in 3GPP TS 36.104) which today are not used for interworking purpose in GERAN would now be used to indicate additional carriers (FDD or TDD), should the 16-bit based EARFCNs address space (Range of NDL in 3GPP TS 36.104) get depleted; and

- Available 16-bit EARFCN values above the currently specified TDD EARFCNs should be used for specifying either TDD or FDD new bands before EARFCNs in the extended range (> 65535) are introduced.

- EARFCNs corresponding to bands that are only deployed in regions not supporting GERAN networks would never be signalled in GERAN thus could be reused for future relevant DL FDD or TDD carriers in GERAN

A companion CR is available in [2] while a draft LS is attached to the present document (in the .zip file).

# REFERENCES

- GP-130844, LS on extending E-UTRA band number and EARFCN numbering space, GERAN2, 3GPP TSG GERAN#59

- R4-134910, CR 36.104 Definition of EARFCN to support additional carrier frequencies in GERAN, Renesas Mobile Europe Ltd, RAN4#68bis

# ANNEX A: EARFCN

The EARFCN identifies a carrier frequency in uplink and downlink as depicted in the table below where NDL indicates the EARFCN of DL carriers and NUL the EARFCN of UL uplink carriers. The yellow shading points to FDD bands (UL and DL spectrums divided in frequency domain i.e. duplex spacing) and the blue shading to TDD bands (same spectrum for UL and DL, divided in time domain) – while FDD UL/DL requires 2x EARFCNs, TDD UL/DL requires 1x EARFCN. An EARFCN is uniquely associated with a carrier frequency and a given band.

| E-UTRA Operating Band | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | FDL_low (MHz) | NOffs-DL | Range of NDL | FUL_low (MHz) | NOffs-UL | Range of NUL |
| 1 | 2110 | 0 | 0 – 599 | 1920 | 18000 | 18000 – 18599 |
| 2 | 1930 | 600 | 600 – 1199 | 1850 | 18600 | 18600 – 19199 |
| 3 | 1805 | 1200 | 1200 – 1949 | 1710 | 19200 | 19200 – 19949 |
| 4 | 2110 | 1950 | 1950 – 2399 | 1710 | 19950 | 19950 – 20399 |
| 5 | 869 | 2400 | 2400 – 2649 | 824 | 20400 | 20400 – 20649 |
| 6 | 875 | 2650 | 2650 – 2749 | 830 | 20650 | 20650 – 20749 |
| 7 | 2620 | 2750 | 2750 – 3449 | 2500 | 20750 | 20750 – 21449 |
| 8 | 925 | 3450 | 3450 – 3799 | 880 | 21450 | 21450 – 21799 |
| 9 | 1844.9 | 3800 | 3800 – 4149 | 1749.9 | 21800 | 21800 – 22149 |
| 10 | 2110 | 4150 | 4150 – 4749 | 1710 | 22150 | 22150 – 22749 |
| 11 | 1475.9 | 4750 | 4750 – 4949 | 1427.9 | 22750 | 22750 – 22949 |
| 12 | 729 | 5010 | 5010 - 5179 | 699 | 23010 | 23010 - 23179 |
| 13 | 746 | 5180 | 5180 – 5279 | 777 | 23180 | 23180 – 23279 |
| 14 | 758 | 5280 | 5280 – 5379 | 788 | 23280 | 23280 – 23379 |
| … |  |  |  |  |  |  |
| 17 | 734 | 5730 | 5730 – 5849 | 704 | 23730 | 23730 – 23849 |
| 18 | 860 | 5850 | 5850 – 5999 | 815 | 23850 | 23850 – 23999 |
| 19 | 875 | 6000 | 6000 – 6149 | 830 | 24000 | 24000 – 24149 |
| 20 | 791 | 6150 | 6150 – 6449 | 832 | 24150 | 24150 – 24449 |
| 21 | 1495.9 | 6450 | 6450 – 6599 | 1447.9 | 24450 | 24450 – 24599 |
| 22 | 3510 | 6600 | 6600 – 7399 | 3410 | 24600 | 24600 – 25399 |
| 23 | 2180 | 7500 | 7500 – 7699 | 2000 | 25500 | 25500 – 25699 |
| 24 | 1525 | 7700 | 7700 - 8039 | 1626.5 | 25700 | 25700 – 26039 |
| 25 | 1930 | 8040 | 8040 - 8689 | 1850 | 26040 | 26040 – 26689 |
| 26 | 859 | 8690 | 8690 - 9039 | 814 | 26690 | 26690 – 27039 |
| 27 | 852 | 9040 | 9040 – 9209 | 807 | 27040 | 27040 – 27209 |
| 282 | 758 | 9210 | 9210 – 9659 | 703 | 27210 | 27210 – 27659 |
| 292 | 717 | 9660 | 9660 – 9769 | N/A |  |  |
| 30 | 2350 | 9770 | 9770 – 9869 | 2305 | 27660 | 27660 – 27759 |
| 31 | 462.5 | 9870 | 9870 – 9919 | 452.5 | 27760 | 27760 – 27809 |
| … |  |  |  |  |  |  |
| 33 | 1900 | 36000 | 36000 – 36199 | 1900 | 36000 | 36000 – 36199 |
| 34 | 2010 | 36200 | 36200 – 36349 | 2010 | 36200 | 36200 – 36349 |
| 35 | 1850 | 36350 | 36350 – 36949 | 1850 | 36350 | 36350 – 36949 |
| 36 | 1930 | 36950 | 36950 – 37549 | 1930 | 36950 | 36950 – 37549 |
| 37 | 1910 | 37550 | 37550 – 37749 | 1910 | 37550 | 37550 – 37749 |
| 38 | 2570 | 37750 | 37750 – 38249 | 2570 | 37750 | 37750 – 38249 |
| 39 | 1880 | 38250 | 38250 – 38649 | 1880 | 38250 | 38250 – 38649 |
| 40 | 2300 | 38650 | 38650 – 39649 | 2300 | 38650 | 38650 – 39649 |
| 41 | 2496 | 39650 | 39650 –41589 | 2496 | 39650 | 39650 –41589 |
| 42 | 3400 | 41590 | 41590 – 43589 | 3400 | 41590 | 41590 – 43589 |
| 43 | 3600 | 43590 | 43590 – 45589 | 3600 | 43590 | 43590 – 45589 |
| 44 | 703 | 45590 | 45590 – 46589 | 703 | 45590 | 45590 – 46589 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. This implies that the first 7, 15, 25, 50, 75 and 100 channel numbers at the lower operating band edge and the last 6, 14, 24, 49, 74 and 99 channel numbers at the upper operating band edge shall not be used for channel bandwidths of 1.4, 3, 5, 10, 15 and 20 MHz respectively. NOTE 2: Restricted to E-UTRA operation when carrier aggregation is configured. |  |  |  |  |  |  |

Table 1. E-UTRAN Channel Numbers [4]

In GERAN specifications, only the EARFCN of DL carriers need to be identified for it is these carriers that are measured for mobility purpose. As a result, half of the FDD EARFCNs address space (NUL range) is unused in GERAN. It should also be noted that 65535 has been used as a special code point for indicating 18 bits coding by RAN2.

UL EARFCN are not deemed needed  (Duplex distance can be used and “RACH” configuration can be obtained once in the target cell)