3GPP TSG-RAN WG4 Meeting #68bis	R4-135515

Riga, Latvia, 7 – 11 October, 2013

Source:	NEC

Title:	Simulation results on in-band blocking for AAS co-existence study

Agenda item:	8.4.2 Deployment scenarios / Co-existence studies

Document for:	Discussion

# Introduction

In this contribution, we provide preliminary simulation results for AAS in-band blocking evaluations based on proposed simulation cases and assumptions for AAS coexistence study in [1].

# Discussion

## Simulation cases

System simulations were performed to evaluate the in-band blocking level. The in-band blocking power level were obtained by system level simulation based on un-coordinated deployment, and the in-band blocking level is the 99.99% level of the CDF of the total received power at the transceiver input.

Simulation cases are listed in Table1. Detailed simulation assumptions are provided in [1].

In cases 2a, Legacy (aggressor) to AAS (victim) performance was evaluated. In cases 2b, Legacy (aggressor) to Legacy (victim) was also evaluated as the baseline case.

Table 1 Simulation cases for in-band blocking

| Case | Aggressor | Victim | Simulated link | Statistics | Target RF requirement |
| --- | --- | --- | --- | --- | --- |
| 2a | Legacy E-UTRA Macro system: | Multi-column AAS E-UTRA Macro system | Uplink | Interferer levels at victim BS | In-band blocking |
| 2b(Baseline) | Legacy E-UTRA Macro system | Legacy E-UTRA Macro system | Uplink | Interferer levels at victim BS | In-band blocking |

Case 2a: Uplink Legacy E-UTRA Macro system aggressor – Multi-column AAS E-UTRA Macro system victim

Simulations are based on the following assumptions:

Aggressor system:	10 MHz E-UTRA with passive antenna system

Victim system:	10 MHz AAS E-UTRA macro system with horizontal cell splitting

Down-tilt angle:	9 degrees down-tilt in aggressor and victim system

Environment:	Macro Cell, Urban Area, uncoordinated deployment

Cell Range		750 m

Case 2b: Uplink Legacy E-UTRA Macro system aggressor – Legacy  E-UTRA Macro system victim

Simulations are based on the following assumptions:

Aggressor system:	10 MHz E-UTRA with passive antenna system

Victim system:	10 MHz E-UTRA with passive antenna system

Down-tilt angle:	9 degrees down-tilt in aggressor and victim system

Environment:	Macro Cell, Urban Area, uncoordinated deployment

Cell Range		750 m

## Simulation results

Table 2, Figures 1 and 2 show the simulation results for in-band blocking level at antenna connector.

Table 2: Case 2b simulation results summary

| Case | PC set 1 |  | PC set 2 |  |
| --- | --- | --- | --- | --- |
|  | Mechanical downtilt | Electrical downtilt | Mechanical downtilt | Electrical downtilt |
| 2a | -47.00 | -46.92 | -58.70 | -54.43 |
| 2b (Baseline) | -47.45 | -47.00 | -60.76 | -57.33 |

Figure 1 In-band blocking received at the transceiver boundary with PC1

Figure 2 In-band blocking received at the transceiver boundary with PC2

## Observation

Simulation results show similar trends with those provided by other companies at RAN4#68 in Barcelona [2,3,4].

# Conclusion

In this contribution, we provided initial simulation results of in-band blocking level based on the cases and assumptions for AAS coexistence study discussed at RAN4 #67 at Fukuoka.

Simulation results confirm that there is no significant difference in in-band blocking for AAS BS macro and legacy macro. It is therefore proposed to adopt in-band blocking values same as that of the legacy specifications.

# References

[1] R4-133123, Simulation scenarios and assumptions for AAS coexistence study, Huawei, Ericsson, Nokia Siemens Networks, NEC

[2] R4-133716, Simulation results for AAS in-band blocking, Huawei

[3] R4-134061, Preliminary AAS Receiver Blocking Simulation Results, NSN

[4] R4-134222, Initial results of AAS coexistence simulation campaign, ZTE, Tejet