3GPP TSG-RAN WG4 #68	R4-135625

Riga, Latvia, 7th – 11th, Oct. 2013

Source: 	Ericsson

Title:		TR 36.863 V0.3.1: Draft TP on average link level results for CRS-IM performance in homogeneous network

Agenda Item:	10.5.1

Document for:	Approval

# Discussion

In RAN4#66bis, system level simulation assumptions are agreed in [1]. After RAN4#67, different companies propose system level simulation based on the agreed system level simulation assumption and the agreed method. Based on the agreed system level side condition, different companies proposed link level simulation.   In this contribution, we propose the draft TP to capture the average link level throughput gain for CRS-IM performance in homogeneous network. The average is over the available results in RAN4#68.

# Reference

[1]	R4-131957, “System level simulation assumptions for LTE UE CRS-IM SI”, Ericsson, RAN4#66bis

# Text Proposal

< Start of draft text proposal for TR36.863 (TR for CRS-IM)>

<Start of the first change>

## Average simulation results

Table 7.3-1 provides the averaged simulation results for the agreed link-level simulation scenario on conditional IorjNoc . The details of simulation result by interested companies are provided in Clause 7.4.

Table 7.3-1: Average link simulation results for agreed link-level simulation scenario on conditional IorjNoc

|  | Set |  | Ior2NocIor2Noc |  | Ior3NocIor3Noc |  | Ior1NocIor1Noc |  | Average throughput Gain of CRS-IM over non-CRS-IM receiver |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10% |  | 1 |  | 2.5 |  | 0 |  | 4 |  | 10.04% |
|  |  | 5 |  | 8.9 |  | 4.6 |  | 8.2 |  | 28.07% |
|  |  | 10 |  | 11.7 |  | 5.7 |  | 10.2 |  | 36.80% |
|  |  | 15 |  | 14.7 |  | 10.1 |  | 13.3 |  | 56.00% |
|  |  | 20 |  | 31.6 |  | 30.2 |  | 30.6 |  | 88.05% |
| 20% |  | 1 |  | 1.7 |  | -0.7 |  | 3.2 |  | 7.69% |
|  |  | 5 |  | 7.7 |  | 3.6 |  | 7 |  | 22.04% |
|  |  | 10 |  | 10.4 |  | 4.6 |  | 8.9 |  | 27.87% |
|  |  | 15 |  | 13.4 |  | 9.2 |  | 12 |  | 42.44% |
|  |  | 20 |  | 30.3 |  | 28.9 |  | 29.4 |  | 82.32% |
| 30% |  | 1 |  | 1.3 |  | -0.7 |  | 3.1 |  | 7.11% |
|  |  | 5 |  | 6.9 |  | 3 |  | 6.3 |  | 17.38% |
|  |  | 10 |  | 9.7 |  | 3.7 |  | 8.2 |  | 24.47% |
|  |  | 15 |  | 12.8 |  | 7.8 |  | 11.2 |  | 34.03% |
|  |  | 20 |  | 29 |  | 27.7 |  | 28.1 |  | 77.70% |
| 40% |  | 1 |  | 0.5 |  | -1.7 |  | 2.1 |  | 5.32% |
|  |  | 5 |  | 6.1 |  | 1.9 |  | 5.4 |  | 16.13% |
|  |  | 10 |  | 8.8 |  | 2.7 |  | 7.2 |  | 19.86% |
|  |  | 15 |  | 11.7 |  | 7.1 |  | 10.3 |  | 27.09% |
|  |  | 20 |  | 28.8 |  | 27.4 |  | 27.8 |  | 72.74% |
| 50% |  | 1 |  | 0.2 |  | -1.6 |  | 1.9 |  | 4.49% |
|  |  | 5 |  | 5.7 |  | 1.2 |  | 4.9 |  | 13.42% |
|  |  | 10 |  | 8.4 |  | 1.7 |  | 6.7 |  | 18.04% |
|  |  | 15 |  | 11.4 |  | 5.9 |  | 9.8 |  | 24.03% |
|  |  | 20 |  | 28.8 |  | 27.4 |  | 27.8 |  | 63.31% |

<End of the first change>

< End of draft text proposal for TR36.863 (TR for CRS-IM)>