3GPP TSG-RAN WG4 #68bis	R1-135628

Riga, Latvia, 7th – 11th, Oct. 2013

Source: 	Ericsson

Title:	TR 36.863 V0.3.1: Draft TP on link and system level summary for CRS-IM performance in homogeneous network

Agenda Item:	10.5

Document for:	Approval

# Discussion

In RAN4#68 meeting, link level simulation results are submitted by several interested companies [2]~[7]. These results are summarized in [1]. Additionally, system level results are also proposed in [9] by some interested companies. In this paper, the conclusion based on the available results is proposed.

# Reference

R4-134481, Summary of link level simulation and side condition for CRS-IM, Ericsson, ST Ericsson, Noted

R4-133997, Link level results for CRS-IM, Ericsson, ST Ericsson, Noted

R4-133144, CRS IC Performance in Homogeneous Network, NEC, Noted

R4-133263, Link level simulation results for CRS-IM in homogeneous network, Huawei, HiSilicon, Noted

R4-133306, Preliminary link level simulaion results, ZTE, Noted

R4-133468, Link-level simulation results of CRS-IM, NSN, Nokia Corporation, Noted

R4-133853, Performance evaluation of CRS-IC and considerations on interference modelling, Renesas Mobile Europe Ltd, Noted

R4-133935, Link level performance  of CRS-IC for Homogeneous network, LG Electronics, Noted

R4-133998, Summary of system level simulation and side condition for CRS-IM, Ericsson, ST Ericsson, Noted

# Text Proposal

< Start of draft text proposal for TR36.863 (TR for CRS-IM)>

<Start of the first change>

## 7.4	Summary

From the link simulation results based on weighted IorjNoc , when the average resource utilization of the aggressor cells was 10% for cell-edge UEs, approximately [9%] gain for CRS based transmission was observed for the lower percentile, i.e., 5%-tile, of  IorjNoc ; For the median percentile, i.e., 50%-tile, of  IorjNoc , approximately [39%] gain can be achieved; For the higher percentile, i.e., 75%-tile, of  IorjNoc , approximately [58%] gain can be obtained. When the average resource utilization of aggressor cells is 50%,  approximately [4%, 16% and 23%] gain can be achieved for the lower percentile, median percentile and higher percentile of   IorjNoc , respectively.  IorjNoc

<End of the first change>

<Start of the second change>

## 8.4	Summary

The study concluded that there is indeed an increase in UPT (user perceived throughput) to be seen with low load (resource utilization is about 10%~50%). For the 5% UPT, the gain is in the order of 10-38% when resource utilization is 30%, additionally approximately 8 – 19 % gain for 50% UPT would be obtained.

<End of the second change>