3GPP TR 36.863 V0.4.0  (2013-11)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Networks;

Feasibility of CRS Interference Mitigation for LTE Homogenous Deployment Study Item

Technical Report;

(Release 12)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

<keyword[, keyword]>

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2011, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

- Contents

Foreword	4

Introduction	4

1	Scope	6

2	References	6

3	Definitions, symbols and abbreviations	7

3.1	Definitions	7

3.2	Symbols	7

3.3	Abbreviations	7

4	Receiver structure	7

5	Network Scenarios	7

6	Interference modelling	9

6.1	General	9

6.2	Statistical measurements	10

6.3	Interference profile	11

6.3.1	General	11

6.3.2	Average evaluation results	11

6.3.3	Evaluation results	14

7	Link performance characterization	31

7.1	General	31

7.2	Detail of simulation assumption for link-level simulation	31

7.3	Average simulation results	34

7.5	Summary	39

8	System performance characterization	39

8.1	General	39

8.2	Simulation results	39

8.3	Summary	41

9	Conclusions	41

Annex A: Change history	42

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

A study item for minimum performance requirements for interference Mitigation of Cell-specific Reference signals (CRS-IM) for LTE/LTE-Advanced (FDD/ TDD) homogeneous network was approved at the 3GPP RAN #59 meeting [1]. This technical report summarizes the work that RAN4 has accomplished in this study item to assess the feasibility of CRS-IM UE receivers for Homogenous deployment. These receivers attempt to mitigate the CRS interference that arises from users operating outside the serving cell. CRS-IM has been studied in the Rel-11 Work Item on FeICIC. Due to its benefits under the condition where interference from CRS dominates but is negligible from data assuming data RE muting, the CRS-IM has been standardized under the above operation conditions for heterogeneous deployments and the corresponding requirements were specified in Release 11. At the same time, the enhanced performance requirements in homogeneous network deployment for MMSE interference rejection combining (MMSE-IRC) receiver were extensively studied and specified in Release 11. The MMSE-IRC receiver can suppress both data and CRS interference without the need to differentiate them. However, the UE performance in homogenous deployment may be further improved by using the same CRS-IM capability as defined for FeICIC, especially when similar interference conditions arise under low traffic loading of neighbouring cells and under similar scope as in FeICIC i.e. synchronous network operation and UE reference receiver assuming single FFT processing.  In past link level evaluations, this type of interference has been modelled as almost blank subframe, as defined in FeICIC WI, or full traffic, as defined in MMSE-IRC receiver. The study item has developed models for this interference in terms of their powers relative to the total other cell interference power, and their resource allocation according to traffic loading levels.  Some of these previously identified scenarios studied in Release 11 for specifying MMSE-IRC receiver performance requirements and the corresponding link/system parameters are reused.

The baseline detector structure is that of an MMSE-IRC detector as defined in [2] combined with a CRS-IM receiver. The same CRS-IM receiver defined for FeICIC is reused here. LTE throughput estimates are developed using link level simulations. In addition, system level performance is assessed to determine the gains that interference mitigation receiver might provide in throughput and coverage. Complexity issues associated with implementing these types of receivers are also discussed. The content of each specific clause of the report is briefly described as follows.

Clause 1 of this document defines the scope and objectives of this feasibility study.

Clause 4 describes the receiver methods that can be applied to CRS-IM receivers.

Clause 5 describes the network scenarios that were defined and used to generate the interference statistics, which were then used to develop the interference models described in clause 6.

Clause 6 defines the interference models/profiles that were developed in order to assess the link level performance of enhanced receivers.

# Scope

The objectives of this Study Items are:

- Identify the partial traffic loading levels and other realistic system level parameters (e.g. traffic and interference models including interference level, time offset between cells and frequency offset between cells) and performance metrics for studying the feasibility of CRS IM in a synchronized homogenous network assuming:

  - 3GPP Case 1 as a starting point

  - non-colliding CRS between serving and dominant aggressor/interfering cells and

  - non-colliding CRS between dominant aggressor/interfering cells

  - The homogeneous deployment and relevant system parameters should reuse as much as possible those defined in the Rel-11 performance study of MMSE-IRC

- Identify the baseline receiver which can be used for evaluating the gain of CRS IM in a synchronized homogenous network considering:

  - Reuse of CRS IM receiver assumed for Release 11 FeICIC.

  - Reuse of MMSE-IRC receiver as the baseline receiver. MMSE-IRC does not differentiate CRS or data interference when suppressing them and was assumed for Release 11 work item on interference rejection combining.

- Agree on interference levels, interference models and simulation parameters for link level evaluations.

- Evaluate the system level and link level gains of CRS IM with respect to the baseline MMSE-IRC receiver in a synchronized homogenous network deployment under the various loading levels identified.

  - Gains of CRS IM from 1 and 2 aggressor cells CRS shall be evaluated and compared.

  - Modeling of realistic CRS IM receiver for system level performance evaluation should be clarified and agreed if possible

- Investigate UE complexity tradeoffs e.g. tradeoff between number of interferers to cancel and the number of transmit antenna ports.

- The introduction of CRS IM based receiver requirements for homogenous deployments will only use the existing Release 11 signaling of CRS assistance information.

Note 1: Interference from neighboring cells’ PDSCH is not the focus of this study item. Any interference from data, which varies according to the traffic loading level, is assumed to be suppressed in the same way as in Rel-11 with the baseline MMSE-IRC receivers.

Note 2: Non-colliding CRS is prioritized and will be studied first while colliding CRS is not precluded. But the study will focus only on UE demodulation aspects.

# References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	RP-130393, “New Study Item proposal: CRS Interference Mitigation for Homogenous Deployments,” Ericsson, RAN #59.

[2]	3GPP TR 36.829, Enhanced performance requirement for LTE User Equipment (UE), 3GPP, 2012

# Definitions, symbols and abbreviations

Delete from the above heading those words which are not applicable.

Clause numbering depends on applicability and should be renumbered accordingly.

## Definitions

Definition format (Normal)

<defined term>: <definition>.

## Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## Abbreviations

Abbreviation format (EW)

<ACRONYM>	<Explanation>

# Receiver structure

The baseline detector structure is that of an MMSE-IRC detector as defined in [2] combined with a CRS-IM receiver. The CRS IM receiver assumed for Release 11 FeICIC [1] is reused. The CRS-IM receiver is with the following assumption:

- Single FFT

- Mitigates CRS interference of up to 2 cells

- No restriction on whether mitigated CRS interference is intra-/inter-site, under the assumption of synchronous networks

- The CRS assistance information, as specified for Release 11, is available for the UE

The MMSE-IRC detector is MMSE-IRC based receiver with interference covariance matrix estimation as defined in TR36.829 as the baseline receiver [2], and additionally with the following assumptions:

- MMSE-IRC does not differentiate CRS or data interference when suppressing them.

# Network Scenarios

To estimate the link gain that UE CRS Interference mitigation (CRS-IM) receivers might provide for LTE/LTE-Advanced downlinks for homogeneous network it is necessary to first define the network scenarios under which the receivers must operate.

A network scenario for downlink performance evaluation is typically defined in terms of eNode B transmit characteristics, UE receive characteristics, traffic mix, inter-site distance, path loss model, etc. Once the network scenario(s) is defined one can then determine the associated interference profile/model that will be used in the actual link level characterization. This clause describes the network scenarios agreed to in this study, while the following clause defines the interference models that were developed based on system level simulations of these network scenarios.

The main system level assumptions are summarized in Table 5.1.

System level simulations were then conducted based on the above assumptions for the purposes of collecting interference statistics. Static system level simulators were deemed sufficient for this exercise, and are preferred over dynamic simulators since they are typically easier to develop and require less computation time. For every ‘iteration’ (or drop) in the static simulator UEs are randomly distributed across the simulated area and the relevant statistics collected. From these collected statistics certain key measures are developed, which provide some insight into how well a CRS-IM receiver might work.

Table 5.1: Simulation assumptions for network scenarios

| Parameter |  | 3GPP Case 1 |
| --- | --- | --- |
| Bandwidth |  | 10 MHz |
| Carrier frequency |  | 2000 MHz |
| Cellular Layout |  | Hexagonal grid, 19 cell sites, 3 sectors per site |
| Inter-site |  | 500 m |
| Distance-dependent path loss |  | L = 128.1 + 37.6log10(R), R: km |
| Shadowing standard deviation |  | 8 dB |
| Shadowing correlation | Between cells | 0.5 |
|  | Between sectors | 1.0 |
| Penetration loss |  | 20 dB |
| Antenna pattern | Horizontal | degrees, |
|  | Vertical | degrees, Antenna height at the base station is set to 32m. Antenna height at the UE is set to 1.5m. |
|  |  | degrees |
|  | Combining method in 3D antenna pattern |  |
|  | Antenna gain + connector loss | eNodeB: 14 dBi, UE: 0 dBi |
| Total BS TX power (Ptotal) |  | 46 dBm |
| Minimum distance between UE and Cell |  | >= 35 meters |
| Hard handover hysteresis |  | 3 dB |
| Traffic model |  | Non-full buffer FTP traffic Model 1 from TR36.814 [5] |
| Network synchronization |  | Synchronized |
| PCI planning |  | Planned cell ID layout with 3-CRS shift patterns (“Shifted CRS”) |
| UE distribution |  | Uniformly distributed, all users outdoors, speed 3 km/h |
| Noise figure |  | 9 dB in UE |
| DL Transmission schemes |  | Closed-loop spatial multiplexing, up to 2 layers, QPSK/16QAM/64QAM |
| File size, S |  | 2 Mbytes (0.5 Mbytes optional) (one user downloads a single file) |
| Target average resource utilization (RU) |  | [0, 10%, 20%,30%,40%,50%, 100% (Low priority)] |
| User arrival rate λ |  | Poisson distributed with arrival rate λ |
| Possible range of λ |  | Each company to adjust λ to match target resource utilization |
| Geometry |  | TBD |
| UE receiver |  | MMSE-IRC without CRS-IM |
|  |  | MMSE-IRC with CRS-IM for 1st strongest interfering cell |
|  |  | MMSE-IRC with CRS-IM for 1st and 2nd strongest interfering cells |

# Interference modelling

## General

In this clause, the interference models/profiles that were developed in order to assess the link level performance of CRS-IM receivers are defined. Clause 6.2 defines a number of statistical measures that were defined during the study, and which provide useful insight into understanding the interference environment. System level simulations were conducted to generate results for the statistical measures defined in clause 6.2. Based on these simulation results interference profiles were developed, which were used in the link level performance and contributed to account performance characterization described in clause 8.

## Statistical measurements

Network interference statistics are computed using the following defined measures. Noc is defined as

Noc=M+2NBSIorj+σ2,

where Iorj is the received power spectral density of the j-th strongest base station measured at serving cell CRS REs (average power obtained within the RE and normalized to the subcarrier spacing). Specifically,  Ior1 denotes the received power spectral density of serving cell, Iorj(j=2,⋯M+1) denotes the received power spectral density of the (j-1)th dominant interfering cells. is the power spectral density of thermal noise (average power per RE normalised), and NBS is the total number of base stations considered including the serving cell, M is the number of dominant interfering cell(s) which interference can be mitigated by the UE. The quantity Noc models the interference from all cells, excluding the contribution of the M dominant interfering cell(s) which interference is mitigated. It is noted that the conducted studies assume M=2.

The interference profiles for the dominant interfering cells are defined by IorjNoc (j=2, ⋯,M+1), and the serving cell signal profiles are characterized by  Ior1Noc.

Geometry G is defined as

G=Ior1Ioc=Ior1j=2j=M+1Iorj+Noc

Here, all the interfering BSs are assumed with full load.

6.2.1 Methodology for statistical measurements

This clause led to the development of a method to derive IorjNoc (j=1, ⋯,M+1)   based on the following steps:

- Step 1: Decide envisioned G value (G) based on the specified percentile of G distribution assuming full load for all the interfering cells.

- Step 2: Select UEs. From a sample of randomly dropped UEs, select those UEs with G close to G with a tolerance of +/- 0.2 dB. IorjNoc (j=1, ⋯,M+1) values are logged for those UEs, and multiple realizations are performed in order to obtain a significant number of samples.

- Step 3: Decide IorjNoc (j=1, ⋯,M+1): After saving the condition IorjNoc (j=1, ⋯,M+1) values from all samples, the Ior2/Noc values are sorted in ascending order. The data set is binned in 5-percentile bands; and a mean of IorjNoc (j=1, ⋯,M+1) inside each 5-percentile band is taken, yielding one characteristic set of  IorjNoc (j=1, ⋯,M+1) values per each 5-percentile band. At the end of the process, 20 characteristic set of  IorjNoc (j=1, ⋯,M+1) values are obtained.

- Note 1: the “mean” of  IorjNoc (j=1, ⋯,M+1) inside each 5-percentile band is performed in the linear domain.

To account for the partial loading on the non-dominant interferers, two methods are developed to compute Noc:

- In the first method, Noc is the Noc averaged across time with target resource utilization assume partial loading and no CRS-IM is assumed.

- In the second method, Noc is obtained as

Noc=M+2NBSαIorj+σ2,

where α∈0,1 is the resource utilization factor in the interfering cells.

## Interference profile

### General

This clause presents interference characterization results leading to the development of the interference profile.

### Average evaluation results

This sub-clause shows the averages of evaluation results. The details of evaluation results by interested companies are shown in sub-clause 6.3.3.

Table 6.3.2-1: Average of the evaluation results on 5%-tile Geometry when RU=0%

| Set | Ior2Noc | Ior3Noc |  | Ior1Noc |
| --- | --- | --- | --- | --- |
| 1 | 3.936651 | 1.026357 | 5.07858 |  |
| 2 | 7.658806 | 4.03515 | 7.805429 |  |
| 3 | 9.387058 | 5.473668 | 9.156139 |  |
| 4 | 10.70342 | 7.04701 | 10.29892 |  |
| 5 | 11.78453 | 7.119031 | 10.91593 |  |
| 6 | 12.64114 | 8.263151 | 11.8317 |  |
| 7 | 13.4074 | 8.616283 | 12.42567 |  |
| 8 | 14.20531 | 9.154887 | 13.0301 |  |
| 9 | 14.97049 | 9.78623 | 13.70621 |  |
| 10 | 15.81172 | 10.5102 | 14.48979 |  |
| 11 | 16.52123 | 11.0318 | 15.11163 |  |
| 12 | 17.30213 | 12.70506 | 16.05681 |  |
| 13 | 18.10986 | 13.20611 | 16.69122 |  |
| 14 | 18.92898 | 14.43306 | 17.57818 |  |
| 15 | 19.91031 | 15.76431 | 18.68795 |  |
| 16 | 21.29979 | 17.10381 | 19.91786 |  |
| 17 | 22.97898 | 18.29432 | 21.36742 |  |
| 18 | 25.18765 | 21.57824 | 23.77741 |  |
| 19 | 28.45329 | 25.59022 | 27.22571 |  |
| 20 | 37.12942 | 35.29573 | 36.07379 |  |

Table 6.3.2-2: Average of the evaluation results on 5%-tile Geometry when RU=10%

| Set | Ior2Noc | Ior3Noc |  | Ior1Noc |
| --- | --- | --- | --- | --- |
| 1 | 2.482849 | -0.0312 | 3.955109 |  |
| 2 | 5.693844 | 2.353362 | 5.986044 |  |
| 3 | 7.035891 | 3.25546 | 6.862847 |  |
| 4 | 8.08061 | 4.380014 | 7.682037 |  |
| 5 | 8.869275 | 4.644348 | 8.157994 |  |
| 6 | 9.61217 | 4.442327 | 8.60485 |  |
| 7 | 10.16532 | 5.571919 | 9.148012 |  |
| 8 | 10.673 | 5.13288 | 9.370735 |  |
| 9 | 11.1823 | 5.240518 | 9.734548 |  |
| 10 | 11.74644 | 5.692049 | 10.18381 |  |
| 11 | 12.30188 | 6.866584 | 10.80951 |  |
| 12 | 12.79584 | 7.279251 | 11.22922 |  |
| 13 | 13.36725 | 8.653107 | 11.926 |  |
| 14 | 13.94071 | 8.957495 | 12.39912 |  |
| 15 | 14.7127 | 10.0816 | 13.28238 |  |
| 16 | 15.98104 | 11.16859 | 14.36077 |  |
| 17 | 17.51079 | 12.12487 | 15.69327 |  |
| 18 | 19.73907 | 15.45547 | 18.12823 |  |
| 19 | 23.63959 | 21.71583 | 22.57779 |  |
| 20 | 31.56395 | 30.1844 | 30.58473 |  |

Table 6.3.2-3: Average of the evaluation results on 5%-tile Geometry when RU=20%

| Set | Ior2Noc | Ior3Noc |  | Ior1Noc |
| --- | --- | --- | --- | --- |
| 1 | 1.688324 | -0.74216 | 3.245691 |  |
| 2 | 4.702463 | 1.411341 | 5.03835 |  |
| 3 | 5.969692 | 2.319278 | 5.8034 |  |
| 4 | 6.886599 | 3.099591 | 6.465793 |  |
| 5 | 7.711274 | 3.623639 | 7.026627 |  |
| 6 | 8.368525 | 3.263815 | 7.336721 |  |
| 7 | 8.927038 | 3.732688 | 7.79261 |  |
| 8 | 9.440027 | 4.18878 | 8.191179 |  |
| 9 | 9.879704 | 3.818742 | 8.388394 |  |
| 10 | 10.44676 | 4.57008 | 8.920975 |  |
| 11 | 10.96829 | 5.49135 | 9.456105 |  |
| 12 | 11.4917 | 6.139498 | 9.952662 |  |
| 13 | 12.04102 | 6.556575 | 10.42053 |  |
| 14 | 12.62303 | 7.520182 | 11.03634 |  |
| 15 | 13.3614 | 9.207155 | 12.02681 |  |
| 16 | 14.63617 | 9.856199 | 13.05097 |  |
| 17 | 16.00842 | 10.7457 | 14.2136 |  |
| 18 | 18.29734 | 13.92034 | 16.6987 |  |
| 19 | 22.42477 | 20.6187 | 21.38866 |  |
| 20 | 30.33475 | 28.92528 | 29.35108 |  |

Table 6.3.2-4: Average of the evaluation results on 5%-tile Geometry when RU=30%

| Set | Ior2Noc | Ior3Noc |  | Ior1Noc |
| --- | --- | --- | --- | --- |
| 1 | 1.340471 | -0.73621 | 3.066401 |  |
| 2 | 3.922121 | 0.993835 | 4.484912 |  |
| 3 | 5.187652 | 1.922775 | 5.238305 |  |
| 4 | 6.05476 | 2.421963 | 5.779836 |  |
| 5 | 6.898605 | 3.003966 | 6.321041 |  |
| 6 | 7.553384 | 3.273023 | 6.799797 |  |
| 7 | 8.160243 | 2.876637 | 7.058987 |  |
| 8 | 8.630169 | 3.315201 | 7.446207 |  |
| 9 | 9.124317 | 3.271485 | 7.711411 |  |
| 10 | 9.693093 | 3.685917 | 8.182885 |  |
| 11 | 10.19666 | 3.780769 | 8.552738 |  |
| 12 | 10.78737 | 5.862466 | 9.369883 |  |
| 13 | 11.30776 | 5.396398 | 9.583789 |  |
| 14 | 11.96284 | 6.454303 | 10.30369 |  |
| 15 | 12.76359 | 7.823391 | 11.21865 |  |
| 16 | 13.99194 | 9.088526 | 12.36336 |  |
| 17 | 15.3589 | 10.74759 | 13.76737 |  |
| 18 | 17.44036 | 13.573 | 16.01893 |  |
| 19 | 21.18595 | 19.28102 | 20.15115 |  |
| 20 | 29.049 | 27.70898 | 28.10518 |  |

Table 6.3.2-5: Average of the evaluation results on 5%-tile Geometry when RU=40%

| Set | Ior2Noc | Ior3Noc |  | Ior1Noc |
| --- | --- | --- | --- | --- |
| 1 | 0.461833 | -1.67387 | 2.133637 |  |
| 2 | 3.25917 | -0.09161 | 3.58993 |  |
| 3 | 4.514175 | 0.946027 | 4.364843 |  |
| 4 | 5.400436 | 1.566172 | 4.956724 |  |
| 5 | 6.133651 | 1.913434 | 5.42123 |  |
| 6 | 6.748279 | 1.7434 | 5.730042 |  |
| 7 | 7.313233 | 2.000899 | 6.153179 |  |
| 8 | 7.748046 | 2.278033 | 6.445109 |  |
| 9 | 8.225228 | 2.145845 | 6.709672 |  |
| 10 | 8.787896 | 2.744118 | 7.222942 |  |
| 11 | 9.285207 | 3.698937 | 7.755658 |  |
| 12 | 9.819048 | 4.645791 | 8.324226 |  |
| 13 | 10.37295 | 5.212933 | 8.840054 |  |
| 14 | 10.94882 | 5.269169 | 9.20864 |  |
| 15 | 11.68908 | 7.118384 | 10.25283 |  |
| 16 | 12.91856 | 8.448295 | 11.38806 |  |
| 17 | 14.2388 | 9.174693 | 12.54038 |  |
| 18 | 16.53651 | 11.93096 | 14.88085 |  |
| 19 | 20.82927 | 19.10101 | 19.81171 |  |
| 20 | 28.79057 | 27.39967 | 27.81409 |  |

Table 6.3.2-6: Average of the evaluation results on 5%-tile Geometry when RU=50%

| Set | Ior2Noc | Ior3Noc |  | Ior1Noc |
| --- | --- | --- | --- | --- |
| 1 | 0.193767 | -1.62617 | 1.905836 |  |
| 2 | 2.812384 | -0.32936 | 3.209827 |  |
| 3 | 4.014475 | 0.432066 | 3.90119 |  |
| 4 | 4.938483 | 1.174056 | 4.54566 |  |
| 5 | 5.712775 | 1.222399 | 4.939935 |  |
| 6 | 6.318586 | 1.865218 | 5.408159 |  |
| 7 | 6.86655 | 1.788057 | 5.761889 |  |
| 8 | 7.332964 | 1.675406 | 6.005829 |  |
| 9 | 7.857275 | 1.295423 | 6.294396 |  |
| 10 | 8.360306 | 1.663436 | 6.69237 |  |
| 11 | 8.863422 | 3.242024 | 7.313906 |  |
| 12 | 9.372357 | 3.993497 | 7.816062 |  |
| 13 | 9.975573 | 4.24023 | 8.329432 |  |
| 14 | 10.61356 | 4.543516 | 8.786584 |  |
| 15 | 11.39361 | 5.931015 | 9.760934 |  |
| 16 | 12.52511 | 8.021431 | 10.95724 |  |
| 17 | 13.88555 | 9.659173 | 12.42197 |  |
| 18 | 15.93891 | 11.59175 | 14.36208 |  |
| 19 | 19.70556 | 17.20405 | 18.49089 |  |
| 20 | 28.81026 | 27.41629 | 27.82856 |  |

### Evaluation results

This sub-clause shows the evaluation results to show IorjNoc (j=1, ⋯,M). Table 6.3.3-1 ~ Table 6.3.3-39 present the evaluation results generated by various interested companies.

Table 6.3.3-1: Evaluation results on 5%-tile Geometry when RU=0% (Ericsson)

| Set | Ior2Noc | Ior3Noc |  | Ior1Noc |
| --- | --- | --- | --- | --- |
| 1 | 4.0965 | 1.7782 | 4.9246 |  |
| 2 | 6.1674 | 1.7369 | 5.8911 |  |
| 3 | 7.1445 | 2.6459 | 6.5477 |  |
| 4 | 8.0094 | 4.9316 | 7.4873 |  |
| 5 | 8.6782 | 4.1177 | 7.6057 |  |
| 6 | 9.1652 | 4.1091 | 7.9513 |  |
| 7 | 9.525 | 4.1407 | 8.2168 |  |
| 8 | 9.947 | 4.0801 | 8.4831 |  |
| 9 | 10.4993 | 4.3961 | 8.883 |  |
| 10 | 11.1632 | 3.3492 | 9.2125 |  |
| 11 | 11.632 | 3.5642 | 9.6213 |  |
| 12 | 12.1829 | 10.0218 | 11.3298 |  |
| 13 | 12.8271 | 7.5578 | 11.1193 |  |
| 14 | 13.6201 | 11.6319 | 12.7272 |  |
| 15 | 14.78 | 13.6389 | 14.2306 |  |
| 16 | 17.4855 | 13.9272 | 15.9238 |  |
| 17 | 20.1342 | 14.4479 | 17.9289 |  |
| 18 | 23.3618 | 21.1991 | 22.0175 |  |
| 19 | 25.8912 | 25.2038 | 25.2038 |  |
| 20 | 30.2131 | 28.3017 | 29.0742 |  |

Table 6.3.3-2: Evaluation results on 5%-tile Geometry when RU=10% (Ericsson)

| Set | Ior2Noc | Ior3Noc |  | Ior1Noc |
| --- | --- | --- | --- | --- |
| 1 | 3.5163 | 1.2147 | 4.3469 |  |
| 2 | 5.5766 | 1.0577 | 5.2623 |  |
| 3 | 6.4835 | 2.5451 | 6.0029 |  |
| 4 | 7.3993 | 3.7687 | 6.7529 |  |
| 5 | 7.9266 | 3.6329 | 6.9649 |  |
| 6 | 8.5183 | 2.7062 | 7.1462 |  |
| 7 | 8.9451 | 4.6591 | 7.8545 |  |
| 8 | 9.2962 | 3.2881 | 7.7796 |  |
| 9 | 9.7268 | 2.0575 | 7.8981 |  |
| 10 | 10.429 | 3.6572 | 8.6226 |  |
| 11 | 11.0701 | 5.5688 | 9.4184 |  |
| 12 | 11.6392 | 7.7812 | 10.2895 |  |
| 13 | 12.3288 | 9.3989 | 11.2083 |  |
| 14 | 12.9538 | 10.3426 | 11.8398 |  |
| 15 | 14.1146 | 12.8221 | 13.5178 |  |
| 16 | 16.7213 | 13.1824 | 15.1644 |  |
| 17 | 19.7537 | 14.2168 | 17.5753 |  |
| 18 | 22.7968 | 20.637 | 21.4566 |  |
| 19 | 25.3857 | 24.6973 | 24.6973 |  |
| 20 | 29.6656 | 27.7942 | 28.5431 |  |

Table 6.3.3-3: Evaluation results on 5%-tile Geometry when RU=20% (Ericsson)

| Set | Ior2Noc | Ior3Noc |  | Ior1Noc |
| --- | --- | --- | --- | --- |
| 1 | 3.0393 | 0.7111 | 3.8656 |  |
| 2 | 4.994 | 0.5338 | 4.7219 |  |
| 3 | 5.9636 | 2.0736 | 5.4303 |  |
| 4 | 6.7551 | 2.8037 | 6.0653 |  |
| 5 | 7.4232 | 3.6852 | 6.5851 |  |
| 6 | 7.9914 | 2.0989 | 6.568 |  |
| 7 | 8.4309 | 3.4433 | 7.1772 |  |
| 8 | 8.8418 | 3.3645 | 7.4437 |  |
| 9 | 9.1376 | 1.5479 | 7.3293 |  |
| 10 | 9.9406 | 3.2458 | 8.1662 |  |
| 11 | 10.4595 | 5.1425 | 8.872 |  |
| 12 | 11.1755 | 7.3394 | 9.806 |  |
| 13 | 11.7827 | 8.8282 | 10.6669 |  |
| 14 | 12.4318 | 9.5146 | 11.2208 |  |
| 15 | 13.576 | 12.4446 | 13.0319 |  |
| 16 | 16.3664 | 12.8714 | 14.8211 |  |
| 17 | 19.0009 | 13.3338 | 16.7986 |  |
| 18 | 22.1964 | 20.7543 | 21.1836 |  |
| 19 | 24.7909 | 23.7677 | 23.9322 |  |
| 20 | 29.1815 | 27.3418 | 28.0702 |  |

Table 6.3.3-4: Evaluation results on 5%-tile Geometry when RU=30% (Ericsson)

| Set | Ior2Noc | Ior3Noc |  | Ior1Noc |
| --- | --- | --- | --- | --- |
| 1 | 2.5834 | 0.27 | 3.42 |  |
| 2 | 4.4712 | -0.06 | 4.17 |  |
| 3 | 5.468 | 1.10 | 4.91 |  |
| 4 | 6.3746 | 3.11 | 5.82 |  |
| 5 | 6.9314 | 2.84 | 5.93 |  |
| 6 | 7.552 | 2.65 | 6.37 |  |
| 7 | 7.9066 | 2.36 | 6.61 |  |
| 8 | 8.2636 | 2.39 | 6.81 |  |
| 9 | 8.6608 | 1.18 | 6.79 |  |
| 10 | 9.3967 | 1.78 | 7.51 |  |
| 11 | 9.9716 | 2.76 | 8.06 |  |
| 12 | 10.6743 | 8.47 | 9.82 |  |
| 13 | 11.3093 | 6.18 | 9.62 |  |
| 14 | 11.9799 | 9.85 | 11.03 |  |
| 15 | 13.1589 | 12.07 | 12.63 |  |
| 16 | 15.8386 | 12.42 | 14.32 |  |
| 17 | 18.5205 | 12.73 | 16.30 |  |
| 18 | 21.693 | 20.27 | 20.69 |  |
| 19 | 24.2764 | 23.25 | 23.41 |  |
| 20 | 28.8015 | 26.96 | 27.69 |  |

Table 6.3.3-5: Evaluation results on 5%-tile Geometry when RU=40% (Ericsson)

| Set | Ior2Noc | Ior3Noc |  | Ior1Noc |
| --- | --- | --- | --- | --- |
| 1 | 2.2 | -0.1034 | 3.0423 |  |
| 2 | 4.067 | -0.4292 | 3.7896 |  |
| 3 | 5.1143 | 1.2151 | 4.614 |  |
| 4 | 5.9796 | 2.2487 | 5.3023 |  |
| 5 | 6.5987 | 2.6773 | 5.6669 |  |
| 6 | 6.971 | 1.5989 | 5.723 |  |
| 7 | 7.4951 | 2.4231 | 6.2627 |  |
| 8 | 7.7815 | 1.8054 | 6.2963 |  |
| 9 | 8.2233 | 0.9184 | 6.3925 |  |
| 10 | 8.9853 | 1.6591 | 7.1154 |  |
| 11 | 9.5494 | 4.6297 | 8.0477 |  |
| 12 | 10.2471 | 7.0822 | 9.079 |  |
| 13 | 10.8842 | 8.481 | 9.9193 |  |
| 14 | 11.6361 | 7.4302 | 10.0691 |  |
| 15 | 12.7594 | 11.6489 | 12.2218 |  |
| 16 | 15.4238 | 12.0631 | 13.9183 |  |
| 17 | 17.9818 | 12.109 | 15.7452 |  |
| 18 | 21.2912 | 19.8444 | 20.2742 |  |
| 19 | 23.8717 | 22.8502 | 23.0141 |  |
| 20 | 28.3591 | 26.5279 | 27.2529 |  |

Table 6.3.3-6: Evaluation results on 5%-tile Geometry when RU=50% (Ericsson)

| Set | Ior2Noc | Ior3Noc |  | Ior1Noc |
| --- | --- | --- | --- | --- |
| 1 | 1.66 | -0.694 | 2.4773 |  |
| 2 | 3.7102 | -0.7832 | 3.3987 |  |
| 3 | 4.7239 | 0.2804 | 4.1639 |  |
| 4 | 5.658 | 2.6694 | 5.1396 |  |
| 5 | 6.2247 | 1.3402 | 5.0694 |  |
| 6 | 6.5929 | 1.7029 | 5.4593 |  |
| 7 | 7.0581 | 2.0396 | 5.8361 |  |
| 8 | 7.4949 | 1.8205 | 6.0462 |  |
| 9 | 7.9326 | 1.2891 | 6.2256 |  |
| 10 | 8.5997 | 0.4051 | 6.6098 |  |
| 11 | 9.2187 | 3.1217 | 7.5238 |  |
| 12 | 9.8034 | 6.2402 | 8.5209 |  |
| 13 | 10.4598 | 8.6844 | 9.6954 |  |
| 14 | 11.2163 | 7.087 | 9.6697 |  |
| 15 | 12.4517 | 11.3328 | 11.9105 |  |
| 16 | 15.0892 | 11.7212 | 13.5821 |  |
| 17 | 17.6509 | 11.9487 | 15.4433 |  |
| 18 | 20.9204 | 19.4938 | 19.9123 |  |
| 19 | 23.6473 | 22.6315 | 22.7923 |  |
| 20 | 27.8753 | 26.0185 | 26.7586 |  |

Table 6.3.3-7: Evaluation results on 5%-tile Geometry when RU=0% (Renesas Mobile Europe Ltd)

| Set | Ior2Noc | Ior3Noc |  | Ior1Noc |
| --- | --- | --- | --- | --- |
| 1 | 0.28 | -1.85 | 2.25 |  |
| 2 | 3.88 | 0.54 | 4.21 |  |
| 3 | 5.17 | 1.79 | 5.24 |  |
| 4 | 6.12 | 3.06 | 5.90 |  |
| 5 | 6.94 | 2.77 | 6.21 |  |
| 6 | 7.62 | 3.79 | 6.86 |  |
| 7 | 8.16 | 4.27 | 7.33 |  |
| 8 | 8.68 | 4.46 | 7.61 |  |
| 9 | 9.20 | 5.05 | 8.09 |  |
| 10 | 9.76 | 5.17 | 8.39 |  |
| 11 | 10.22 | 6.02 | 8.89 |  |
| 12 | 10.77 | 5.48 | 9.17 |  |
| 13 | 11.35 | 7.03 | 9.93 |  |
| 14 | 11.88 | 6.26 | 10.08 |  |
| 15 | 12.57 | 7.53 | 10.88 |  |
| 16 | 13.38 | 8.23 | 11.57 |  |
| 17 | 14.44 | 8.73 | 12.42 |  |
| 18 | 16.29 | 12.62 | 14.69 |  |
| 19 | 19.84 | 17.38 | 18.42 |  |
| 20 | 28.25 | 26.51 | 26.98 |  |

Table 6.3.3-8: Evaluation results on 5%-tile Geometry when RU=10% (Renesas Mobile Europe Ltd)

| Set | Ior2Noc | Ior3Noc |  | Ior1Noc |
| --- | --- | --- | --- | --- |
| 1 | -0.23 | -2.25 | 1.87 |  |
| 2 | 3.30 | 0.22 | 3.79 |  |
| 3 | 4.53 | 1.29 | 4.56 |  |
| 4 | 5.51 | 2.12 | 5.16 |  |
| 5 | 6.30 | 2.22 | 5.68 |  |
| 6 | 6.91 | 2.66 | 6.08 |  |
| 7 | 7.47 | 3.65 | 6.56 |  |
| 8 | 7.99 | 4.09 | 7.00 |  |
| 9 | 8.48 | 3.90 | 7.23 |  |
| 10 | 9.03 | 4.47 | 7.71 |  |
| 11 | 9.51 | 5.06 | 8.13 |  |
| 12 | 9.96 | 5.17 | 8.48 |  |
| 13 | 10.50 | 6.11 | 8.99 |  |
| 14 | 11.06 | 5.72 | 9.33 |  |
| 15 | 11.61 | 6.79 | 9.93 |  |
| 16 | 12.36 | 6.78 | 10.44 |  |
| 17 | 13.28 | 7.52 | 11.27 |  |
| 18 | 15.01 | 11.09 | 13.28 |  |
| 19 | 18.63 | 16.47 | 17.30 |  |
| 20 | 27.39 | 25.82 | 26.18 |  |

Table 6.3.3-9: Evaluation results on 5%-tile Geometry when RU=20% (Renesas Mobile Europe Ltd)

| Set | Ior2Noc | Ior3Noc |  | Ior1Noc |
| --- | --- | --- | --- | --- |
| 1 | -0.70 | -2.73 | 1.48 |  |
| 2 | 2.76 | -0.34 | 3.26 |  |
| 3 | 4.01 | 0.70 | 3.98 |  |
| 4 | 4.97 | 1.50 | 4.64 |  |
| 5 | 5.72 | 1.70 | 5.09 |  |
| 6 | 6.34 | 1.94 | 5.46 |  |
| 7 | 6.90 | 2.91 | 5.97 |  |
| 8 | 7.39 | 3.49 | 6.40 |  |
| 9 | 7.87 | 3.22 | 6.59 |  |
| 10 | 8.37 | 3.74 | 7.03 |  |
| 11 | 8.87 | 4.48 | 7.51 |  |
| 12 | 9.34 | 5.32 | 8.00 |  |
| 13 | 9.85 | 4.91 | 8.21 |  |
| 14 | 10.37 | 5.35 | 8.69 |  |
| 15 | 10.88 | 5.77 | 9.14 |  |
| 16 | 11.56 | 6.17 | 9.68 |  |
| 17 | 12.40 | 6.78 | 10.40 |  |
| 18 | 14.13 | 9.71 | 12.26 |  |
| 19 | 17.87 | 15.93 | 16.62 |  |
| 20 | 26.77 | 25.28 | 25.59 |  |

Table 6.3.3-10: Evaluation results on 5%-tile Geometry when RU=30% (Renesas Mobile Europe Ltd)

| Set | Ior2Noc | Ior3Noc |  | Ior1Noc |
| --- | --- | --- | --- | --- |
| 1 | -1.13 | -3.18 | 1.05 |  |
| 2 | 2.28 | -0.92 | 2.76 |  |
| 3 | 3.54 | 0.37 | 3.57 |  |
| 4 | 4.49 | 1.07 | 4.15 |  |
| 5 | 5.22 | 1.23 | 4.61 |  |
| 6 | 5.83 | 1.40 | 4.95 |  |
| 7 | 6.38 | 1.89 | 5.35 |  |
| 8 | 6.85 | 2.93 | 5.84 |  |
| 9 | 7.36 | 2.89 | 6.11 |  |
| 10 | 7.83 | 3.10 | 6.45 |  |
| 11 | 8.32 | 3.55 | 6.87 |  |
| 12 | 8.81 | 4.96 | 7.49 |  |
| 13 | 9.30 | 4.79 | 7.78 |  |
| 14 | 9.79 | 4.41 | 8.02 |  |
| 15 | 10.26 | 5.42 | 8.56 |  |
| 16 | 10.90 | 5.39 | 9.00 |  |
| 17 | 11.74 | 5.76 | 9.65 |  |
| 18 | 13.44 | 8.90 | 11.54 |  |
| 19 | 17.26 | 15.48 | 16.06 |  |
| 20 | 26.27 | 24.82 | 25.10 |  |

Table 6.3.3-11: Evaluation results on 5%-tile Geometry when RU=40% (Renesas Mobile Europe Ltd)

| Set | Ior2Noc | Ior3Noc |  | Ior1Noc |
| --- | --- | --- | --- | --- |
| 1 | -1.52 | -3.47 | 0.72 |  |
| 2 | 1.86 | -1.28 | 2.33 |  |
| 3 | 3.11 | -0.21 | 3.11 |  |
| 4 | 4.06 | 0.63 | 3.73 |  |
| 5 | 4.76 | 0.81 | 4.16 |  |
| 6 | 5.38 | 1.00 | 4.50 |  |
| 7 | 5.91 | 1.13 | 4.83 |  |
| 8 | 6.39 | 2.32 | 5.34 |  |
| 9 | 6.90 | 2.30 | 5.62 |  |
| 10 | 7.36 | 2.73 | 5.99 |  |
| 11 | 7.84 | 3.44 | 6.45 |  |
| 12 | 8.33 | 4.30 | 6.98 |  |
| 13 | 8.82 | 4.16 | 7.25 |  |
| 14 | 9.26 | 4.23 | 7.57 |  |
| 15 | 9.75 | 5.11 | 8.09 |  |
| 16 | 10.35 | 4.13 | 8.30 |  |
| 17 | 11.20 | 5.28 | 9.12 |  |
| 18 | 12.85 | 8.35 | 10.95 |  |
| 19 | 16.74 | 15.15 | 15.62 |  |
| 20 | 25.83 | 24.40 | 24.68 |  |

Table 6.3.3-12: Evaluation results on 5%-tile Geometry when RU=50% (Renesas Mobile Europe Ltd)

| Set | Ior2Noc | Ior3Noc |  | Ior1Noc |
| --- | --- | --- | --- | --- |
| 1 | -1.88 | -3.75 | 0.38 |  |
| 2 | 1.48 | -1.54 | 1.98 |  |
| 3 | 2.72 | -0.78 | 2.67 |  |
| 4 | 3.66 | 0.22 | 3.34 |  |
| 5 | 4.34 | 0.51 | 3.75 |  |
| 6 | 4.96 | 0.27 | 4.04 |  |
| 7 | 5.50 | 0.85 | 4.44 |  |
| 8 | 5.98 | 1.68 | 4.87 |  |
| 9 | 6.48 | 2.02 | 5.23 |  |
| 10 | 6.94 | 2.49 | 5.62 |  |
| 11 | 7.41 | 3.23 | 6.07 |  |
| 12 | 7.89 | 3.65 | 6.49 |  |
| 13 | 8.38 | 3.80 | 6.83 |  |
| 14 | 8.80 | 3.88 | 7.15 |  |
| 15 | 9.30 | 4.23 | 7.52 |  |
| 16 | 9.87 | 4.21 | 7.93 |  |
| 17 | 10.73 | 4.59 | 8.60 |  |
| 18 | 12.34 | 7.72 | 10.41 |  |
| 19 | 16.30 | 14.80 | 15.22 |  |
| 20 | 25.45 | 24.04 | 24.31 |  |

Table 6.3.3-13: Evaluation results on 5%-tile Geometry when RU=0% (Nokia Siemens network)

| Set | Ior2Noc | Ior3Noc |  | Ior1Noc |
| --- | --- | --- | --- | --- |
| 1 | 4.7 | 0.6 | 5.6 |  |
| 2 | 9.5 | 4.4 | 9.1 |  |
| 3 | 12.1 | 7.2 | 11.7 |  |
| 4 | 14.2 | 8.9 | 13.4 |  |
| 5 | 15.7 | 9.5 | 14.6 |  |
| 6 | 16.8 | 11.4 | 15.9 |  |
| 7 | 17.9 | 11.2 | 16.6 |  |
| 8 | 19.0 | 11.7 | 17.5 |  |
| 9 | 20.1 | 13.9 | 18.8 |  |
| 10 | 21.0 | 14.5 | 19.7 |  |
| 11 | 22.0 | 14.6 | 20.5 |  |
| 12 | 23.0 | 16.5 | 21.6 |  |
| 13 | 23.9 | 17.1 | 22.5 |  |
| 14 | 25.0 | 18.9 | 23.7 |  |
| 15 | 26.1 | 18.8 | 24.5 |  |
| 16 | 27.6 | 22.0 | 26.1 |  |
| 17 | 29.3 | 24.0 | 28.0 |  |
| 18 | 31.5 | 25.5 | 29.9 |  |
| 19 | 34.6 | 30.1 | 33.4 |  |
| 20 | 43.8 | 40.6 | 42.6 |  |

Table 6.3.3-14: Evaluation results on 5%-tile Geometry when RU=10% (Nokia Siemens network)

| Set | Ior2Noc | Ior3Noc |  | Ior1Noc |
| --- | --- | --- | --- | --- |
| 1 | 3.4 | 0.0 | 4.8 |  |
| 2 | 7.4 | 3.5 | 7.5 |  |
| 3 | 9.0 | 4.7 | 8.8 |  |
| 4 | 10.2 | 5.2 | 9.7 |  |
| 5 | 11.1 | 6.6 | 10.5 |  |
| 6 | 12.1 | 6.1 | 11.2 |  |
| 7 | 12.7 | 7.0 | 11.6 |  |
| 8 | 13.3 | 6.4 | 11.9 |  |
| 9 | 13.9 | 7.4 | 12.5 |  |
| 10 | 14.4 | 7.1 | 12.7 |  |
| 11 | 14.9 | 7.5 | 13.3 |  |
| 12 | 15.3 | 7.8 | 13.6 |  |
| 13 | 15.8 | 8.9 | 14.2 |  |
| 14 | 16.4 | 8.3 | 14.5 |  |
| 15 | 17.2 | 9.7 | 15.4 |  |
| 16 | 18.1 | 11.4 | 16.4 |  |
| 17 | 19.1 | 12.4 | 17.5 |  |
| 18 | 21.3 | 13.5 | 19.3 |  |
| 19 | 26.1 | 23.2 | 25.0 |  |
| 20 | 34.5 | 33.4 | 33.7 |  |

Table 6.3.3-15: Evaluation results on 5%-tile Geometry when RU=20% (Nokia Siemens network)

| Set | Ior2Noc | Ior3Noc |  | Ior1Noc |
| --- | --- | --- | --- | --- |
| 1 | 2.3 | -0.8 | 3.9 |  |
| 2 | 5.8 | 2.3 | 6.2 |  |
| 3 | 7.2 | 2.9 | 7.0 |  |
| 4 | 8.2 | 3.6 | 7.7 |  |
| 5 | 9.1 | 4.1 | 8.4 |  |
| 6 | 9.8 | 4.0 | 8.8 |  |
| 7 | 10.5 | 4.0 | 9.3 |  |
| 8 | 11.1 | 5.0 | 9.8 |  |
| 9 | 11.6 | 4.9 | 10.1 |  |
| 10 | 12.0 | 4.9 | 10.4 |  |
| 11 | 12.5 | 5.3 | 10.9 |  |
| 12 | 13.0 | 4.5 | 11.1 |  |
| 13 | 13.5 | 5.6 | 11.6 |  |
| 14 | 14.1 | 6.3 | 12.2 |  |
| 15 | 14.7 | 8.2 | 13.1 |  |
| 16 | 15.6 | 8.3 | 13.9 |  |
| 17 | 16.6 | 10.0 | 14.9 |  |
| 18 | 18.8 | 10.9 | 16.8 |  |
| 19 | 24.0 | 21.4 | 23.0 |  |
| 20 | 32.3 | 31.1 | 31.5 |  |

Table 6.3.3-16: Evaluation results on 5%-tile Geometry when RU=30% (Nokia Siemens network)

| Set | Ior2Noc | Ior3Noc |  | Ior1Noc |
| --- | --- | --- | --- | --- |
| 1 | 1.3 | -1.27 | 3.1 |  |
| 2 | 4.7 | 1.08 | 5.1 |  |
| 3 | 6.0 | 1.77 | 5.8 |  |
| 4 | 6.9 | 1.77 | 6.3 |  |
| 5 | 7.6 | 3.03 | 7.1 |  |
| 6 | 8.4 | 2.74 | 7.4 |  |
| 7 | 9.1 | 2.40 | 7.8 |  |
| 8 | 9.6 | 3.29 | 8.3 |  |
| 9 | 10.1 | 3.13 | 8.5 |  |
| 10 | 10.5 | 3.92 | 9.1 |  |
| 11 | 11.0 | 3.13 | 9.3 |  |
| 12 | 11.5 | 3.28 | 9.6 |  |
| 13 | 12.0 | 3.85 | 10.1 |  |
| 14 | 12.6 | 4.55 | 10.7 |  |
| 15 | 13.2 | 6.23 | 11.5 |  |
| 16 | 14.0 | 7.96 | 12.6 |  |
| 17 | 15.0 | 8.31 | 13.3 |  |
| 18 | 17.3 | 9.34 | 15.3 |  |
| 19 | 22.7 | 20.04 | 21.6 |  |
| 20 | 31.0 | 29.72 | 30.1 |  |

Table 6.3.3-17: Evaluation results on 5%-tile Geometry when RU=40% (Nokia Siemens network)

| Set | Ior2Noc | Ior3Noc |  | Ior1Noc |
| --- | --- | --- | --- | --- |
| 1 | 0.6 | -1.5 | 2.5 |  |
| 2 | 3.7 | -0.1 | 4.1 |  |
| 3 | 5.0 | 0.9 | 4.9 |  |
| 4 | 5.9 | 0.9 | 5.4 |  |
| 5 | 6.6 | 1.5 | 5.9 |  |
| 6 | 7.3 | 1.6 | 6.3 |  |
| 7 | 8.0 | 1.4 | 6.8 |  |
| 8 | 8.5 | 2.1 | 7.1 |  |
| 9 | 9.0 | 2.4 | 7.5 |  |
| 10 | 9.4 | 2.7 | 7.9 |  |
| 11 | 9.9 | 1.9 | 8.2 |  |
| 12 | 10.4 | 2.5 | 8.6 |  |
| 13 | 10.9 | 2.1 | 8.9 |  |
| 14 | 11.5 | 3.4 | 9.6 |  |
| 15 | 12.1 | 4.9 | 10.4 |  |
| 16 | 12.9 | 7.5 | 11.5 |  |
| 17 | 13.8 | 7.3 | 12.3 |  |
| 18 | 16.2 | 7.9 | 14.1 |  |
| 19 | 21.6 | 19.1 | 20.6 |  |
| 20 | 30.0 | 28.7 | 29.1 |  |

Table 6.3.3-18: Evaluation results on 5%-tile Geometry when RU=50% (Nokia Siemens network)

| Set | Ior2Noc | Ior3Noc |  | Ior1Noc |
| --- | --- | --- | --- | --- |
| 1 | -0.1 | -2.1 | 1.8 |  |
| 2 | 3.0 | -0.7 | 3.4 |  |
| 3 | 4.2 | 0.1 | 4.1 |  |
| 4 | 5.1 | 0.1 | 4.6 |  |
| 5 | 5.8 | 0.7 | 5.1 |  |
| 6 | 6.5 | 0.8 | 5.5 |  |
| 7 | 7.1 | 0.8 | 5.9 |  |
| 8 | 7.6 | 1.1 | 6.3 |  |
| 9 | 8.1 | 1.2 | 6.5 |  |
| 10 | 8.6 | 1.8 | 7.0 |  |
| 11 | 9.0 | 1.2 | 7.3 |  |
| 12 | 9.5 | 1.5 | 7.7 |  |
| 13 | 10.0 | 1.4 | 8.0 |  |
| 14 | 10.6 | 2.5 | 8.7 |  |
| 15 | 11.3 | 3.5 | 9.4 |  |
| 16 | 12.0 | 6.7 | 10.7 |  |
| 17 | 12.9 | 6.8 | 11.4 |  |
| 18 | 15.3 | 6.7 | 13.2 |  |
| 19 | 20.8 | 18.6 | 19.8 |  |
| 20 | 29.2 | 27.9 | 28.3 |  |

Table 6.3.3-19: Evaluation results on 5%-tile Geometry when RU=0% (LG Electronics)

| Set | Ior2Noc | Ior3Noc |  | Ior1Noc |
| --- | --- | --- | --- | --- |
| 1 | 4.58 | 1.90 | 5.71 |  |
| 2 | 8.47 | 6.13 | 9.13 |  |
| 3 | 10.48 | 7.28 | 10.53 |  |
| 4 | 11.60 | 7.80 | 11.16 |  |
| 5 | 12.53 | 9.03 | 12.09 |  |
| 6 | 13.48 | 9.28 | 12.63 |  |
| 7 | 14.29 | 9.88 | 13.44 |  |
| 8 | 14.89 | 10.80 | 13.92 |  |
| 9 | 15.59 | 10.99 | 14.41 |  |
| 10 | 16.16 | 11.79 | 14.92 |  |
| 11 | 16.69 | 12.61 | 15.47 |  |
| 12 | 17.30 | 12.07 | 15.80 |  |
| 13 | 17.95 | 13.27 | 16.44 |  |
| 14 | 18.56 | 13.62 | 16.94 |  |
| 15 | 19.21 | 14.44 | 17.63 |  |
| 16 | 20.06 | 15.98 | 18.56 |  |
| 17 | 21.28 | 17.76 | 19.86 |  |
| 18 | 23.17 | 20.11 | 21.83 |  |
| 19 | 26.94 | 24.42 | 25.64 |  |
| 20 | 37.57 | 36.59 | 36.81 |  |

Table 6.3.3-20: Evaluation results on 5%-tile Geometry when RU=10% (LG Electronics)

| Set | Ior2Noc | Ior3Noc |  | Ior1Noc |
| --- | --- | --- | --- | --- |
| 1 | 3.22 | 0.94 | 4.77 |  |
| 2 | 6.55 | 4.60 | 7.34 |  |
| 3 | 8.16 | 4.49 | 8.05 |  |
| 4 | 9.23 | 6.41 | 9.11 |  |
| 5 | 10.15 | 6.17 | 9.46 |  |
| 6 | 10.92 | 6.35 | 10.00 |  |
| 7 | 11.50 | 7.00 | 10.54 |  |
| 8 | 12.05 | 6.73 | 10.79 |  |
| 9 | 12.61 | 7.59 | 11.31 |  |
| 10 | 13.17 | 7.52 | 11.65 |  |
| 11 | 13.77 | 9.35 | 12.41 |  |
| 12 | 14.25 | 8.37 | 12.55 |  |
| 13 | 14.80 | 10.21 | 13.31 |  |
| 14 | 15.33 | 11.43 | 13.93 |  |
| 15 | 15.94 | 11.05 | 14.26 |  |
| 16 | 16.76 | 13.35 | 15.41 |  |
| 17 | 17.88 | 14.34 | 16.46 |  |
| 18 | 19.90 | 16.60 | 18.47 |  |
| 19 | 24.49 | 22.45 | 23.33 |  |
| 20 | 34.68 | 33.67 | 33.91 |  |

Table 6.3.3-21: Evaluation results on 5%-tile Geometry when RU=20% (LG Electronics)

| Set | Ior2Noc | Ior3Noc |  | Ior1Noc |
| --- | --- | --- | --- | --- |
| 1 | 2.17 | -0.13 | 3.77 |  |
| 2 | 5.21 | 3.12 | 5.96 |  |
| 3 | 6.72 | 3.62 | 6.78 |  |
| 4 | 7.65 | 4.49 | 7.42 |  |
| 5 | 8.64 | 5.02 | 8.05 |  |
| 6 | 9.35 | 5.00 | 8.48 |  |
| 7 | 9.91 | 4.54 | 8.73 |  |
| 8 | 10.42 | 4.95 | 9.11 |  |
| 9 | 10.93 | 5.61 | 9.56 |  |
| 10 | 11.47 | 6.43 | 10.05 |  |
| 11 | 12.04 | 7.04 | 10.55 |  |
| 12 | 12.48 | 7.37 | 10.92 |  |
| 13 | 13.05 | 6.93 | 11.23 |  |
| 14 | 13.61 | 8.94 | 12.02 |  |
| 15 | 14.26 | 10.39 | 12.80 |  |
| 16 | 15.04 | 12.07 | 13.84 |  |
| 17 | 16.07 | 12.88 | 14.74 |  |
| 18 | 18.09 | 14.34 | 16.54 |  |
| 19 | 23.01 | 21.36 | 22.00 |  |
| 20 | 33.04 | 32.01 | 32.27 |  |

Table 6.3.3-22: Evaluation results on 5%-tile Geometry when RU=30% (LG Electronics)

| Set | Ior2Noc | Ior3Noc |  | Ior1Noc |
| --- | --- | --- | --- | --- |
| 1 | 1.33 | -0.93 | 2.97 |  |
| 2 | 4.19 | 2.07 | 4.92 |  |
| 3 | 5.64 | 2.78 | 5.70 |  |
| 4 | 6.54 | 3.25 | 6.29 |  |
| 5 | 7.50 | 3.82 | 6.92 |  |
| 6 | 8.22 | 3.77 | 7.30 |  |
| 7 | 8.75 | 3.63 | 7.62 |  |
| 8 | 9.24 | 3.17 | 7.82 |  |
| 9 | 9.74 | 4.15 | 8.31 |  |
| 10 | 10.31 | 4.93 | 8.80 |  |
| 11 | 10.80 | 5.47 | 9.27 |  |
| 12 | 11.26 | 6.21 | 9.72 |  |
| 13 | 11.81 | 6.07 | 10.05 |  |
| 14 | 12.36 | 7.45 | 10.70 |  |
| 15 | 13.08 | 7.81 | 11.28 |  |
| 16 | 13.87 | 11.47 | 12.85 |  |
| 17 | 14.86 | 12.04 | 13.64 |  |
| 18 | 16.81 | 12.76 | 15.18 |  |
| 19 | 21.95 | 20.34 | 20.96 |  |
| 20 | 31.89 | 30.85 | 31.11 |  |

Table 6.3.3-23: Evaluation results on 5%-tile Geometry when RU=40% (LG Electronics)

| Set | Ior2Noc | Ior3Noc |  | Ior1Noc |
| --- | --- | --- | --- | --- |
| 1 | 0.61 | -1.67 | 2.28 |  |
| 2 | 3.37 | 1.41 | 4.14 |  |
| 3 | 4.81 | 1.91 | 4.85 |  |
| 4 | 5.65 | 2.52 | 5.44 |  |
| 5 | 6.58 | 2.63 | 5.95 |  |
| 6 | 7.34 | 2.73 | 6.40 |  |
| 7 | 7.86 | 3.06 | 6.75 |  |
| 8 | 8.31 | 2.92 | 7.01 |  |
| 9 | 8.83 | 2.95 | 7.33 |  |
| 10 | 9.37 | 3.87 | 7.87 |  |
| 11 | 9.87 | 4.82 | 8.36 |  |
| 12 | 10.31 | 4.71 | 8.67 |  |
| 13 | 10.87 | 6.15 | 9.30 |  |
| 14 | 11.39 | 5.99 | 9.63 |  |
| 15 | 12.13 | 6.80 | 10.32 |  |
| 16 | 12.97 | 10.12 | 11.80 |  |
| 17 | 13.94 | 11.98 | 13.04 |  |
| 18 | 15.83 | 11.63 | 14.15 |  |
| 19 | 21.09 | 19.29 | 20.01 |  |
| 20 | 30.99 | 29.97 | 30.22 |  |

Table 6.3.3-24: Evaluation results on 5%-tile Geometry when RU=50% (LG Electronics)

| Set | Ior2Noc | Ior3Noc |  | Ior1Noc |
| --- | --- | --- | --- | --- |
| 1 | 0.00 | -2.11 | 1.73 |  |
| 2 | 2.68 | 0.50 | 3.38 |  |
| 3 | 4.10 | 1.28 | 4.17 |  |
| 4 | 4.94 | 1.64 | 4.67 |  |
| 5 | 5.83 | 2.00 | 5.22 |  |
| 6 | 6.57 | 2.17 | 5.67 |  |
| 7 | 7.13 | 2.13 | 6.01 |  |
| 8 | 7.57 | 2.20 | 6.24 |  |
| 9 | 8.08 | 2.04 | 6.57 |  |
| 10 | 8.59 | 1.83 | 6.89 |  |
| 11 | 9.09 | 4.46 | 7.66 |  |
| 12 | 9.55 | 3.88 | 7.90 |  |
| 13 | 10.10 | 5.23 | 8.50 |  |
| 14 | 10.60 | 5.51 | 8.91 |  |
| 15 | 11.35 | 5.95 | 9.53 |  |
| 16 | 12.23 | 9.28 | 11.02 |  |
| 17 | 13.20 | 11.37 | 12.35 |  |
| 18 | 15.04 | 10.87 | 13.37 |  |
| 19 | 20.38 | 18.59 | 19.31 |  |
| 20 | 30.25 | 29.23 | 29.48 |  |

Table 6.3.3-25: Evaluation results on 5%-tile Geometry when RU=0% (Huawei)

| Set | Ior2Noc | Ior3Noc |  | Ior1Noc |
| --- | --- | --- | --- | --- |
| 1 | 6 | 2.7 | 6.9 |  |
| 2 | 10.3 | 7.4 | 10.7 |  |
| 3 | 12 | 8.4 | 11.72 |  |
| 4 | 13.6 | 10.5 | 13.5 |  |
| 5 | 15.1 | 10.2 | 14.1 |  |
| 6 | 16.1 | 12.7 | 15.8 |  |
| 7 | 17.2 | 13.6 | 16.5 |  |
| 8 | 18.5 | 14.7 | 17.6 |  |
| 9 | 19.5 | 14.6 | 18.4 |  |
| 10 | 21 | 17.7 | 20.2 |  |
| 11 | 22.1 | 18.4 | 21.1 |  |
| 12 | 23.3 | 19.4 | 22.4 |  |
| 13 | 24.5 | 21.1 | 23.5 |  |
| 14 | 25.6 | 21.7 | 24.4 |  |
| 15 | 26.9 | 24.4 | 26.2 |  |
| 16 | 28 | 25.4 | 27.4 |  |
| 17 | 29.7 | 26.5 | 28.6 |  |
| 18 | 31.6 | 28.5 | 30.4 |  |
| 19 | 35 | 30.8 | 33.5 |  |
| 20 | 45.8 | 44.5 | 44.9 |  |

Table 6.3.3-26: Evaluation results on 5%-tile Geometry when RU=10% (Huawei)

| Set | Ior2Noc | Ior3Noc |  | Ior1Noc |
| --- | --- | --- | --- | --- |
| 1 | 4.4 | 1.2 | 5.7 |  |
| 2 | 7.2 | 5.5 | 8.4 |  |
| 3 | 8.9 | 6.7 | 9.3 |  |
| 4 | 10.1 | 6.8 | 10 |  |
| 5 | 11.1 | 6.6 | 10.4 |  |
| 6 | 11.9 | 8 | 11.3 |  |
| 7 | 12.8 | 7.9 | 11.8 |  |
| 8 | 13.4 | 7.8 | 12.2 |  |
| 9 | 13.9 | 9 | 12.8 |  |
| 10 | 14.5 | 10.1 | 13.2 |  |
| 11 | 15.2 | 10.1 | 13.8 |  |
| 12 | 15.9 | 12.2 | 14.6 |  |
| 13 | 16.5 | 12.6 | 15.2 |  |
| 14 | 17.4 | 10.8 | 15.5 |  |
| 15 | 18.4 | 12.3 | 16.5 |  |
| 16 | 19.4 | 15.4 | 17.9 |  |
| 17 | 20.5 | 17.7 | 19.3 |  |
| 18 | 22.2 | 21.1 | 21.7 |  |
| 19 | 23.9 | 21.3 | 22.8 |  |
| 20 | 35.1 | 33.6 | 34.1 |  |

Table 6.3.3-27: Evaluation results on 5%-tile Geometry when RU=20% (Huawei)

| Set | Ior2Noc | Ior3Noc |  | Ior1Noc |
| --- | --- | --- | --- | --- |
| 1 | 2.7 | 0.55 | 4.5 |  |
| 2 | 5.7 | 3.38 | 6.5 |  |
| 3 | 7.2 | 3.98 | 7.4 |  |
| 4 | 7.9 | 4.52 | 7.9 |  |
| 5 | 8.8 | 5.35 | 8.5 |  |
| 6 | 9.6 | 5.23 | 8.9 |  |
| 7 | 10.1 | 5.73 | 9.3 |  |
| 8 | 10.9 | 5.66 | 9.7 |  |
| 9 | 11.4 | 6.18 | 10.3 |  |
| 10 | 12.1 | 5.93 | 10.6 |  |
| 11 | 12.9 | 8.1 | 11.5 |  |
| 12 | 13.6 | 9.74 | 12.3 |  |
| 13 | 14.2 | 11.2 | 13.1 |  |
| 14 | 15 | 10.64 | 13.4 |  |
| 15 | 15.9 | 9.76 | 14 |  |
| 16 | 17.4 | 13.89 | 16 |  |
| 17 | 18.9 | 17.77 | 18.3 |  |
| 18 | 20.1 | 18.5 | 19.4 |  |
| 19 | 22.1 | 20.47 | 21.3 |  |
| 20 | 32.1 | 30.35 | 31 |  |

Table 6.3.3-28: Evaluation results on 5%-tile Geometry when RU=30% (Huawei)

| Set | Ior2Noc | Ior3Noc |  | Ior1Noc |
| --- | --- | --- | --- | --- |
| 1 | 2.6 | 1.42 | 4.80 |  |
| 2 | 4 | 2.80 | 5.50 |  |
| 3 | 5.3 | 3.60 | 6.20 |  |
| 4 | 6 | 2.90 | 6.30 |  |
| 5 | 7.2 | 4.10 | 7.10 |  |
| 6 | 7.8 | 5.80 | 8.00 |  |
| 7 | 8.7 | 4.10 | 7.90 |  |
| 8 | 9.2 | 4.80 | 8.50 |  |
| 9 | 9.8 | 5.00 | 8.80 |  |
| 10 | 10.4 | 4.70 | 9.10 |  |
| 11 | 10.9 | 4.00 | 9.30 |  |
| 12 | 11.7 | 6.40 | 10.20 |  |
| 13 | 12.1 | 6.10 | 10.40 |  |
| 14 | 13.1 | 6.00 | 11.10 |  |
| 15 | 14.1 | 7.60 | 12.10 |  |
| 16 | 15.3 | 8.20 | 13.10 |  |
| 17 | 16.7 | 14.90 | 15.90 |  |
| 18 | 18 | 16.60 | 17.40 |  |
| 19 | 19.8 | 17.30 | 18.70 |  |
| 20 | 27.3 | 26.20 | 26.50 |  |

Table 6.3.3-29: Evaluation results on 5%-tile Geometry when RU=40% (Huawei)

| Set | Ior2Noc | Ior3Noc |  | Ior1Noc |
| --- | --- | --- | --- | --- |
| 1 | 1 | -1.4 | 2.6 |  |
| 2 | 3.9 | 1.5 | 4.6 |  |
| 3 | 5 | 2 | 5.2 |  |
| 4 | 6.2 | 2.1 | 5.9 |  |
| 5 | 7.2 | 2.9 | 6.5 |  |
| 6 | 7.8 | 2.9 | 6.9 |  |
| 7 | 8.4 | 2.6 | 7.2 |  |
| 8 | 9.1 | 3.8 | 7.8 |  |
| 9 | 9.6 | 3.9 | 8.2 |  |
| 10 | 10.2 | 3.9 | 8.5 |  |
| 11 | 10.9 | 5.6 | 9.4 |  |
| 12 | 11.6 | 6.7 | 10 |  |
| 13 | 12.3 | 7.7 | 10.7 |  |
| 14 | 13.1 | 7.8 | 11.4 |  |
| 15 | 14.6 | 12.8 | 13.8 |  |
| 16 | 16.4 | 14.7 | 15.6 |  |
| 17 | 17.8 | 16.9 | 17.4 |  |
| 18 | 19.5 | 18.8 | 19.1 |  |
| 19 | 21.1 | 20.9 | 21 |  |
| 20 | 27.9 | 26 | 26.8 |  |

Table 6.3.3-30: Evaluation results on 5%-tile Geometry when RU=50% (Huawei)

| Set | Ior2Noc | Ior3Noc |  | Ior1Noc |
| --- | --- | --- | --- | --- |
| 1 | 1.30 | 0.50 | 3.1 |  |
| 2 | 3.20 | 0.90 | 3.9 |  |
| 3 | 4.30 | 1.30 | 4.4 |  |
| 4 | 5.30 | 1.20 | 5 |  |
| 5 | 6.40 | 1.60 | 5.6 |  |
| 6 | 7.00 | 4.40 | 6.4 |  |
| 7 | 7.50 | 3.10 | 6.6 |  |
| 8 | 8.00 | 1.60 | 6.6 |  |
| 9 | 8.70 | -0.10 | 6.9 |  |
| 10 | 9.10 | 1.80 | 7.3 |  |
| 11 | 9.60 | 4.20 | 8 |  |
| 12 | 10.10 | 4.70 | 8.5 |  |
| 13 | 10.90 | 2.10 | 8.6 |  |
| 14 | 11.80 | 3.70 | 9.5 |  |
| 15 | 12.60 | 4.60 | 10.4 |  |
| 16 | 13.40 | 8.20 | 11.6 |  |
| 17 | 14.90 | 13.60 | 14.3 |  |
| 18 | 16.10 | 13.20 | 14.9 |  |
| 19 | 17.40 | 11.40 | 15.3 |  |
| 20 | 31.30 | 29.90 | 30.3 |  |

# Link performance characterization

## General

The purpose of this clause is to analyse the performance gain from link-level simulation results. Emphasis is on results that were based on agreed set of simulation assumption. This clause introduces an evaluation framework for link-level simulations. As described in clause 6, interference modelling was performed by interested companies in order to reach an agreement of link-level simulation assumptions.

## Detail of simulation assumption for link-level simulation

The link-level assumptions are summarized in Table 7.2-1.

Table 7.2-1: Link level simulation assumptions for CRS-IM investigations in homogeneous deployments

| Parameter |  |  |  | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Carrier frequency |  |  |  | 2 GHz |  |  |  |  |  |
| System bandwidth |  |  |  | 10 MHz |  |  |  |  |  |
| Duplex mode |  |  |  | FDD |  |  |  |  |  |
| Transmission mode in serving cell |  |  |  | Option A (baseline): TM2 Option B (interested companies): TM4 |  |  |  |  |  |
| Transmission mode in interfering cells |  |  |  | Use TM3 for TM2 serving, and use TM4 for TM4 serving |  |  |  |  |  |
| MIMO configuration |  |  |  | 2x2, low correlation |  |  |  |  |  |
| Channel model and Doppler frequency for target and interfering cells |  |  |  | EVA5 |  |  |  |  |  |
|  |  |  |  | Use different channel seed for between cells |  |  |  |  |  |
| Number of explicitly modeled interfering cells |  |  |  | Option A (baseline): 2 interfering cells Option B (interested companies): 3 or more interfering cells are investigated in order to check whether 2 cells model sufficiently well non-full-buffer interference |  |  |  |  |  |
| Signal level for serving cell CRS (Es/Noc) |  |  |  | Range of Es/Noc: TBD |  |  |  |  |  |
| Signal level for interfering cells CRS (interference over Noc) |  |  |  | 1st interfering cell |  |  | 2nd interfering cell |  | 3rd, 4th etc interferers (option B) |
|  |  |  |  | INR1=TBD |  |  | INR2=TBD |  | INR3, INR4 = TBD |
| Other cells interference |  |  |  | AWGN with 1 Noc level |  |  |  |  |  |
| Network synchronization in time |  |  |  | All cells are synchronous |  |  |  |  |  |
|  |  |  |  | Time-delay wrt. serving cell |  |  |  |  |  |
|  |  |  |  | 1st interfering cell |  | 2nd interfering cell |  |  | 3rd, 4th interfering cells (option B) |
|  |  |  |  | [3 us] |  | [-1 us] |  |  | FFS |
| Network synchronization in frequency |  |  |  | Frequency shift wrt. serving cell |  |  |  |  |  |
|  |  |  |  | 1st interfering cell | 2nd interfering cell |  |  |  | 3rd, 4th interfering cells (option B) |
|  |  |  |  | 300 Hz | -100 Hz |  |  |  | FFS |
| CRS configuration |  |  |  | 2 CRS ports per cell with planning, non-colliding CRS between explicitly modeled serving and the first two interfering cells |  |  |  |  |  |
| Downlink power allocation (cf. Chapter 8 of TS36.101) | A |  |  | -3 dB in all modeled cells |  |  |  |  |  |
|  | B |  |  | -3 dB in all modeled cells (PB=1) |  |  |  |  |  |
|  |  |  |  | 0 dB in all modeled cells |  |  |  |  |  |
| CSI reference signals |  |  |  | N/A |  |  |  |  |  |
| CSI-RS periodicity and subframe offset (TCSI-RS / ICSI-RS) |  |  |  | N/A |  |  |  |  |  |
| CSI reference signal configuration |  |  |  | N/A |  |  |  |  |  |
| Subframes for demodulation |  |  |  | All subframes scheduled for demodulation except subframe #5 |  |  |  |  |  |
| HARQ |  |  |  | 8 HARQ processes and max 4 transmissions |  |  |  |  |  |
| Feedback mode |  |  |  | PUCCH 1-0 for TM2 and PUCCH 1-1 for TM4 |  |  |  |  |  |
| Feedback periodicity & delay for target signal |  |  |  | Feedback periodicity |  |  |  | Feedback delay |  |
|  |  |  |  | 5 milliseconds |  |  |  | 8 milliseconds |  |
| Channel and interference estimation at UE |  |  |  | Practical and realizable channel and interference covariance estimates with no a-priori knowledge of the channel state information |  |  |  |  |  |
| Time/frequency tracking at the UE |  |  |  | Practical algorithms should be used |  |  |  |  |  |
| Physical channels transmitted in serving cell |  |  |  | PSS/SSS/PBCH |  |  |  |  |  |
| PCFICH |  |  |  | CFI = 2 in all cells |  |  |  |  |  |
| PCFICH/PDCCH detection |  |  |  | Not considered |  |  |  |  |  |
| Physical channels transmitted in interfering cells |  |  |  | PDCCH PDSCH PSS/SSS/PBCH |  |  |  |  |  |
| Desired PDSCH parameterization |  | Resource allocation |  | 50 PRB |  |  |  |  |  |
|  |  | Rank |  | Rank-1 |  |  |  |  |  |
|  |  | PMI |  | TM2: N/A |  |  |  | TM4: Follow wideband PMI |  |
|  |  | Modulation |  | Option 1 (baseline): Inner- and outer-link adaptation targeting 10% BLER for the 1st transmission Option 2 (interested companies): Fixed MCS (MCS set TBD) (Note : The baseline assumption does not preclude the use of fixed MCS for any future performance requirement which may be developed for homogeneous CRS-IM) |  |  |  |  |  |
|  |  | Code rate |  |  |  |  |  |  |  |
|  |  | Channel coding, rate matching |  | As specified in TS36.212 |  |  |  |  |  |
|  |  | CRC |  |  |  |  |  |  |  |
| Interfering PDSCH parameterization |  | Resource allocation |  | Random full band (50PRB) on/off model, proportional to the average resource utilization in the interfering cells; ON/OFF pattern depends on the Possion distribution |  |  |  |  |  |
|  |  | Rank |  | Randomly changing rank per allocated subband from subframe to subframe: 80% rank-1, 20% rank-2 |  |  |  |  |  |
|  |  | PMI |  | TM3: N/A |  |  |  | TM4: Random PMI per allocated subband |  |
|  |  | Modulation |  | Randomly modulated 16QAM symbols over allocated interfering resources |  |  |  |  |  |
|  |  | Code rate |  | - |  |  |  |  |  |
|  |  | Channel coding, rate matching |  | - |  |  |  |  |  |
|  |  | CRC |  |  |  |  |  |  |  |
| Non-full buffer interference |  | Model |  | Interfering PDSCH transmissions in interfering cells are randomly & independently active over the full band with an activity in time domain equal on average to the targeted resource utilization |  |  |  |  |  |
|  |  | Average resource utilization |  | {0%, 10%, 20%, 30%, 40%, 50%} |  |  |  |  |  |
| Tx EVM |  |  |  | 6% in both alignment and impairment simulations |  |  |  |  |  |
| Noc at antenna port |  |  |  | [-98 dBm] |  |  |  |  |  |
| Cyclic prefix |  |  |  | Normal |  |  |  |  |  |
| Simulation length |  |  |  | 10000 sub-frames at minimum |  |  |  |  |  |
| Simulation output |  |  |  | PDSCH throughput vs. serving cell Es/Noc |  |  |  |  |  |
| UE receiver |  |  | Detector | MMSE-IRC with CRS based interference covariance estimation as defined in TR36.829 |  |  |  |  |  |
|  |  |  | CRS-IM | Without CRS-IM |  |  |  |  |  |
|  |  |  |  | With CRS-IM for the 1st strongest interfering cell |  |  |  |  |  |
|  |  |  |  | With CRS-IM for the 1st and 2nd strongest interfering cells |  |  |  |  |  |

## Average simulation results

Table 7.3-1 provides the averaged simulation results for the agreed link-level simulation scenario on conditional IorjNoc . The details of simulation result by interested companies are provided in Clause 7.4.

Table 7.3-1: Average link simulation results for agreed link-level simulation scenario on conditional IorjNoc

|  | Set | Ior2NocIor2Noc | Ior3NocIor3Noc | Ior1NocIor1Noc | Average throughput Gain of CRS-IM over non-CRS-IM receiver |
| --- | --- | --- | --- | --- | --- |
| 10% |  | 1 |  | 2.5 |  | 0 |  |  | 4 |  | 10.04% |
|  |  | 5 |  | 8.9 |  | 4.6 |  |  | 8.2 |  | 28.07% |
|  |  | 10 |  | 11.7 |  | 5.7 |  |  | 10.2 |  | 36.80% |
|  |  | 15 |  | 14.7 |  | 10.1 |  |  | 13.3 |  | 56.00% |
|  |  | 20 |  | 31.6 |  | 30.2 |  |  | 30.6 |  | 88.05% |
| 20% |  | 1 |  | 1.7 |  | -0.7 |  |  | 3.2 |  | 7.69% |
|  |  | 5 |  | 7.7 |  | 3.6 |  |  | 7 |  | 22.04% |
|  |  | 10 |  | 10.4 |  | 4.6 |  |  | 8.9 |  | 27.87% |
|  |  | 15 |  | 13.4 |  | 9.2 |  |  | 12 |  | 42.44% |
|  |  | 20 |  | 30.3 |  | 28.9 |  |  | 29.4 |  | 82.32% |
| 30% |  | 1 |  | 1.3 |  | -0.7 |  |  | 3.1 |  | 7.11% |
|  |  | 5 |  | 6.9 |  | 3 |  |  | 6.3 |  | 17.38% |
|  |  | 10 |  | 9.7 |  | 3.7 |  |  | 8.2 |  | 24.47% |
|  |  | 15 |  | 12.8 |  | 7.8 |  |  | 11.2 |  | 34.03% |
|  |  | 20 |  | 29 |  | 27.7 |  |  | 28.1 |  | 77.70% |
| 40% |  | 1 |  | 0.5 |  | -1.7 |  |  | 2.1 |  | 5.32% |
|  |  | 5 |  | 6.1 |  | 1.9 |  |  | 5.4 |  | 16.13% |
|  |  | 10 |  | 8.8 |  | 2.7 |  |  | 7.2 |  | 19.86% |
|  |  | 15 |  | 11.7 |  | 7.1 |  |  | 10.3 |  | 27.09% |
|  |  | 20 |  | 28.8 |  | 27.4 |  |  | 27.8 |  | 72.74% |
| 50% |  | 1 |  | 0.2 |  | -1.6 |  |  | 1.9 |  | 4.49% |
|  |  | 5 |  | 5.7 |  | 1.2 |  |  | 4.9 |  | 13.42% |
|  |  | 10 |  | 8.4 |  | 1.7 |  |  | 6.7 |  | 18.04% |
|  |  | 15 |  | 11.4 |  | 5.9 |  |  | 9.8 |  | 24.03% |
|  |  | 20 |  | 28.8 |  | 27.4 |  |  | 27.8 |  | 63.31% |

  - Link level simulation results

Table 7.4-1 ~ Table 7.4-5 provides the simulation results generated by various interested companies. The side condition refers to Clause 6.3.2.

Table 7.4-1: Link simulation results for agreed link-level simulation scenario on conditional IorjNoc (Ericsson)

|  | Set | 2-cell CRS IC | No CRS-IC | Gain |
| --- | --- | --- | --- | --- |
| 10% | 1 | 8678124.22 | 7717126.43 | 12.45% |
|  | 5 | 12191329.45 | 9503970.25 | 28.28% |
|  | 10 | 14051487.84 | 9589590.35 | 46.53% |
|  | 15 | 16842838.19 | 8391413.87 | 100.72% |
|  | 20 | 21206346.93 | 8435427.54 | 151.40% |
| 20% | 1 | 7404488.84 | 6719384.92 | 10.20% |
|  | 5 | 9875567.44 | 7969337.09 | 23.92% |
|  | 10 | 11251061.71 | 8199710.55 | 37.21% |
|  | 15 | 13242312.36 | 7508786.33 | 76.36% |
|  | 20 | 16821033.97 | 6942544.72 | 142.29% |
| 30% | 1 | 6810570.45 | 6256038.59 | 8.86% |
|  | 5 | 8347268.34 | 6925697.29 | 20.53% |
|  | 10 | 9458286.63 | 7183179.1 | 31.67% |
|  | 15 | 10994416.88 | 6826685.43 | 61.05% |
|  | 20 | 13598789.95 | 5699064.92 | 138.61% |
| 40% | 1 | 6112454.27 | 5657651.46 | 8.04% |
|  | 5 | 7755792.56 | 6442178.09 | 20.39% |
|  | 10 | 8709308.14 | 6874042.21 | 26.70% |
|  | 15 | 10222485.23 | 6976908.54 | 46.52% |
|  | 20 | 13472980.9 | 5838986.93 | 130.74% |
| 50% | 1 | 5812951.16 | 5400988.14 | 7.63% |
|  | 5 | 7205450.45 | 6068996.98 | 18.73% |
|  | 10 | 8055873.77 | 6465934.47 | 24.59% |
|  | 15 | 9435281.21 | 6489136.88 | 45.40% |
|  | 20 | 11970636.38 | 5330785.13 | 124.56% |

Table 7.4-2: Link simulation results for agreed link-level simulation scenario on conditional IorjNoc (ZTE)

|  | Set | 2-cell CRS IC | No CRS-IC | Gain |
| --- | --- | --- | --- | --- |
| 10% | 1 | 5548768 | 4994849 | 11.09% |
|  | 5 | 8163787.5 | 6818250 | 19.73% |
|  | 10 | 9010660.5 | 7002418 | 28.68% |
|  | 15 | 9879011.7 | 7068326 | 39.76% |
|  | 20 | 9770558.5 | 5577800 | 75.17% |
| 20% | 1 | 4322129.5 | 3940162 | 9.69% |
|  | 5 | 7110729 | 6063392 | 17.27% |
|  | 10 | 7610320 | 6334285 | 20.14% |
|  | 15 | 8466757.4 | 6546125 | 29.34% |
|  | 20 | 9221530.5 | 5092184 | 81.09% |
| 30% | 1 | 4103113 | 3860839 | 6.28% |
|  | 5 | 6320388 | 5515071 | 14.60% |
|  | 10 | 6835658 | 5873568 | 16.38% |
|  | 15 | 7252017 | 5892117 | 23.08% |
|  | 20 | 8245043 | 4584061 | 79.86% |
| 40% | 1 | 3009599 | 2815151 | 6.91% |
|  | 5 | 5526006 | 4943076 | 11.79% |
|  | 10 | 6144031 | 5060749 | 21.41% |
|  | 15 | 6635474 | 5377026 | 23.40% |
|  | 20 | 7243585 | 3994678.5 | 81.33% |
| 50% | 1 | 3091212 | 2863061.5 | 7.97% |
|  | 5 | 4947728 | 4501394 | 9.92% |
|  | 10 | 5530989 | 4734923.5 | 16.81% |
|  | 15 | 6086049 | 5037107.5 | 20.82% |
|  | 20 | 6196158 | 3610607 | 71.61% |

Table 7.4-3: Link simulation results for agreed link-level simulation scenario on conditional IorjNoc (Renesas)

|  | Set | 2-cell CRS IC | No CRS-IC | Gain |
| --- | --- | --- | --- | --- |
| 10% | 1 | 8556634.22 | 7977262.47 | 7.26% |
|  | 5 | 12612881.18 | 9589972.43 | 31.52% |
|  | 10 | 14197176.28 | 9864003.18 | 43.93% |
|  | 15 | 15489844.78 | 10100301.13 | 53.36% |
|  | 20 | 17475669.57 | 8683801.7 | 101.24% |
| 20% | 1 | 7432968.9 | 7101008.38 | 4.67% |
|  | 5 | 10314550.73 | 8223672.11 | 25.43% |
|  | 10 | 11759382.05 | 8766848.05 | 34.13% |
|  | 15 | 12935536.35 | 8528041.1 | 51.68% |
|  | 20 | 13878503.85 | 7306422.05 | 89.95% |
| 30% | 1 | 7108237.9 | 6704186.45 | 6.03% |
|  | 5 | 8772262.55 | 7610215.22 | 15.27% |
|  | 10 | 10244520 | 7733108.97 | 32.48% |
|  | 15 | 11275903.65 | 7958588.25 | 41.68% |
|  | 20 | 12069482.82 | 6467640.42 | 86.61% |
| 40% | 1 | 6066459.85 | 5796932.28 | 4.65% |
|  | 5 | 7609429.28 | 6696661.38 | 13.63% |
|  | 10 | 8490755.13 | 7070396.78 | 20.09% |
|  | 15 | 9828148.82 | 7469501.95 | 31.58% |
|  | 20 | 10922649.63 | 6105781.53 | 78.89% |
| 50% | 1 | 5568648.45 | 5486648.65 | 1.49% |
|  | 5 | 6843657 | 6098729.75 | 12.21% |
|  | 10 | 7654198.38 | 6482530.35 | 18.07% |
|  | 15 | 8781832.22 | 6967841.25 | 26.03% |
|  | 20 | 9507925.15 | 5822405.45 | 63.30% |

Table 7.4-4: Link simulation results for agreed link-level simulation scenario on conditional IorjNoc (NSN)

|  | Set | 2-cell CRS IC | No CRS-IC | Gain |
| --- | --- | --- | --- | --- |
| 10% | 1 | 8190000 | 7287000 | 12.39% |
|  | 5 | 11538000 | 8039000 | 43.53% |
|  | 10 | 12778000 | 8621000 | 48.22% |
|  | 15 | 13615000 | 9191000 | 48.13% |
|  | 20 | 14006000 | 9552000 | 46.63% |
| 20% | 1 | 7531000 | 6869000 | 9.64% |
|  | 5 | 10084000 | 7798000 | 29.32% |
|  | 10 | 11196000 | 8452000 | 32.47% |
|  | 15 | 12269000 | 9053000 | 35.52% |
|  | 20 | 13096000 | 9463000 | 38.39% |
| 30% | 1 | 7392000 | 6820000 | 8.39% |
|  | 5 | 9274000 | 7596000 | 22.09% |
|  | 10 | 10562000 | 8313000 | 27.05% |
|  | 15 | 11629000 | 8965000 | 29.72% |
|  | 20 | 12536000 | 9383000 | 33.60% |
| 40% | 1 | 6640000 | 6280000 | 5.73% |
|  | 5 | 8504000 | 7287000 | 16.70% |
|  | 10 | 9576000 | 8025000 | 19.33% |
|  | 15 | 10941000 | 8848000 | 23.66% |
|  | 20 | 12119000 | 9147000 | 32.49% |
| 50% | 1 | 6483000 | 6228000 | 4.09% |
|  | 5 | 8061000 | 7114000 | 13.31% |
|  | 10 | 9285000 | 7902000 | 17.50% |
|  | 15 | 10711000 | 8833000 | 21.26% |
|  | 20 | 11919000 | 9159000 | 30.13% |

Table 7.4-5: Link simulation results for agreed link-level simulation scenario on conditional IorjNoc (LGE)

|  | Set | 2-cell CRS IC | No CRS-IC | Gain |
| --- | --- | --- | --- | --- |
| 10% | 1 | 6660086.547 | 6187748.822 | 7.63% |
|  | 5 | 9621232.811 | 7749329.743 | 24.16% |
|  | 10 | 9289724.012 | 7437761.323 | 24.90% |
|  | 15 | 9202484.854 | 6184010.001 | 48.81% |
|  | 20 |  |  |  |
| 20% | 1 | 5552149.245 | 5249304.741 | 5.77% |
|  | 5 | 7881434.753 | 6795930.378 | 15.97% |
|  | 10 | 7635918.838 | 6441988.653 | 18.53% |
|  | 15 | 7753068.564 | 6223890.759 | 24.57% |
|  | 20 |  |  |  |
| 30% | 1 | 5057378.594 | 4708421.964 | 7.41% |
|  | 5 | 6370951.053 | 5636895.855 | 13.02% |
|  | 10 | 7016520.819 | 5979621.117 | 17.34% |
|  | 15 | 6384660.063 | 5401350.13 | 18.20% |
|  | 20 |  |  |  |
| 40% | 1 | 4369435.523 | 4282196.365 | 2.04% |
|  | 5 | 5906090.97 | 5016251.563 | 17.74% |
|  | 10 | 6456943.937 | 5831314.549 | 10.73% |
|  | 15 | 5998315.223 | 5351499.183 | 12.09% |
|  | 20 |  |  |  |
| 50% | 1 | 4126412.155 | 4056620.829 | 1.72% |
|  | 5 | 4885392.826 | 4284688.912 | 14.02% |
|  | 10 | 5316603.52 | 4705929.416 | 12.98% |
|  | 15 | 5700455.813 | 5170789.499 | 10.24% |
|  | 20 |  |  |  |

Table 7.4-6: Link simulation results for agreed link-level simulation scenario on conditional IorjNoc (Huawei)

|  | Set | 2-cell CRS IC | No CRS-IC | Gain |
| --- | --- | --- | --- | --- |
| 10% | 1 | 7570000 | 6920000 | 9.39% |
|  | 5 | 10980000 | 9060000 | 21.19% |
|  | 10 | 12470000 | 9700000 | 28.56% |
|  | 15 | 14900000 | 10260000 | 45.22% |
|  | 20 | 17080000 | 10300000 | 65.83% |
| 20% | 1 | 6380000 | 6010000 | 6.16% |
|  | 5 | 9120000 | 7580000 | 20.32% |
|  | 10 | 10180000 | 8160000 | 24.75% |
|  | 15 | 12030000 | 8770000 | 37.17% |
|  | 20 | 14340000 | 8970000 | 59.87% |
| 30% | 1 | 5950000 | 5630000 | 5.68% |
|  | 5 | 7780000 | 6550000 | 18.78% |
|  | 10 | 8690000 | 7130000 | 21.88% |
|  | 15 | 9980000 | 7650000 | 30.46% |
|  | 20 | 11910000 | 7950000 | 49.81% |
| 40% | 1 | 5050000 | 4830000 | 4.55% |
|  | 5 | 6340000 | 5440000 | 16.54% |
|  | 10 | 7110000 | 5880000 | 20.92% |
|  | 15 | 8130000 | 6490000 | 25.27% |
|  | 20 | 9580000 | 6830000 | 40.26% |
| 50% | 1 | 4650000 | 4470000 | 4.03% |
|  | 5 | 5560000 | 4950000 | 12.32% |
|  | 10 | 6280000 | 5310000 | 18.27% |
|  | 15 | 7070000 | 5870000 | 20.44% |
|  | 20 | 7770000 | 6120000 | 26.96% |

## Summary

From the link simulation results based on weighted IorjNoc , when the average resource utilization of the aggressor cells was 10% for cell-edge UEs, approximately [9%] gain for CRS based transmission was observed for the lower percentile, i.e., 5%-tile, of  IorjNoc ; For the median percentile, i.e., 50%-tile, of  IorjNoc , approximately [39%] gain can be achieved; For the higher percentile, i.e., 75%-tile, of  IorjNoc , approximately [58%] gain can be obtained. When the average resource utilization of aggressor cells is 50%, approximately [4%, 16% and 23%] gain can be achieved for the lower percentile, median percentile and higher percentile of   IorjNoc , respectively.

# System performance characterization

## General

The purpose of this clause is to analyse the performance gain from system-level simulation results based on the agreed set of simulation assumption.

## Simulation results

This section presents the evaluation results for system level simulation generated by various interested companies to show the contribution.

Table 8.2-1: System simulation results (R4-132603, Ericsson)

|  |  | User Throughput |  | Relative gain in user throughput |
| --- | --- | --- | --- | --- |
| Resource Utilization | Performance metrics | No CRS IC (M=0) | CRS IC from the two strongest interferers (M=2) | CRS IC from the two strongest interferer (M=2) |  |
| About 10% | 5%-ile | 10.1 Mbps | 13.9 Mbps | 37% |  |
|  | 50%-ile | 20.2 Mbps | 23.2 Mbps | 14% |  |
| About ~20% | 5%-ile | 8.1Mbps | 11.2 Mbps | 36% |  |
|  | 50%-ile | 16.8Mbps | 19.7Mbps | 17% |  |
| About ~30% | 5%-ile | 6.5Mbps | 9Mbps | 38% |  |
|  | 50%-ile | 14.1Mbps | 16.9Mbps | 19% |  |
| About ~40% | 5%-ile | 5.0Mbps | 7.1Mbps | 41% |  |
|  | 50%-ile | 11.6Mbps | 14.3Mbps | 22% |  |
| About ~50% | 5%-ile | 3.7Mbps | 5.4Mbps | 46% |  |
|  | 50%-ile | 9.2Mbps | 11.8Mbps | 28% |  |
| Note: The transmission mode is TM4 |  |  |  |  |  |  |

Table 8.2-3: System simulation results (R4-132384, Huawei)

|  |  | Relative gain in user throughput |  |
| --- | --- | --- | --- |
| Resource Utilization | Performance metrics | CRS IC from the two strongest interferer (M=2) | CRS IC from all interferers (M=∞) |
| About ~20% | 5%-ile | 10.2% | 14.6% |
|  | 50%-ile | 8.3% | 13.7% |
|  | 95%-ile | 0% | 0% |
| About ~50% | 5%-ile | 14.4% | 14.4% |
|  | 50%-ile | 7.4% | 7.9% |
|  | 95%-ile | 1.9% | 1.9% |
| About ~70% | 5%-ile | 2.5% | 3.5% |
|  | 50%-ile | 2.5% | 3.0% |
|  | 95%-ile | 1.3% | 3.4% |

Table 8.2-3: System simulation results (R4-133469, NSN)

|  |  | User Throughput |  |  | Relative gain in user throughput |  |
| --- | --- | --- | --- | --- | --- | --- |
| Resource Utilization | Performance metrics | No CRS IC (M=0) | CRS IC from the two strongest interferers (M=2) | CRS IC from all interferers (M=∞) | CRS IC from the two strongest interferer (M=2) | CRS IC from all interferers (M=∞) |
| About 5% | 5%-ile | 18.7Mbps | 19.8Mbps | 20.6Mbps | 5.9% | 10.2% |
|  | 50%-ile | 38.0Mbps | 41.1Mbps | 45.6Mbps | 8.1% | 20% |
| About ~20% | 5%-ile | 12.7Mbps | 13.5Mbps | 14.3Mbps | 6.3% | 12.6% |
|  | 50%-ile | 30.3Mbps | 31.5Mbps | 33.9Mbps | 3.9% | 11.9% |
| About ~50% | 5%-ile | 6.4Mbps | 6.7Mbps | 7.2Mbps | 4.7% | 12.5% |
|  | 50%-ile | 18.6Mbps | 19.4Mbps | 20.8Mbps | 4.3% | 11.8% |
| About ~70% | 5%-ile | 2.7Mbps | 2.8Mbps | 3.0Mbps | 3.7% | 11.1% |
|  | 50%-ile | 8.7Mbps | 9.0Mbps | 9.9Mbps | 3.4% | 13.8% |

## Summary

The study concluded that there is indeed an increase in UPT (user perceived throughput) to be seen with low load (resource utilization is about 10%~50%). For the 5% UPT, the gain is in the order of 10-38% when resource utilization is 30%, additionally approximately 8 – 19 % gain for 50% UPT would be obtained.

# Conclusions

In this technical report we have documented the work that was accomplished by RAN4 as part of the feasibility study on CRS interference mitigation for LTE homogeneous network. The major work includes the definition of CRS-IM receiver, investigation of interference model/profiles for partial load networks, and performance evaluation with link-level simulation and system-level simulation.

Receiver structures based on MMSE-IRC detector combined with a CRS-IM receiver were defined. This type of receiver attempts to mitigate the CRS interference along with the data/control interference from neighbouring cells. Under partial-load networks, interference models/profiles were developed for this interference in terms of the number of dominant interfering cells, and their powers relative to the noise plus the total other cell interference power excluding the considered dominant interference cells. This ratio is referred to as  IorjNoc . For the purpose of this study it was determined that two dominant interfering cells should be taken into account in the interference models. The interference profiles were defined for a wide range of operating conditions when the interfering cells have low traffic loading. Throughput estimates were developed using link level simulations, which included the ON/OFF model for interfering cells based on the results of interference modelling. System level results were also provided.

Based on the link simulation results based on weighted IorjNoc , when the average resource utilization of aggressor cells was 10% for cell-edge UEs, approximately [9%] gain for CRS based transmission was observed for the lower percentile, i.e., 5%-tile, of  IorjNoc ; For the median percentile, i.e, 50%-tile, of  IorjNoc , approximately [39%] gain can be achieved; For the higher percentile, i.e, 75%-tile, of  IorjNoc , approximately [58%] gain can be obtained. When the average resource utilization of aggressor cells is 50%, approximately [4%, 16% and 23%] gain can be achieved for the lower percentile, median percentile and higher percentile of   IorjNoc , respectively.  The system study concluded that there is an indeed increase in user perceived throughput (UPT) when the traffic load is low at the interfering cells. With regard to UE implementation issues, the CRS-IM receiver is based upon well-defined receivers used for meeting existing MMSE-IRC and FeICIC performance requirements. The reuse of these receivers’ structure minimizes the UE complexity.

Based on the observations cited above, RAN4 has concluded that moderate to significant throughput gain can be obtained for cell-edge UE by CRS-IM receiver in LTE homogeneous network under low traffic loading.

# Annex A: Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 2013-04 | 66bis | R4-131738 |  |  | First version approved | N/A | 0.1.0 |
| 2013-08 | 68 | R4-134463 |  |  | Agreed text proposals from RAN4 #67, the following TPs have been implemented: R4-132616, TP for TR 36.863 Vx.y.z: Draft TP on CRS-IM performance in homogeneous network | 0.1.0 | 0.2.0 |
| 2013-10 | 68bis | R4-135329 |  |  | Agreed text proposals from RAN4 #68, the following TPs have been implemented: R4-133990, TP for TR 36.863 V xxx: Link level simulation assumption for CRS-IM in homogeneous network R4-133991, TP for TR 36.863 V xxx: System simulation assumptions for CRS-IM in homogeneous network, R4-133992, TP for TR 36.863 V xxx: Average side condtion for link level simulation assumption for CRS-IM in homogeneous network R4-133994 TP for TR 36.863 V xxx: side conditions for CRS-IM from different companies R4-134479, TP for TR 36.863 V xxx: Interference modelling for CRS-IM R4-134463, TP for TR 36.863 V 0.1.0 Editorial updates by the rapporteur: - Version number changed - Add section tile for 6.3 - Change section 6.3.2 of R4-133994 into section 6.3.3 and change the corresponding Table index. - Remove the duplicated tables | 0.2.0 | 0.3.1 |
| 2013-11 | 69 | R4-13xxxx |  |  | Agreed text proposals from RAN4#68bis, the following TPs have been implemented: R4-135623, TR 36.863 V0.3.1: Draft TP on final summary for CRS-IM performance in homogeneous network R4-135628, TR 36.863 V0.3.1: Draft TP on link and system level summary for CRS-IM performance in homogeneous network R4-135624, TR 36.863 V0.3.1: Draft TP on receiver structure for CRS-IM performance in homogeneous network R4-135352, TR 36.863 V0.3.1: Draft TP on average system level simulation resutls R4-135626, TR 36.863 V0.3.1: Draft TP on Link level simulation resutls from different companies, Ericsson R4-135625, TR 36.863 V0.3.1: Draft TP on average Link level simulation resutls | 0.3.1 | 0.4.0 |