3GPP TSG-WG4 Meeting #72bis	R4-146432

# Singapore, 6th – 10th October, 2014

Agenda item: 	7.15.1

Source: 	Qualcomm Incorporated

Title: 	UE demodulation performance results due to introduction of DCH enhancements

Document for:	Discussion

# 1	Introduction

This contribution presents performance results due to introduction of DCH enhancements. The simulations have been run with realistic receiver assumptions and the DPCH Ec/Ior has been calculated only during the ON duration (excluding DTX). The UE is assumed to be in basic configuration unless other wise specified. Also the UE targets a BLER of 1% at 20ms when DCCH is On and the UE targets a BLER of 1% at 10ms when DCCH is Off.

# 2	DCH performance results

## 2.1	Single-link DCH performance

It was agreed in [3] that new requirements for DCH enahcnements will be introduced corresponding to Sections 8.2 and 8.3 of TS25.101 using existing framework but with the new RMCs. The tests in TS25.101 Sections 8.2 and 8.3 impose DPCH_Ec/Ior requirements for various fixed reference measurement channels (RMC) in various propagation channels with power-control off. These tests can be extended to the newly defined RMCs using the existing test framework.

The key parameters for this simulation are as shown below.

# Table 1: Test parameters

| Parameter | Unit | Full | Null |
| --- | --- | --- | --- |
| Phase reference |  | P-CPICH |  |  |
|  | dBm/3.84 MHz | -60 |  |  |
| Information Data Rate | kbps | 12.2 | 0 |

The parameter is set to -1dB for Static channel test, 9dB for Case 1, and, -3dB for Case 2, Case 3 and Case 6 channels. The following are the simulation results for the basic configuration showing the average downlink  power ratio to achieve BLER of 0.01. The section number, table number, channel and geometry

| Section # | Subsection | Table | Channel | Geometry |
| --- | --- | --- | --- | --- |
| 8.2 | 8.2.1 | 8.5 | AWGN | -1 |
|  | FULL+DCCH | FULL-DCCH | NULL+DCCH | NULL-DCCH |
| Ec/Ior (dB) | -19.6 | -17.5 | -21.1 | -19 |
| Section # | Subsection | Table | Channel | Geometry |
| 8.3 | 8.3.1 | 8.7 | Case 1 | 9 |
|  | FULL+DCCH | FULL-DCCH | NULL+DCCH | NULL-DCCH |
| Ec/Ior (dB) | -18.3 | -15.4 | -21.2 | -18.6 |
| Section # | Subsection | Table | Channel | Geometry |
| 8.3 | 8.3.1 | 8.9 | Case 2 | -3 |
|  | FULL+DCCH | FULL-DCCH | NULL+DCCH | NULL-DCCH |
| Ec/Ior (dB) | -9.6 | -6.8 | -12.7 | -10.2 |
| Section # | Subsection | Table | Channel | Geometry |
| 8.3 | 8.3.1 | 8.11 | Case 3 | -3 |
|  | FULL+DCCH | FULL-DCCH | NULL+DCCH | NULL-DCCH |
| Ec/Ior (dB) | -14.4 | -11.3 | -16.3 | -13.8 |
| Section # | Subsection | Table | Channel | Geometry |
| 8.3 | 8.3.1 | 8.13 | Case 1 | 9 |
|  | FULL+DCCH | FULL-DCCH | NULL+DCCH | NULL-DCCH |
| Ec/Ior (dB) | -18.6 | -15.4 | -21.5 | -18.6 |
| Section # | Subsection | Table | Channel | Geometry |
| 8.3 | 8.3.1 | 8.14A | Case 6 | -3 |
|  | FULL+DCCH | FULL-DCCH | NULL+DCCH | NULL-DCCH |
| Ec/Ior (dB) | -13.9 | -10.9 | -15.9 | -13.4 |

## 2.2	Handover performance

It was agreed in [3] that new requirements for DCH enhancements will be introduced corresponding to Section 8.7.1 of TS25.101 and also the TPC combining performance requirements of Sections 8.7.2 and 8.7.3 of TS25.101 that test the UE obeys UL power control commands transmitted on the DL. Here the DL RMC does not play a role, since DL DCH demodulation performance is not tested and DL power control is disabled. Thus, the goal of the new test is primarily to re-test the existing functionality with the new pilot-free slot-format. It was agreed that the uplink transmits Full packet in 20ms transmission mode, so that UE has to receive all the TPC bits sent on the downlink.

For the test on demodulation of DCH in Inter-Cell Soft Handover corresponding to section 8.7.1, the key parameters for this simulation are as shown below.

# Table 2: Test parameters

| Parameter | Unit | Full | Null |
| --- | --- | --- | --- |
| Phase reference |  | P-CPICH |  |  |
|  | dBm/3.84 MHz | -60 |  |  |
| Information Data Rate | kbps | 12.2 | 0 |

The  and parameters is set to 0dB.

The following are the simulation results for the average downlink  power ratio to achieve BLER of 0.01.

| Section # | Subsection | Table | Channel | Geometry |
| --- | --- | --- | --- | --- |
| 8.7 | 8.7.1 | 8.25 | Case 3 | 0 |
|  | FULL-DCCH |
| EcIor(dB) | -16.3 |

## 2.3	DL power control performance

It was agreed that new requirements are defined corresponding to Sections 8.8.1,8.8.2, 8.8.3 and 8.8.4 of TS25.101 for DL power-control, using existing framework, where uplink is set up so that Ack sent on UL is reliably received.

The requirements of Sections 8.8.1, 8.8.2 and 8.8.3 all specify a DPCH_Ec/Ior level required for certain RMCs with power control on. With DL FET active, the DPCH_Ec/Ior measurement will depend on whether the NodeB DTXes the DL DPDCH in response to Ack received on the uplink. The uplink in the test setup is ensured to be strong enough so that the test equipment can receive the uplink transport blocks as well as Ack channel reliably.

## 2.3.1 Power control in the downlink, constant BLER target

For the test on Power control in the downlink, constant BLER target corresponding to section 8.8.1, the key parameters for this simulation are as shown below.

# Table 3: Test parameters

| Parameter | Unit | Full | Null |
| --- | --- | --- | --- |
| Phase reference |  | P-CPICH |  |  |
|  | dBm/3.84 MHz | -60 |  |  |
| Information Data Rate | kbps | 12.2 | 0 |

Table 4: Test parameter for downlink power control

| Parameter | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- |
|  | dB | 9 | -1 | 4 | 9 |
|  | dBm/3.84 MHz | -60 |  | -60 |  |
| Information Data Rate | Kbps | 12.2 |  | 0 |  |
| Target quality value on DTCH | BLER | 0.01 |  | 0.1 | 0.001 |
| Target quality value on DCCH | BLER | - |  | 0.1 | 0.1 |
| Propagation condition |  | Case 4 |  |  |  |
| Maximum_DL_Power * | dB | 7 |  |  |  |
| Minimum_DL_Power * | dB | -18 |  |  |  |
| DL Power Control step size, TPC | dB | 1 |  |  |  |
| Limited Power Increase | - | "Not used" |  |  |  |

The following are the simulation results for the 90% downlink power ratio

| Section # | Subsection | Table | Channel | Geometry |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 8.8.1 | 8.8.1.1 | 8.29 | AWGN | 9,-1 |  |  |  |
|  |  | Basic Config 20ms |  |  | Basic Config 10ms |  |  |
|  |  | EcIor | DTCH BLER | 90% EcIor | EcIor | DTCH BLER | 90% EcIor |
| Test1 | FULL-DCCH | -21.73 | 0.011 | -19.11 | -18.63 | 0.0108 | -16.13 |
|  | NULL-DCCH | -23.45 | 0.0082 | -21.03 | -20.54 | 0.0112 | -18.09 |
| Test2 | FULL-DCCH | -14.61 | 0.0112 | -12.03 | -10.8 | 0.01 | -7.24 |
|  | NULL-DCCH | -16.2 | 0.0086 | -13.19 | -13.11 | 0.0114 | -10.12 |

| Section # | Subsection | Table | Channel |  |  | Geometry |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8.8.1 | 8.8.1.1 | 8.29 | AWGN |  |  | 9,-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Full Config 10ms |  |  |  |  | Full Config 10ms |  |  |  |  |  |
|  |  | EcIor |  | DTCH BLER | 90% EcIor |  | EcIor |  | DTCH BLER |  |  | 90% EcIor |
| Test1 | FULL-DCCH | -21.75 |  | 0.011 | -19.13 |  | -21.71 |  | 0.011 |  |  | -19.12 |
|  | NULL-DCCH | -23.41 |  | 0.0078 | -21.04 |  | -23.34 |  | 0.008 |  |  | -21.02 |
| Test2 | FULL-DCCH | -14.42 |  | 0.0112 | -11.23 |  | -14.31 |  | 0.011 |  |  | -11.2 |
|  | NULL-DCCH | -16.04 |  | 0.0092 | -13.13 |  | -15.98 |  | 0.009 |  |  | -13.12 |

## 2.3.2 Power control in the downlink, initial convergence

For the test on Power control in the downlink, initial convergence corresponding to section 8.8.2, the key parameters for this simulation are as shown below.

Table 5: Test parameter for downlink power control

| Parameter | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- |
| Target quality value on DTCH | BLER | 0.01 | 0.01 | 0.1 | 0.1 |
| Initial DPCH_Ec/Ior | dB | -5.9 | -25.9 | -3 | -22.8 |
| Information Data Rate | kbps | 12.2 | 12.2 | 64 | 64 |
|  | dB | -1 |  |  |  |
|  | dBm/3.84 MHz | -60 |  |  |  |
| Propagation condition |  | Static |  |  |  |
| Maximum_DL_Power | dB | 7 |  |  |  |
| Minimum_DL_Power | dB | -18 |  |  |  |
| DL Power Control step size, TPC | dB | 1 |  |  |  |
| Limited Power Increase | - | "Not used" |  |  |  |

The the simulation results showing CDF for the average downlink  power ratio for each test case during time T1 and time T2 are presented in the Appendix section.

## 2.3.3 Power control in the downlink, wind-up

For the test on Power control in the downlink, wind-up corresponding to section 8.8.3, the key parameters for this simulation are as shown below. Power control of the UE is ON during the test.

Table 6: Test parameter for downlink power control, wind-up effects

| Parameter | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | Stage 1 | Stage 2 | Stage 3 |
| Time in each stage | s | 5 | 5 | 0.5 |
|  | dB | 5 |  |  |
|  | dBm/3.84 MHz | -60 |  |  |
| Quality target on DTCH | BLER | 0.01 |  |  |
| Propagation condition |  | Case 4 |  |  |
| Maximum_DL_Power | dB | 7 | min(-6.2,P). Note 1 | 7 |
| Minimum_DL_Power | dB | -18 |  |  |
| DL Power Control step size, TPC | dB | 1 |  |  |
| Limited Power Increase | - | "Not used" |  |  |
| Note 1: P is the level corresponding to the average power ratio - 2 dB compared to the P-CPICH level. The average power ratio is measured during the initialisation stage after the power control loop has converged before the actual test starts. |  |  |  |  |

The following are the simulation results for the 90% downlink  power ratio

| Section # | Subsection | Table | Channel | Geometry |  |
| --- | --- | --- | --- | --- | --- |
| 8.8 | 8.8.3 | 8.33 | Case 4 | 5 |  |
|  |  | Basic Config 20ms |  | Basic Config 10ms |  |
|  |  | EcIor | 90% EcIor | EcIor | 90% EcIor |
| Stage 3 | FULL-DCCH | -20.22 | -16.88 | -18.55 | -16.15 |
|  | NULL-DCCH | -21.41 | -18.43 | -19.63 | -16.43 |

| Section # | Subsection | Table | Channel |  |  | Geometry |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8.8 | 8.8.3 | 8.33 | Case 4 |  |  | 5 |  |  |  |
|  |  | Full Config 10ms |  |  | Full Config 10ms NULL UL |  |  |  |
|  |  | EcIor |  | 90% EcIor | EcIor |  | 90% EcIor |  |
| Stage 3 | FULL-DCCH | -20.1 |  | -16.63 | -20.14 |  | -17.5 |  |
|  | NULL-DCCH | -21.97 |  | -19.15 | -21.27 |  | -18.26 |  |

## 2.3.4 Power control in the downlink, different transport formats

For the test on Power control in the downlink, different transport formats corresponding to section 8.8.4, the key parameters for this simulation are as shown below. Power control in downlink is ON during the test.

Table 7: Test parameter for downlink power control

| Parameter | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | Stage 1 | Stage 2 | Stage 3 |
| Time in each stage | s | 5 | 5 | 0.5 |
|  | dB | 5 |  |  |
|  | dBm/3.84 MHz | -60 |  |  |
| Information Data Rate | kbps | 12.2 |  |  |
| Quality target on DTCH | BLER | 0.01 |  |  |
| Propagation condition |  | Case 4 |  |  |
| Maximum_DL_Power | dB | 7 | min(-6.2,P). Note 1 | 7 |
| Minimum_DL_Power | dB | -18 |  |  |
| DL Power Control step size, TPC | dB | 1 |  |  |
| Limited Power Increase | - | "Not used" |  |  |
| Note 1: P is the level corresponding to the average power ratio - 2 dB compared to the P-CPICH level. The average power ratio is measured during the initialisation stage after the power control loop has converged before the actual test starts. |  |  |  |  |

The following are the simulation results for the 90% downlink  power ratio

| Section # | Subsection | Table | Channel | Geometry |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 8.8.4 | 8.8.4.1 | 8.34A | Case 4 | 9 |  |  |  |
|  |  | Basic Config 20ms |  |  | Basic Config 10ms |  |  |
|  |  | EcIor | DTCH BLER | 90% EcIor | EcIor | DTCH BLER | 90% EcIor |
| Test1 | FULL-DCCH | -21.73 | 0.011 | -19.11 | -18.63 | 0.0108 | -16.13 |
| Test2 | NULL-DCCH | -16.2 | 0.0086 | -13.19 | -13.11 | 0.0114 | -10.12 |

| Section # | Subsection | Table | Channel |  |  | Geometry |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8.8.4 | 8.8.4.1 | 8.34A | Case 4 |  |  | 9 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Full Config 10ms |  |  |  |  | Full Config 10ms NULL UL |  |  |  |  |  |
|  |  | EcIor |  | DTCH BLER | 90% EcIor |  | EcIor |  | DTCH BLER |  |  | 90% EcIor |
| Test1 | FULL-DCCH | -21.75 |  | 0.011 | -19.13 |  | -21.71 |  | 0.011 |  |  | -19.12 |
| Test2 | NULL-DCCH | -16.04 |  | 0.0092 | -13.13 |  | -15.98 |  | 0.009 |  |  | -13.12 |

# 4	Conclusion

This contribution presents simulations results to show required average DPCH Ec/Ior with DCH enhancements for

- Single-link DCH performance

- Demodulation of DCH in Inter-Cell Soft Handover

- Power control in the downlink, constant BLER target

- Power control in the downlink, initial convergence

- Power control in the downlink, wind-up

- Power control in the downlink, different transport formats

# 5	Reference

- R4-145140, “Further discussion on UE RAN4 requirements due to introduction of DCH enhancements”, Qualcomm Incorporated

- RP-145141, “New DL reference measurement channels for DCH enhancements," Qualcomm Incorporated

- R1-145387, “WF on UE Requirements for DCH enhancements”, Qualcomm Incorporated

# 6	Appendix

CDF Results for the test on Power control in the downlink, initial convergence corresponding to section 8.8.2 are as follows:

| Section # | Subsection | Table | Channel | Geometry |
| --- | --- | --- | --- | --- |
| 8.8.2 | 8.8.2.1 | 8.31 | AWGN | 9,-1 |

| Test 1 | T1 | 20ms | basic | Full-DCCH | Test 2 | T1 | 20ms | basic | Full-DCCH |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 1 | T2 | 20ms | basic | Full-DCCH | Test 2 | T2 | 20ms | basic | Full-DCCH |
| Test 1 | T1 | 20ms | basic | Null-DCCH | Test 2 | T1 | 20ms | basic | Null-DCCH |
| Test 1 | T2 | 20ms | basic | Null-DCCH | Test 2 | T2 | 20ms | basic | Null-DCCH |
| Test 1 | T1 | 10ms | basic | Full-DCCH | Test 2 | T1 | 10ms | basic | Full-DCCH |

| Test 1 | T2 | 10ms | basic | Full-DCCH | Test 2 | T2 | 10ms | basic | Full-DCCH |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 1 | T1 | 10ms | basic | Null-DCCH | Test 2 | T1 | 10ms | basic | Null-DCCH |

| Test 1 | T2 | 10ms | basic | Null-DCCH | Test 2 | T2 | 10ms | basic | Null-DCCH |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 1 | T1 | 10ms | full config | Full-DCCH | Test 2 | T1 | 10ms | full config | Full-DCCH |

| Test 1 | T2 | 10ms | full config | Full-DCCH | Test 2 | T2 | 10ms | full config | Full-DCCH |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 1 | T1 | 10ms | full config | Null-DCCH | Test 2 | T1 | 10ms | full config | Null-DCCH |

| Test 1 | T2 | 10ms | full config | Null-DCCH | Test 2 | T2 | 10ms | full config | Null-DCCH |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Test 1 | T1 | 10ms | full config UL null | Full-DCCH | Test 2 | T1 | 10ms | full config UL null | Full-DCCH |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Test 1 | T2 | 10ms | full config UL null | Full-DCCH | Test 2 | T2 | 10ms | full config UL null | Full-DCCH |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Test 1 | T1 | 10ms | full config UL null | Null-DCCH | Test 2 | T1 | 10ms | full config UL null | Null-DCCH |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 1 | T2 | 10ms | full config UL null | Null-DCCH | Test 2 | T2 | 10ms | full config UL null | Null-DCCH |