3GPP TSG-RAN WG4 Meeting #74bis	R4-151922

Rio de Janeiro, Brazil, 20 - 24 April 2015

Source: 	Nokia Networks

Title: 					Initial performance evaluation for RRM measurement in enhanced coverage

Agenda Item: 	7.7.5

Document for:	Discussion

# Introduction

The Rel-13 WI on further MTC enhancements [1] is currently under investigation in RAN1. On RRM measurement, two LSs were sent by RAN1#80 meeting [2][3].

In [2], RAN1 is asking

RAN4 to provide feedback on RSRP and RSRQ measurement performance for Rel-13 low complexity UEs in normal coverage and enhanced coverage.

In this paper, we will provide initial link level simulation results for the RRM measurement in enhanced coverage.

# Discussion

2.1 Simulation assumptions

This is the first time the WI is discussed in RAN4 RRM/Demod session, and there is no agreed simulation assumptions to evaluate the performance of RRM measurement in enhanced coverage.

The worst case for the RRM measurement is when 1 Rx is used and with the maximum MCL of 155.7dB. In [4] we derive the SNR condition for serving cell measurement as -14.2dB with 15dB coverage enhancement. The SNR side condition may be even worse considering the neighbor cell measurement, but as it has not been discussed in RAN4 we will focus on serving cell measurement in this paper.

We also simulate the -9.2dB SNR condition which corresponds to 10dB coverage enhancement, and -6/-3dB which are used as side conditions fro current requirements for reference. The sampling period is 80ms as for Cat-0 UE in Rel-12, and the measurement period is 400ms as baseline. In addition, considering that extending the measurement period is the most straightforward way to improve the accuracy, we also simulate measurement period of 800ms and 1600ms while keeping the same sampling rate.

For other parameter settings, we are re-using the agreed simulation assumptions for Rel-12 study. The whole simulation assumptions are summarized in Table 1.

Table 1 Simulation assumptions for RRM measurement in enhanced coverage

| Parameters | Value |
| --- | --- |
| SNR | {-14.2, -9.2, -6, -3} dB |
| Measurement Bandwidth | 6 RBs |
| Number of Tx Antennas | 1 |
| Number of Rx Antennas | 1 |
| Measurement Period | 400ms, 800ms, 1600ms |
| Sampling Period | 80ms |
| L3 filtering | Disable |
| DRX | OFF |
| Propagation Condition | AWGN, EPA5, ETU70 |
| CP Length | Normal |

2.2 Simulation results

The CDF curves of the delta RSRP/RSRQ (the difference between the measured value and the nominal value which is estimated with perfect channel information) are derived from the simulation, and the 5%-, 50%- and 95%-tile of the curves are summarized in Table 2 to Table 3.

Table 2 Simulation results of Delta RSRP with 400ms measurement period

| Channel model | SNR (dB) | 5%-tile | 50%-tile | 95%-tile |
| --- | --- | --- | --- | --- |
| AWGN | -3 | -1.2329 | 0.1039 | 1.1512 |
|  | -6 | -2.8969 | 0.1086 | 1.8225 |
|  | -9.2 | -4.1728 | 0.6757 | 3.2611 |
|  | -14.2 | -2.3850 | 3.2422 | 6.5975 |
| EPA5 | -3 | -1.6056 | 0.0963 | 1.4847 |
|  | -6 | -2.9619 | 0.1745 | 2.4806 |
|  | -9.2 | -3.7647 | 1.0226 | 4.2321 |
|  | -14.2 | -1.7422 | 3.9768 | 8.8768 |
| ETU70 | -3 | -1.4682 | 0.0577 | 1.3217 |
|  | -6 | -2.5977 | 0.2496 | 2.3332 |
|  | -9.2 | -4.2205 | 0.8540 | 3.6038 |
|  | -14.2 | -2.2504 | 3.7058 | 7.3914 |

Table 3 Simulation results of Delta RSRQ with 400ms measurement period

| Channel model | SNR (dB) | 5%-tile | 50%-tile | 95%-tile |
| --- | --- | --- | --- | --- |
| AWGN | -3 | -0.8612 | 0.4434 | 1.4631 |
|  | -6 | -2.6901 | 0.3123 | 2.0388 |
|  | -9.2 | -4.0287 | 0.7920 | 3.3567 |
|  | -14.2 | -2.3586 | 3.2734 | 6.5921 |
| EPA5 | -3 | -1.3616 | 0.4541 | 1.7194 |
|  | -6 | -2.7618 | 0.4035 | 2.5981 |
|  | -9.2 | -3.6370 | 1.1294 | 4.2563 |
|  | -14.2 | -1.6935 | 4.0182 | 8.9359 |
| ETU70 | -3 | -1.1735 | 0.4137 | 1.5765 |
|  | -6 | -2.4236 | 0.4720 | 2.4771 |
|  | -9.2 | -4.1833 | 0.9838 | 3.6692 |
|  | -14.2 | -2.2069 | 3.7473 | 7.4473 |

From the tables above, it can be seen that the accuracy of the RSRP/RSRQ measurement is significantly degraded with low SNR condition. For example, if we use the max(5%, 95%) across all channel models to derive the requirement, the RSRP performance requirement would be ±12.9dB including a 4dB RF margin.

Observation 1: The accuracy of the RSRP/RSRQ measurement in enhanced coverage is significantly degraded due to low SNR condition.

Simulation results are summarized through Table 4 to Table 7 for RSRP/RSRQ with 800/1600ms measurement period.

Table 4 Simulation results of Delta RSRP with 800ms measurement period

| Channel model | SNR (dB) | 5%-tile | 50%-tile | 95%-tile |
| --- | --- | --- | --- | --- |
| AWGN | -3 | -0.92716 | 0.025985 | 0.84802 |
|  | -6 | -1.7968 | 0.087266 | 1.4224 |
|  | -9.2 | -4.2161 | 0.27433 | 2.3344 |
|  | -14.2 | -3.1129 | 2.2197 | 5.4173 |
| EPA5 | -3 | -1.1565 | 0.021492 | 0.93082 |
|  | -6 | -2.1986 | 0.17282 | 1.872 |
|  | -9.2 | -2.8459 | 0.51602 | 2.6841 |
|  | -14.2 | -3.274 | 2.4954 | 5.9355 |
| ETU70 | -3 | -1.1308 | 0.006575 | 0.83863 |
|  | -6 | -1.8586 | 0.058175 | 1.4507 |
|  | -9.2 | -3.365 | 0.41432 | 2.7173 |
|  | -14.2 | -3.2912 | 2.4058 | 5.7415 |

Table 5 Simulation results of Delta RSRQ with 800ms measurement period

| Channel model | SNR (dB) | 5%-tile | 50%-tile | 95%-tile |
| --- | --- | --- | --- | --- |
| AWGN | -3 | -0.58676 | 0.36283 | 1.1813 |
|  | -6 | -1.6128 | 0.28414 | 1.6153 |
|  | -9.2 | -4.1051 | 0.35365 | 2.4377 |
|  | -14.2 | -3.0597 | 2.2605 | 5.4481 |
| EPA5 | -3 | -0.84602 | 0.35536 | 1.2048 |
|  | -6 | -2.0228 | 0.38964 | 1.9961 |
|  | -9.2 | -2.7403 | 0.62852 | 2.7631 |
|  | -14.2 | -3.2559 | 2.5338 | 5.9362 |
| ETU70 | -3 | -0.79492 | 0.34205 | 1.1269 |
|  | -6 | -1.6316 | 0.26895 | 1.6319 |
|  | -9.2 | -3.2376 | 0.53156 | 2.787 |
|  | -14.2 | -3.1963 | 2.4305 | 5.7359 |

Table 6 Simulation results of Delta RSRP with 1600ms measurement period

| Channel model | SNR (dB) | 5%-tile | 50%-tile | 95%-tile |
| --- | --- | --- | --- | --- |
| AWGN | -3 | -0.67975 | 0.03317 | 0.68955 |
|  | -6 | -1.2657 | 0.13917 | 1.0522 |
|  | -9.2 | -2.8708 | -0.05808 | 1.8547 |
|  | -14.2 | -3.8303 | 1.5774 | 4.2654 |
| EPA5 | -3 | -0.6628 | 0.027432 | 0.66687 |
|  | -6 | -1.2747 | 0.057859 | 1.1323 |
|  | -9.2 | -2.5421 | 0.15264 | 1.9669 |
|  | -14.2 | -3.9336 | 1.7456 | 4.6256 |
| ETU70 | -3 | -0.81509 | 0.008751 | 0.5526 |
|  | -6 | -1.1309 | 0.028059 | 1.0915 |
|  | -9.2 | -2.4865 | 0.19256 | 1.8936 |
|  | -14.2 | -3.6103 | 1.5971 | 4.5011 |

Table 7 Simulation results of Delta RSRQ with 1600ms measurement period

| Channel model | SNR (dB) | 5%-tile | 50%-tile | 95%-tile |
| --- | --- | --- | --- | --- |
| AWGN | -3 | -0.3493 | 0.36164 | 0.99458 |
|  | -6 | -1.0671 | 0.33986 | 1.254 |
|  | -9.2 | -2.773 | 0.03964 | 1.9563 |
|  | -14.2 | -3.8168 | 1.6301 | 4.2897 |
| EPA5 | -3 | -0.31258 | 0.38308 | 0.97347 |
|  | -6 | -1.0974 | 0.25082 | 1.296 |
|  | -9.2 | -2.4538 | 0.27875 | 2.0911 |
|  | -14.2 | -3.9072 | 1.7902 | 4.6324 |
| ETU70 | -3 | -0.4446 | 0.35017 | 0.88442 |
|  | -6 | -0.95936 | 0.2272 | 1.2589 |
|  | -9.2 | -2.4152 | 0.29593 | 1.987 |
|  | -14.2 | -3.5998 | 1.6026 | 4.5213 |

It can be seen there is obvious accuracy improvement when the measurement period is extended from 400ms to 800ms. For example, if we use the same methodology to derive the accuracy requirement for -14.2dB SNR, the requirement can be ±9.9dB, i.e. the gain is 3dB.

However, when the measurement period is extended further to 1600ms, the gain becomes less. Still considering -14.2dB SNR, the accuracy requirement would be ±8.6dB, so the gain is only 1.3dB.

Observation 2: 3dB gain can be achieved by extending the measurement period from 400ms to 800ms, while the gain by further extending measurement period to 1600ms is less.

# Conclusions

In this paper, we derived the SNR condition for serving cell measurement, and provided simulation results on the accuracy of the RRM measurement with 1 Rx in enhanced coverage. From the simulation results, we observed that

Observation 1: The accuracy of the RSRP/RSRQ measurement in enhanced coverage is significantly degraded due to low SNR condition.

Observation 2: 3dB gain can be achieved by extending the measurement period from 400ms to 800ms, while the gain by further extending measurement period to 1600ms is less.

# References

- RP-141660,	New WI proposal: further LTE physical layer enhancements for MTC, Ericsson, Nokia Networks

- R1-150919, LS on measurement performance for MTC

- R1-150920, LS on PRACH coverage enhancement

- R4-151921, RRM measurement for coverage enhancement for MTC, Nokia Networks