3GPP TSG-RAN WG4 #74bis	R4-152060

Rio de Janeiro, Brazil 20th April– 24th April 2015

Source: 	Ericsson

Title:	Calibration results for RMC regarding UE reference sensitivity

Agenda Item:	6.5.2

Document for:	Discussion

# Introduction

In RAN4#74 meeting, the first round alignment for RMC for REFSENS was performed. Based on the collected results captured in [1], the following span was observed:

- PSDCH		: Span of 1.24dB

- PSSCH (10MHz)	: Span of 1.97dB

- PSSCH (5MHz)	: Span of 2.22dB

To have further alignment and avoid any misunderstanding for the simulation assumption, one WF paper is approved in [2]. In [2], seven test cases are proposed for further alignment, as shown in Table 1. In this paper, we provide our simulation results for these cases.

Table 1: simulation case

| Case | D2D channel | Time offset | Frequency offset | Channel | HARQ Retransmissions | Time/Frequency offset estimation and compensation |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | PSDCH | 0 us | 0 Hz | AWGN | 0 | {No, Yes} |
| 2 | PSSCH (5MHz) | 0 us | 0 Hz | AWGN | 0 | {No, Yes} |
| 3 | PSDCH | 0 us | 0 Hz | EVA | 0 | {No, Yes} |
| 4 | PSSCH (5MHz) | 0 us | 0 Hz | EVA | 0 | {No, Yes} |
| 5 | PSDCH | +23 Ts | Tx: 200Hz | AWGN | 0 | {No, Yes} |
| 6 | PSSCH (5MHz) | +23 Ts | Tx: 200Hz | AWGN | 0 | {No, Yes} |
| 7 | PSSCH (5MHz) | 0 us | 0 Hz | AWGN | 3 | {No, Yes} |

# Simulation results

In [3], the simulation parameters are given for discovery and communication. These parameters are recapped in Appendix A. With the additional setup from Table 1, link level simulation results for Case 1~ Case 7 are provided. The target SNR at BLER=5% is tabulated in Table 2.

Figure 1: Link level simulation results for Case 1

Figure 2: Link level simulation results for Case 2

Figure 3: Link level simulation results for Case 3

Figure 4: Link level simulation results for Case 4

Figure 5: Link level simulation results for Case 5

Figure 6: Link level simulation results for Case 6

Figure 7: Link level simulation results for Case 7

Table 2: Target SNR when BLER=5% for each calibration case

| Case | D2D channel | Time offset | Frequency offset | Channel | HARQ Retransmissions | SNR@5%BLER (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | PSDCH | 0 us | 0 Hz | AWGN | 0 | -0.5 |
| 2 | PSSCH (5MHz) | 0 us | 0 Hz | AWGN | 0 | -3.35 |
| 3 | PSDCH | 0 us | 0 Hz | EVA70 | 0 | 0 |
| 4 | PSSCH (5MHz) | 0 us | 0 Hz | EVA70 | 0 | -2 |
| 5 | PSDCH | +23 Ts | Tx: 200Hz | AWGN | 0 | -0.5 |
| 6 | PSSCH (5MHz) | +23 Ts | Tx: 200Hz | AWGN | 0 | -3.3 |
| 7 | PSSCH (5MHz) | 0 us | 0 Hz | AWGN | 3 | -8.9 |

# Conclusion

In this paper, we propose some link level results for RMC of REFSENS for calibration. We hope group can consider these results in the final SNR decision for the RMC of REFSENS.

# Appendix A: Simulation parameters

Table 1: D2D discovery reference measurement channel for receiver requirements (FDD)

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 1.4 | 3 | 5 | 10 | 15 | 20 |
| Allocated resource blocks |  |  |  | [2] | [2] | [2] | [2] |
| Subcarriers per resource block |  |  |  | 12 | [12] | [12] | [12] |
| Allocated subframes per Discovery period |  |  |  | [1] | [1] | [1] | [1] |
| Modulation |  |  |  | QPSK | QPSK | QPSK | QPSK |
| Transport Block Size |  |  |  | [224] | [224] | [224] | [224] |
| Transport block CRC | Bits |  |  | [24] | [24] | [24] | [24] |
| Maximum number of HARQ transmissions |  |  |  | [1] | [1] | [1] | [1] |
| Binary Channel Bits | Bits |  |  | [576] | [576] | [576] | [576] |
| Max. Throughput averaged over 1 Discovery period | kbps |  |  | [TBD] | [TBD] | [TBD] | [TBD] |
| UE Category |  |  |  | [≥ 1] | [≥ 1] | [≥ 1] | [≥ 1] |
| Note 1: For PSDCH transmission, the last symbol shall be punctured as per TS 36.211. Editor’s Note: Throughput (in kbps) will depend on discovery period configuration. Editor’s Note: Further details for specifications will be required (e.g., discovery pool configuration, etc.). However, the above details are sufficient to simulate the RMCs. Editor’s Note: TBS size may increase by 1 byte due to additional 4 bits in discovery message requested by SA3 [S3-142355]. |  |  |  |  |  |  |  |

Table 2: D2D communications reference measurement channel for receiver requirements (FDD)

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 1.4 | 3 | 5 | 10 | 15 | 20 |
| Allocated resource blocks |  |  |  | [25] | [50] |  |  |
| Subcarriers per resource block |  |  |  | 12 | 12 |  |  |
| Packets per SA period |  |  |  | [TBD] | [TBD] |  |  |
| Modulation |  |  |  | [QPSK] | [QPSK] |  |  |
| Transport Block Size |  |  |  | [2216] | [4392] |  |  |
| Transport block CRC | Bits |  |  | [24] | [24] |  |  |
| Maximum number of HARQ transmissions |  |  |  | [4] | [4] |  |  |
| Binary Channel Bits | Bits |  |  | [7200] | [14400] |  |  |
| Max. Throughput averaged over 1 SA period | kbps |  |  | [TBD] | [TBD] |  |  |
| UE Category |  |  |  | [≥ 1] | [≥ 1] |  |  |
| Note 1: For PSSCH transmission, the last symbol shall be punctured as per TS 36.211. Editor’s Note: Throughput (in kbps) will depend on discovery period configuration. Editor’s Note: Further details for specifications will be required (e.g., SA and data pool configuration, etc.). However, the above details are sufficient to simulate the RMCs. |  |  |  |  |  |  |  |

Table 3: Main assumption for discovery and communication

| D2D Test/Simulation parameter | Proposals |
| --- | --- |
|  | Rx RF characteristics |
| AGC settling time (not used for demodulation) | QPSK: 1 symbol |
| Tx EVM | 10% |
| UE RRC state | Not required for simulation purpose. Will be needed when test procedure is defined, and is FFS |
| Propagation channel | Static H = [ 1; 1] |
| Doppler spectrum | N/A |
| Timing error (eNodeB DL and D2D Tx) | [±12Ts] |
| Frequency error (eNodeB UL and D2D Tx) | Option 1: ±10 Hz, Option 2: ±200Hz |
| HARQ retransmissions | Discovery: 0 Communications: 3 |
| Soft-combining | No soft combining for Discovery 1 Communiction: Option 1: No soft combining Option 2: Use sof combining |
| Performance metric | SNR @ x% [BLER]/[Throughput] |

# Reference

- R4-151062, Summary of demodulation simulation results of D2D RMC for REFSENS, Feb, 2015.

- R4-151143, WF on D2D RMC performance evaluation for future meetings, Qualcomm, Feb. 2015.

- R4-148062, WF on simulation assumptions for D2D REFSENS demodulation, Intel, Dec. 2014.