3GPP TSG-RAN WG4 (Radio) Meeting #76	R4-153995

Beijing, China, 24 – 28 August 2015

Agenda Item:	7.39.2

Source: 	Alcatel-Lucent, U.S. Cellular

Title: 	Required BS studies of Harmonics and Intermodulation Products caused by LTE Advanced 4DL/1UL Carrier Aggregation with Bands 2, 4, 5 and 12

Document for:	Discussion

# 1.	Introduction

5 WI proposals to support the LTE Advanced (LTE-A) 4DL/1UL Carrier Aggregation (CA) with Bands 2, 4, 5 and 12 were approved in RAN#68 [1-5]. One objective of the WIs is to specify the band-combination specific Radio Frequency (RF) requirements for the new inter-band CA scenarios. Base Station (BS) studies required to be performed on the new inter-band CA scenarios are the impacts of harmonics and InterModulation Distortion (IMD) products caused by LTE-A BS supporting 4DL CA configurations to the receiver of own or different BS.

In this contribution, we provide an analysis on the required BS studies of harmonics and IMD products for the 5 WI proposals approved in RAN#68 on LTE-A 4DL/1UL CA with Bands 2, 4, 5 and 12.

# 2.	Discussion

As we discussed and agreed in the past [4], no additional BS study of 2nd and 3rd harmonics and IMD products will be required for a 4DL CA configuration, if it does not use a larger carrier channel bandwidth or aggregated channel bandwidth in any of the CA bands when compared to the constituent 3 DL CA configurations, and the BS studies of 2nd and 3rd harmonics and IMD products for all the constituent 3 DL CA configurations have been performed..

In the following, we provide an overview on the required BS studies of harmonics and IMD products for the 5 WI proposals approved in RAN#68 on LTE-A 4DL/1UL CA with Bands 2, 4, 5 and 12, considering the BS studies that have been performed for the constituent 3 DL CA configurations.

## 2.1	Band (2 + 4 + 5 + 12)

The CA scenario is provided in Table 1 below [1]:

Table 1 – Interband 4 Band CA Scenario (single uplink)

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | E-UTRA Band or E-UTRA CA configuration | 1.4 | 3 | 5 | 10 | 15 | 20 |  | Maximum aggregated bandwidth | Bandwidth combination set |
|  |  | MHz | MHz | MHz | MHz | MHz | MHz |  | [MHz] |  |
| CA_2A-4A-5A-12A | 2 |  |  | Yes | Yes | Yes | Yes |  | 60 | 0 |
|  | 4 |  |  | Yes | Yes | Yes |  | Yes |  |  |
|  | 5 |  |  | Yes | Yes |  |  |  |  |  |
|  | 12 |  |  | Yes | Yes |  |  |  |  |  |

For the constituent 3 DL CA configurations:

1.	Band (2 + 4 + 5) is already specified with the same CC channel bandwidth in each CA band.

2.	Band (2 + 4 + 12) is already specified with the same CC channel bandwidth in each CA band.

3.	Band (2 + 5 + 12) is already specified with the same CC channel bandwidth in each CA band.

4.	Band (4 + 5 + 12) is already specified with the same CC channel bandwidth in each CA band.

Therefore, no additional BS study of harmonics and IMD products is required for this WI.

## 2.2	Band (2 + 2 + 5 + 12)

The CA scenario is provided in Table 2 below [2]:

Table 2 – Interband 4 Band CA Scenario (single uplink)

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | E-UTRA Band or E-UTRA CA configuration | 1.4 | 3 |  |  | 5 | 10 |  | 15 |  | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
|  |  | MHz | MHz |  |  | MHz | MHz |  | MHz |  | MHz | [MHz] |  |
| CA_2A-2A-5A-12A | CA_2A-2A | See 36.101 V12.7.0 table 5.6A.1-3 BW combination set 0 |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  | 5 |  |  |  | Yes |  |  | Yes |  |  |  |  |  |
|  | 12 |  |  |  | Yes |  |  | Yes |  |  |  |  |  |

For the constituent 3 DL CA configurations:

1.	Band (2 + 2 + 5) is already specified with the same CC channel bandwidth in each CA band.

2.	Band (2 + 2 + 12) is already specified with the same CC channel bandwidth in each CA band.

3.	Band (2 + 5 + 12) is already specified with the same CC channel bandwidth in each CA band.

Therefore, no additional BS study of harmonics and IMD products is required for this WI.

## 2.3	Band (2 + 5 + 12 + 12)

The CA scenario is provided in Table 3 below [3]:

Table 3 – Interband 4 Band CA Scenario (single uplink)

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | E-UTRA Band or E-UTRA CA configuration | 1.4 | 3 |  | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
|  |  | MHz | MHz |  | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_2A-5A-12B | 2 |  |  | Yes |  | Yes | Yes | Yes | 45 | 0 |
|  | 5 |  |  | Yes |  | Yes |  |  |  |  |
|  | 12B | See 36.101 V12.7.0 table 5.6A.1-1 BW combination set 0 |  |  |  |  |  |  |  |  |

For the constituent 3 DL CA configurations:

1.	Band (2 + 5 + 12) is already specified with the same CC channel bandwidth in each CA band.

2.	Band (2 + 12 + 12) for contiguous CC in Band 12 is already specified with the same CC channel bandwidth in each CA band.

3.	Band (5 + 12 + 12) for contiguous CC in Band 12 is not specified.

Therefore, the required BS studies of harmonics and IMD products for this WI are:

1.	Band (5 + 12 + 12) for contiguous CC in Band 12.

## 2.4	Band (2 + 2 + 4 + 5)

The CA scenario is provided in Table 4 below [4]:

Table 4 – Interband 4 Band CA Scenario (single uplink)

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | E-UTRA Band or E-UTRA CA configuration | 1.4 | 3 |  |  | 5 | 10 |  | 15 |  | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
|  |  | MHz | MHz |  |  | MHz | MHz |  | MHz |  | MHz | [MHz] |  |
| CA_2A-2A-4A-5A | CA_2A-2A | See 36.101 V12.7.0 table 5.6A.1-3 BW combination set 0 |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  | 4 |  |  |  | Yes |  |  | Yes |  | Yes | Yes |  |  |
|  | 5 |  |  |  | Yes |  |  | Yes |  |  |  |  |  |

For the constituent 3 DL CA configurations:

1.	Band (2 + 2 + 4) is already specified with the same CC channel bandwidth in each CA band.

2.	Band (2 + 2 + 5) is already specified with the same CC channel bandwidth in each CA band.

3.	Band (2 + 4 + 5) is already specified with the same CC channel bandwidth in each CA band.

Therefore, no additional BS study of harmonics and IMD products is required for this WI.

## 2.5	Band (2 + 2 + 12 + 12)

The CA scenario is provided in Table 5 below [5]:

Table 5 – Interband 4 Band CA Scenario (single uplink)

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | E-UTRA Band or E-UTRA CA configuration | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
|  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_2A-2A-12B | CA_2A-2A | See 36.101 V12.7.0 table 5.6A.1-3 BW combination set 0 |  |  |  |  |  | 55 | 0 |
|  | 12B | See 36.101 V12.7.0 table 5.6A.1-1 BW combination set 0 |  |  |  |  |  |  |  |

For the constituent 3 DL CA configurations:

1.	Band (2 + 2 + 12) is already specified with the same CC channel bandwidth in each CA band.

2.	Band (2 + 12 + 12) for contiguous CC in Band 12 is already specified with the same CC channel bandwidth in each CA band.

Therefore, no additional BS study of harmonics and IMD products is required for this WI.

# 3.	Conclusions

In this contribution, we have provided an overview on the required BS studies of harmonics and IMD products for the 5 WI proposals approved in RAN#68 on LTE-A 4DL/1UL CA with Bands 2, 4, 5 and 12, considering the BS studies that have been performed for the constituent 3 DL CA configurations.

As a summary, the following BS studies of harmonics and IMD products are required to complete the 3 WIs:

1.	Band (5 + 12 + 12) for contiguous CC in Band 12.

# References

[1]	RP-150728, “New WI proposal: LTE Advanced 4-Band Carrier Aggregation of Band 2, Band 4, Band 5 and Band 12”, U.S. Cellular.

[2]	RP-151018, “New WI proposal: LTE Advanced 4-Band Carrier Aggregation of Band 2, Band 2, Band 5 and Band 12”, U.S. Cellular.

[3]	RP-151019, “New WI proposal: LTE Advanced 4-Band Carrier Aggregation of Band 2, Band 5, Band 12 and Band 12”, U.S. Cellular.

[4]	RP-150731, “New WI proposal: LTE Advanced 4-Band Carrier Aggregation of Band 2, Band 2, Band 4 and Band 5”, U.S. Cellular.

[5]	RP-150732, “New WI proposal: LTE Advanced 4-Band Carrier Aggregation of Band 2, Band 2, Band 12 and Band 12”, U.S. Cellular.

[6]	R4-151293, “TP for TR 36.854-13: Required BS studies of Harmonics and Intermodulation Products caused by LTE Advanced 4DL/1UL Carrier Aggregation”, Alcatel-Lucent.