3GPP TSG-RAN WG4 #76	R4-154636

Beijing, China, 24 - 28 Aug 2015

Source: 	Ericsson

Title:	Preliminary link level simulation results for maximum sidelink process

Agenda Item:	6.2.2

Document for:	Discussion

# Introduction

In RAN4#75 meeting, the test for maximum sidelink process was extensively discussed. Parameters for the test were agreed in [1]. In the paper, we provide our preliminary link level simulation for the test and corresponding proposals for the test metric.

# Simulation assumption

Simulation parameters are shown in Table 1.

Table 1: Simulation parameters for maximum sidelink process test

| Test / Simulation Parameters |  | Test 1 (Discovery; 50 Sidelink Processes) | Test 2 (Discovery; 400 Sidelink Processes) | Test 3 (Communication) |
| --- | --- | --- | --- | --- |
| # E-UTRA cells |  | 1 (serving cell) |  |  |
| # D2D UEs |  | 50 | 400 | 16 |
| D2D UE | Transmission | Option 1: PSDCH (3 ReTx) | Option 1: PSDCH (3 ReTx) | PSCCH/PSSCH |
|  | PSSCH RMC | N/A | N/A | 5 MHz: 25 PRBs and 18336 TBS 10 MHz: 50 PRBs and 25456 TBS |
|  | Timing offset w.r.t. serving cell | 0 us | 0 us | 0 us |
|  | Freq offset w.r.t. serving cell | 0 Hz | 0 Hz | 0 Hz |
| Propagation Channel | Serving cell | Clean channel |  |  |
|  | D2D UE |  |  |  |
| Channel BWs * |  | 5/10/15/20 MHz | 10/15/20 MHz | 5 MHz and 10MHz |
| Rx UE assumptions (for simulation) | Timing reference | Serving cell |  |  |
|  | Frequency reference |  |  |  |
| Resource pool allocation |  | R4-153081(5.1.1., 5.1.2.) |  | R4-152781(2.4) |
| Simulation results needed |  | PSDCH BLER |  | PSSCH BLER |

# Link level simulation results

Link level simulation results for test 1 and 2 are shown in Figure 1. Link level simulation results for test 3 are shown in Figure 2. The required SNR at BLER 5% are shown in Table 2.

Figure 1: Simulation results for test 1 and test 2

Figure 2: Simulation results for Test 3

Table 2: Required SNR for each test case at 5% BLER

| Test 1 and Test 2 | 5MHz | -6.3 dB |
| --- | --- | --- |
|  | 10 MHz | -6.8 dB |
|  | 15MHz | -6.9 dB |
|  | 20 MHz | -6.9 dB |
| Test 3 | 5 MHz | 2.55 dB |
|  | 10 MHz | 0.15 dB |

## Remark on how to set up the test with the given SNR

In principle, the purpose of the maximum sidelink process test is to verify the UE capability to handle the maximum sidelink process. It is more like traditional sustained data rate (SDR) test. In traditional SDR test, noise free channel are setup and higher signal power are set. However, if the same noise free channel and high signal power are used in this test, it will lead to some UE can pass the test but cannot actually support so many HARQ processes in practical usage case.

For test 2 and test 3, the challenge case is UE needs to buffer all the receptions until the maximum retransmissions are finished and then release the computation and storage resource for each HARQ process. If noise free channel and very high power signal is setup, it is not necessary to buffer any retransmissions and can release the computation and storage resource in advance. For discovery, with this implementation, for a UE who only supports 50 HARQ processes, it can pass the test for 400 HARQ processes. For communication, with this implementation, for a UE who only supports 2 HARQ processes can pass the test designed for 16 HARQ processes. It is not desirable. In order to avoid this problem, we propose to add noise in the test and set the signal power according to the required SNR.

Proposal 1: Noise is added in the test and the input signal is set according to the required SNR

# Conclusion

In this contribution, we provide preliminary results for the test of maximum side link process. For the test setup, we have the following proposal:

Proposal 1: Noise is added in the test and the input signal is set according to the required SNR

# Reference

- R4-153676, WF on D2D Demodulation with Multiple sidelinks, LG Electronics, Huawei, HiSilicon, Qualcomm, Intel, Samsung, May 2014.