3GPP TSG RAN4 Meeting #77	R4-157909

Anaheim, USA, 16th – 20th Nov, 2015

Title: 	Analysis of the delta between average and min performance in UMTS BHH

Source: 	Vodafone

Agenda item:	X

Document for:	Approval

# 1	Introduction

This contribution is a resubmission of [1] and analyses the distribution of the difference between the average requirement and the minimum requirement.

# 2	Discussion

In order to understand the difference, a data set of about 120 UMTS BHH measurements has been collected. From this data set, the devices that do not pass the min requirement should be removed. Considering the devices in this data set passed the value that was identified in RAN4 AH in July meeting, then all are qualified to be part of the delta analysis. Hence, the average measurement across the 6 channels (Low,Mid,High for Left and Right measurements) is calculated. Then, the delta is defined as delta = (TRP/TRS_average_of_the_6) – (min_of_the_6)

The Delta for TRP is:

Delta=10log10Pleft-low/10+10Pleft-mid/10+10Pleft-high/10+10Pright-low/10+10Pright-mid/10+10Pright-high/10/6-min(Pleft-low,Pleft-mid,Pleft-high,Pright-low,Pright-mid,Pright-high)

Whereas for TRS is

Delta=-10log10-Pleft-low/10+10-Pleft-mid/10+10-Pleft-high/10+10-Pright-low/10+10-Pright-mid/10+10-Pright-high/10/6-max⁡(Pleft-low,Pleft-mid,Pleft-high,Pright-low,Pright-mid,Pright-high)

As a result, the following table captures the main statistics of the data set considering the computed delta as above:

|  | TRP |  | TRS |  |
| --- | --- | --- | --- | --- |
|  | 900 | 2100 | 900 | 2100 |
| min | 0,09 | 0,19 | 0,25 | 0,15 |
| max | 3,25 | 3,80 | 3,85 | 4,46 |
| avg | 0,84 | 1,19 | 1,50 | 1,28 |
| median | 0,71 | 0,97 | 1,42 | 1,20 |
| 0,9 | 1,51 | 2,18 | 2,39 | 2,20 |
| 0,8 | 1,16 | 1,76 | 1,93 | 1,77 |

In consequence we propose the following deltas based on BHH UMTS measurements:

|  | TRP | TRS |
| --- | --- | --- |
| 900 | 1.5dB | 2.5dB |
| 2100 | 2dB | 2dB |

# 3	Conclusion

This contribution has analysed the statistics of the delta between average measurement of the 6 channels against the minimum measurement across any of the 6 channels.

Out of the statistics, a proposal has been made based on the 90th percentile in order to remove outliers in the data sample.

The proposal is as follows:

|  | TRP | TRS |
| --- | --- | --- |
| 900 | 1.5dB | 2.5dB |
| 2100 | 2dB | 2dB |

# 6	References

[1] R4-155303	Analysis of the delta between average and min performance in UMTS BHH, Vodafone, RAN4#76