3GPP TSG-RAN WG4 #80bis	R4-167748

October 10th ‒ 14th, 2016

Ljubljana, SI

Agenda item:	X.Y.Z

Source: 	Qualcomm Incorporated

Title: 	Proposal to update eNB Total Power Dynamic Range requirement for LAA

Document for:	Approval

# Introduction

In this contribution we review the existing Total Power Dynamic Range (TPDR) and we propose an update of the requirement specific to LAA band, i.e. Band 46. Given the specific nature of the 5GHz unlicensed band, we propose to exclude Band 46 from the TPDR test.

# Discussion

The existing 3GPP Total Power Dynamic Range (TPDR) test defined in sec 6.3.2 of [1] aims at testing a very dynamic transmitter power behavior which is typical of a licensed band. In particular, it relies on test signals that cover the nominal bandwidth (E-TM 3.1) as well as the single RB (E-TM 2) [2], i.e. the lower limit of the dynamic range is the OFDM symbol power for a BS when one resource block is transmitted.

Base station behavior in Band 46, i.e. in 5GHz unlicensed spectrum, is fundamentally different compared to the operating conditions in licensed bands. First of all unlicensed regulation present in different regions applies. Considering the more stringent regulation, i.e. the ETSI harmonized standard for broadband access in 5GHz [3], a minimum allocation of 80% of the declared nominal channel bandwidth is required. Note that in order to fulfill this requirement LAA UE adopts a new waveform where the minimum allocation unit is called interlace and allows to meet the 80% minimum allocation requirement. In other regions, like US for instance, such requirement does not exist, however transmission of single RB is in practice precluded by the stringent Power Spectral Density (PSD) requirements. In other words, the amount of power back off needed to meet PSD requirements with single RB allocation would strongly degrade the link budget. Another consideration is related to the Listen Before Talk behavior. Having very small allocation will affect the overall system behavior by altering the channel access mechanism functionalities.

Based on the above observations, it is clear that the existing TPDR test does not present a meaningful test for unlicensed operation since it is naturally expected that the base stations use very large. As a consequence we propose to update the TPDR requirements by excluding band 46.

Proposal 1: B46 shall be exempt from the TPDR requirement.

A possible modification of TS 36.104 is provided below:

# <Start of Changes>

### 6.3.2	Total power dynamic range

The total power dynamic range is the difference between the maximum and the minimum transmit power of an OFDM symbol for a specified reference condition.

NOTE 1:	The upper limit of the dynamic range is the OFDM symbol power for a BS at maximum output power. The lower limit of the dynamic range is the OFDM symbol power for a BS when one resource block is transmitted. The OFDM symbol shall carry PDSCH and not contain RS, PBCH or synchronisation signals.

NOTE 2:	The requirement does not apply to Band 46.

# <End of Changes>

A CR implementing the proposed changes to TS 36.104 is provided in [4].

# Conclusions

In this contribution we discussed the applicability of Total Power Dynamic Range to LAA band, i.e. Band 46. Based on the specific nature of operation in 5GHz unlicensed spectrum and taking into account regulatory considerations, we proposed to exclude B46 from TPDR requirement and test.

Proposal 1: B46 shall be exempt from the TPDR requirement.

# Reference

- 3GPP TS 36.104 v13.5.0 “Technical specification - Evolved Universal Terrestrial Radio Access – Base Station (BS) radio transmission and release”.

- 3GPP TS 36.141 v13.5.0 “Technical specification - Evolved Universal Terrestrial Radio Access – Base Station (BS) conformance testing”.

- ETSI EN 301 893 V1.8.1 (2015-03).

- R4-167559, “Update of the Total Power Dynamic Range requirement for Band 46”, Qualcomm Incorporated.