3GPP TSG-RAN WG4 Meeting #78BIS	R4-161633

St. San Jose del Cabo, Mexico, 11th – 15th April, 2016

Agenda Item:	7.12.3

Source: 	Intel Corporation

Title: 	Discussion on the baseline of receivers in high speed scenarios

Document for:	Discussion

- Introduction

The work item (WI) “Performance enhancements for high speed scenario” was approved at RAN plenary meeting #70 [1]. One objective of this WI is to identify downlink demodulation requirements under the identified high speed scenarios. In this contribution, we discuss the baseline behaviour for UE in high speed scenarios.

- Three types of HST SFN channel

So far, there have been three channel models in discussion for the bidirectional HST SFN with RRH deployment, namely, 2-path, 3-path and reflection channel models.

- 2-path model

In high speed scenario’s study item (SI) phase, a 2-path HST SFN channel model was provided [2], as shown in Fig.1.

Fig. 1 “2-path” channel model of HST in bidirectional RRH SFN

In 2-path channel model, it is assumed that:

- Transmission link from a RRH to a train is a single path, non-fading, line-of-sight propagation channel;

- A train receives signals from the two nearest RRHs only.

Moreover, a “wrap around” method is used to facilitate the implementation of the 2-path HST channel model. Based on this model, the UE always sees two opposite Doppler shifts from two nearest RRHs in front of and behind it. The Doppler shifts, together with the tap power and delay, would change dynamically as a train is running at a high speed, up to 350km/h, resulting in a Doppler shift’s range from -875Hz to +875Hz.

- 3-path model

Recently, in RAN4 #78 Meeting, a “3-path” HST channel model was proposed in [3], as shown in Fig.2. The justification for the “3-path” HST channel model is that, when a set of three RRHs shares the same cell ID and the distance from the RRH to the railway, “Dmin”, is large, such as a few hundreds of meters, there would be more than 2 paths of reasonable strength received by HST UE. It is important because multiple paths would bring detrimental impact on demodulation performance of UE, if the receiver is only robust to the “2-path” channel model.

Fig. 2 “3-path” channel model of HST in bidirectional RRH SFN

- Reflection model

In [4], a HST SFN channel model is proposed, where the presence of reflection surface in the environment surrounding, such as trees and walls aligned with the railway, is considered. It suggests that:

- HST channel model should consider multiple paths due to reflection;

- The fading effect in HST channels may be considered when we investigate the receiver’s robustness in high speed scenarios.

Fig. 3 Reflection channel model of HST in bidirectional RRH SFN

Based on the above observations, we have:

- Proposal 1: Baseline receiver should be able to provide robust demodulation performance in all three channel models, and other scenarios/channels may also be considered;

- Proposal 2: Limited number of input parameters are required by HST UE for channel estimation (CE), for example, HST UE only requires the path’s time delay and maximum Doppler shift for CE, in case that there are multiple (more than 2) paths.

- Conclusion

In this contribution, we discuss the baseline behaviour for UE in high speed scenarios:

- Proposal 1: Baseline receiver should be able to provide robust demodulation performance in all three channel models, and other scenarios/channels may also be considered;

- Proposal 2: Limited number of input parameters are required by HST UE for channel estimation (CE), for example, HST UE only requires the path’s time delay and maximum Doppler shift for CE, in case that there are multiple (more than 2) paths.

- Reference

[1] RP-152263, “New WI proposal: Performance enhancements for high speed scenario”, NTT DoCoMo Inc., Huawei, HiSilicon, Dec. 2015.

[2] R4-151587, “Channel model for SFN deployment with RRH for HST scenario”, Qualcomm, April 2015.

[3] R4-160992, “UE demodulation performance in high speed scenarios in SFN deployment”, Qualcomm, February 2016.

[4] R4-161632, “SFN channel model in high speed scenarios”, Intel Corporation, April 2016.