3GPP TSG-RAN WG4 Meeting #79	R4-163183

Nanjing, China, 23 - 27 May 2016

Agenda item:			6.5.4

Source:	Nokia

Title:	Cat-M1 PRACH Practical Simulation Results

Document for:		Discussion

# Introduction

PRACH simulation assumptions for supporting Cat-M1 UEs were agreed in RAN4#78 [1]. In RAN4#78bis, we provided the initial Cat-M1 PRACH link level simulation results based on the agreed simulation assumptions [2]. In this paper, we present the updated ideal Cat-M1 PRACH link level simulation results and practical Cat-M1 PRACH link level simulation results with implementation margin.

# Simulation Assumptions

Tables 1 and 2 provide the simulation assumptions for Cat-M PRACH detection performance based on the PRACH simulation assumptions agreed in [1]. Additionally, prach-ConfigurationIndex of 3 and PRACH Frequency Hopping Offsets of 24 PRBs and 40 PRBs were assumed in the PRACH simulations.

| Parameters | Values | Comments |
| --- | --- | --- |
| PRACH format | Format 0, Format 1, Format 2, Format 3 | PRACH Configuration indexes (3, 19, 35, 51) for {Format 0, Format 1, Format 2, Format 3} |
| Propagation conditions | AWGN, EPA1 | Due to the low mobility of Cat-M UEs, using EPA1 as in RLM requirements |
| Number of TX antennas | 1 |  |
| Number of RX antennas | 2 |  |
| Antenna correlation | Low |  |
| Noise model | AWGN |  |
| Timing offset | Randomly in the range of [0~1]us. | The timing offset base value is set to 50% of Ncs (see Section 8.4.1.4.2 in TS 36.141) |
| Repetition level | Basline: Format 0 and Format 1: {8,32} Format 2 and Format 3: {4,16} Other options are not precluded | May not need to simulate all levels of {1,2,4, 8,16,32,64,128} |
| Number of signatures available for Cat-M UEs | 64 | The size of preamble pool has impact on detection performance |
| Frequency offset | 0Hz for AWGN, 270 Hz for EPA1 | More discussion may be needed for frequency offset |
| Frequency hopping | ON/OFF |  |
| Prach Configuration Index | (3, 19, 35, 51) for Format 0, Format 1, Format 2, and Format 3) respectively | There could be significant difference when different configuration indexes are used under fading channels, especially with high PRACH repetitions |

Table 2. Test preambles for Normal Mode (From Table A.6-1 in TS 36.104 [2])

| Burst format | Ncs | Logical sequence index | v |
| --- | --- | --- | --- |
| 0 | 13 | 22 | 32 |
| 1 | 167 | 22 | 2 |
| 2 | 167 | 22 | 0 |
| 3 | 0 | 22 | 0 |

# Simulation Results

Tables 3 and 4 provide the ideal simulation results for Cat-M PRACH detection.

Table 3 PRACH missed detection performance for Cat-M1 Mode (Frequency Hopping OFF)

| Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Frequency offset | Number of Repetitions | SNR [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 | Burst format 1 | Burst format 2 | Burst format 3 |
| 2 | AWGN | 0 | 4 |  |  | -23.38 | -23.08 |
|  |  |  | 8 | -23.76 | -23.35 |  |  |
|  |  |  | 16 |  |  | -27.26 | -27.01 |
|  |  |  | 32 | -27.74 | -27.38 |  |  |
|  | EPA1 Low | 270 Hz | 4 |  |  | -14.34 | -14.08 |
|  |  |  | 8 | -15.23 | -14.83 |  |  |
|  |  |  | 16 |  |  | -19.26 | -18.94 |
|  |  |  | 32 | -21.23 | -20.83 |  |  |

Table 4 PRACH missed detection performance for Cat-M1 Mode (Frequency Hopping ON)

| Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Frequency offset | Number of Repetitions | SNR [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 | Burst format 1 | Burst format 2 | Burst format 3 |
| 2 | AWGN | 0 | 4 |  |  | -23.38 | -23.08 |
|  |  |  | 8 | -23.76 | -23.35 |  |  |
|  |  |  | 16 |  |  | -27.26 | -27.01 |
|  |  |  | 32 | -27.74 | -27.38 |  |  |
|  | EPA1 Low | 270 Hz | 4 |  |  | -18.07 | -18.07 |
|  |  |  | 8 | -19.08 | -19.08 |  |  |
|  |  |  | 16 |  |  | -23.12 | -23.12 |
|  |  |  | 32 | -24.63 | -24.63 |  |  |

Tables 5 and 6 provide the practical simulation results for Cat-M PRACH detection with implementation margin.

Table 3 PRACH missed detection performance for Cat-M1 Mode (Frequency Hopping OFF)

| Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Frequency offset | Number of Repetitions | SNR [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 | Burst format 1 | Burst format 2 | Burst format 3 |
| 2 | AWGN | 0 | 4 |  |  | -21.38 | -21.08 |
|  |  |  | 8 | -21.76 | -21.35 |  |  |
|  |  |  | 16 |  |  | -25.26 | -25.01 |
|  |  |  | 32 | -25.74 | -25.38 |  |  |
|  | EPA1 Low | 270 Hz | 4 |  |  | -12.34 | -12.08 |
|  |  |  | 8 | -13.23 | -12.83 |  |  |
|  |  |  | 16 |  |  | -17.26 | -16.94 |
|  |  |  | 32 | -19.23 | -18.83 |  |  |

Table 4 PRACH missed detection performance for Cat-M1 Mode (Frequency Hopping ON)

| Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Frequency offset | Number of Repetitions | SNR [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 | Burst format 1 | Burst format 2 | Burst format 3 |
| 2 | AWGN | 0 | 4 |  |  | -21.38 | -21.08 |
|  |  |  | 8 | -21.76 | -21.35 |  |  |
|  |  |  | 16 |  |  | -25.26 | -25.01 |
|  |  |  | 32 | -25.74 | -25.38 |  |  |
|  | EPA1 Low | 270 Hz | 4 |  |  | -16.07 | -16.07 |
|  |  |  | 8 | -17.08 | -17.08 |  |  |
|  |  |  | 16 |  |  | -21.12 | -21.12 |
|  |  |  | 32 | -22.63 | -22.63 |  |  |

# Summary

In this contribution, we provided the updated ideal and practical simulation results for Cat-M1 PRACH detection performance. We suggest these results be considered in defining the Cat-M1 PRACH detection performance requirements.

# References

- R4-161227, Cat-M PRACH Simulation Assumptions, Nokia Networks, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, Ericsson, Huawei, Hisilicon

- R4-162704, Cat-M1 PRACH Initial Simulation Results, Nokia

- R4-160350, Work plan for WI on Rel-13 eMTC BS performance requirements Ericsson

- R4-161215, Way forward on eMTC BS demodulation performance requirements, Huawei, HiSilicon, Ericsson, Nokia Networks, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell

- 3GPP TS 36.104, E-UTRA: Base Station (BS) radio transmission and reception.