3GPP TSG-RAN WG4 Meeting #79	                                         R4-163622

Nanjing, China, 23 - 27 May, 2016

Source: 	Huawei, Hisilicon

Title: 	eLAA UE RF TX requirements

Agenda Item:	7.20.2

Document for:	Discussion

# 1. Introduction

This contribution provides our initial views on eLAA [1] UE RF TX requirements. This is mainly a resubmission of [2] but includes also some updates.

# 2. Discussion

LAA operates at significantly higher frequencies (5150-5925MHz) than any LTE band. Due to this high frequency some requirements need special attention, and LTE requirements cannot necessarily be directly reused.

This high frequencies pose significant challenges on TX side. In general, the front-end losses tend to be higher albeit the LAA filter itself is a relatively low loss component. Even with the latest state of the art PA technologies the PA output power, gain, linearity and efficiency are all limited within certain boundaries and with certain inter-dependencies.

This paper is organized into sections according to Chapter 6 in 36.101. As a baseline, we intend to use generic LTE requirements when possible, with some exceptions explained below. Furthermore we recognize that some upcoming RAN1 decisions may have impacts to the observations made in this contribution.

6.2 Transmit power

We conducted a brief survey about PA vendors plans on LAA capable PA, and the result was that they are working on it and it is too premature to give any promises on the achievable performance. The approach of defining the LAA maximum power should be based on the estimates from PA vendors. Based on initial information we find it too early to decide anything on LAA maximum output power. Early indications show that the LAA PA’s could achieve high enough output power to meet 23dBm UE max power with reasonable linearity, but the efficiency is yet to be further studied. Thus one of the important factors impacting LAA maximum power is the PA output power vs. PA efficiency. Similarly, MPR requires further study because it depends on the ACLR, SEM and so on.

The PCMAX tolerances defined in 6.2.5 may need to be revised if the maximum output power is less than 23dBm.

Observation1: More work is needed before the LAA maximum power and MPR can be decided

Observation2: The PCMAX tolerances defined in 6.2.5 may need to be revised if the maximum output power is less than 23dBm

6.3 Output power dynamics

This chapter is quite generic and can be tentatively applied into eLAA.

Observation3: Output power dynamics requirements can be tentatively reused for eLAA

6.5 Transmit signal quality

In [2] is was assume that only one user is using each channel at a time, but now the consensus is that there can be multiple concurrent users. Thus in-band emission requirements are needed. As described in [3] and [4], due to interlaced waveform the definition of in-band emissions is rather challenging.

Observation4: In-band emissions in 6.5.2.2 and 6.5.2.3 (General, IQ image, Carrier leakage) are needed for eLAA. Definition due to interlaced waveform is challenging.

6.6 Output RF spectrum emissions

Our initial view is that spectrum emission mask can be reused. Due to high frequency, intermodulations are likely a bit higher level than in case of the other E-UTRA bands but still initially we believe current SEM is pretty ok for eLAA as well. The only exception of not reusing LTE SEM would relate to regulatory emission requirements defined for these frequencies.

In ACLR, only the E-UTRA ACLR is needed because there is no UTRA operating at these frequencies. It is yet to be seen what kind of ACLR performance the LAA capable PA’s have, so the ACLR power needs further study.

Spurious emission limits shall apply because generic spurious requirements are regulatory and eLAA has to protect other bands in similar fashion than any other E-UTRA bands.

Observation5: SEM can be tentatively reused for eLAA, when allowed by regulatory emission limits

Osbervation6: Only the E-UTRA ACLR requirement shall apply for eLAA. E-UTRA ACLR power needs further study

Observation7: Spurious emission limits shall apply for eLAA. eLAA has to protect other LTE bands in spurious emissions band UE co-existence

6.7 Transmit intermodulations

Our initial view is that transmit intermodulation requirements can be used for eLAA.

Observation8: Current transmit intermodulation requirements can tentatively apply for eLAA

# 3. Conclusions

A brief analysis on the eLAA UE TX requirements is provided. The following observations are made.

Observation1: More work is needed before the LAA maximum power and MPR can be decided

Observation2: The PCMAX tolerances defined in 6.2.5 may need to be revised if the maximum output power is less than 23dBm

Observation3: Output power dynamics requirements can be tentatively reused for eLAA

Observation4: In-band emissions in 6.5.2.2 and 6.5.2.3 (General, IQ image, Carrier leakage) are needed for eLAA. Definition due to interlaced waveform is challenging.

Observation5: SEM can be tentatively reused, when allowed by regulatory emission limits

Osbervation6: Only the E-UTRA ACLR requirement shall apply for eLAA. E-UTRA ACLR power needs further study

Observation7: Spurious emission limits shall apply for eLAA. eLAA has to protect other LTE bands in spurious emissions band UE co-existence

Observation8: Current transmit intermodulation requirements can tentatively apply for eLAA

# 4. References

[1] RP-152272, “New Work Item on enhanced LAA for LTE”, RAN #70, Ericsson, Huawei

[2] R4-162208, “eLAA UE RF TX requirements”, Huawei, Hisilicon

[3] R4-162474, “On UE uplink requirements for eLAA: single-CC in Band 46”, Ericsson

[4] R4-162652, “eLAA specification changes for UL operation”, Qualcomm Inc.