3GPP TSG-RAN WG4 meeting #79	R4-163726

Nanjing, China, 23 – 27 May, 2016

Source:	Ericsson

Title:	TP to 36.714-05-01 on co-existence analysis for 2A-46A-46D

Agenda item:				7.5.1

Document for:	Approval

# 1. Introduction

This is a text proposal for 36.714-05-01 for co-existence analysis for 5DL/1UL CA combination 2A-46A-46D.

# 2. Text Proposal for TR 36.714-05-01

##### ---Start of changes---

### 6.x		CA_4DL_2A-46A-46D_1UL_BCS0

##### ---Text omitted---

##### 6.x.1.2	Co-existence studies for CA_2A-46A-46D (5DL/1UL)

Table 6.x.1.2-1 gives the UL to DL harmonic products for CA band 2 + band 46 with 1UL. It can be seen that 2nd harmonics fall into band 43 and that 3rd harmonics falls into the band 46.

Table 6.x.1.2-1: Band 2 and Band 46 1UL harmonic products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 5150 | 5925 |
| 2nd order harmonics frequency range (MHz) | 3700 to 3820 |  | 10300 to 11850 |  |
| 3rd order harmonics frequency range (MHz) | 5550 to 5730 |  | 15450 to 17775 |  |

The 2nd and 3rd order DL to UL harmonics and IMD products caused in the BS by transmitting of band 2 and band 46 DL carriers is calculated and shown in table 6.x.1.2-2 below:

Table 6.x.1.2-2: Band 2 and Band 46 DL harmonics and IMD products

| BS DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 1930 | 1990 | 5150 | 5925 |
| 2nd order harmonics frequency range (MHz) | 3860 to 3980 |  | 10300 to 11850 |  |
| 3rd order harmonics frequency range (MHz) | 5790 to 5970 |  | 15450 to 17775 |  |
| Two-tone 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency range (MHz) | 3160 to 3995 |  | 7080 to 7915 |  |
| Two-tone 3rd order IMD products | 2*f1_low – f2_high & (2*f1_low + f2_low) | 2*f1_high – f2_low & (2*f1_high + f2_high) | 2*f2_low – f1_high & (2*f2_low + f1_low) | 2* f2_high – f1_low & (2*f2_high + f1_high) |
| IMD frequency range (MHz) | 1170 to 2065 9010 to 9905 |  | 8310 to 9920 12230 to 13840 |  |
| Three-tone 3rd order IMD products | (f1_low – max BW f2) | (f1_high + max BW f2) | (f2_low – max BW f1) | (f2_high + max BW f1) |
| IMD frequency range (MHz) | 1910 to 2010 |  | 5130 to 5945 |  |

It can be seen from Table 6.x.1.2-2 that none of 2rd harmonics fall into the any E-UTRA operating BS receive band, and it can be seen that 3rd harmonics fall into the E-UTRA operating BS receive band 46.

It can also be seen that two-tone 3rd order IMD products may fall into Band 22, 42 and 43,  and that the three-tone 3rd order IMD products may fall into Band 1, 2, 3, 4, 9, 10, 11, 21, 23, 24, 25, 33, 34, 35, 36, 37, 39, 65 and 66. With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the harmonics and IMD interference generated within the Band 1, 2, 3, 4, 9, 10, 11, 21, 23, 24, 25, 33, 34, 35, 36, 37, 39, 65 and 66 receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Band 2 and Band 46 BS transmitters do not share the same antenna with any of these receivers.

##### ---End of changes---