3GPP TSG-RAN WG4 #79	R4-163928

Nanjing, China, May.23~May.27, 2016

Source: 	Ericsson

Title:	Simulation assumption for LAA demodulation

Agenda Item:	6.10.4

Document for:	Approval

# Introduction

In this paper, simulation assumption for LAA demodulation is provided.

# Common parameters for Scell

Table 1 provides all common parameters for Scell. Without special mention, the parameters in Table 1 will be applied for all the following sections.

Table 1: Common parameters for Scell

| Parameters | unit | Value |
| --- | --- | --- |
| MBSFN configuration |  | No MBSFN subframe is configured |
| DRS periodicity | ms | 80 |
| Frequency offset related to primary carrier | Hz | 750 |
| Time offset related to primary carrier | us | 30.26 |
| Bandwidth | MHz | 20 |
| Number of allocated resource blocks (Note 2) | PRB | 100 |
| Number of control OFDM symbols |  | 2 |
| Signal transmission model |  | Randomness model is used as baseline, predefined pattern model can be used if any problem is identified for randomness model |

# Simulation assumption for DMRS-based transmission scheme

For Pcell, the parameters defined in Table 8.2.1.3.1-4 and Table 8.2.1.3.1-3 of 36.101 are reused and recapped in Table 2. For Scell, the parameters are shown in Table 3.

Table 2: Parameters for PCell

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (NOTE 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| bandwidth |  | MHz | 20 |
| Propagation condition |  |  | EVA70 |
| Correlation matrix |  |  | 2x2 Low |
| Reference channel |  |  | R.30 |
| PDSCH transmission mode |  |  | 3 |
| NOTE 1: . NOTE 2: PUCCH format 1b with channel selection is used to feedback ACK/NACK for Tests in Table 8.2.1.3.1-4, PUCCH format 3 is used to feedback ACK/NACK for Tests in Table 8.2.1.3.1-6. NOTE 3: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 3: Parameters for Scell

| parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | -3 |
| Cell-specific reference signals |  |  | Antenna ports 0 and 1 |
| CSI reference signals |  |  | Antenna ports 15,16 |
| DMRS ports (dual layer transmission) |  |  | port 7 and port 8 |
| Beamforming model |  |  | Annex B.4.2 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 2 |
| CSI reference signal configuration |  |  | 8 |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | 3 / 0010000000000000 |
| at antenna port |  | dBm/15kHz | -98 |
| Correlation matrix |  |  | 2x2 Low |
| Propagation condition |  |  | EVA70 |
| TBS for each TTI |  |  | A desire coding rate R is given and TBS for each TTI is derived according to A.2.1.2 of 36.101. |
| MCS |  |  | Option 1: 16QAM 1/2 Option 2: 64QAM 1/2 |
| PDSCH transmission mode |  |  | 9 |
| Note 1: Note 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. |  |  |  |

# Simulation assumption for CRS-based transmission scheme

## Open-loop spatial multiplexing performance

For Pcell, the parameters defined in Table 8.2.1.3.1-4 and Table 8.2.1.3.1-3 of 36.101 are reused and recapped in Table 2. For Scells, the parameters are given in Table 4.

Table 4: Parameters for Scell when TM3 is configured

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (NOTE 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| bandwidth |  | MHz | 20 |
| Propagation condition |  |  | EVA70 |
| Correlation matrix |  |  | 2x2 Low |
| MCS |  |  | Option 1: 16QAM 1/2 Option 2: 64QAM 1/2 |
| PDSCH transmission mode |  |  | 3 |
| NOTE 1: . NOTE 2: PUCCH format 1b with channel selection is used to feedback ACK/NACK for Tests in Table 8.2.1.3.1-4, PUCCH format 3 is used to feedback ACK/NACK for Tests in Table 8.2.1.3.1-6. NOTE 3: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

## Closed-loop spatial multiplexing performance

The parameters for Pcell are the same as Table 8.2.1.4.3-3 and recapped in Table 5, and the parameters for Scell are provided in Table 6.

Table 5: Test Parameters for Multi-Layer Spatial Multiplexing (FRC) for CA (Table 8.2.1.4.3-3)

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | 3 |
| at antenna port |  | dBm/15kHz | -98 |
| Precoding granularity |  | PRB | 4 for 3MHz and 5MHz CCs, 6 for 10MHz CCs, 8 for 15MHz and 20MHz CCs |
| PMI delay (Note 2) |  | ms | 8 |
| Reporting interval |  | ms | 1 |
| Reporting mode |  |  | PUSCH 1-2 |
| CodeBookSubsetRestriction bitmap |  |  | 0000000000000000000000000000000011111111111111110000000000000000 |
| CSI request field (Note 3) |  |  | ‘10’ |
| PDSCH transmission mode |  |  | 4 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: Multiple CC-s under test are configured as the 1st set of serving cells by higher layers. Note 4: ACK/NACK bits are transmitted using PUSCH with PUCCH format 1b with channel selection configured for Tests in Table 8.2.1.4.3-4, and with PUCCH format 3 for Tests in Table 8.2.1.4.3-6. Note 5: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 6: Test Parameters for Multi-Layer Spatial Multiplexing (FRC) for Scell

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | 3 |
| at antenna port |  | dBm/15kHz | -98 |
| Precoding granularity |  | PRB | 8 |
| PMI delay (Note 2) |  | ms | 8 |
| Reporting interval |  | ms | 1 |
| Reporting mode |  |  | PUSCH 1-2 |
| CodeBookSubsetRestriction bitmap |  |  | 0000000000000000000000000000000011111111111111110000000000000000 |
| CSI request field (Note 3) |  |  | ‘10’ |
| TBS for each TTI |  |  | A desire coding rate R is given and TBS for each TTI is derived according to A.2.1.2 of 36.101. |
| MCS |  |  | Refer to R.14-3 FDD |
| Propagation condition |  |  | EVA5 |
| Correlation matrix |  |  | 4x2 Low |
| PDSCH transmission mode |  |  | 4 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: Multiple CC-s under test are configured as the 1st set of serving cells by higher layers. Note 4: ACK/NACK bits are transmitted using PUSCH with PUCCH format 1b with channel selection configured for Tests in Table 8.2.1.4.3-4, and with PUCCH format 3 for Tests in Table 8.2.1.4.3-6. Note 5: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

# Simulation assumption for (e)PDCCH

## PDCCH

The parameters for Scell in PDCCH test is given in Table 7.

Table 7: The parameters for PDCCH (Scells)

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (NOTE 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| Propagation condition |  |  | EVA70 |
| Correlation matrix |  |  | 2x2 Low |
| Number of CCE |  |  | 4 |
| Format 2A |  | bits | 48 (not included CRC) |
| PDSCH transmission mode |  |  | 3 |
| NOTE 1: . NOTE 2: PUCCH format 1b with channel selection is used to feedback ACK/NACK for Tests in Table 8.2.1.3.1-4, PUCCH format 3 is used to feedback ACK/NACK for Tests in Table 8.2.1.3.1-6. NOTE 3: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

## ePDCCH

The parameters for ePDCCH are reused from Table 8.8.2.1-1 with minor modification for Scell.

Table 8: Test Parameters for Localized EPDCCH with TM9 (recap from Table 8.8.2.1-1 with minor

Modification)

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Number of PDCCH symbols |  | symbols | 1 (Note 1) |
| EPDCCH starting symbol |  | symbols | 2 (Note 1) |
| PHICH duration |  |  | Normal |
| Unused RE-s and PRB-s |  |  | OCNG |
| Cell ID |  |  | 0 |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 |
|  |  | dB | -3 |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| Cyclic prefix |  |  | Normal |
| Subframe Configuration |  |  | Non-MBSFN |
| Precoder Update Granularity |  | PRB | 1 |
|  |  | ms | 1 |
| Beamforming Pre-Coder |  |  | Annex B.4.5 |
| Cell Specific Reference Signal |  |  | Port 0 and 1 |
| CSI-RS Reference Signal |  |  | Port 15 and 16 |
| CSI-RS reference signal resource configuration |  |  | 0 |
| CSI reference signal subframe configuration ICSI-RS |  |  | 2 |
| ZP-CSI-RS configuration bitmap |  |  | 0000010000000000 |
| ZP-CSI-RS subframe configuration IZP-CSI-RS |  |  | 2 |
| Number of EPDCCH Sets |  |  | 2 (Note 2) |
| EPDCCH Subframe Monitoring pattern subframePatternConfig-r11 |  |  | 1111111110 1111111101 1111111011 1111110111 (Note 3) |
| Number of CCE |  | CCE | 2 |
| Propagation Condition |  |  | EVA5 |
| Antenna configuration and correlation Matrix |  |  | 2 x 2 Low |
| DCI Format |  |  | DCI Format 2C |
| Number of DCI bits without CRC |  |  | 50 |
| PDSCH TM |  |  | TM9 |
| Note 1: The starting symbol for EPDCCH is signalled with epdcch-StartSymbol-r11. However, CFI is set to 1. Note 2: The first set is distributed transmission with PRB = {0, 49} and the second set is localized transmission with PRB = {0, 7, 14, 21, 28, 35, 42, 49}. ePDCCH is scheduled in the second set for all tests. Note 3: EPDCCH is scheduled in every SF. UE is required to monitor ePDCCH for UE-specific search space only in SFs configured by subframePatternConfig-r11. Legacy PDCCH is not scheduled. |  |  |  |

# Conclusion

In this paper, simulation parameters are provided. We hope the group approves the above parameters for simulation results alignment.