3GPP TSG-RAN WG4 Meeting #80	R4-165036

Gothenburg, Sweden, 22 - 26 Aug, 2016

Agenda Item:	8.10.3.4

Source: 	Intel Corporation

Title: 	Analysis on CQI reporting in high speed scenarios

Document for:	Discussion

- Introduction

The work item (WI) “Performance enhancements for high speed scenario” was approved at RAN plenary meeting #70 [1]. According to the high speed Work Item Description (WID) [1], the target high speed train (HST) moving speed is at least 350km/h, and two objectives in the high speed WID are:

- Specify new CSI requirements considered for the final solutions to enhance the downlink demodulation performance, if needed;

- Receiver robustness in other scenarios/channels should be considered (besides SFN channels).

In RAN4#78bis Meeting, the CSI reporting issue was first discussed in [2]. It was suggested that the CSI test should be precluded in the high speed WI due to the nature of the high speed channel model. In this contribution, we further discuss the CSI reporting, in particular, two CQI reporting issues:

- Coherent time in fading channel conditions;

- CQI feedback reporting in bidirectional HST SFN with the normalized received power assumption.

- Issues on CQI reporting in high speed scenarios

  - Coherent time in conventional fading channels

As specified in the high speed WID [1], besides the SFN channels which have been the main focus and extensively discussed in the high speed SI and WI, receiver robustness in other scenarios/channels should also be considered. This means that a high speed train with speed at least 350km/h can also run under the conventional fading channel conditions, such as urban areas.

If a train is running in the urban area under fading channel conditions, conventionally, the Doppler spectrum for the received signal at the UE side would be of the U-shaped spectrum for isotropic scattering, as shown in Fig.1.

Fig.1 U-shaped Doppler spectrum

Under conventional fading channel conditions, the coherent time Tc can be theoretically given by [3]

Tc=916π1fd≈12·fd

Assume the carrier frequency fc = 2.7GHz, and train speed is v = 350km/h, the maximum Doppler shift fd is given by (c - speed of light)

fd=fc×vc=875Hz.

Therefore, the coherent time for this high speed scenario would be

Tc=916π1fd≈0.5ms

With this observation, if the CQI feedback reporting delay time is configured as 8ms or longer like TS 36.101 Section 9.3 tests [4], channel becomes uncorrelated within CQI reporting delay. It implies that CQI reporting can be outdated within CQI reporting delay, especially when a few taps are overlapped in the middle of RRH. At the moment, channel quality as well as channel correlation changes in wideband.

In order to enhance the fluctuated CQI report, there has been discussions in previous WIs regarding CQI averaging. It can be considered as one of an UE implementation solution. Extending to non-normalized SNR case, it is obvious that the UE will measure more fluctuated CQI under a dynamic SNR profile. In spite of these channel properties, an HST UE behaviour will not be changed due to the HST channel, and we don’t find a special HST UE behaviour comparing to the legacy UE.

Observation 1: We don’t find a special HST UE CSI measurement behaviour comparing to a legacy UE. The UE is allowed to handle fluctuated CQIs by an UE implementation solution.

  - CQI feedback reporting in bidirectional HST SFN with the normalized received power assumption

To demonstrate the impact on the CQI feedback of the bidirectional HST SFN, we consider a RRH deployment scenario as summarised in Table 1.

| Parameters | Value |
| --- | --- |
| Ds | 500m |
| Dmin | 5m |
| v | 350km/h |
| fd | 875Hz |

Table 1. Parameters for HST SFN Scenario

Fig. 2 Bidirectional HST SFN

Fig. 3 CQI feedback index vs. train position

Fig. 2 shows a train running in the bidirectional HST SFN from RRH2 (blue) to RRH3 (black). Fig.3 shows one realization of the CQI feedback index, tracking continuously as the train running from 710m to 790m w.r.t. to the original position 0m. The CQI feedback simulation results are based on the bidirectional HST SFN channel model assumptions:

- There is only single-path line of sight transmission from each RRH to the train;

- Non-fading channel, only path loss is considered for transmission from each RRH to the train;

- Received power is normalized at the UE side for signals received from multiple RRHs.

Observation 2: The CQI feedback index fluctuates significantly when the train is around the very middle of the RRHs (i.e. 750m) due to the constructive/destructive impact of the overlap of multiple signals from different RRHs; however, when the train is away from the middle position, relatively stable CQI feedback index can be achieved.

Observation 3: The area with considerably fluctuated CQI feedback index is very short (less than 10 meters).

Proposal 1: We propose not to introduce a CSI test for a HST UE.

- Conclusion

In this contribution, we further discuss the CSI reporting.

Observation 1 : We don’t find a special HST UE CSI measurement behaviour comparing to a legacy UE. A UE is allowed to handle fluctuated CQIs by an UE implementation solution.

Observation 2: The CQI feedback index fluctuates significantly when the train is around the very middle of the RRHs (i.e. 750m) due to the constructive/destructive impact of the overlap of multiple signals from different RRHs; however, when the train is away from the middle position, relatively stable CQI feedback index can be achieved.

Observation 3: The area with considerably fluctuated CQI feedback index is very short (less than 10 meters).

Proposal 1: We propose not to introduce a CSI test for a HST UE.

- Reference

[1] RP-152263, “New WI proposal: Performance enhancements for high speed scenario”, NTT DoCoMo Inc., Huawei, HiSilicon, Dec. 2015.

[2] R4-161634, “Discussion on CSI reporting in high speed scenarios”, Intel, April 2016.

[3] T.S. Rappaport, Wireless Communications: Principles and practice, Second Edition, Prentice Hall, 2002.

[4] TS 36.101, “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception”, 3GPP LTE, Rel. 12, May, 2015.