3GPP TSG-RAN WG4 Meeting #80 	R4-165556

# Gothenburg, Sweden, 22 – 26 Aug, 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 3718 | rev | - | Current version: | 13.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduce aperiodic CSI test for LAA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_LAA-Perf |  |  |  |  |  |  | Date: |  | 2016-06-21 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | RAN4 agreed to have aperiodic CSI test for LAA, but there is no specfication for it |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce aperiodic CSI test for LAA |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is no aperiodic CSI test in LAA |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | Y |  | Test specifications |  |  | TS36.521 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------- Beginning of Change ------------------------------------------------------------

#### 9.2.6	Minimum requirement PUSCH 3-0 (Cell-Specific Reference Symbols)

#### 9.2.6.1 	Frame structure type 3 with FDD Pcell

The following requirements apply to UE Category ≥1. For the parameters specified in Table 9.2.6.1-1, Table 9.2.6.1-2, and using the downlink physical channels specified in tables C.3.6-1 and C.3.6-2, two sets of CQI reports are obtained for LAA Scell. The first one is obtained by reports whose reference resource is in the downlink subframes with [6] dB transmission power boost, i.e., high power subframes. The second one is obtained by reports whose reference resource is in the downlink subframe with [0] dB transmission power boost, i.e., low power subframe. In the test, PDSCH transport format in high power subframe is determined by first set of CQI reports and PDSCH transport format in low power subframe is determined by second set of CQI reports.

The reported CQI value in the first set of reports shall be in the range of ±1 of the reported median more than 90% of the first set of reports. The reported CQI value in the second set of reports shall be in the range of ±1 of the reported median more than 90% of the second set of reports.

If the PDSCH BLER in the high power subframes using the transport format indicated by wideband CQI median is less than or equal to 0.1, the BLER in high power subframes using the transport format indicated by the (wideband CQI median + 1) shall be greater than 0.1. If the PDSCH BLER in the high power subframes using the transport format indicated by the wideband CQI median is greater than 0.1, the BLER in high power subframes using transport format indicated by (wideband CQI median – 1) shall be less than or equal to 0.1.

If the PDSCH BLER in the low power subframes using the transport format indicated by wideband CQI median is less than or equal to 0.1, the BLER in low power subframes using the transport format indicated by the (wideband CQI median + 1) shall be greater than 0.1. If the PDSCH BLER in the low power subframes using the transport format indicated by the wideband CQI median is greater than 0.1, the BLER in low power subframes using transport format indicated by (wideband CQI median – 1) shall be less than or equal to 0.1.

The value of the wideband CQI median for first set of CQI reports minus the wideband CQI median for second set of CQI reports shall be larger than or equal to [2] in Test 1 and Test 2.

Table 9.2.6.1-1: Parmeters for PUSCH 3-0 static test on FDD Pcell

| Parameter |  | Unit | Value |  |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 20 |  |
| PDSCH transmission mode |  |  | 3 |  |
| Downlink power allocation |  | dB | -3 |  |
|  |  | dB | -3 |  |
|  |  | dB | 0 |  |
| Propagation condition and antenna configuration |  |  | Clause B.1 (2 x 2) |  |
| SNR (Note 2) |  | dB | [20] |  |
|  |  | dB[mW/15kHz] | [-78] |  |
|  |  | dB[mW/15kHz] | [-98] |  |
| Max number of HARQ transmissions |  |  | 1 |  |
| Reporting mode |  |  | PUSCH 3-0 |  |
| CSI request field |  |  | ‘10’ |
| trigger1 |  |  | 01000000 |
| trigger2 |  |  | 00000000 |
| Note 1: PCell is used for HARQ ACK/NACK feedback and aperiodic CSI triggering/reporting. PDSCH is not transmitted on PCell. |  |  |  |  |

Table 9.2.6.1-2: PUSCH 3-0 static test on LAA Scell

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 20 |  |  |  |
| PDSCH transmission mode |  |  |  | 3 |  |  |  |
| Downlink power allocation |  |  | dB | -3 |  |  |  |
|  |  |  | dB | -3 |  |  |  |
|  |  |  | dB | 0 |  |  |  |
| Propagation condition and antenna configuration |  |  |  | Clause B.1 (2x2) |  |  |  |
| SNR in subframes with [6] dB power boost (Note 2) |  |  | dB | [TBD] | [TBD] | [TBD] | [TBD] |
| SNR in subframes with [0] dB power boost (Note 2) |  |  | dB | [TBD] | [TBD] | [TBD] | [TBD] |
| in subframes with [6] dB power |  |  | dB[mW/15kHz] | [TBD] | [TBD] | [TBD] | [TBD] |
| in subframes with [0] dB power |  |  | dB[mW/15kHz] | [TBD] | [TBD] | [TBD] | [TBD] |
|  |  |  | dB[mW/15kHz] | -98 |  |  |  |
| MBSFN subframe Configuration |  |  |  | Non-MBSFN |  |  |  |
| Cell Id |  |  |  | 0 |  |  |  |
| dmtc-Periodicity |  |  | ms | 80 |  |  |  |
| dmtc-Offset |  |  |  | 0 |  |  |  |
| Number of control OFDM symbols |  |  |  | 3 |  |  |  |
| Max number of HARQ transmissions |  |  |  | 1 |  |  |  |
| Reporting mode |  |  |  | PUSCH 3-0 |  |  |  |
| PDSCH transmission model | Basic model |  |  | As specified in Section B.7 |  |  |  |
|  | subframeStartPosition |  |  | s0 |  |  |  |
|  | Number of occupied symbols per subframe |  |  | 14 |  |  |  |
|  | The number of subframes set (S1) per burst |  |  | {3,8} |  |  |  |
|  | Power configuration for each burst |  |  | is randomly selected from [6]dB power boosting or [0]dB power boosting with equal probability |  |  |  |
| Note 1: Reference measurement channel RC.1 FDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1, except for category 1 UE use RC.4 FDD with two sided dynamic OCNG Pattern OP.2 FDD as described in Annex A.5.1.2. Note 2: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 3: PDCCH DCI format 0 with a trigger for aperiodic CQI shall be transmitted in PCell on second subframe of each LAA SCell burst. |  |  |  |  |  |  |  |

#### 9.2.6.2 	Frame structure type 3 with TDD Pcell

The following requirements apply to UE Category ≥1. For the parameters specified in Table 9.2.6.2-1, Table 9.2.6.2-2, and using the downlink physical channels specified in tables C.3.6-1 and C.3.6-2, two sets of CQI reports are obtained for LAA Scell. The first one is obtained by reports whose reference resource is in the downlink subframes with [6] dB transmission power boost, i.e., high power subframes. The second one is obtained by reports whose reference resource is in the downlink subframe with [0] dB transmission power boost, i.e., low power subframe. In the test, PDSCH transport format in high power subframe is determined by first set of CQI reports and PDSCH transport format in low power subframe is determined by second set of CQI reports.

The reported CQI value in the first set of reports shall be in the range of ±1 of the reported median more than 90% of the first set of reports. The reported CQI value in the second set of reports shall be in the range of ±1 of the reported median more than 90% of the second set of reports.

If the PDSCH BLER in the high power subframes using the transport format indicated by wideband CQI median is less than or equal to 0.1, the BLER in high power subframes using the transport format indicated by the (wideband CQI median + 1) shall be greater than 0.1. If the PDSCH BLER in the high power subframes using the transport format indicated by the wideband CQI median is greater than 0.1, the BLER in high power subframes using transport format indicated by (wideband CQI median – 1) shall be less than or equal to 0.1.

If the PDSCH BLER in the low power subframes using the transport format indicated by wideband CQI median is less than or equal to 0.1, the BLER in low power subframes using the transport format indicated by the (wideband CQI median + 1) shall be greater than 0.1. If the PDSCH BLER in the low power subframes using the transport format indicated by the wideband CQI median is greater than 0.1, the BLER in low power subframes using transport format indicated by (wideband CQI median – 1) shall be less than or equal to 0.1.

The value of the wideband CQI median for first set of CQI reports minus the wideband CQI median for second set of CQI reports shall be larger than or equal to [2] in Test 1 and Test 2.

Table 9.2.6.2-1: Parmeters for PUSCH 3-0 static test on TDD Pcell

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 20 |
| Uplink downlink configuration |  |  | 2 |
| Special subframe configuration |  |  | 4 |
| PDSCH transmission mode |  |  | 3 |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 |
|  |  | dB | 0 |
| Propagation condition and antenna configuration |  |  | Clause B.1 (2 x 2) |
| SNR (Note 2) |  | dB | [20] |
|  |  | dB[mW/15kHz] | [-78] |
|  |  | dB[mW/15kHz] | [-98] |
| Max number of HARQ transmissions |  |  | 1 |
| Reporting mode |  |  | PUSCH 3-0 |
| CSI request field |  |  | ‘10’ |
| trigger1 |  |  | 01000000 |
| trigger2 |  |  | 00000000 |
| Note 1: PCell is used for HARQ ACK/NACK feedback and aperiodic CSI triggering/reporting. PDSCH is not transmitted on PCell. |  |  |  |

Table 9.2.6.2-2: PUSCH 3-0 static test on LAA Scell

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 20 |  |  |  |
| PDSCH transmission mode |  |  |  | 3 |  |  |  |
| Downlink power allocation |  |  | dB | -3 |  |  |  |
|  |  |  | dB | -3 |  |  |  |
|  |  |  | dB | 0 |  |  |  |
| Propagation condition and antenna configuration |  |  |  | Clause B.1 (2x2) |  |  |  |
| SNR in subframes with [6] dB power boost (Note 2) |  |  | dB | [TBD] | [TBD] | [TBD] | [TBD] |
| SNR in subframes with [0] dB power boost (Note 2) |  |  | dB | [TBD] | [TBD] | [TBD] | [TBD] |
| in subframes with [6] dB power |  |  | dB[mW/15kHz] | [TBD] | [TBD] | [TBD] | [TBD] |
| in subframes with [0] dB power |  |  | dB[mW/15kHz] | [TBD] | [TBD] | [TBD] | [TBD] |
|  |  |  | dB[mW/15kHz] | -98 |  |  |  |
| MBSFN subframe Configuration |  |  |  | Non-MBSFN |  |  |  |
| Cell Id |  |  |  | 0 |  |  |  |
| dmtc-Periodicity |  |  | ms | 80 |  |  |  |
| dmtc-Offset |  |  |  | 0 |  |  |  |
| Number of control OFDM symbols |  |  |  | 3 |  |  |  |
| Max number of HARQ transmissions |  |  |  | 1 |  |  |  |
| Reporting mode |  |  |  | PUSCH 3-0 |  |  |  |
| PDSCH transmission model | Basic model |  |  | As specified in Section B.7 |  |  |  |
|  | subframeStartPosition |  |  | s0 |  |  |  |
|  | Number of occupied symbols per subframe |  |  | 14 |  |  |  |
|  | The number of subframes set (S1) per burst |  |  | {3,8} |  |  |  |
|  | Power configuration for each burst |  |  | is randomly selected from [6]dB power boosting or [0]dB power boosting with equal probability |  |  |  |
| Note 1: Reference measurement channel RC.1 FDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1, except for category 1 UE use RC.4 FDD with two sided dynamic OCNG Pattern OP.2 FDD as described in Annex A.5.1.2. Note 2: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 3: PDCCH DCI format 0 with a trigger for aperiodic CQI shall be transmitted in PCell on second subframe of each LAA SCell burst. |  |  |  |  |  |  |  |

------------------------------------------------- Unchanged sections omitted --------------------------------------------------------

#### 9.2.7	Minimum requirement PUSCH 3-1 (CSI Reference Symbol)

#### 9.2.7.1	Frame structure type 3 wth FDD Pcell

The following requirements apply to UE Category ≥1. For the parameters specified in Table 9.2.7.1-1, Table 9.2.7.1-2and using the downlink physical channels specified in tables C.3.6-1 and C.3.6-2, two sets of CQI reports are obtained for LAA Scell, The first one is obtained by reports whose reference resource is in the downlink subframes with [6] dB transmission power boost, i.e., high power subframes. The second one is obtained by reports whose reference resource is in the downlink subframe with [0] dB transmission power boost, i.e., low power subframe. In the test, PDSCH transport format in high power subframe is determined by first set of CQI reports and PDSCH transport format in low power subframe is determined by second set of CQI reports.

The reported CQI value in the first set of reports shall be in the range of ±1 of the reported median more than 90% of the first set of reports. The reported CQI value in the second set of reports shall be in the range of ±1 of the reported median more than 90% of the second set of reports.

If the PDSCH BLER in the high power subframes using the transport format indicated by wideband CQI median is less than or equal to 0.1, the BLER in high power subframes using the transport format indicated by the (wideband CQI median + 1) shall be greater than 0.1. If the PDSCH BLER in high power subframes using the transport format indicated by the wideband CQI median is greater than 0.1, the BLER in high power subframes using transport format indicated by (wideband CQI median – 1) shall be less than or equal to 0.1.

If the PDSCH BLER in the low power subframes using the transport format indicated by wideband CQI median is less than or equal to 0.1, the BLER in low power subframes using the transport format indicated by the (wideband CQI median + 1) shall be greater than 0.1. If the PDSCH BLER in the low power subframes using the transport format indicated by the wideband CQI median is greater than 0.1, the BLER in low power subframes using transport format indicated by (wideband CQI median – 1) shall be less than or equal to 0.1.

The value of the wideband CQI for the first set of CQI report minus the wideband CQI median  for second set of CQI shall be larger than or equal to [2] in Test 1 and Test 2.

Table 9.2.7.1-1: Parmeters for PUSCH 3-1 static test on FDD Pcell

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 20 |
| PDSCH transmission mode |  |  | 9 |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 |
|  |  | dB | 0 |
|  |  | dB | 0 |
| Propagation condition and antenna configuration |  |  | Clause B.1 (2 x 2) |
| SNR (Note 2) |  | dB | [20] |
|  |  | dB[mW/15kHz] | [-78] |
|  |  | dB[mW/15kHz] | [-98] |
| Beamforming Model |  |  | As specified in Section B.4.3 |
| CRS reference signals |  |  | Antenna ports 0 |
| CSI reference signals |  |  | Antenna ports 15, 16 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/ 1 |
| CSI-RS reference signal configuration |  |  | 4 |
| CodeBookSubsetRestriction bitmap |  |  | 000001 |
| Number of control OFDM symbols |  |  | 3 |
| Max number of HARQ transmissions |  |  | 1 |
| Reporting mode |  |  | PUSCH 3-1 |
| CSI request field |  |  | ‘10’ |
| trigger1 |  |  | 01000000 |
| trigger2 |  |  | 00000000 |
| Note 1: PCell is used for HARQ ACK/NACK feedback and aperiodic CSI triggering/reporting. PDSCH is not transmitted on PCell. |  |  |  |

Table 9.2.7.1-2: PUSCH 3-1 static test on LAA Scell

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 20 MHz |  |  |  |
| Transmission mode |  |  |  | 9 |  |  |  |
| Downlink power allocation |  |  | dB | 0 |  |  |  |
|  |  |  | dB | 0 |  |  |  |
|  |  |  | dB | 0 |  |  |  |
|  |  |  | dB | 0 |  |  |  |
| SNR in subframes with [6] dB power boost (Note 3) |  |  | dB | TBD | TBD | TBD | TBD |
| SNR in subframes with [0] dB power boost (Note 3) |  |  |  | TBD | TBD | TBD | TBD |
| in subframes with [6] dB power |  |  |  |  |  |  |  |
| in subframes with [0] dB power |  |  | dB[mW/15kHz] | TBD | TBD | TBD | TBD |
|  |  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| MBSFN subframe Configuration |  |  |  | Non-MBSFN |  |  |  |
| Cell Id |  |  |  | 0 |  |  |  |
| dmtc-Periodicity |  |  | ms | 80 |  |  |  |
| dmtc-Offset |  |  |  | 0 |  |  |  |
| Propagation condition and antenna configuration |  |  |  | Clause B.1 (2x2) |  |  |  |
| Beamforming Model |  |  |  | As specified in Section B.4.3 |  |  |  |
| CRS reference signals |  |  |  | Antenna ports 0 |  |  |  |
| CSI reference signals |  |  |  | Antenna ports 15, 16 |  |  |  |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  |  | 5/ 1 |  |  |  |
| CSI-RS reference signal configuration |  |  |  | 4 |  |  |  |
| CodeBookSubsetRestriction bitmap |  |  |  | 000001 |  |  |  |
| Number of control OFDM symbols |  |  |  | 3 |  |  |  |
| Max number of HARQ transmissions |  |  |  | 1 |  |  |  |
| Reporting mode |  |  |  | PUSCH 3-1 |  |  |  |
| PDSCH transmission model | Basic model |  |  | As specified in Section B.7 |  |  |  |
|  | subframeStartPosition |  |  | s0 |  |  |  |
|  | Number of occupied symbols per subframe |  |  | 14 |  |  |  |
|  | The number of subframes set (S1) per burst |  |  | {3, 8} |  |  |  |
|  | Power configuration for each burst |  |  | is randomly selected from [6]dB power boosting or [0]dB power boosting with equal probability |  |  |  |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink subframe not later than SF#(n-4), this reported subband or wideband CQI cannot be applied at the eNB downlink before SF#(n+4) Note 2: Reference measurement channel RC.8 FDD according to Table A.4-1 with one/two sided dynamic OCNG Pattern OP.1/2 FDD as described in Annex A.5.1.1/2. Note 3: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 4: PDCCH DCI format 0 with a trigger for aperiodic CQI shall be transmitted in the subframe where CSI-RS is configured for transmission in the LAA Scell. |  |  |  |  |  |  |  |

#### 9.2.7.2	Frame structure type 3 wth TDD Pcell

The following requirements apply to UE Category ≥1. For the parameters specified in Table 9.2.7.2-1, Table 9.2.7.2-2and using the downlink physical channels specified in tables C.3.6-1 and C.3.6-2, two sets of CQI reports are obtained for LAA Scell, The first one is obtained by reports whose reference resource is in the downlink subframes with [6] dB transmission power boost, i.e., high power subframes. The second one is obtained by reports whose reference resource is in the downlink subframe with [0] dB transmission power boost, i.e., low power subframe. In the test, PDSCH transport format in high power subframe is determined by first set of CQI reports and PDSCH transport format in low power subframe is determined by second set of CQI reports.

The reported CQI value in the first set of reports shall be in the range of ±1 of the reported median more than 90% of the first set of reports. The reported CQI value in the second set of reports shall be in the range of ±1 of the reported median more than 90% of the second set of reports.

If the PDSCH BLER in the high power subframes using the transport format indicated by wideband CQI median is less than or equal to 0.1, the BLER in high power subframes using the transport format indicated by the (wideband CQI median + 1) shall be greater than 0.1. If the PDSCH BLER in high power subframes using the transport format indicated by the wideband CQI median is greater than 0.1, the BLER in high power subframes using transport format indicated by (wideband CQI median – 1) shall be less than or equal to 0.1.

If the PDSCH BLER in the low power subframes using the transport format indicated by wideband CQI median is less than or equal to 0.1, the BLER in low power subframes using the transport format indicated by the (wideband CQI median + 1) shall be greater than 0.1. If the PDSCH BLER in the low power subframes using the transport format indicated by the wideband CQI median is greater than 0.1, the BLER in low power subframes using transport format indicated by (wideband CQI median – 1) shall be less than or equal to 0.1.

The value of the wideband CQI for the first set of CQI report minus the wideband CQI median  for second set of CQI shall be larger than or equal to [2] in Test 1 and Test 2.

Table 9.2.7.2-1: Parmeters for PUSCH 3-1 static test on TDD Pcell

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 20 |
| PDSCH transmission mode |  |  | 9 |
| Uplink downlink configuration |  |  | 2 |
| Special subframe configuration |  |  | 4 |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 |
|  |  | dB | 0 |
|  |  | dB | 0 |
| Propagation condition and antenna configuration |  |  | Clause B.1 (2 x 2) |
| SNR (Note 2) |  | dB | [20] |
|  |  | dB[mW/15kHz] | [-78] |
|  |  | dB[mW/15kHz] | [-98] |
| Beamforming Model |  |  | As specified in Section B.4.3 |
| CRS reference signals |  |  | Antenna ports 0 |
| CSI reference signals |  |  | Antenna ports 15, 16 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/ 1 |
| CSI-RS reference signal configuration |  |  | 4 |
| CodeBookSubsetRestriction bitmap |  |  | 000001 |
| Number of control OFDM symbols |  |  | 3 |
| Max number of HARQ transmissions |  |  | 1 |
| Reporting mode |  |  | PUSCH 3-1 |
| CSI request field |  |  | ‘10’ |
| trigger1 |  |  | 01000000 |
| trigger2 |  |  | 00000000 |
| Note 1: PCell is used for HARQ ACK/NACK feedback and aperiodic CSI triggering/reporting. PDSCH is not transmitted on PCell. |  |  |  |

Table 9.2.7.2-2: PUSCH 3-1 static test on LAA Scell

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 20 MHz |  |  |  |
| Transmission mode |  |  |  | 9 |  |  |  |
| Downlink power allocation |  |  | dB | 0 |  |  |  |
|  |  |  | dB | 0 |  |  |  |
|  |  |  | dB | 0 |  |  |  |
|  |  |  | dB | 0 |  |  |  |
| SNR in subframes with [6] dB power boost (Note 3) |  |  | dB | TBD | TBD | TBD | TBD |
| SNR in subframes with [0] dB power boost (Note 3) |  |  |  | TBD | TBD | TBD | TBD |
| in subframes with [6] dB power |  |  |  |  |  |  |  |
| in subframes with [0] dB power |  |  | dB[mW/15kHz] | TBD | TBD | TBD | TBD |
|  |  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| MBSFN subframe Configuration |  |  |  | Non-MBSFN |  |  |  |
| Cell Id |  |  |  | 0 |  |  |  |
| dmtc-Periodicity |  |  | ms | 80 |  |  |  |
| dmtc-Offset |  |  |  | 0 |  |  |  |
| Propagation condition and antenna configuration |  |  |  | Clause B.1 (2x2) |  |  |  |
| Beamforming Model |  |  |  | As specified in Section B.4.3 |  |  |  |
| CRS reference signals |  |  |  | Antenna ports 0 |  |  |  |
| CSI reference signals |  |  |  | Antenna ports 15, 16 |  |  |  |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  |  | 5/ 1 |  |  |  |
| CSI-RS reference signal configuration |  |  |  | 4 |  |  |  |
| CodeBookSubsetRestriction bitmap |  |  |  | 000001 |  |  |  |
| Number of control OFDM symbols |  |  |  | 3 |  |  |  |
| Max number of HARQ transmissions |  |  |  | 1 |  |  |  |
| Reporting mode |  |  |  | PUSCH 3-1 |  |  |  |
| PDSCH transmission model | Basic model |  |  | As specified in Section B.7 |  |  |  |
|  | subframeStartPosition |  |  | s0 |  |  |  |
|  | Number of occupied symbols per subframe |  |  | 14 |  |  |  |
|  | The number of subframes set (S1) per burst |  |  | {3, 8} |  |  |  |
|  | Power configuration for each burst |  |  | is randomly selected from [6]dB power boosting or [0]dB power boosting with equal probability |  |  |  |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink subframe not later than SF#(n-4), this reported subband or wideband CQI cannot be applied at the eNB downlink before SF#(n+4) Note 2: Reference measurement channel RC.8 FDD according to Table A.4-1 with one/two sided dynamic OCNG Pattern OP.1/2 FDD as described in Annex A.5.1.1/2. Note 3: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 4: PDCCH DCI format 0 with a trigger for aperiodic CQI shall be transmitted in the subframe where CSI-RS is configured for transmission in the LAA Scell. |  |  |  |  |  |  |  |

------------------------------------------------- Unchanged sections omitted --------------------------------------------------------

## C.3.6	Measurement of Performance requirements (FS3)

Table C.3.6-1 is applicable for measurements in which uniform RS-to-EPRE boosting is assumed for all downlink physical channels.

# Table C.3.6-1: Downlink Physical Channels transmitted during a connection (FS3)

| Physical Channel | EPRE Ratio |
| --- | --- |
| PSS | PSS_RA = 0 (Note 3) |
| SSS | SSS_RA = 0 (Note 3) |
| PCFICH | PCFICH_RB = B+ |
| PDCCH | PDCCH_RA = A+ |
|  | PDCCH_RB = B+ |
| EPDCCH | EPDCCH_RA = A+δ |
|  | EPDCCH_RB = B+δ |
| PDSCH | PDSCH_RA = A |
|  | PDSCH_RB = B |
| OCNG | OCNG_RA = A+ |
|  | OCNG_RB = B+ |

NOTE 1:	A= B = 0 dB means no RS boosting.

NOTE 2:	OCNG is not defined downlink physical channels in [4].

NOTE 3: Assuming PSS and SSS transmitted on a single antenna port.

NOTE 4: A, B, , and δ are test specific.

NOTE 5: For TM 8, TM 9 and TM10  A, B are used for the purpose of the test set up only.

# Table C.3.6-2: Power allocation for OFDM symbols and reference signals

| Parameter | Unit | Value | Note |
| --- | --- | --- | --- |
| Total transmitted power spectral density | dBm/15 kHz | Test specific | 1. shall be kept constant throughout all OFDM symbols |
| Cell-specific reference signal power ratio |  | Test specific | 1. Applies for antenna port p |
| Energy per resource element EPRE |  | Test specific | 1. The complex-valued symbols and defined in [4] shall conform to the given EPRE value. 2. For TM8, TM9 and TM10 the reference point for EPRE is before the precoder in Annex B.4. |

-----------------------------------------------------------------End of Change ------------------------------------------------------------