3GPP TSG-RAN WG4 Meeting #80	   R4-165957

Gothenburg, SE, 22 – 26 Aug, 2016

Agenda Item:	8.11.3.2

Source: 	Samsung

Title: 	Discussion on V2V UE Rx RF requirements at 5.9GHz

Document for:	Discussion

Introduction

In RAN4#79 meeting there were several contributions on V2V UE RF aspect. However, there is no consensus achieved on any specific requirement in Nanjing. Hence this contribution is submitted to provide further consideration especially on UE RF Rx requirement for Band 47, which seems the only available operating band dedicated for V2V communication. In addition, considering operator’s interest in MCC operation the contribution involves all three scenarios for,

- Case1: Band 47 SCC operation

- Case2: Band 47 intra-band MCC operation

- Case3: Band 47 and WAN inter-band operation

Discussion

Relevant considerations on UE RF front-end could be found in companion contribution [7] with respect to UE RF reference architecture and implementation challenge, which is applicable for both TRX chain. In this contribution we just directly go into the discussion on specific UE RX requirements as below.

REFSENS

The REFSENS for Band 46 with 20MHz BW is -90dBm, which is equivalent to -4dB SNR, 2.5dB margin and 4dB additional relaxation due to implementation condition of 5.9GHz. The SNR value suitable for V2V would be evaluated according agreement in [5]. For V2V 5.9 GHz TDD operating band the 2.5dB margin and 4dB additional implementation relaxation could be reused for both 10MHz and 20MHz channel bandwidth. Then the REFSENS would be

REFSENS10MHz= -174 +10log(9*106)+NF(9dB)+SNRV2V+2.5+4= -89+SNRV2V [dBm]

REFSENS20MHz= -174 +10log(18*106)+NF(9dB)+SNRV2V+2.5+4= -86+SNRV2V [dBm]

It should be mentioned that if Band 47 is restrained dedicatedly for V2V operation between terminals then the REFSENS could not be captured in Table 7.3.1-1: Reference sensitivity QPSK PREFSENS which is applicable for general WAN operation.

Maximum input level

It is proposed to reuse -25dBm legacy requirement as baseline for SCC V2V operation for 64QAM. And the requirement of -22dBm for CA intra-band class C CA requirement could apply for MCC intra-band V2V operation.

ACS

For selectivity requirement of V2V 5.9GHz operation, 10MHz interference should be considered for wanted signal with 10MHz channel bandwidth as below tables. The requirement agreed for Band 46 is considered here, however the final parameters are still pend on conclusion of co-existence study.

Table 2.3-1: Adjacent channel selectivity for V2V

| V2V band | Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| 47 | ACS | dB |  |  |  | [33] |  | [27] |

Table 2.3-2: Test parameters for Adjacent channel selectivity for V2V, Case 1

| V2V Band | Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| 47 | Power in Transmission Bandwidth Configuration | dBm | REFSENS + 14 dB |  |  |  |  |  |
|  | PInterferer | dBm |  |  |  | [REFSENS +45.5dB] |  | [REFSENS +39.5dB] |
|  | BWInterferer | MHz |  |  |  | 10 |  | 20 |
|  | FInterferer (offset) | MHz |  |  |  | 10+0.0125 / -10-0.0125 |  | 20+0.0025 / -20-0.0025 |

Table 2.3-3: Test parameters for Adjacent channel selectivity for V2V, Case 2

| V2V band | Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| 47 | Power in Transmission Bandwidth Configuration | dBm |  |  |  | [-56.5] |  | [-50.5] |
|  | PInterferer | dBm | -25 |  |  |  |  |  |
|  | BWInterferer | MHz |  |  |  | 10 |  | 20 |
|  | FInterferer (offset) | MHz |  |  |  | 10+0.0125 / -10-0.0125 |  | 20+0.0025 / -20-0.0025 |

ACS requirement for contiguous MCC operation within Band 47 could also refer to intra-band contiguous class C ACS requirement of Band 46.

Blocking requirement

In band blocking

For SCC V2V operation the corresponding requirement of Band46 is suggested to be reused here with the extension of 10MHz BW interferer for 10MHz system bandwidth as below.

Table 2.4-1: In band blocking parameters for V2V operating band

| V2V band | Rx parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| 47 | Power in Transmission Bandwidth Configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |  |
|  |  |  |  |  |  | 6 |  | 9 |
|  | BWInterferer | MHz |  |  |  | 10 |  | 20 |
|  | FIoffset, case 1 | MHz |  |  |  | 15+0.0125 |  | 30+0.0125 |
|  | FIoffset, case 2 | MHz |  |  |  | 25+0.0075 |  | 50+0.0075 |

Table 2.4-2: In-band blocking for V2V operating bands

| V2V band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -50 | -44 |
|  | FInterferer (offset) | MHz | =-BW/2 – FIoffset,case 1 & =+BW/2 + FIoffset,case 1 | ≤-BW/2 – FIoffset,case 2 & ≥+BW/2 + FIoffset,case 2 |
| 47 | FInterferer | MHz | (Note 2) | FDL_low – 60 to FDL_high + 60 |

In-band blocking requirement for contiguous MCC operation within Band 47 could reference to intra-band contiguous class C In-band blocking requirement of Band 46.

Out-of-band blocking

Current SCC requirement could be reused with modification on NOTE2 to indicate that the power level of the interferer (Pinterferer) for Range 3 of Band 47 shall be modified to -20dBm for Finterfer>4400MHz.

Regulation requirement as below should be also considered for V2V operation.

Blocking is a measure of the capability of the receiver to operate satisfactorily in the presence of a signal in frequency band further away and it shall be tested at ± 50 MHz, ± 100 MHz, and ± 200 MHz. Blocking testing shall be performed at least at 6 different frequency offset positions. The manufacturer of the equipment can add additional frequency offsets positions.

The blocking level shall not be less than -30 dBm.

Narrow band blocking

No narrow band blocking is suggested for band 47 the same as agreed for Band 46.

Spurious response

No impact on specification.

RX IM

For SCC V2V operation the corresponding requirement of Band46 is suggested to be reused here with the extension of 10MHz BW interferer for 10MHz system bandwidth as below.

Table 2.6-1: Wide band intermodulation

| E-UTRA band | Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1.4 MHz |  | 3 MHz |  | 5 MHz | 10 MHz |  | 15 MHz |  | 20 MHz |  |
| 47 | Power in Transmission Bandwidth Configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 6 |  |  |  | 9 |  |
|  | PInterferer 1 (CW) | dBm | -46 |  |  |  |  |  |  |  |  |  |  |
|  | PInterferer 2 (Modulated) | dBm | -46 |  |  |  |  |  |  |  |  |  |  |
|  | BWInterferer 2 |  |  |  |  |  |  |  | 10 |  |  |  | 20 |
|  | FInterferer 1 (Offset) | MHz |  |  |  |  |  |  | -BW/2 – 15 / +BW/2 + 15 |  |  |  | -BW/2 – 30 / +BW/2 + 30 |
|  | FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |  |  |  |  |  |  |  |  |  |  |

IM requirement for contiguous MCC operation within Band 47 could reference to intra-band contiguous class C IM requirement of Band 46.

Spurious emission

The only modification of specification is that additional requirement in the frequency range of 12.75GHz~26GHz should be added for 5.9GHz V2V operation band.

Receiver image

The related requirement of CA class C can apply for UE supporting contiguous MCC operation within Band 47.

Conclusion

In this contribution we generally discussed the V2V UE Rx requirements with some initial proposal on specification draft, which RAN4 could take into account in V2V RAN4 standard. Some key points are summarized as blow,

- For V2V 5.9 GHz TDD operating band the 2.5dB margin and 4dB additional implementation relaxation could be reused for REFSENS of both 10MHz and 20MHz channel bandwidth.

- Reuse -25dBm legacy maximum input requirement as baseline for SCC V2V 5.9 GHz operation for 64QAM.

- Reuse ACS, blocking and RX IM requirement of Band 46 as starting point for V2V 5.9 GHz operation with the extension to utilize 10MHz BW interferer for 10MHz system bandwidth in ACS, in-band blocking and RX IM requirements.

- Introduction of V2V 5.9GHz Band would have no impact on requirement of Spurious response in spec.

- Additional requirement of spurious emission in the frequency range of 12.75GHz~26GHz should be added for 5.9GHz V2V operation band.

- The related Receiver image requirement of CA class C can apply for UE supporting contiguous MCC operation within 5.9GHz V2V operation band.

Reference

[1] R4-164804, “TP on V2V operating scenarios and operating band and channel bandwidths”

[2] TR36.785: V2V Services based on LTE sidelink User Equipment (UE) radio transmission and reception

[3] R4-164443 LS on V2X multicarrier configuration

[4] TR36.885: Study on LTE-based V2X Services

[5] R4-164849, WF on simulation assumptions for V2V REFSENS demodulation

[6] R4-165955, Discussion on multiple component carrier operation in V2V

[7] R4-165956, Discussion on V2V UE Tx RF requirement at 5.9GHz