3GPP TSG-RAN WG4 Meeting #80                                   R4-166103

Gothenburg, Sweden, 22 - 27 Aug, 2016

Agenda Item:	10.4

Source: 	CMCC

Title: 			   Discussion on co-existence study for frequency below 6GHz

Document for:	Discussion

- Introduction

At previous meetings, NR co-existence simulation assumption for mm-wave was discussed [1]. However, for the sake of less time, simulation for frequency below 6GHz was not discussed extensively. This paper is intended to trigger this discussion.

- Discussion

RAN4 hadn’t done any co-existence study on mm-wave before the NR SI started, but a large amount of work had been done for frequency below 6GHz, such as in TR 36.942, TR 25.942, TR 36.931, TR 37.809 and etc. RAN4 had accumulated a lot of experience on system modeling below 6GHz. Hence, what we should do at this time is to identify the difference of system modeling between E-UTRA and NR.

Observation#1: RAN4 has to identify the difference of simulation modeling between E-UTRA and NR below 6GHz.

If we look at the TR 38.913, we will find some difference for NR.

First of all, some new deployment scenarios such as extreme rural and highway coverage scenario are new for RAN4 study. Although frequency below 6GHz can support nearly all the scenario, RAN4 has to focus on finding worst case for co-existence purpose.

Observation#2: RAN4 has to identify if the deployment scenario newly introduced is worth evaluating

Secondly, larger number of antenna elements for both BS and UE may be adopted. The number of antenna elements is mainly limited by size of the antennal panel, that higher frequency is more likely to use larger number of antenna elements. RAN4 has to investigate the appropriate number of antenna elements for each frequency range from both BS and UE implementation point of view.

Observation#3: RAN4 has to identify the frequency range of large number of antenna elements could be implemented.

Finally, Larger carrier bandwidth especially for frequency around 4GHz is introduced for NR. Although RAN4 had done LTE advanced carrier aggregation co-existence study, the NR may have even larger carrier bandwidth. Due to the limitation of spectrum resource, lower frequency usually doesn’t have large contiguous bandwidth.

Observation#4: RAN4 has to identify which frequency range may have extremely wide bandwidth.

We don’t exclude other difference of system modeling between NR and E-UTRA. Considering the three characteristics above, if we find one particular co-existence NR case doesn’t have big difference from E-UTRA, we may conclude that the existing study could cover that. For example, for frequency below 1GHz, large number of antenna elements is hard to be implemented and it is also hard for operator to get such wide contiguous bandwidth on the frequency. We may conclude that the co-existence study may be very similar to that we have done for E-UTRA 900MHz in Macro, Micro or Pico scenario case. On the other side, such as frequency around 4GHz, large number of antenna elements and extreme large bandwidth are most likely to be used. Then the simulation should be conducted.

Observation#5: reuse the simulation results if no significant difference to E-UTRA study is found for a certain co-existence case

Observation#6: conduct simulation if significant difference to E-UTRA study is found for a certain co-existence case

- Conclusion

Observation#1: RAN4 has to identify the difference of simulation modeling between E-UTRA and NR below 6GHz.

Observation#2: RAN4 has to identify if the deployment scenario newly introduced is worth evaluating

Observation#3: RAN4 has to identify the frequency range of large number of antenna elements could be implemented.

Observation#4: RAN4 has to identify which frequency range may have extremely wide bandwidth.

Observation#5: reuse the simulation results if no significant difference to E-UTRA study is found for a certain co-existence case

Observation#6: conduct simulation if significant difference to E-UTRA study is found for a certain co-existence case

- Reference

- R4-164541, Evening AH minutes for WP 5D co-existence simulation assumptions, NTT DOCOMO, INC