3GPP TSG-RAN WG4 Meeting #80                                   R4-166105

Gothenburg, China, 22 - 26 Aug, 2016

Agenda Item:		8.1.2

Source: 		CMCC

Title: 	TP on TR 36.714-00-00 for CA_40D and CA_41D

Document for:		Discussion

- Introduction

Intra-band contiguous CA of 3UL in B40 and B41 were approved in Ran#72 meeting [1]. This TP is based on the analysis in [2].

- Reference

- RP-160822, Revised WID: LTE Advanced intra-band CA Rel-14 for xDL/yUL including contiguous and non-contiguous spectrum

- R4-166106, Discussion on RF requirements for 3UL intra-band contiguous CA of CA_40D and CA_41D, CMCC

- Text Proposal

<Start of Text Proposal for TR 36.714-00-00>

- 6.x	CA_40D_3UL_BCS0

- 6.x.1	Channel bandwidths per operating band

Table 6.x.1-1 defines the supported E-UTRA CA configurations and bandwidth combination set for LTE-advanced 3UL intra-band contiguous CA in Band 40.

Table 6.x.1-1: E-UTRA CA configurations and bandwidth combination sets for CA_40D

|  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Uplink CA configurations (NOTE 3) | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] Channel bandwidths for carrier [MHz] |  |  |
| CA_40D | CA_40D | 10, 15, 20 | 20 | 20 | 60 | 0 |
|  |  | 20 | 10, 15 | 20 |  |  |
|  |  | 20 | 20 | 10, 15 |  |  |

- 6.x.2	UE maximum output power for

The UE maximum output power for CA_40D is proposed to reuse requirement for bandwidth class C.

Table 6.x.2-1 CA UE Power Class for intra-band contiguous CA_40D

| E-UTRA CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_40D |  |  |  |  | 23 | +2/-2 |  |  |

- 6.x.3	UE Maximum Output power for modulation / channel bandwidth

Table 6.x.3-1 shows the MPR requirement for bandwidth class D, Some of the requirements could be reused. Requirement with more than 200 RB is FFS.

Table 6.x.3-1 Maximum Power Reduction (MPR) for CA bandwidth class D with Power Class 3

| Modulation | CA bandwidth Class D |  |  |  |  | MPR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
|  | 50 RB + 75 RB + 100RB | 50 RB + 100 RB + 100 RB | 75 RB + 75 RB + 100 RB | 75 RB + 100 RB + 100 RB | 100 RB + 100 RB + 100 RB |  |
| QPSK | > 12 and ≤ 50 | > 12 and ≤ 50 | > 16 and ≤ 75 | > 16 and ≤ 75 | > 18 and ≤ 100 | ≤ 1 |
| QPSK | > 50 and ≤ TBD | > 50 and ≤ TBD | > 75 and ≤ TBD | > 75 and ≤ TBD | > 100 and ≤ TBD | ≤ 2 |
| QPSK | > TBD | > TBD | > TBD | > TBD | > TBD | ≤ TBD |
| 16 QAM | ≤ 8 | ≤ 12 | ≤ 8 | ≤ 12 | ≤ 16 | ≤ 1 |
| 16 QAM | > 12 and ≤ 50 | > 12 and ≤ 50 | > 16 and ≤ 75 | > 16 and ≤ 75 | > 18 and ≤ 100 | ≤ 2 |
| 16 QAM | > 50 and ≤ TBD | > 50 and ≤ TBD | > 75 and ≤ TBD | > 75 and ≤ TBD | > 100 and ≤ TBD | ≤ 3 |
| 16 QAM | > TBD | > TBD | > TBD | > TBD | > TBD | ≤ TBD |
| 64 QAM | ≤ 12 allocation wholly contained within a single CC | ≤ 12 and allocation wholly contained within a single CC | ≤ 16 and allocation wholly contained within a single CC | ≤ 16 and allocation wholly contained within a single CC | ≤ 18 and allocation wholly contained within a single CC | ≤ 2 |
| 64 QAM | > 12 allocation wholly contained within a single CC or allocation extends across two CC’s | > 12 allocation wholly contained within a single CC or allocation extends across two CC’s | > 16 allocation wholly contained within a single CC or allocation extends across two CC’s | > 16 allocation wholly contained within a single CC or allocation extends across two CC’s | > 18 allocation wholly contained within a single CC or allocation extends across two CC’s | ≤ 3 |
| 64 QAM | allocation extends across Three CC’s | allocation extends across Three CC’s | allocation extends across Three CC’s | allocation extends across Three CC’s | allocation extends across Three CC’s | ≤ TBD |

- 6.x.4 Power control

Requirements for bandwidth class B and C can be reused.

- 6.x.5 Transmit modulation quality

Requirements for bandwidth class B and C can be reused.

- 6.x.6 Spectrum emission mask

Table 6.x.6-1 shows the SEM requirement for bandwidth class D.

Table 6.x.6-1: General E-UTRA CA spectrum emission mask for Bandwidth Class D

| Spectrum emission limit [dBm]/BWChannel_CA |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 50RB+75RB+100RB (44.1MHz) | 50RB+100RB+100 RB (49.2MHz) | 75RB+75RB+100RB (49.6MHz) | 75RB+100RB+100RB (54.4MHz) | 100RB+100RB+100 RB (59.6MHz) | Measurement bandwidth |
| 0-1 | -22 | -22.5 | -22.5 | -23.5 | -24 | 30 kHz |
| 1-5 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| 5-44.1 | -13 | -13 | -13 | -13 | -13 | 1 MHz |
| 44.1-49.1 | -25 | -13 | -13 | -13 | -13 | 1 MHz |
| 49.1-49.2 |  | -13 | -13 | -13 | -13 | 1 MHz |
| 49.2-49.6 |  | -25 | -13 | -13 | -13 | 1 MHz |
| 49.6-54.2 |  | -25 | -25 | -13 | -13 | 1 MHz |
| 54.2-54.4 |  |  | -25 | -13 | -13 | 1 MHz |
| 54.4-54.6 |  |  | -25 | -25 | -13 | 1 MHz |
| 54.6-59.4 |  |  |  | -25 | -13 | 1 MHz |
| 59.4-59.6 |  |  |  |  | -13 | 1 MHz |
| 59.6-64.6 |  |  |  |  | -25 | 1 MHz |

- 6.x.7 UTRA and E-UTRA ACLR requirements

Requirements for bandwidth class B and C can be reused.

- 6.x.8 Spurious emission band UE co-existence

Table 6.x.8-1 shows the spurious emission band UE co-existence for CA_40D.

Table 6.x.8-1: Spurious emission band UE co-existence Requirements

| E-UTRA CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_40D | E-UTRA Band 1, 3, 7, 8, 20, 22, 26, 27, 32, 33, 34, 38, 39, 41, 42, 43, 44, 65, 67 | FDL_low | - | FDL_high | -50 | 1 |  |

- 6.x.8 Transmit intermodulation

Requirements for bandwidth class B and C can be reused.

- 6.y	CA_41D_3UL_BCS0

- 6.y.1	Channel bandwidths per operating band

Table 6.y.1-1 defines the supported E-UTRA CA configurations and bandwidth combination set for LTE-advanced 3UL intra-band contiguous CA in Band 41.

Table 6.y.1-1: E-UTRA CA configurations and bandwidth combination sets for CA_41D

|  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Uplink CA configurations (NOTE 3) | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] Channel bandwidths for carrier [MHz] |  |  |
| CA_41D | CA_41D | 10 | 20 | 15 | 60 | 0 |
|  |  | 10 | 15, 20 | 20 |  |  |
|  |  | 15 | 20 | 10, 15 |  |  |
|  |  | 15 | 10, 15, 20 | 20 |  |  |
|  |  | 20 | 15, 20 | 10 |  |  |
|  |  | 20 | 10, 15, 20 | 15, 20 |  |  |

- 6.y.2	UE maximum output power

The UE maximum output power for CA_41D is proposed to reuse requirement for bandwidth class C.

Table 6.y.2-1 CA UE Power Class for intra-band contiguous CA_41D

| E-UTRA CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_41D |  |  |  |  | 23 | +2/-22 |  |  |
| NOTE 2: If all transmitted resource blocks (Figure 5.6A-1) over all component carriers are confined within FUL_low and FUL_low + 4 MHz or/and FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |  |  |  |  |  |  |

- 6.y.3 Spurious emission band UE co-existence

Table 6.x.8-1 shows the spurious emission band UE co-existence for CA_41D.

Table 6.y.3-1: Spurious emission band UE co-existence for CA_41D

| E-UTRA CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_41D | E-UTRA Band 1, 2, 3, 4, 5, 8, 10, 12, 13 , 14, 17, 23, 24, 25, 26, 27, 28, 29, 30, 34, 39, 40, 42, 44, 65, 66 | FDL_low | - | FDL_high | -50 | 1 |  |

- 6.x.4 UE maximum output power with additional requirements

A New CA network signaling needs to be added for CA_41D.

2.5-1: Additional Maximum Power Reduction (A-MPR) for intra-band contiguous CA

| CA Network Signalling value | Requirements (subclause) | Uplink CA Configuration | A-MPR [dB] (subclause) |
| --- | --- | --- | --- |
| CA_NS_09 | TBD | CA_41D | TBD |

- 6.y.5 Additional Spectrum emission mask for CA_41D

FFS

<End of Text Proposal for TR 36.714-00-00>