3GPP TSG RAN4 Meeting #80	R4-166140

Gothenburg, Sweden, 22 – 26 August, 2016

Title: 	MSD for 1+5+40 3DL 2UL CA

Source: 	Huawei, HiSilicon

Agenda item:	8.7.2

Document for:	Discussion

# 1	Introduction

This contribution discusses MSD requirements for 1+5+40 3DL 2 UL CA.

# 2	Discussion

This CA combination has only one intermodulation product, IMD4, falling on top of own B40 RX channel

| CA | UL CA | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL | 3rd DL Fc (MHz) | DL BW (MHz) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | RB # |  |  |
| B1+B5+B40 | B1 | IMD4 | 2*fB1 – 2*fB5 | 1977.5 | 5 | 25 | 2305 | 10 |
|  | B5 |  |  | 826.5 | 5 | 25 |  |  |
|  | B41 |  |  | 2655 | 10 | 25 |  |  |

Table 1 Example test points

We use the following RF front-end architecture in our analysis. A triplexer based architecture would be an alternative.

Figure 1 Common diplexer -architecture

We analyze the amount of IMD using the following component linearity assumptions:

| Component | IP4 (dBm) |
| --- | --- |
| Antenna switch | 56 |
| Diplexer | 55 |
| Triplexer | 55 |
| PA forward mixing | 28 |
| PA reverse mixing | 33 |
| LNA | -6 |

Table 2 General linearity parameters

B5 duplexer TX rejection at B40	40

B40 filter attenuation at B8 TX		36

Triplexer B5 TX rejection    			30

Antenna isolation 					10

PCB isolation PA-PA 				60

PCB isolation PAout-RX				65

RFIC Pout B5/B40					0

Triplexer X-band Iso B1->B40		43

Diplexer isolation						15

PA gain								28

Based on these parameters we get the following MSD. Please note that the CF is the same as in [1]. B8 B40 LNA is the dominant factor.

| CA | UL CA | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL | 3rd DL Fc (MHz) | DL BW (MHz) | CF | MSD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | RB # |  |  |  |  |
| B1+B5+B40 | B1 | IMD4 | 2*fB1 – 2*fB5 | 1977.5 | 5 | 25 | 2305 | 10 | N/A |  |
|  | B5 |  |  | 826.5 | 5 | 25 |  |  | N/A |  |
|  | B40 |  |  | 2655 | 10 | 25 |  |  | 1.21 | 9.0 |

# 3	Conclusion

Example test points and MSD for B1+B5+B40 3DL 2UL CA MSD is provided.

| CA | UL CA | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL | 3rd DL Fc (MHz) | DL BW (MHz) | CF | MSD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | RB # |  |  |  |  |
| B1+B5+B40 | B1 | IMD4 | 2*fB1 – 2*fB5 | 1977.5 | 5 | 25 | 2305 | 10 | N/A |  |
|  | B5 |  |  | 826.5 | 5 | 25 |  |  | N/A |  |
|  | B40 |  |  | 2655 | 10 | 25 |  |  | 1.21 | 9.0 |

# 4	References

[1] R4-161889, “XDL 2UL IMDs, testpoints and CFs”, Qualcomm Inc., RAN4#78bis