TSG-RAN Working Group 4 (Radio) meeting #80		R4-166193

Gothenburg, Sweden, 22 - 26 August, 2016

Agenda Item:	8.11.3

Source: 	Ericsson

Title:	RF regulatory requirements for V2V in European 5.9 GHz ITS band

Document for:	Discussion

# Introduction

One of the objectives of the V2V sidelink WI in [1] is

- To specify UE Tx and Rx RF requirement covering operations at up to 6 GHz carrier [RAN4]

In this contribution, we look at the RF regulatory requirements for ITS in the 5.9 GHz ITS band. In particular, we discuss the harmonized European standard for 5.9 GHz ITS spectrum in Europe. The summary below is based on draft ETSI EN 302 571 V2.0.3 (2016-03) with latest modifications [2]. This draft is currently under approval procedure and will be the next version of the ETSI harmonized standard.

# Disscussion

As shown in Table 1, ECC Decision (08)01 [3] defines the spectrum usage conditions in the frequency range 5 875 GHz to 5 905 MHz for non-safety ITS and proposes CEPT administrations to consider within a future review of the Decision to include the designation of the frequency sub-band 5905-5925 MHz for an extension of ITS spectrum. ECC Recommendation (08)01 [4] recommends the spectrum usage of the frequency range 5 855 MHz to 5 875 MHz for non-asfety ITS. The Commission Decision 2008/671/EC [5] mandates a harmonized use of the frequency band 5 875 MHz to 5 905 MHz dedicated to safety-related ITS applications throughout the member states of the European Union.

Table 1: 5.9 GHz ITS frequency band segmentation in Europe

| Frequency band | Frequency range | Usage | Regulation |
| --- | --- | --- | --- |
| ITS-G5B | 5 855 MHz to 5 875 MHz | ITS non-safety applications | ECC Recommendation (08)01 [4] |
| ITS-G5A | 5 875 MHz to 5 905 MHz | ITS road safety | Commission Decision 2008/671/EC [5], ECC Decision (08)01 [3] |
| ITS-G5D | 5 905 MHz to 5 925 MHz | Future ITS applications | ECC Decision (08)01 [3] |

In the following, we review the regulatory comformance requirements specified in the harmonized standard draft EN 302 571 V2.0.3 (2016-03) [2], which applies to radio transmitters and receivers for ITS operating in the frequency range 5 855 MHz to 5 925 MHz in Europe.

## TX requirements

### RF output power

The radio frequency (RF) output power is the mean equivalent isotropically radiated power (e.i.r.p.) during transmission bursts.

The maximum RF output power shall not exceed 33 dBm e.i.r.p. [2]

### Power spectral density

The power spectral density (PSD) is the mean e.i.r.p. spectral density during transmission bursts.

The maximum power spectral density shall not exceed 23 dBm/MHz e.i.r.p. [2]

### Transmit power control

The TPC range shall at least be 3 dBm up to the maximum specified RF output power e.i.r.p of the equipment. [2]

### Transmitter frequency stability

The nominal carrier frequencies, fc , for ITS channels are shown in Table 2. The actual carrier centre frequency for any given channel given in Table 2 shall be maintained within the range fc ± 20 ppm. [2]

Table 2: Nominal carrier frequency allocations [2]

|  | Carrier centre frequency fc (MHz) | Maximum channel bandwidth (MHz) |
| --- | --- | --- |
|  | 5 860 | 10 |
|  | 5 870 | 10 |
|  | 5 880 | 10 |
|  | 5 890 | 10 |
|  | 5 900 | 10 |
|  | 5 910 | 10 |
|  | 5 920 | 10 |

### Transmitter unwanted emissions

Table 3 and Table 4 tabulate transmitter unwanted emission limits in the spurious domain below 1 GHz and above 1 GHz, respectively.

Table 3: Transmitter unwanted emission limits in the spurious domain below 1 GHz [2]

| Frequency range | Maximum power, (e.r.p.) (dBm) | Reference bandwidth |
| --- | --- | --- |
| 30 MHz f 1 GHz | -36 | 100 kHz |

Table 4. Transmitter unwanted emissions in the spurious domain above 1 GHz [2]

| Frequency range | Maximum power, (e.i.r.p.) (dBm) | Bandwidth |
| --- | --- | --- |
| 1 GHz to 5,795 GHz | -30 dBm | 1 MHz |
| 5,795 GHz to 5,815 GHz | see Clause 4.2.9.2 | 1 MHz |
| 5,815 GHz to 5,835 GHz | -30 dBm | 1 MHz |
| 5,945 GHz to 18 GHz | -30 dBm | 1 MHz |

Table 5 tabluates transmitter unwanted emission limits in the out-of-band domain of the 5 GHz ITS frequency band.

The out-of-band domain is defined as ±250 % of the channel bandwidth, and then the out-of-band (OOB) domain for the 5 GHz ITS frequency band is 5,835 GHz to 5,855 GHz at the lower part and 5,925 GHz to 5,945 GHz at the higher part of the frequency band.

Table 5: Transmitter unwanted emission limits in the out-of-band domain of the 5 GHz ITS frequency band [2]

| Frequency range | Maximum power, (e.i.r.p.) (dBm) | Reference bandwidth |
| --- | --- | --- |
| 5,835 GHz to 5,855 GHz | -30 | 1 MHz |
| 5,925 GHz to 5,945 GHz | -30 | 1 MHz |

In Figure 1, the transmitter unwanted emission limits are depicted for the 5 GHz ITS frequency band (ITS-G5A, ITS-G5B, and ITS-G5D frequency bands).

Figure 1: Transmitter unwanted emission limits for the 5 GHz ITS frequency band [2]

Observation 1: TX requirements by ETSI have unwanted emission limit down to -65 dBm between 5795MHz and 5815MHz.

Proposal 1: TX requirements for LTE-based ITS shall meet the unwanted emission requirement of -65dBm between 5795MHz and 5815MHz.

### Transmitter spectrum mask within the 5 GHz ITS frequency band for 10 MHz channels

The spectrum mask in Table 6 applies within 5 855 MHz to 5 925 MHz according to draft ETSI EN 302 571 [2].

Table 6: Transmitter spectrum mask for 10 MHz channel bandwidth [2]

| Carrier frequency fc (dBc) | ± 4,5 MHz offset (dBc) | ± 5,0 MHz offset (dBc) | ± 5,5 MHz offset (dBc) | ± 10 MHz offset (dBc) | ± 15 MHz offset (dBc) |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | -26 | -32 | -40 | -50 |

The relative power values given in Table 6 are valid for the maximum allowed output power as given in 2.1.1. For devices with lower maximum output power values, absolute limits shall be calculated at the maximum allowed output power given in Clause 2.1.1 and these absolute values shall be used for testing devices with lower output power values. It has to be noted that the spectrum mask is only applicable within 5 855 MHz to 5 925 MHz.

## RX requirements

### Receiver spurious emissions

Receiver spurious emissions are emissions at any frequency when the equipment is in receive mode. Table 7 tablulates the limits that the receiver shall not exceed.

Table 7: Spurious radiated emission limits

| Frequency range | Maximum power | Measurement bandwidth | Special requirement |
| --- | --- | --- | --- |
| 30 MHz f 1 GHz | -57 dBm (e.r.p.) | 100 kHz | n/a |
| 1 GHz f 5,795 GHz | -47 dBm (e.i.r.p.) | 1 MHz | n/a |
| 5,795 GHz f 5,815 GHz | -60 dBm (e.i.r.p.) | 1 MHz | Applicable for an intended antenna installation ≤ 2 meters above the ground level. |
|  | -65 dBm (e.i.r.p.) | 1 MHz | Applicable for an intended antenna installation > 2 meters above the ground level. |
| 5,815 GHz f 18 GHz | -47 dBm (e.i.r.p.) | 1 MHz | n/a |

Observation 2: RX requirements by ETSI have receiver spurious emissions down to -65 dBm between 5795MHz and 5815MHz.

Proposal 2: RX requirements for LTE-based ITS shall be able to limit the receiver spurious emission down to -65dBm between 5795MHz and 5815MHz.

### Receiver selectivity

#### Adjacent channel rejection

The adjacent channel rejection is a measure of the capability of the receiver to operate satisfactorily in the presence of a signal in the adjacent channel, which differs in frequency from the wanted signal by ± 10 MHz. [2]

The receiver adjacent channel rejection parameters under specified conditions shall be equal to or greater than the limits in Table 8.

#### Alternate channel rejection

The alternate channel rejection is a measure of the capability of the receiver to operate satisfactorily in the presence of a signal in the alternate adjacent channel, which differs in frequency from the wanted signal by ± 20 MHz.

The receiver alternate channel rejection parameters under specified conditions shall be equal to or greater than the limits in Table 8.

Table 8: Limits for receiver adjacent channel rejection and alternate adjacent channel rejection [2]

| Modulation | Coding rate | Adjacent channel rejection (dB) | Alternate adjacent channel rejection (dB) |
| --- | --- | --- | --- |
| BPSK | 1/2 | 16 | 32 |
| BPSK | 3/4 | 15 | 31 |
| QPSK | 1/2 | 13 | 29 |
| QPSK | 3/4 | 11 | 27 |
| 16-QAM | 1/2 | 8 | 24 |
| 16-QAM | 3/4 | 4 | 20 |
| 64-QAM | 2/3 | 0 | 16 |
| 64-QAM | 3/4 | -1 | 15 |

#### Blocking

Blocking is a measure of the capability of the receiver to operate satisfactorily in the presence of a signal in frequency band further away and it shall be tested at ± 50 MHz, ± 100 MHz, and ± 200 MHz. Blocking testing shall be performed at least at 6 different frequency offset positions. The manufacturer of the equipment can add additional frequency offsets positions.

The blocking level shall not be less than -30 dBm. [2]

### Receiver sensitivity

The receiver sensitivity is defined as the minimum receive signal level at the antenna connector required for a given packet error rate and modulation scheme (noise factor of 10 dB and 5 dB implementation margins are assumed).

The receiver sensitivity shall be less or equal to the values given in Table 9 for a packet error rate (PER) of 10-1 for 1 000 octet frames assuming stationary, non-fading channel conditions. [2]

Table 9: Receiver sensitivity [2]

| Modulation | Coding rate | Minimum sensitivity for 10 MHz channel spacing (dBm) |
| --- | --- | --- |
| BPSK | 1/2 | –85 |
| BPSK | 3/4 | –84 |
| QPSK | 1/2 | –82 |
| QPSK | 3/4 | –80 |
| 16-QAM | 1/2 | –77 |
| 16-QAM | 3/4 | –73 |
| 64-QAM | 2/3 | –69 |
| 64-QAM | 3/4 | –68 |

Proposal 3: The RX selectivity and sensitivity requirements of LTE-based ITS shall be superior to the RX selectivity and sensitivity requirements defined by the ETSI ITS HS draft EN 302 571 V2.0.3 (2016-03) for the same modulation and coding scheme.

# Conclusion

In this contribution, we present a summary of the regulatory requirements for the 5.9GHz ITS band in Europe defined in Draft ETSI EN 302 571 V2.0.3 (2016-03) [2], which is currently under approval and is scheduled to be made available publicly by the end of 2016

Observation 1: TX requirements by ETSI have unwanted emission limit down to -65 dBm between 5795MHz and 5815MHz.

Proposal 1: TX requirements for LTE-based ITS shall meet the unwanted emission requirement of -65dBm between 5795MHz and 5815MHz.

Observation 2: RX requirements by ETSI have receiver spurious emissions down to -65 dBm between 5795MHz and 5815MHz.

Proposal 2: RX requirements for LTE-based ITS shall be able to limit the receiver spurious emission down to -65dBm between 5795MHz and 5815MHz.

Proposal 3: The RX selectivity and sensitivity requirements of LTE-based ITS shall be superior to the RX selectivity and sensitivity requirements defined by the ETSI ITS HS draft EN 302 571 V2.0.3 (2016-03) for the same modulation and coding scheme.

# References

- RP-152293, “New WI proposal: Support for V2V services based on LTE sidelink”, LG Electronics, Huawei, HiSilicon, CATT, CATR

- Draft ETSI EN 302 571 V2.0.0 (2016-03): Intelligent Transport Systems (ITS); Radiocommunications equipment operating  in the 5 855 MHz to 5 925 MHz frequency band; Harmonised Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU, URL: https://docbox.etsi.org/ERM/ERMTG37/05-CONTRIBUTIONS/2016/ERMTG37(16)000038r2_NV__EN_302_571___HS_ITS_G5_.zip (last accessed 10.08.2016, ETSI membership required for the access)

- ECC Decision (08)01: "ECC Decision of 14 March 2008 on the harmonised use of the 587-5925 frequency band for Intelligent Transport Systems (ITS)", approved 14 March 2008 and amended 3 July 2015.

- ECC Recommendation (08)01:"Use of band 5855-5875 MHz for Intelligent Transport Systems (ITS)," approved 21 February 2008 and amended 3 July 2015.

- Commission Decision 2008/671/EC of 5 August on the harmonised use of radio spectrum in the 5875-5905 MHz frequency band for safety related application of Intelligent Transport Systems (ITS).