TSG-RAN Working Group 4 (Radio) meeting #80	R4-166194

Gothenburg, Sweden, 22nd – 26th August 2016

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	OTA EVM of AAS base station transmitters

Agenda item:	8.12.2.1 [AASenh_BS_LTE_UTRA]

Document for:	Discussion

## Introduction

Presently, the conducted EVM requirements of AAS base station transmitters are reused from the current requirements for EVM as specified in TS 36.104 and TS 25.104 for UTRA and E-UTRA, respectively. References [1],[2] and [3] studied spatial variations of EVM due to beamforming.

Non-ideal analogue devices used in transmitters introduce phase and amplitude errors that subsequently lead to deteriorated EVM. One major source of phase errors is phase noise of local oscillators. This document provides an insight into the effect of correlated phase-noise sources on OTA EVM. It is important to note that the amount of correlation among the phase-noise sources present in each base station transceiver is implementation specific.

## Discussion

In conducted testing of AAS transmitters, the EVM requirement is specified per TAB connector. As shown in Figure 1, the signal from one TAB connector is fed to a signal processing chain. The output signal of the per-subcarrier amplitude/phase correction block is used to calculate the EVM. Table 1 presents the minimum conducted EVM requirements as specified in TS 36.104. Unlike the conducted EVM, the OTA EVM exhibit spatial variations because beams are steered towards desired user devices. References [1], [2] and [3] showed the spatial distribution of EVM in both the azimuth and elevation directions. The conclusion of [3] is that OTA EVM requirements should be specified spatially according to declared beams; it also pointed out the possibility of reusing declared EIRP beams in specifying such EVM requirements. A question arises as to whether the declared EIRP beams correspond to good EVM in all test scenarios. There could exist test scenarios in which beams with the best EIRP but degraded EVM. Thus, the declared EIRP beams should be supplemented with, e.g., antenna-array radiation patterns.

While it is generally sufficient to consider the beamsteered signal in the main lobe for the EVM measurement, it is not known how the signal impairments (e.g., due to phase noise of local oscillators) affect the OTA EVM tested at such a center position. Let us consider an AAS base station with n transceiver units. The received signal at a given time instance is a superposition of the signals radiated by all antenna-array elements is [4]

s= k=1nakej(θk+φk)																								(1)

where θk = the phase shifts due to a carrier frequency

ak= the amplitude

φk = the phase noise

The error vector ∆s is obtained by subtracting the ideal signal from equation (1)

∆s= k=1nak ej(θk+φk)- k=1nak ejθk

= k=1nak ejθk(ejφk- 1)																				(2)

We can calculate its root mean square value as

EVMRMS2= E[|∆s|2]|sideal|2= var∆s+ |E[∆s]|2|sideal|2 																				(3)

where  var  =  variance

E[.] = the mean value

AAS Base Station under TX test

1

TRXU 1

2

OTA EVM measurement

TRXU 2

.

.

.

Antenna Array

Radio Distribution Network

.

.

.

TRXU NTRXU

NTABC

TRXU Array

CP Removal

FFT

Per-subcarrier Amplitude/Phase correction

Conducted EVM measurement

Symbol detection

Figure 1: Conducted and OTA AAS BS EVM measurement

| Modulation scheme for PDSCH | Required EVM [%] |
| --- | --- |
| QPSK | 17.5 % |
| 16QAM | 12.5 % |
| 64QAM | 8 % |
| 256QAM | 3.5 % |

Table 1: Minimum Conducted EVM requirements for E-UTRA carriers (Table 6.5.2.-1, TS 36.104)

In order to analyze the impact of phase noise on EVM, we consider two cases, namely correlated and uncorrelated phase noise.

Case 1: Correlated phase noise

In case the phase noise is fully correlated in the different transmitters, φk= φ for all k. Now, considering only the signal in the center of the main lobe, that is when the phases of all antenna-array elements align in a point of constructive interference, i.e., when θk=0 for all k, equation (2) simplifies to

∆scor= (ejφ- 1) k=1nak 																					(4)

Let x= ejφ- 1 and using equation (3), the RMS EVM value is given by (see [4] for a complete derivation)

EVMcor,RMS2= σx2+ |μx|2 																						(5)

where σx = standard deviation of x

μx = mean of x

Case 2: Uncorrelated phase noise

In case of uncorrelated phase noise, equation (4) becomes

∆suncor= k=1nak (ejφk-1) 																				(6)

Let xk= ejφk- 1 and using equation (3), the RMS EVM is

EVMuncor,RMS2= σx2σa2na2+ σx2n+ |μx|2 																		(7)

where σx = standard deviation of xk

μx = mean of xk

a  = the average of the amplitude coefficients ak

σa = standard deviation of ak

In the special case where all the amplitudes are the same, i.e., when ak=a for all k and thus σa=0, equation (7) reduces to

EVMuncor,RMS2= σx2n+ |μx|2 																					(8)

As the phase noise is relatively small, i.e., μx ≈ 0, and n ≫1, we can deduce from equations (5) and (8) that the correlated RMS EVM is greater than the uncorrelated one, i.e.,

EVMuncor,RMS2 ≤ EVMcor,RMS2 																					(9)

From equation (9), we can observe that a high correlation of phase noise sources will result in bad EVM. Therefore, the OTA requirement of AAS base station transmitters should capture the worst-case scenario; however, the present of phase noise in an AAS base station is implementation dependent. Thus, further studies are needed to determine if one EVM value is sufficient (as in the conducted requirements) to define the OTA EVM requirements or a range of EVM values because the AAS base station under test is viewed as a black box.

## Conclusions

This document has examined the impact of phase noise on OTA EVM of the user-specific beamforming. The OTA EVM of correlated phase-noise sources is in general worse than uncorrelated ones; however, the degree of phase-noise correlation depends on AAS base station implementation. Further research is needed on specifying OTA EVM requirements in order determine if one OTA EVM value is sufficient as in the conducted requirements.

## References

[1]  Ericsson, Considerations for an OTA EVM requirement, R4-162088, RAN4#78bis

[2]  Huawei, Discussion on EVM for REL14, R4-164290, RAN4#79

[3]  Ericsson, OTA EVM, R4-164161, RAN4#79

[4]  T. Höhne, and V. Ranki, Phase Noise in Beamforming, IEEE Transactions on Wireless Communications, vol. 9, no. 12, December 2010