3GPP TSG-RAN WG4 Meeting #80 	R4-166410

# Gothenburg, Sweden, 22-26 August, 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 3933 | rev |  | Current version: | 14.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Intra-frequency absolute and relative CSI-RSRP accuracies for SCell with FS3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_LAA-Perf |  |  |  |  |  |  | Date: |  | 2016-08-12 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Test cases for LAA are introduced in LTE_LAA-Perf work item. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test cases for Intra-frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with PCell in FDD/TDD and SCells in FS3 are introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CSI-RSRP accuracy test cases for LAA will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New sections A.9.1.57 and A.9.1.58 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  | TS3 36.521-3 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*Start of change*

### A.9.1.57	FS3 absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal with FDD PCell

#### A.9.1.57.1	Test Purpose and Environment

The purpose of this test is to verify that CSI- RSRP measurement accuracy is within the specified limits. This test will verify the absolute intra-frequency CSI-RSRP accuracy requirements of the SCells defined in Section 9.1.18.4.4 for intra-frequency measurements under FS3, and the relative intra-frequency CSI-RSRP accuracy requirements between SCells defined in Section 9.1.18.4.5.

#### A.9.1.57.2	Test parameters

In this set of cases Cell 1 is PCell on the primary component carrier, Cell 2 using FS3 is SCell on the secondary component carrier and activated, and Cell 3 using FS3 is the neighbouring cell on the secondary component carrier. The test parameters are given in Table A.9.1.57.2-1. Intra-frequency measurements are supported by a DMTC configuration.

A.9.1.57.2-1: CSI-RSRP carrier aggregation test parameters with FDD PCell and FS3 SCells

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell3 |
| Frame structure |  |  | FDD | FS3 | FS3 |
| E-UTRA RF Channel Number |  |  | 1 | 2 | 2 |
| BWchannel |  | MHz | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 | 20 MHz: NRB,c = 100 | 20 MHz: NRB,c = 100 |
| Timing offset to Cell 1 |  | s | - | 0 | 3 |
| Uplink/downlink configuration |  |  | - | Note10 | Note10 |
| Time alignment error between cell 2 and cell 1 |  |  | - | Time alignment error as specified in 3GPP TS 36.104 [30] clause 6.5.3.1. | - |
| DMTC period |  | ms | - | 40 | 40 |
| DMTC period offset |  | ms | - | 10 | 10 |
| Discovery signal occasion duration |  | ms | - | 1 | 1 |
| CSI-RS resource configuration |  |  | - | 4 | 6 |
| CSI-RS periodicity |  | ms | - | 10 | 10 |
| CSI-RS subframe offset |  | ms | - | 0 | 0 |
| CSI-RS individual offset[2] |  | dB | - | 0 | 0 |
| CSI-RS muting |  |  | - | Enable | Enable |
| LBT model |  |  | - | - | A.3.17 |
| Measurement bandwidth |  |  | 5 MHz: 10—15 10 MHz: 22—27 20 MHz: 47—52 | 47—52 | 47—52 |
| PDSCH Reference measurement channel defined in A.3.1.1.1 (R.6 FDD) and A.3.1.1.6 (R.0 FS3) |  |  | 5 MHz: R.5 FDD 10 MHz: R.0 FDD 20 MHz: R.4 FDD | R.0 FS3 | R.0 FS3 |
| PDSCH allocation |  |  | 5 MHz: 7-17 10 MHz: 13-36 20 MHz: 38-61 | 38—61 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 (R.6 FDD, R.10 FDD, R.11 FDD) and A.3.1.2.3 (R.0 FS3) |  |  | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD | R.0 FS3 | R.0 FS3 |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD), A.3.2.1.15 (OP.15 FDD), A.3.2.1.11 (OP.11 FDD), A.3.2.2.7 (OP.7 TDD) and A.3.2.2.8 (OP.8 TDD) |  |  | 5 MHz: OP.15 FDD 10 MHz: OP.1 FDD 20 MHz: OP.11 FDD | OP.7 TDD | OP.8 TDD |
| PBCH_RA |  | dB | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |
| OCNG_RBNote |  |  |  |  |  |
| p-C-r10[2] |  | dB | 6 | 6 | 6 |
| Note2 | Bands FDD_A | dBm/15 kHz | -117 | - | - |
|  | Bands FDD_B |  | -116.5 |  |  |
|  | Bands FDD_C |  | -116 |  |  |
|  | Bands FDD_D |  | -115.5 |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  | -115 |  |  |
|  | Bands FDD_G |  | -114 |  |  |
|  | Bands FDD_H |  | -113.5 |  |  |
|  | Bands FS3_G |  | - | (for Channel 1 + [1] dB) | (for Channel 1 + [1] dB) |
| CRS |  | dB | [-4] | [0.46] | [-5.76] |
| CSI-RS |  | dB | - | [6.46] | [0.24] |
| RSRPNote3 | Bands FDD_A | dBm/15 kHz | -121 | - | - |
|  | Bands FDD_B |  | -120.5 |  |  |
|  | Bands FDD_C |  | -120 |  |  |
|  | Bands FDD_D |  | -119.5 |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  | -119 |  |  |
|  | Bands FDD_G |  | -118 |  |  |
|  | Bands FDD_H |  | -117.5 |  |  |
|  | Bands FS3_G |  | - | RSRP for Cell 1 + [8] dB) | RSRP for Cell 1 + [4] dB) |
| CSI-RSRPNote3 | Bands FDD_A | dBm/15 kHz | -115 | - | - |
|  | Bands FDD_B |  | -114.5 |  |  |
|  | Bands FDD_C |  | -114 |  |  |
|  | Bands FDD_D |  | -113.5 |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  | -113 |  |  |
|  | Bands FDD_G |  | -112 |  |  |
|  | Bands FDD_H |  | -111.5 |  |  |
|  | Bands FS3_G |  | - | CSI-RSRP for Cell 1 + [8] dB) | CSI-RSRP for Cell 1 + [4] dB) |
| IoNote3 | Bands FDD_A | dBm/ BWchannel | -87.76+10log(NRB,c/50) | - | - |
|  | Bands FDD_B |  | -87.26+10log(NRB,c/50) |  |  |
|  | Bands FDD_C |  | -86.76+10log(NRB,c/50) |  |  |
|  | Bands FDD_D |  | -86.26+10log(NRB,c/50) |  |  |
|  | Bands FDD_E, FDD_F Note 6 |  | -85.76 +10log(NRB,c/50) |  |  |
|  | Bands FDD_G |  | -84.76 +10log(NRB,c/50) |  |  |
|  | Bands FDD_H |  | -84.26 +10log(NRB,c/50) |  |  |
|  | Bands FS3_G |  | - | (Io for Channel 1 + [5.33] dB +10log (NRB channel2 / NRB channel 1)) | (Io for Channel 1 + [5.33] dB +10log (NRB channel2 / NRB channel 1)) |
| CRS |  | dB | [-4] | [3] | [-1] |
| CSI-RS |  | dB | - | [9] | [5] |
| Propagation condition |  | - | AWGN | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Io levels are calculated in CRS symbols of measurement subframe. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 6: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 7: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. Note 8: E-UTRA operating band groups are as defined in Section 3.5. Note9: DMTC configurations are provided to the UE in the measDS-Config (in TS36.331) before the beginning of the test. Note10: Downlink only configuration |  |  |  |  |  |

#### A.9.1.57.3	Test Requirements

In the test, the performance of CSI-RSRP measurements is verified from following four perspectives:

-	The absolute accuracy of intra-frequency CSI-RSRP measurements for Cell 2 on the secondary component carrier shall fulfil the requirements defined in clause 9.1.18.4.4.

-	The absolute accuracy of intra-frequency CSI-RSRP measurements for Cell 3 on the secondary component carrier shall fulfil the requirements defined in clause 9.1.18.4.4.

-	The relative accuracy of intra-frequency CSI-RSRP measurements for Cell 3 relative to Cell 2 on the secondary component carrier shall fulfil the requirements defined in clause 9.1.18.4.5.

*End of change*

*Start of change*

### A.9.1.58	FS3 absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal with TDD PCell

#### A.9.1.58.1	Test Purpose and Environment

The purpose of this test is to verify that CSI- RSRP measurement accuracy is within the specified limits. This test will verify the absolute intra-frequency CSI-RSRP accuracy requirements of the SCells defined in Section 9.1.18.4.4 for intra-frequency measurements under FS3, and the relative intra-frequency CSI-RSRP accuracy requirements between SCells defined in Section 9.1.18.4.5.

#### A.9.1.58.2	Test parameters

In this set of cases Cell 1 is PCell on the primary component carrier, Cell 2 using FS3 is SCell on the secondary component carrier and activated, and Cell 3 using FS3 is the neighboring cell on the secondary component carrier. The test parameters are given in Table A.9.1.58.2-1. The intra-frequency measurements are supported by a DMTC configuration.

A.9.1.58.2-1: CSI-RSRP carrier aggregation test parameters with TDD PCell and FS3 SCells

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell3 |
| Frame structure |  |  | TDD | FS3 | FS3 |
| E-UTRA RF Channel Number |  |  | 1 | 2 | 2 |
| BWchannel |  | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 20 MHz: NRB,c = 100 | 20 MHz: NRB,c = 100 |
| Timing offset to cell1 |  | s | - | 0 | 3 |
| Special subframe configurationNote1 |  |  | 6 | Note10 | Note10 |
| Uplink/downlink configurationNote1 |  |  | 1 | Note10 | Note10 |
| Time alignment error between cell 2 and cell 1 |  |  | - | Time alignment error as specified in 3GPP TS 36.104 [30] clause 6.5.3.1. | - |
| DMTC period |  | ms | - | 40 | 40 |
| DMTC period offset |  | ms | - | 10 | 10 |
| Discovery signal occasion duration |  | ms | - | 1 | 1 |
| CSI-RS resource configuration |  |  | - | 4 | 6 |
| CSI-RS periodicity |  | ms | - | 10 | 10 |
| CSI-RS subframe offset |  | ms | - | 0 | 0 |
| CSI-RS individual offset[2] |  | dB | - | 0 | 0 |
| CSI-RS muting |  |  | - | Enable | Enable |
| LBT model |  |  | - | - | A.3.17 |
| Measurement bandwidth |  |  | 5 MHz: 10—15 10 MHz: 22—27 20 MHz: 47—52 | 47—52 | 47—52 |
| PDSCH Reference measurement channel defined in A.3.1.1.2 (R.3 TDD) and A.3.1.1.6 (R.0 FS3) |  |  | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD | R.0 FS3 | R.0 FS3 |
| PDSCH allocation |  |  | 5 MHz: 7-17 10 MHz: 13-36 20 MHz: 38-61 | 38—61 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 (R.6 TDD, R.10 TDD, R.11 TDD) and A.3.1.2.3 (R.0 FS3) |  |  | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD | R.0 FS3 | R.0 FS3 |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) , A.3.2.2.9 (OP.9 TDD), A.3.2.2.7 (OP.7 TDD) and A.3.2.2.8 (OP.8 TDD) |  |  | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD | OP.7 TDD | OP.8 TDD |
| PBCH_RA |  | dB | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |
| OCNG_RBNote |  |  |  |  |  |
| p-C-r10[2] |  | dB | 6 | 6 | 6 |
| Note2 | Bands TDD_A | dBm/15 kHz | -117 | - | - |
|  | Bands TDD_C |  | -116 |  |  |
|  | Bands TDD_E |  | -115 |  |  |
|  | Bands FS3_G |  | - | (for Channel 1 + [1] dB) | (for Channel 1 + [1] dB) |
| CRS |  | dB | [-4] | [0.46] | [-5.76] |
| CSI-RS |  | dB | - | [6.46] | [0.24] |
| RSRPNote3 | Bands TDD_A | dBm/15 kHz | -121 | - | - |
|  | Bands TDD_C |  | -120 |  |  |
|  | Bands TDD_E |  | -119 |  |  |
|  | Bands FS3_G |  | - | RSRP for Cell 1 + [8] dB) | RSRP for Cell 1 + [4] dB) |
| CSI-RSRPNote3 | Bands TDD_A | dBm/15 kHz | -115 | - | - |
|  | Bands TDD_C |  | -114 |  |  |
|  | Bands TDD_E |  | -113 |  |  |
|  | Bands FS3_G |  | - | CSI-RSRP for Cell 1 + [8] dB) | CSI-RSRP for Cell 1 + [4] dB) |
| IoNote3 | Bands TDD_A | dBm/ BWchannel | -87.76+10log(NRB,c/50) | - | - |
|  | Bands TDD_C |  | -86.76+10log(NRB,c/50) |  |  |
|  | Bands TDD_E |  | -85.76+10log(NRB,c/50) |  |  |
|  | Bands FS3_G |  | - | (Io for Channel 1 + [5.33] dB +10log (NRB channel2 / NRB channel 1)) | (Io for Channel 1 + [5.33] dB +10log (NRB channel2 / NRB channel 1)) |
| CRS |  | dB | [-4] | [3] | [-1] |
| CSI-RS |  | dB | - | [9] | [5] |
| Propagation condition |  | - | AWGN | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Io levels are calculated in CRS symbols of measurement subframe. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The selection of the bands for testing depends on the configuration of the carrier aggregation supported by the UEs. Note 6: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 7: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. Note 8: E-UTRA operating band groups are as defined in Section 3.5. Note9: DMTC configurations are provided to the UE in the measDS-Config (in TS36.331) before the beginning of the test. Note10: Downlink only configuration |  |  |  |  |  |

#### A.9.1.58.3	Test Requirements

In the test, the performance of CSI-RSRP measurements is verified from following four perspectives:

-	The absolute accuracy of intra-frequency CSI-RSRP measurements for Cell 2 on the secondary component carrier shall fulfil the requirements defined in clause 9.1.18.4.4.

-    The absolute accuracy of intra-frequency CSI-RSRP measurements for Cell 2 on the secondary component carrier shall fulfil the requirements defined in clause 9.1.18.4.4.

-	The relative accuracy of intra-frequency CSI-RSRP measurements for Cell 3 relative to Cell 2 on the secondary component carrier shall fulfil the requirements defined in clause 9.1.18.4.5.

*End of change*