3GPP TSG-RAN WG4 Meeting #80	R4-166612

# Gothenburg, Sweden, 22 - 26 Aug, 2016

Source:	Nokia

Title:	PUSCH ETU600 simulation results with IM

Agenda item:	8.10.4

Document for:	Discussion

### 1	Introduction

This contribution provides simulation results with impairment margin (IM) for PUSCH ETU600 under the Rel-14 high speed WI [1].  This contribution follows the simulation assumptions of [2].

### 2	FRC table for 16QAM rate 1/2

The FRC table shall base based on the new 16QAM rate ½ defined for BS MMSE-IRC in [3].  The FRC table is listed below.

Table A.13-1 FRC parameters for performance requirements (16QAM 1/2)

| Reference channel | A13-1 | A13-2 | A13-3 | A13-4 | A13-5 | A13-6 |
| --- | --- | --- | --- | --- | --- | --- |
| Allocated resource blocks | 6 | 15 | 25 | 50 | 75 | 100 |
| DFT-OFDM Symbols per subframe | 12 | 12 | 12 | 12 | 12 | 12 |
| Modulation | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM |
| Code rate | 0.51 | 0. 50 | 0.50 | 0.49 | 0.50 | 0.49 |
| MCS index | 15 | 15 | 15 | 15 | 15 | 15 |
| Payload size (bits) | 1736 | 4264 | 7224 | 14112 | 21384 | 28336 |
| Transport block CRC (bits) | 24 | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 0 | 0 | 24 | 24 | 24 | 24 |
| Number of code blocks - C | 1 | 1 | 2 | 3 | 4 | 5 |
| Coded block size including 12bits trellis termination (bits) | 5292 | 12876 | 10956 | 14220 | 16140 | 17100 |
| Total number of bits per sub-frame | 3456 | 8640 | 14400 | 28800 | 43200 | 57600 |
| Total symbols per sub-frame | 864 | 2160 | 3600 | 7200 | 10800 | 14400 |

### 3	ETU600 simulation results

The simulation shall cover all BW: 1.4, 3, 5, 10, 15, 20 MHz.  The simulation results with IM are shown in Table 1.

Table 1    PUSCH ETU600 Simulaiton results with IM

| Bandwidth (MHz) | (Tx, Rx) configuration | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- |
| 1.4 | 1x4 | ETU 600Hz Low | A13-1 | 30% | -1.8 |
|  |  |  |  | 70% | 5.8 |
| 3 | 1x4 | ETU 600Hz Low | A13-2 | 30% | -2.1 |
|  |  |  |  | 70% | 5.5 |
| 5 | 1x4 | ETU 600Hz Low | A13-3 | 30% | -2.0 |
|  |  |  |  | 70% | 6.0 |
| 10 | 1x4 | ETU 600Hz Low | A13-4 | 30% | -2.1 |
|  |  |  |  | 70% | 5.7 |
| 15 | 1x4 | ETU 600Hz Low | A13-5 | 30% | -2.0 |
|  |  |  |  | 70% | 5.9 |
| 20 | 1x4 | ETU 600Hz Low | A13-6 | 30% | -2.1 |
|  |  |  |  | 70% | 5.8 |

# Reference

[1]	RP-152263	“New WI proposal: Performance enhancements for high speed scenario”, NTT DOCOMO, Huawei, HiSilicon, Dec 2015

[2]	R4-161255	“Way forward on high speed BS demodulation assumptions”, Nokia Networks, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, RAN4 #78

[3]	R4-162794	“36.104 CR for BS MMSE-IRC receiver - FRC definitions”, China Telecom, Huawei, ZTE, April 2016.