3GPP TSG-RAN WG4 Meeting #80bis 	R4-167338

Ljubliana, Slovenia, 10 – 14 October, 2016

Source:	SoftBank Corp.

Title:	Consideration on RB restriction/A-MPR schemes

Agenda item:	9.2.2

Document for:	Approval

1.  Introduction

The study item of global application of LTE Band 11and Band 21 UEs was approved in RAN#72[1]. In the last meeting, there was discussion on how the actual operating conditions are determined. This paper is intended for clarifying some prerequisites for this study and attempting a preliminary study on RB restriction/A-MPR based on the prerequisites.

2.  On ITU-R requirement

As a requirement to a UE operating in Band 11/21 region, ITU-R document [2] defines in Table 1-1 that “−62 dBW in the 27 MHz of the EESS (passive) band for IMT mobile stations” and relevant footnote 3 explains “The unwanted emission power level is to be understood here as the level measured with the mobile station transmitting at an average output power of 15dBm”.

Our understanding is then ITU-R’s requirement says that a single UE (“the mobile station” in footnote 3) should satisfy -62dBW/27MHz at average output power of 15dBm. Then the simplest testing scheme is that a UE is tested for unwanted emission in EESS region at fixed output power of 15dBm (apparently this satisfies 15dBm in average).

[Proposal-1] A UE is assumed to be tested for unwanted emission in EESS region at fixed output power of 15dBm.

Then any schemes (RB restriction, A-MPR and etc.) to be employed for B11/B21 globalization should be designed to guarantee this requirement in the first place.

[Proposal-2] Any schemes (RB restriction, A-MPR and etc.) to be employed for B11/B21 globalization should be designed to guarantee the requirement (-62dBW/27MHz at 15dBm) in the first place.

3.  Operations other than ITU-R requirements

The requirement does not mention anything about UE’s transmitting power above or below 15dBm. There were opinions in the last meeting that we should/could be based on average of a number of UEs as in an ITU-R document dealing with interference from terrestrial to satellite interference.  Our view is that it is not likely because (a) such co-ex study has already done in ITU-R to draw the conclusion above and (b) the agreed value might include some “compromise” which could not always be traced or justified from a technical standpoint.

It is also our understanding that, based on co-ex over average of many UEs, variation of UE power, such as up to 23dBm or also smaller than 15dBm, has already been prerequisite of this requirement.

[Proposal-3] It should be assumed that a UE can take any power up to 23dBm in actual operation.

On the other hand, it is quite difficult to derive/estimate how RB allocation is permitted or the allocation can be justified in such wide range of Tx power from the requirement of the single point. A simple way is to extend the limitation at 15dBm, i.e. whatever the Tx power would be, using the RBs which are permitted under protection requirements at 15dBm and prohibiting the rest.

In real operation of LTE systems where frequency reuse = 1 is employed and uplink transmission power control is motivated to take the balance between cell throughput and inter-cell interference, it is true in general that there is quite a limited part of UEs transmitted at higher power. This could justify the proposal above.

[Proposal-4] It is proposed, as a baseline, to apply limitations at 15dBm (RB position, length) to any Tx power.

In addition, there might be some room for prohibited region (in section 4) to be used with appropriate back-off. This is not suited for RB restriction but could be for A-MPR to be discussed in section 5.

[Proposal-5] The handling of prohibited regions is left for further discussion.

4.  Evaluation from the simulation results – RB restriction

In the following sections (section 4 and 5), as per initial simulation results, the lowest 10MHz of Band 11 (1427.9 – 1437.9MHz) is considered.  This study is based on initial simulation results presented in the last meeting [3, 4].

Firstly, to derive backoff at 15dBm from 23dBm max power simulations, this paper assumes that the region where more than 8dB (23dBm – 15dBm) A-MPR is required should be excluded from operation for EESS protection.  (Also note that, considering MPR for QPSK, portions of more than 7dB should be excluded for LCRB>12 in 10MHz BW). As mentioned in the previous section, it is also assumed that permitted RB allocations at 15dBm can be used in any output power up to 23dBm.

Two results show similar tendencies in terms of >8dB (or 7dB) backoff region, i.e.,

- Up to 8RBs, no restriction is required.

- The border of the region is almost linearly extended from (RBSTART, LCRB) = (0, 9) toward (20, 30), via (4,12),

based on the stringent result [4]. This trend is depicted in figure 1. The white-colored region is “permitted” in any power while yellow-colored region means “prohibited” due to 15dBm results.

Figure 1  Permitted and prohibited regions based on 15dBm Tx

From these observations, it can be concluded that in this 10MHz operation:

- up to 8RBs can be used without restriction.

This leads to the following outcomes:

- no PUCCH overprovisioning is required, and

- eMTC (6RB) or in-band NB-IoT (1RB) could also be accommodated without restriction.

[Observation-1] Up to 8RB can be used without restriction.

[Observation-2] eMTC or in-band NB-IoT can be used without restriction.

RB restriction could not control Tx power. Then operation is limited to the white colored region in figure 1 and in principle this region can be used without any restriction as long as scheduler in an eNB can support. However, the degree of freedom in RB allocation is a trade off with the complexity of implementation or testability. Practically speaking, considering 5-6 RBs on both ends are occupied by PUCCH, it is likely to introduce a combination like (RBSTART≧ 8, LCRB≦16) or  (RBSTART≧12, LCRB≦20) in addition to LCRB≦8 for any RBSTART. An example is shown in figure. 2 with possible schemes indicated in green.

Figure 2   Possible RB restriction scheme

5.  Consideration on A-MPR

In the last meeting, application of NS signaling/A-MPR was also discussed [5]. As proposed in the sections above, this A-MPR should firstly exclude the prohibited RBs at 15dBm and this requires to put at least 9dB (or 8dB for larger RBs) backoff for the yellow-colored region in figure 1. As it seems that current A-MPR does not consider to indicate the prohibition of transmission (63[dB] = max - min?), probably we should put appropriate values for A-MPR.

Based on the Figure 1, a possible A-MPR scheme is shown in Figure 3 and Table 1. (Note that backoff regions are not always precise because it is not easy to read the backoff values from color gradations: so the A-MPR here is not a precise reflection of simulation result in [4] and mainly for illustrative purpose.)

Figure 3   Example of A-MPR scheme

Table 1   Example of A-MPR table

| Parameters | Region A |  | Region B |  | Region C |  |
| --- | --- | --- | --- | --- | --- | --- |
| RBstart | 0 - 4 |  | 5 – [11] |  | [12] – 20 |  |
| LCRB [RBs] | 9-15 | ≥16 | 12-[16] | ≥ [18] | [18]-[24] | ≥ [25] |
| A-MPR [dB] | ≤ 13 | ≤ 15 | ≤ 9 | ≤ 13 | ≤ 9 | ≤ 13 |

Apparently, A-MPR’s advantage is to be able to control Tx power based on RB position so its benefit is pronounced especially when prohibited region could be assigned to a UE. Even if the allocation is restricted in the permitted area, this could be more flexible than RB restriction, but at the cost of incompatibility with legacy UEs which do not understand this new NS/A-MPR.

Not to mention, we need to conclude on Proposal-5 in advance, whether or not the region can be used.

[Observation - 3] A-MPR could offer flexible resource usage but this is at the cost of lack of backward compatibility in UEs.

This document does not use the new A-MPR scheme proposed in [5]. The document [5] seems to assume that the protection requirements should also be somewhat extended for Tx power other than 15dBm but this contribution is based on the Proposal-4 above and then could follow conventional way, i.e. selecting RBSTART and then LCRB to give appropriate backoff. It seems we can get along with the traditional A-MPR scheme when the prohibited region is to be used.

6.  Conclusion

This paper proposes some prerequisites essential for this study and runs preliminary evaluations based the prerequisites and initial simulation results. Proposals are:

[Proposal-1] A UE is assumed to be tested for unwanted emission in EESS region at fixed output power of 15dBm.

[Proposal-2] Any schemes (RB restriction, A-MPR and etc.) to be employed for B11/B21 globalization should be designed to guarantee the requirement (-62dBw/27MHz at 15dBm) in the first place.

[Proposal-3] It should be assumed that a UE can take any power up to 23dBm in actual operation.

[Proposal-4] It is proposed, as a baseline, to apply limitations at 15dBm (RB position, length) to any Tx power.

[Proposal-5] The handling of prohibited regions is left for further discussion.

7.  Reference

[1]      RP-161220    New Study Item proposal: Feasibility study on global application of LTE Band 11 and of LTE Band 21 UEs, SoftBank Corp.

[2]     “Final Acts WRC-15”, WORLD RADIOCOMMUNICATION CONFERENCE (GENEVA, 2015)

[3]       R4-166173,  Initial simulation results for the feasibility study on global application of LTE Band 11 and of LTE Band 21 UEs, Nokia

[4]       R4-166324, Discussion on Band 11 UE emission to protect EESS, Huawei, HiSilicon

[5]       R4-165236, A solution for maximum use of the 1.5GHz band, NTT DOCOMO INC

form change history:

2013-12-06 v1.14.1 modified §11 to read: <FamilyName>, <GivenName>, (If the person is new to 3GPP work, give full contact coordinates, in particular, email address.)

2013-10-03 v1.14.0 removal of embedded help text

v1.13.2: adds tdoc header

v1.13.1: minor changes resulting from discussions at CT#41 & SA#41

v1.13.0: mods to enforce linkage amongst stages 1, 2, 3

draft mods Scarrone-Meredith 2008-07 ff

v1.12.1: removes revision marks following approval at SP-29
v1.12.0: includes provision for Study Items (SP-29)

v1.11.0: includes those changes from v1.8.0 agreed at SP-25.

v1.10.0: full circle

v1.9.0: a clean sheet

v1.8.0: includes comments from SA#24

v1.7.0: includes comments from RAN, CN and T #24; also includes “early implementation” data

v1.6.0: includes comments made during review period prior to TSGs#24

v1.5.0: includes comments made at TSGs#23 (Phoenix)

v1.4.0: offered to SA#23 for approval

v1.3.0: offered to CN#23, RAN#23 and T#23 for comments

DRAFT4 v1.3.0: 2004-03-09: Incorporation of comments from Leaders list

DRAFT3 v1.3.0: 2004-02-19: Incorporation of comments from MCC members

DRAFT2 v1.3.0: 2004-01-29: Complete redraft:

v1.2.0: 2002-07-04: "USIM" box changed to "UICC apps"

2003-05-28: spelling of “rapporteur” corrected

2002-07-04: "USIM" box changed to "UICC apps"