3GPP TSG-RAN WG4 Meeting #80bis 	R4-167339

Ljubliana, Slovenia, 10 – 14 October, 2016

Source:	SoftBank Corp.

Title:	TP for TR36.714-02-01: the support of CA_3A-11A BCS0

Agenda item:	8.2.2

Document for:	Approval

1.  Introduction

B3+B11 was approved in 2DL/1UL basket WID in the last RAN-P[1]. This paper is to evaluate IM/harmonics, relaxation required to provide TP for the completion of this combination.

2. Basic Configurations

As written in the WID, the following configurations are assumed for B3+B11.

Table 1: Inter-band CA Frequencies

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) band |  |  |  | Downlink (DL) band |  |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | Channel BW (MHz) | BS transmit / UE receive |  |  | Channel BW (MHz) |  |
|  |  | FUL_low – FUL_high |  |  |  | FDL_low – FDL_high |  |  |  |  |
| CA_3-11 | 3 | 1710 MHz | – | 1785 MHz | 5, 10, 15, 20 | 1805 MHz | – | 1880 MHz | 5, 10, 15, 20 | FDD |
|  | 11 | 1427.9 MHz | – | 1447.9 MHz | 5, 10 | 1475.9 MHz | – | 1495.9 MHz | 5, 10 |  |

Table 2: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| CA operating / channel bandwidth |  |  |  |  |  |  |  | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |
| CA_3A-11A | 3 |  |  | Yes | Yes | Yes | Yes | 0 |
|  | 11 |  |  | Yes | Yes |  |  |  |

3.  Co-existence

The harmonic relations of Band 3 and Band 11 are calculated as Table 3.  It can be concluded that no harmful fall down toward DL bands of interest is observed.

Table 3: Impact of UL Harmonic Interference

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 3 | 1710 | 1785 | 1805 | 1880 | 3420 | 3570 | 5130 | 5355 | 3610 | 3760 | 5415 | 5640 |
| 11 | 1427.9 | 1447.9 | 1475.9 | 1495.9 | 2855.8 | 2895.8 | 4283.7 | 4343.7 | 2951.8 | 2991.8 | 4427.7 | 4487.7 |

Table 4: Band 3 and Band 11 DL harmonics and IMD products

| BS DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 1475.9 | 1495.9 | 1805 | 1880 |
| 2nd order harmonics frequency range (MHz) | 2951.8 | 2991.8 | 3610 | 3760 |
| 3rd order harmonics frequency range (MHz) | 4427.7 | 4487.7 | 5415 | 5640 |
| 2nd order IMD products | ¦f2_low – f1_high¦ | ¦f2_high – f1_low¦ | ¦f2_low + f1_low¦ | ¦f2_high + f1_high¦ |
| IMD frequency limits (MHz) | 309.1 | 404.1 | 3280.9 | 3375.9 |
| 3rd order IMD products | ¦f2_high – 2*f1_low¦ | ¦f2_low – 2*f1_high¦ | ¦2*f2_low – f1_high¦ | ¦2*f2_high – f1_low¦ |
| IMD frequency limits (MHz) | 1071.8 | 1186.8 | 2114.1 | 2284.1 |
| 3rd order IMD products | ¦2*f1_low + f2_low¦ | ¦2*f1_high + f2_high¦ | ¦2*f2_low + f1_low¦ | ¦2*f2_high + f1_high¦ |
| IMD frequency limits (MHz) | 4756.8 | 4871.8 | 5085.9 | 5255.9 |
| 3rd order IMD products | ¦f1_low – f2_high + f2_low¦ | ¦f1_high + f2_high – f2_low¦ | ¦f2_low – f1_high + f1_low¦ | ¦f2_high + f1_high – f1_low¦ |
| IMD frequency limits (MHz) | 1400.9 | 1570.9 | 1785 | 1900 |
| 3rd order IMD products (with maximum channel bandwidth) | (f1_low – f2_BWmax) | (f1_high + f2_BWmax) | (f2_low – f1_BWmax) | (f2_high + f1_BWmax) |
| IMD frequency limits (MHz) | 1455.9 | 1515.9 | 1795 | 1890 |

It can be seen from Table 4 that:

- the 2nd harmonics of BS transmitting in Bands 3 and 11 may fall into the BS receive band of Band 43,

- the 3rd harmonics may fall into Band 46

- the 2nd IMD products may fall into none of 3GPP bands

- the 3rd IMD products may fall into the BS receive band of Bands 2, 11, 21, 25, 33, 35. 39, 45 and 46

assuming (except the last row) the whole 75MHz DL frequency of Band 3 and the whole 20 MHz DL frequency of Band 11.

If the BS is only transmitting up to 20 MHz DL in Band 3 and 10MHz DL in Band 11 as stated in the WID, then the 3rd IMD products will not fall into the BS receive band of Bands 11 and 33 as shown in the last row in table 4.

It should be noted that Bands 2, 25, 35, 39, 43, 45 and 46 are not intended for use in the same geographical area as Bands 3 and 11. In addition, based on the current spectrum allocation in Japan, Band 11 and Band 21 are not expected to be operated in the same BS. Therefore, there is no harmful fall down expected in this combination in BS side also.

4.  ΔTIB and ΔRIB

Up to REL-13, mid band (Band 11 and 21) was assumed to be stand-alone or use extended High-Low diplexer where the high band branch also covers 1.5GHz bands with additional 0.1dB relaxation inΔTIB in B11/B21 as shown in Figure 1.

In REL-14, there is a completed combination with almost identical frequency arrangement: Band 3 + Band 21 conducted by NTT DOCOMO. The combination however assumed a new RF architecture, i.e. Band 3 RF bundled with Band 1 (using B1+B3 quadplexer) under common triplexer [2]. The new proposal above requires the low portion of the common triplexer to cover 1.5GHz assuming that B3 filter could not be modified further. This is depicted in Figure 2.

Figure 1: Diplexer assumption for mid-band up to REL-13

Figure 2: Modified common triplexer for CA_3-21

While this seems to worsen insertion loss in general, the new architecture looks well-aligned with a current trend where Bands around 2GHz (Band 1-4) are supported in an integrated module. Thus, this study follows the way B3+B21 was defined.

Based on [2], the following relaxations are proposed which are identical to those for CA_3-21:

Table 5: Proposed ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_3-11 | 3 | 0.8 |
|  | 11 | 0.9 |

Table 6: Proposed ΔRIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_3-11 | 3 | 0.3 |
|  | 11 | 0.5 |

In addition, desense between B3 and B21 due to the new filter configuration was concluded that there is no issue [3]. Then it is proposed not to introduce REFSENS exception or MSD to this combination also.

[Proposal-1] The same relaxation values of B3+B21 are applied to this combination.

5.  Summary

This paper investigates all the necessary elements to complete B3+B11 based on B3+B21 proposal. The relevant TP is attached in the Annex.

6.  Reference

[1]      RP-161481   LTE Advanced inter-band CA Rel-14 for 2DL/1UL, Qualcomm (rapporteur).

[2]      R4-162925    TP for TR 36.714-02-01: LTE Advanced Carrier Aggregation of Band Combination (3+21: 2DL/1UL), NTT DOCOMO,

[3]      R4-163450	    TP for TR 36.714-02-01: 2DL/1UL CA_3A-21A, NTT DOCOMO

Annex: TP for TR36.714-02-01 (v0.3.0)

[Unchanged portions skipped]

6.X	CA_3A-11A_BCS0

6.X.1	Operating bands for CA

Table 6.X.1-1: Inter-band CA

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) band |  |  |  | Downlink (DL) band |  |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | Channel BW (MHz) | BS transmit / UE receive |  |  | Channel BW (MHz) |  |
|  |  | FUL_low – FUL_high |  |  |  | FDL_low – FDL_high |  |  |  |  |
| CA_3-11 | 3 | 1710 MHz | – | 1785 MHz | 5, 10, 15, 20 | 1805 MHz | – | 1880 MHz | 5, 10, 15, 20 | FDD |
|  | 11 | 1427.9 MHz | – | 1447.9 MHz | 5, 10 | 1475.9 MHz | – | 1495.9 MHz | 5, 10 |  |

6.X.2	Channel bandwidths per operating band for CA

Table 6.X.2-1: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| CA operating / channel bandwidth |  |  |  |  |  |  |  | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |
| CA_3A-11A | 3 |  |  | Yes | Yes | Yes | Yes | 0 |
|  | 11 |  |  | Yes | Yes |  |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in table 5.6.1-1 of TS 36.101 [6] when operating in single carrier mode.

6.X.3	Co-existence studies

Based on table 6.X.3-1, it is concluded that there is no issue on harmonic interference for CA_3A-11A.

Table 6.X.3-1: Impact of UL Harmonic Interference

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 3 | 1710 | 1785 | 1805 | 1880 | 3420 | 3570 | 5130 | 5355 | 3610 | 3760 | 5415 | 5640 |
| 11 | 1427.9 | 1447.9 | 1475.9 | 1495.9 | 2855.8 | 2895.8 | 4283.7 | 4343.7 | 2951.8 | 2991.8 | 4427.7 | 4487.7 |  |

Table 6.X.3-2: Band 3 and Band 11 DL harmonics and IMD products

| BS DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 1475.9 | 1495.9 | 1805 | 1880 |
| 2nd order harmonics frequency range (MHz) | 2951.8 | 2991.8 | 3610 | 3760 |
| 3rd order harmonics frequency range (MHz) | 4427.7 | 4487.7 | 5415 | 5640 |
| 2nd order IMD products | ¦f2_low – f1_high¦ | ¦f2_high – f1_low¦ | ¦f2_low + f1_low¦ | ¦f2_high + f1_high¦ |
| IMD frequency limits (MHz) | 309.1 | 404.1 | 3280.9 | 3375.9 |
| 3rd order IMD products | ¦f2_high – 2*f1_low¦ | ¦f2_low – 2*f1_high¦ | ¦2*f2_low – f1_high¦ | ¦2*f2_high – f1_low¦ |
| IMD frequency limits (MHz) | 1071.8 | 1186.8 | 2114.1 | 2284.1 |
| 3rd order IMD products | ¦2*f1_low + f2_low¦ | ¦2*f1_high + f2_high¦ | ¦2*f2_low + f1_low¦ | ¦2*f2_high + f1_high¦ |
| IMD frequency limits (MHz) | 4756.8 | 4871.8 | 5085.9 | 5255.9 |
| 3rd order IMD products | ¦f1_low – f2_high + f2_low¦ | ¦f1_high + f2_high – f2_low¦ | ¦f2_low – f1_high + f1_low¦ | ¦f2_high + f1_high – f1_low¦ |
| IMD frequency limits (MHz) | 1400.9 | 1570.9 | 1785 | 1900 |
| 3rd order IMD products (with maximum channel bandwidth) | (f1_low – f2_BWmax) | (f1_high + f2_BWmax) | (f2_low – f1_BWmax) | (f2_high + f1_BWmax) |
| IMD frequency limits (MHz) | 1455.9 | 1515.9 | 1795 | 1890 |

It can be seen from Table 6.X.3-2 that:

- the 2nd harmonics of BS transmitting in Bands 3 and 11 may fall into the BS receive band of Band 43,

- the 3rd harmonics may fall into Band 46

- the 2nd IMD products may fall into none of 3GPP bands

- the 3rd IMD products may fall into the BS receive band of Bands 2, 11, 21, 25, 33, 35. 39, 45 and 46

assuming (except the last row) the whole 75MHz DL frequency of Band 3 and the whole 20 MHz DL frequency of Band 11.

If the BS is only transmitting up to 20 MHz DL in Band 3 and 10MHz DL in Band 11 as stated in the WID, then the 3rd IMD products will not fall into the BS receive band of Bands 11 and 33 as shown in the last row in table 4.

It should be noted that Bands 2, 25, 35, 39, 43, 45 and 46 are not intended for use in the same geographical area as Bands 3 and 11. In addition, based on the current spectrum allocation in Japan, Band 11 and Band 21 are not expected to be operated in the same BS. Therefore, there is no harmful fall down expected in this combination in BS side also.

6.X.4	∆TIB and ∆RIB values

In REL-14, there is a completed combination with almost identical frequency arrangement: Band 3 + Band 21 (see section 6.15). The combination however assumed a new RF architecture, i.e. Band 3 RF bundled with Band 1 (using B1+B3 quadplexer) under common triplexer. The new proposal above requires the low portion of the common triplexer to cover 1.5GHz assuming that B3 filter could not be modified further.

While this seems to worsen insertion loss in general, the new architecture looks well-aligned with a current trend where Bands around 2GHz (Band 1-4) are supported in an integrated module. Thus, this study follows the way B3+B21 was defined.

Based on section 6.15, the following relaxations are proposed which are identical to those for CA_3-21:

Table 6.X.4-1: Proposed ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_3-11 | 3 | 0.8 |
|  | 11 | 0.9 |

Table 6.X.4-2: Proposed ΔRIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_3-11 | 3 | 0.3 |
|  | 11 | 0.5 |

6.X.5	REFSENS requirements

<Text will be added if it’s necessary.>

6.X.6	Blocking requirements

<Text will be added if it’s necessary.>

[Unchanged portions skipped]

form change history:

2013-12-06 v1.14.1 modified §11 to read: <FamilyName>, <GivenName>, (If the person is new to 3GPP work, give full contact coordinates, in particular, email address.)

2013-10-03 v1.14.0 removal of embedded help text

v1.13.2: adds tdoc header

v1.13.1: minor changes resulting from discussions at CT#41 & SA#41

v1.13.0: mods to enforce linkage amongst stages 1, 2, 3

draft mods Scarrone-Meredith 2008-07 ff

v1.12.1: removes revision marks following approval at SP-29
v1.12.0: includes provision for Study Items (SP-29)

v1.11.0: includes those changes from v1.8.0 agreed at SP-25.

v1.10.0: full circle

v1.9.0: a clean sheet

v1.8.0: includes comments from SA#24

v1.7.0: includes comments from RAN, CN and T #24; also includes “early implementation” data

v1.6.0: includes comments made during review period prior to TSGs#24

v1.5.0: includes comments made at TSGs#23 (Phoenix)

v1.4.0: offered to SA#23 for approval

v1.3.0: offered to CN#23, RAN#23 and T#23 for comments

DRAFT4 v1.3.0: 2004-03-09: Incorporation of comments from Leaders list

DRAFT3 v1.3.0: 2004-02-19: Incorporation of comments from MCC members

DRAFT2 v1.3.0: 2004-01-29: Complete redraft:

v1.2.0: 2002-07-04: "USIM" box changed to "UICC apps"

2003-05-28: spelling of “rapporteur” corrected

2002-07-04: "USIM" box changed to "UICC apps"