3GPP TSG-RAN WG4 Meeting #80bis 	R4-167341

Ljubliana, Slovenia, 10 – 14 October, 2016

Source:	SoftBank Corp.

Title:	TP for TR36.714-03-01: the support of CA_3A-8A-11A BCS0

Agenda item:	8.3.2

Document for:	Approval

1.  Introduction

B3+B8+B11 was approved in 3DL/1UL basket WID in the last RAN-P [1]. This paper is to evaluate IM/harmonics and relaxation required. This includes the relevant TP to complete this combination.

Note that this 3DL includes an outstanding 2DL of 3A-11A, which is to be dealt with a companion contribution for approval in this meeting [2].

2. Basic Configurations

As written in the WID, the following configurations are assumed for B3+B8+B11.

Table 1: 3DL Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_3-8-11 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz |  |
|  | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz |  |

Table 2: Supported E-UTRA bandwidths per CA configuration for 3DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_3A-8A-11A | 3 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  | 8 |  |  | Yes | Yes |  |  |  |  |
|  | 11 |  |  | Yes | Yes |  |  |  |  |

3.  Co-existence

Firstly, co-existence of BS side is evaluated with Table.3.

Table 3: Band 3, Band 8 and Band 11 DL 3rd order IMD products

| BS DL carriers | f1-low | f1-high | f2-low | f2-high | f3-low | f3-high |
| --- | --- | --- | --- | --- | --- | --- |
| DL frequency (MHz) | 925 | 960 | 1475.9 | 1495.9 | 1805 | 1880 |
| 3rd order IMD products | ¦f1-high + f2-high – f3-low¦ |  |  | ¦f1-low + f2-low – f3-high¦ |  |  |
| IMD frequency limits (MHz) | 520.9 |  |  | 650.9 |  |  |
| 3rd order IMD products | ¦f1-low + f3-low – f2-high¦ |  |  | ¦f1-high + f3-high – f2-low¦ |  |  |
| IMD frequency limits (MHz) | 1234.1 |  |  | 1364.1 |  |  |
| 3rd order IMD products | ¦f2-low + f3-low – f1-high¦ |  |  | ¦f2-high + f3-high – f1-low¦ |  |  |
| IMD frequency limits (MHz) | 2320.9 |  |  | 2450.9 |  |  |
| 3rd order IMD products | ¦f1-low + f2-low + f3-low¦ |  |  | ¦f1-high + f2-high + f3-high¦ |  |  |
| IMD frequency limits (MHz) | 4205.9 |  |  | 4335.9 |  |  |

It can be seen from Table 3 that in addition to the impacts of harmonics and IMD products from the constituent 2DL CA configurations, the 3rd order IMD products supporting 3 DL CA of Band (3 + 8 + 11) may fall into BS receive band of Band 40. However, Band 40 is not intended for use in the same geographical area as Bands 3, 8 and 11. Therefore, it is clarified that the 3rd order IMD will not cause desensitization.

For UE side, table 4 gives the 2nd and 3rd harmonic for CA with B3+ B8 + B11. The 2nd harmonics of Band 8 will fall into the UE receive portion of B3 but this issue was addressed in the previous releases and it was agreed not to specify this issue. Other than this, it is found that none of the products falls into the B3, B8 or B11 UE receiving bands.

Table 4: UE harmonic table for 3DL/1UL

| UE DL Carriers with B3, B8, B11 | [MHz] |
| --- | --- |
| 2*fx_low to 2*fx_high | 3420 to 3570 |
| 2*fy_low to 2*fy_high | 1760 to 1830 |
| 2*fz_low to 2*fz_high | 2855.8 to 2895.8 |
| 3*fx_low to 3*fx_high | 5130 to 5355 |
| 3*fy_low to 3*fy_high | 2640 to 2745 |
| 3*fz_low to 3*fz_high | 4283.7 to 4343.7 |

4.  ΔTIB and ΔRIB

As written in the companion document [2], 2DL of B3+B11 assumes a common triplexer with low band portion extended to 1.5GHz. Then a diplexer or a quadplexer is needed between Band 8 and Band 11 to split as shown in Figure 1. We believe that this could be realized with a diplexer since it is quite similar to “extended High-Low” diplexer in the sense that 1.4GHz and 1GHz are the border frequencies to split. ThenΔTIB and ΔRIB for this portion can be reused from CA_8A-11A which assumes an extended High-Low diplexer.

[Proposal-1] For B8+B11 portion, the same relaxation values are applied for the new architecture since B8/B11 diplexer performance is expected to be quite similar to “extended High-Low” diplexer.

Figure 1: RF assumption for CA_3-8-11

Thus, the followingΔTIB and ΔRIB are proposed based on its constituent 2DL/1ULs:

Table 5: ΔTIB of constituent 2DL/1UL

| ΔTIB | 3+8 | 8+11 | 3+11 | Proposed Value |
| --- | --- | --- | --- | --- |
| B3 | 0.3 | - | 0.8 | 0.8 |
| B8 | 0.3 | 0.3 | - | 0.3 |
| B11 | - | 0.4 | 0.9 | 0.9 |

Table 6: ΔRIB of constituent 2DL/1UL

| ΔRIB | 3+8 | 8+11 | 3+11 | Proposed Value |
| --- | --- | --- | --- | --- |
| B3 | 0 | - | 0.3 | 0.3 |
| B8 | 0 | 0 | - | 0 |
| B11 | - | 0 | 0.5 | 0.5 |

Then the following relaxations are proposed which are identical to those for CA_3-8-11:

Table 7: ΔTIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c Inter-band with one active UL serving cell [dB] |
| --- | --- | --- |
| CA_3A-8A-11A | 3 | 0.8 |
|  | 8 | 0.3 |
|  | 11 | 0.9 |

Table 8: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c Inter-band with one active UL serving cell [dB] |
| --- | --- | --- |
| CA_3A-8A-11A | 3 | 0.3 |
|  | 8 | 0 |
|  | 11 | 0.5 |

5.  REFSENS

As mentioned above, B3+B8 is a combination of harmonic fall down in UE but it was agreed that this issue is not specified. Because of that, Band 3 REFSENS is “NA” when Band 8 Tx hits the B3 Rx. Then, the following exception is required to be defined in Table 7.3.1A-0a and 0b of [3]:

Table 9 (7.3.1A-0a): Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_3A-8A-11A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
|  | 8 |  |  | -97 | -94 |  |  |  |
|  | 11 |  |  | -100 | -97 |  |  |  |
| NOTE 4: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity is only verified when this is not the case (the requirements specified in clause 7.3.1 apply). |  |  |  |  |  |  |  |  |

Table 10 (7.3.1A-0b): Uplink configuration for the low band (exceptions)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_3A-8A-11A | 8 |  |  | 8 | 16 | 25 | 25 | FDD |

6.  Summary

This paper investigates all the necessary elements to complete CA_3A-8A-11A. The relevant TP is shown in Annex.

7.  Reference

[1]      RP-161674   Revised WID: LTE Advanced inter-band CA Rel-14 for 3DL/1UL, Huawei (Rapporteur).

[2]      R4-167339    TP for TR36.714-02-01: the support of CA_3A-11A BCS0, SoftBank Corp.

[3]      TS36.101(v14.1.0)

Annex: TP for TR36.714-03-01 (v0.4.0)

[Unchanged portions skipped]

6.Z 	CA_3A-8A-11A_BCS0

6.Z.1	Operating bands for CA

Table 6.Z.1-1: 3DL Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_3-8-11 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz |  |
|  | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz |  |

6.Z.2	Channel bandwidths per operating band for CA

Table 6.Z.2-1: Supported E-UTRA bandwidths per CA configuration for 3DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_3A-3A-11A | 3 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  | 8 |  |  | Yes | Yes |  |  |  |  |
|  | 11 |  |  | Yes | Yes |  |  |  |  |
| NOTE 1: For the UE that signals support of a 3DL CA bandwidth combination set, the UE shall support all single carrier channel bandwidths for the constituent bands as defined in table 5.6.1-1 of TS 36.101 and all the 2DL CA bandwidth combination sets in Table 5.6A.2-1 of TS 36.101 whose channel bandwidths are entirely covered by the signaled 3DL CA bandwidth combination set. |  |  |  |  |  |  |  |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all for the constituent bands as defined in Table 5.6.1-1 of TS 36.101 [2] when operating in single carrier mode.

6.Z.3 	Harmonics and IMD study

6.Z.3.1	For the BS

The 3rd order IMD products caused in the BS by transmitting of Band 3, Band 8 and Band 11 DL carriers (i.e. 3DL CA with one component carrier in each band) can be calculated as shown in Table 6.Z.3.1-1 below:

Table 6.Z.3.1-1: Band 3, Band 8 and Band 11 DL 3rd order IMD products

| BS DL carriers | f1-low | f1-high | f2-low | f2-high | f3-low | f3-high |
| --- | --- | --- | --- | --- | --- | --- |
| DL frequency (MHz) | 925 | 960 | 1475.9 | 1495.9 | 1805 | 1880 |
| 3rd order IMD products | ¦f1-high + f2-high – f3-low¦ |  |  | ¦f1-low + f2-low – f3-high¦ |  |  |
| IMD frequency limits (MHz) | 520.9 |  |  | 650.9 |  |  |
| 3rd order IMD products | ¦f1-low + f3-low – f2-high¦ |  |  | ¦f1-high + f3-high – f2-low¦ |  |  |
| IMD frequency limits (MHz) | 1234.1 |  |  | 1364.1 |  |  |
| 3rd order IMD products | ¦f2-low + f3-low – f1-high¦ |  |  | ¦f2-high + f3-high – f1-low¦ |  |  |
| IMD frequency limits (MHz) | 2320.9 |  |  | 2450.9 |  |  |
| 3rd order IMD products | ¦f1-low + f2-low + f3-low¦ |  |  | ¦f1-high + f2-high + f3-high¦ |  |  |
| IMD frequency limits (MHz) | 4205.9 |  |  | 4335.9 |  |  |

It can be seen from Table 6.Z.3.1-1 that in addition to the impacts of harmonics and IMD products from the constituent 2DL CA configurations, the 3rd order IMD products supporting 3 DL CA of Band (3 + 8 + 11) may fall into BS receive band of Band 40. However, Band 40 is not intended for use in the same geographical area as Bands 3, 8 and 11. Therefore, it is clarified that the 3rd order IMD will not cause desensitization.

6.Z.3.2	For the UE

Table 6.Z.3.2-1 gives the 2nd and 3rd harmonic for CA with B3+ B8 + B11. The 2nd harmonics of Band 8 will fall into the UE receive portion of B3 but this issue was addressed in the previous releases and it was agreed not to specify this issue. Other than this, it is found that none of the products falls into the B3, B8 or B11 UE receiving bands.

Table 6.Z.3.2-1: UE harmonic table for 3DL/1UL

| UE DL Carriers with B3, B8, B11 | [MHz] |
| --- | --- |
| 2*fx_low to 2*fx_high | 3420 to 3570 |
| 2*fy_low to 2*fy_high | 1760 to 1830 |
| 2*fz_low to 2*fz_high | 2855.8 to 2895.8 |
| 3*fx_low to 3*fx_high | 5130 to 5355 |
| 3*fy_low to 3*fy_high | 2640 to 2745 |
| 3*fz_low to 3*fz_high | 4283.7 to 4343.7 |

6.Z.4	ΔTIB,c and ΔRIB,c values

Since a constituent 2DL of B3+B11 assumes a common triplexer with low band portion extended to 1.5GHz, a diplexer or a quadplexer is needed between Band 8 and Band 11 to split these. This could be realized with a diplexer since it is quite similar to extended High-Low diplexer, assumed in CA_8A-11A also, in the sense that 1.4GHz and 1GHz are the border frequencies to split. ThenΔTIB and ΔRIB for this portion can be reused from CA_8A-11A.

With that, for the UE which supports inter-band carrier aggregation configurations with uplink assigned to one E-UTRA band the ΔTIB,c is defined for applicable bands in Table 6.Z.4-1.

Table 6.Z.4-1: ΔTIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c Inter-band with one active UL serving cell [dB] |
| --- | --- | --- |
| CA_3A-8A-11A | 3 | 0.8 |
|  | 8 | 0.3 |
|  | 11 | 0.9 |

For the UE which supports inter-band carrier aggregation configuration in Table 6.Z.4-2 with uplink in one E-UTRA band, the minimum requirement for reference sensitivity in Table 7.3.1-1 (of TS 36.101 [2]) shall be increased by the amount given in ΔRIB,c in Table 6.Z.4-2 for the applicable E-UTRA bands.

Table 6.Z.4-2: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c Inter-band with one active UL serving cell [dB] |
| --- | --- | --- |
| CA_3A-8A-11A | 3 | 0.3 |
|  | 8 | 0 |
|  | 11 | 0.5 |

6.Z.5	REFSENS

B3+B8 is a combination of harmonic fall down in UE but it was agreed that this issue is not specified. Because of that, Band 3 REFSENS is “NA” when Band 8 Tx hits the B3 Rx. Then, the following exception is required to be defined:

Table 6.Z.5-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_3A-8A-11A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
|  | 8 |  |  | -97 | -94 |  |  |  |
|  | 11 |  |  | -100 | -97 |  |  |  |
| NOTE 4: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity is only verified when this is not the case (the requirements specified in clause 7.3.1 apply). |  |  |  |  |  |  |  |  |

Table 6.Z.5-2: Uplink configuration for the low band (exceptions)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_3A-8A-11A | 8 |  |  | 8 | 16 | 25 | 25 | FDD |

[Unchanged portions skipped]

form change history:

2013-12-06 v1.14.1 modified §11 to read: <FamilyName>, <GivenName>, (If the person is new to 3GPP work, give full contact coordinates, in particular, email address.)

2013-10-03 v1.14.0 removal of embedded help text

v1.13.2: adds tdoc header

v1.13.1: minor changes resulting from discussions at CT#41 & SA#41

v1.13.0: mods to enforce linkage amongst stages 1, 2, 3

draft mods Scarrone-Meredith 2008-07 ff

v1.12.1: removes revision marks following approval at SP-29
v1.12.0: includes provision for Study Items (SP-29)

v1.11.0: includes those changes from v1.8.0 agreed at SP-25.

v1.10.0: full circle

v1.9.0: a clean sheet

v1.8.0: includes comments from SA#24

v1.7.0: includes comments from RAN, CN and T #24; also includes “early implementation” data

v1.6.0: includes comments made during review period prior to TSGs#24

v1.5.0: includes comments made at TSGs#23 (Phoenix)

v1.4.0: offered to SA#23 for approval

v1.3.0: offered to CN#23, RAN#23 and T#23 for comments

DRAFT4 v1.3.0: 2004-03-09: Incorporation of comments from Leaders list

DRAFT3 v1.3.0: 2004-02-19: Incorporation of comments from MCC members

DRAFT2 v1.3.0: 2004-01-29: Complete redraft:

v1.2.0: 2002-07-04: "USIM" box changed to "UICC apps"

2003-05-28: spelling of “rapporteur” corrected

2002-07-04: "USIM" box changed to "UICC apps"