3GPP TSG-RAN WG4 Meeting #80bis 	R4-167344

Ljubliana, Slovenia, 10 – 14 October, 2016

Source:	SoftBank Corp.

Title:	TP for TR36.714-00-02: the support of CA_3DL_41A-42C_2UL_41A-42A_BCS0

Agenda item:	8.7.2

Document for:	Approval

1.  Introduction

CA_3DL_41A-42C_2UL_41A-42A_BCS0 was approved in xDL/2UL basket WID in the last RAN-P [1]. This paper is to evaluate IM/harmonics and relaxation required. This also includes the relevant TP to complete this combination.

2. Summary of the study

Since this WI simply adds another DL contiguous CC in Band 42 to 2UL/2DL relevant [2], study necessary for 2UL portion has already covered in [2]. Essential findings are:

- Desense won’t occur since B41 and B42 are assumed to be synchronized,

- Interference to ISM/GNSS should be the same as in 2DL/2UL,

- TIB,c and RIB,c in [2] can be reused since there is no need to change RF configuration.

TP is attached to reflect the findings.

3.  Reference

[1]     RP-161487   Revised WID: LTE Advanced inter-band CA Rel-14 for xDL/2UL with x=3,4,5, LG Electronics Inc (Rapporteur).

[2]     R4-165069    TP for TR36.714-02-02: Discussions to complete 2UL/2DL of 41A-42A, SoftBank Corp.

.

Annex: TP for TR36.714-00-02 (v0.4.0)

[Unchanged portions skipped]

6.X	LTE Advanced Carrier Aggregation: Band 41 and Band 42 and Band 42 with 2 ULs

6.X.1	List of specific combination issues

6.X.1.1	Channel bandwidths per operating band for CA

Table 6.X.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_41A-42C | CA_41A-42A | 41 |  |  |  | Yes | Yes | Yes | 60 | 0 |
|  |  | CA_42C | See CA_42C BCS1 in Table 6.X.1.1-2 |  |  |  |  |  |  |  |

Note that in this WI, B41 and B42 are assumed to be synchronized (i.e. without simultaneous Rx/Tx).

Table 6.X.1.1-2: E-UTRA CA_42C configurations and bandwidth combination sets defined for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_42C | 10, 15, 20 | 20 | 40 | 1 |
|  | 20 | 10, 15 |  |  |

6.X.1.2	Co-existence studies for LTE-A UL CA_41A-42A and DL CA_41A-42C

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.X.1.2-1.

Table 6.X.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2496 | 2690 | 3400 | 3600 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 4992 | 5380 | 6800 | 7200 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7488 | 8070 | 10200 | 10800 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 710 | 1104 | 5896 | 6290 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1392 | 1980 | 4110 | 4704 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8392 | 8980 | 9296 | 9890 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3888 | 4670 | 7510 | 8304 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 10888 | 11670 | 12696 | 13490 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1420 | 2208 | 11792 | 12580 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 10910 | 11904 | 6384 | 7360 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 16096 | 17090 | 13384 | 14360 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4820 | 5808 | 288 | 1270 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 15192 | 16180 | 14288 | 15270 |

Based on the table above, no fall down into own Rx of B41 and B42 is observed. It should be noted that, as long as self-desense is concerned, there would be no problem since this 2UL CA assumes non simultaneous Rx/Tx, i.e. Rx is not operated while Tx is active. This means that extra REFSENS requirement such as MSD is not needed for this 2UL CA. This fact was already covered in 2UL/2DL study of Rel-14 TR 36.714-02-02. Impacts for ISM and GNSS bands are already captured in Rel-14 TR 36.714-02-02 also.

6.X.1.3	MSD

When uplink CA configurations CA_41A-42A is paired with downlink CA configuration CA_41A-42C, as mentioned above, there is no concern from 2 uplink operation which would interfere the additional (second) downlink of the Band 42 since Band 41 and Band 42 are assumed to be synchronized.

6.X.1.4	∆TIB and ∆RIB values

For inter-band CA_41-42, same requirements on values for ∆TIB and ∆RIB can apply with 2DL/1UL and 2DL/2UL cases. Therefore, for two simultaneous UL, the TIB,c and RIB,c values are shown in table 6.X.1.4-1 and in table 6.X.1.4-2 which are the same as 2DL/2UL:

Table 6.X.1.4-1: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_41-42 | 41 | 0.3 |
|  | 42 | 0.8 |
| Note: Only applicable for UE supporting inter-band carrier aggregation with uplink in two E-UTRA bands and without simultaneous Rx/Tx. |  |  |

Table 6.X.1.4-2: ΔRIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_41-42 | 41 | 0 |
|  | 42 | 0.5 |
| Note: Only applicable for UE supporting inter-band carrier aggregation with uplink in two E-UTRA bands and without simultaneous Rx/Tx. |  |  |

[Unchanged portions skipped]

form change history:

2013-12-06 v1.14.1 modified §11 to read: <FamilyName>, <GivenName>, (If the person is new to 3GPP work, give full contact coordinates, in particular, email address.)

2013-10-03 v1.14.0 removal of embedded help text

v1.13.2: adds tdoc header

v1.13.1: minor changes resulting from discussions at CT#41 & SA#41

v1.13.0: mods to enforce linkage amongst stages 1, 2, 3

draft mods Scarrone-Meredith 2008-07 ff

v1.12.1: removes revision marks following approval at SP-29
v1.12.0: includes provision for Study Items (SP-29)

v1.11.0: includes those changes from v1.8.0 agreed at SP-25.

v1.10.0: full circle

v1.9.0: a clean sheet

v1.8.0: includes comments from SA#24

v1.7.0: includes comments from RAN, CN and T #24; also includes “early implementation” data

v1.6.0: includes comments made during review period prior to TSGs#24

v1.5.0: includes comments made at TSGs#23 (Phoenix)

v1.4.0: offered to SA#23 for approval

v1.3.0: offered to CN#23, RAN#23 and T#23 for comments

DRAFT4 v1.3.0: 2004-03-09: Incorporation of comments from Leaders list

DRAFT3 v1.3.0: 2004-02-19: Incorporation of comments from MCC members

DRAFT2 v1.3.0: 2004-01-29: Complete redraft:

v1.2.0: 2002-07-04: "USIM" box changed to "UICC apps"

2003-05-28: spelling of “rapporteur” corrected

2002-07-04: "USIM" box changed to "UICC apps"