3GPP TSG-RAN WG4 Meeting #80bis 	R4-167345

Ljubliana, Slovenia, 10 – 14 October, 2016

Source:	SoftBank Corp.

Title:	TP for TR36.714-00-02: the support of CA_3DL_41C-42A_2UL_41A-42A_BCS0

Agenda item:	8.7.2

Document for:	Approval

1.  Introduction

CA_3DL_41C-42A_2UL_41A-42A_BCS0 was approved in xDL/2UL basket WID in the last RAN-P [1]. This paper is to evaluate IM/harmonics, relaxation required and includes the relevant TP to complete this combination.

2. Summary of the study

Since this WI simply adds another DL contiguous CC in Band 41 to 2UL/2DL relevant [2], study necessary for 2UL portion has already covered in [2]. Essential findings are:

- Desense won’t occur since B41 and B42 are assumed to be synchronized,

- Interference to ISM/GNSS should be the same as in 2DL/2UL,

- TIB,c and RIB,c in [2] can be reused since there is no need to change RF configuration.

TP is attached to reflect the findings.

3.  Reference

[1]      RP-161487   Revised WID: LTE Advanced inter-band CA Rel-14 for xDL/2UL with x=3,4,5, LG Electronics Inc (Rapporteur).

[2]      R4-165069    TP for TR36.714-02-02: Discussions to complete 2UL/2DL of 41A-42A, SoftBank Corp.

Annex: TP for TR36.714-00-02 (v0.4.0)

[Unchanged portions skipped]

6.Y	LTE Advanced Carrier Aggregation: Band 41 and Band 41 and Band 42 with 2 ULs

6.Y.1	List of specific combination issues

6.Y.1.1	Channel bandwidths per operating band for CA

Table 6.Y.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_41C-42A | CA_41A-42A | 41 | See CA_41C BCS0 in Table 6.Y.1.1-2 |  |  |  |  |  | 60 | 0 |
|  |  | 42 |  |  |  | Yes | Yes | Yes |  |  |

Note that in this WI, B41 and B42 are assumed to be synchronized (i.e. without simultaneous Rx/Tx).

Table 6.Y.1.1-2: E-UTRA CA_41C configurations and bandwidth combination sets defined for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_41C | 10 | 20 | 40 | 0 |
|  | 15 | 15, 20 |  |  |
|  | 20 | 10, 15, 20 |  |  |

6.Y.1.2	Co-existence studies for LTE-A UL CA_41A-42A and DL CA_41C-42A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.Y.1.2-1.

Table 6.Y.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2496 | 2690 | 3400 | 3600 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 4992 | 5380 | 6800 | 7200 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7488 | 8070 | 10200 | 10800 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 710 | 1104 | 5896 | 6290 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1392 | 1980 | 4110 | 4704 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8392 | 8980 | 9296 | 9890 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3888 | 4670 | 7510 | 8304 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 10888 | 11670 | 12696 | 13490 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1420 | 2208 | 11792 | 12580 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 10910 | 11904 | 6384 | 7360 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 16096 | 17090 | 13384 | 14360 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4820 | 5808 | 288 | 1270 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 15192 | 16180 | 14288 | 15270 |

Based on the table above, no fall down into own Rx of B41 and B42 is observed. It should be noted that, as long as self-desense is concerned, there would be no problem since this 2UL CA assumes non simultaneous Rx/Tx, i.e. Rx is not operated while Tx is active. This means that extra REFSENS requirement such as MSD is not needed for this 2UL CA. This fact was already covered in 2UL/2DL study of Rel-14 TR 36.714-02-02. Impacts for ISM and GNSS bands are already captured in Rel-14 TR 36.714-02-02 also.

6.Y.1.3	MSD

When uplink CA configurations CA_41A-42A is paired with downlink CA configuration CA_41C-42A, as mentioned above, there is no concern from 2 uplink operation which would interfere the additional (second) downlink of the Band 41 since Band 41 and Band 42 are assumed to be synchronized.

6.Y.1.4	∆TIB and ∆RIB values

For inter-band CA_41-42, same requirements on values for ∆TIB and ∆RIB can apply with 2DL/1UL and 2DL/2UL cases. Therefore, for two simultaneous UL, the TIB,c and RIB,c values are shown in table 6.Y.1.4-1 and in table 6.Y.1.4-2 which are the same as 2DL/2UL:

Table 6.Y.1.4-1: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_41-42 | 41 | 0.3 |
|  | 42 | 0.8 |
| Note: Only applicable for UE supporting inter-band carrier aggregation with uplink in two E-UTRA bands and without simultaneous Rx/Tx. |  |  |

Table 6.Y.1.4-2: ΔRIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_41-42 | 41 | 0 |
|  | 42 | 0.5 |
| Note: Only applicable for UE supporting inter-band carrier aggregation with uplink in two E-UTRA bands and without simultaneous Rx/Tx. |  |  |

[Unchanged portions skipped]

form change history:

2013-12-06 v1.14.1 modified §11 to read: <FamilyName>, <GivenName>, (If the person is new to 3GPP work, give full contact coordinates, in particular, email address.)

2013-10-03 v1.14.0 removal of embedded help text

v1.13.2: adds tdoc header

v1.13.1: minor changes resulting from discussions at CT#41 & SA#41

v1.13.0: mods to enforce linkage amongst stages 1, 2, 3

draft mods Scarrone-Meredith 2008-07 ff

v1.12.1: removes revision marks following approval at SP-29
v1.12.0: includes provision for Study Items (SP-29)

v1.11.0: includes those changes from v1.8.0 agreed at SP-25.

v1.10.0: full circle

v1.9.0: a clean sheet

v1.8.0: includes comments from SA#24

v1.7.0: includes comments from RAN, CN and T #24; also includes “early implementation” data

v1.6.0: includes comments made during review period prior to TSGs#24

v1.5.0: includes comments made at TSGs#23 (Phoenix)

v1.4.0: offered to SA#23 for approval

v1.3.0: offered to CN#23, RAN#23 and T#23 for comments

DRAFT4 v1.3.0: 2004-03-09: Incorporation of comments from Leaders list

DRAFT3 v1.3.0: 2004-02-19: Incorporation of comments from MCC members

DRAFT2 v1.3.0: 2004-01-29: Complete redraft:

v1.2.0: 2002-07-04: "USIM" box changed to "UICC apps"

2003-05-28: spelling of “rapporteur” corrected

2002-07-04: "USIM" box changed to "UICC apps"