3GPP TSG-RAN WG4 RAN4 #80bis                                       R4-167364

Ljubljana, Slovenia, 10 - 14 Oct, 2016

Agenda item:			9.3.3

Source:			Samsung

Title:	Updated Simulation Results on NB-IoT Coexistence Study with CDMA system for Uplink Scenarios

Document for:			Discussion

# 1	Introduction

The study item on NB-IoT RF requirement for coexistence with CDMA was approved at RAN #72. According to WID [1], the study item has set up objectives as follows:

- Identify operating bands for interference evaluation on NB-IoT coexistence with CDMA, such as bands around 800M (B5, B26 etc.) - For the NB-IoT stand-alone operation mode, evaluate coexistence requirement between NB-IoT and CDMA, and further evaluate whether R13 NB-IoT RF requirements could be reused or not. - Identify necessary additional RF requirements to ensure NB-IoT for co-existence with CDMA.

In last RAN4 meeting, simulation assumptions and evaluation methodology for the coexistence study between CDMA and NB-IoT are agreed in the WF [2] as follows:

- Standalone NB-IoT to coexistence with CDMA 1x should be evaluated in this study. Cases; Aggressor system; Victim system; Link Direction; Deployment. 1; NB-IoT; CDMA1x; DL; Uncoordinated. 2; CDMA1x; NB-IoT; DL; Uncoordinated. 3; NB-IoT; CDMA1x; UL; Uncoordinated. 4; CDMA1x; NB-IoT; UL; Uncoordinated. - The evaluation methodology applied for coexistence study of NB-IoT system in TR36.802 is reused. - Static system simulation - The results should be presented as a function of ACS (when NB-IoT is victim) and as a function of ACLR (when NB-IoT is aggressor) - Metrics and criteria for coexistence study - For NB-IoT, SINR loss s less or equal to 1dB - For CDMA , Capacity loss is less or equal to 5% - Simulation parameters of Standalone NB-IoT for R13 coexistence study are reused. - Simulation parameters of CDMA 1x are mainly refer to TR25.942, while the CDMA1x BS and UE’s ACLR and ACS values are derived from 3GPP2 specs in C.S0010-D and C.S0011-D.

In last RAN4 meeting, we had provided the initial simulation results for the uplink coexistence simulation scenarios between NB-IoT and CDMA system in [3]. In this contribution, we provide the updated simulation results for the uplink cases, since the simulation parameters UE ACLR and BS ACS were updated in last meeting. And we also provide the simulation results for the downlink cases in our accompanying contribution [4].

# Coexistence Simulation Results

As described in [2], there are total 4 coexistence simulation cases between NB-IoT and CDMA2000 1x system. In this paper, we provide the simulation results for the two uplink cases, i.e. Case 3 and Case 4 in below table.

Table 2.1 Simulation cases of coexistence study for NB-IoT with CDMA

| Cases | Aggressor system | Victim system | Link Direction | Deployment |
| --- | --- | --- | --- | --- |
| 1 | NB-IoT | CDMA1x | DL | Uncoordinated |
| 2 | CDMA1x | NB-IoT | DL | Uncoordinated |
| 3 | NB-IoT | CDMA1x | UL | Uncoordinated |
| 4 | CDMA1x | NB-IoT | UL | Uncoordinated |

The detailed simulation assumptions and parameters are agreed in the WF [2] as listed in the Appendix.

The simulation results are provided in Table 2.2 and Table 2.3 for Case 3 and Case 4 respectively. In below simulation results, 3 types of NB-IoT UE configurations are provided, i.e. 3/12/48 UEs per cell with 60/15/3.75 KHz bandwidth per UE. For Case 3 in Table 2.2, the results are CDMA capacity loss (%) at different NB-IoT UE ACLR value. For Case 4 in Table 2.3, the results are NB-IoT SINR loss (dB) at different NB-IoT BS ACS value.

Note that the NB-IoT UE ACLR value and NB-IoT BS ACS value are both defined on adjacent measurement channel bandwidth 180 kHz.

Table 2.2 Simulation results for Case 3: Uplink, NB-IoT aggressor, CDMA victim

| # of NB-IoT UE | CDMA capacity loss (%) vs. NB-IoT UE ACLR |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20 | 25 | 30 | 35 | 40 | 45 | 50 |
| 3 UEs (60 KHz/UE) | 95.52 | 92.00 | 57.09 | 27.66 | 8.25 | 2.82 | 0.75 |
| 12 UEs (15 KHz/UE) | 95.52 | 95.52 | 80.90 | 41.55 | 12.34 | 3.91 | 1.06 |
| 48 UEs (3.75 KHz/UE) | 95.52 | 95.52 | 86.14 | 41.48 | 13.15 | 4.15 | 1.39 |

Table 2.3 Simulation results for Case 4: Uplink, CDMA aggressor, NB-IoT victim

| # of NB-IoT UE | # of NB-IoT UE | NB-IoT SINR loss (dB) vs. NB-IoT BS ACS |  |  |
| --- | --- | --- | --- | --- |
|  |  | 40 | 45 | 50 |
| 3 UEs (60 KHz/UE) | SINR loss 5%-tile | 0.01 | 0.01 | 0.01 |
|  | SINR loss 50%-tile | 0.03 | 0.02 | 0.02 |
|  | SINR loss 95%-tile | 0.10 | 0.09 | 0.08 |
|  | SINR loss 99%-tile | 0.10 | 0.09 | 0.08 |
| 12 UEs (15 KHz/UE) | SINR loss 5%-tile | 0.01 | 0.01 | 0.01 |
|  | SINR loss 50%-tile | 0.03 | 0.03 | 0.03 |
|  | SINR loss 95%-tile | 0.09 | 0.08 | 0.08 |
|  | SINR loss 99%-tile | 0.12 | 0.11 | 0.10 |
| 48 UEs (3.75 KHz/UE) | SINR loss 5%-tile | 0.01 | 0.01 | 0.01 |
|  | SINR loss 50%-tile | 0.03 | 0.03 | 0.03 |
|  | SINR loss 95%-tile | 0.09 | 0.08 | 0.07 |
|  | SINR loss 99%-tile | 0.10 | 0.08 | 0.08 |

Based on the above results, it can be observed that:

- For Case 3, i.e. NB-IoT interfering CDMA in uplink, when NB-IoT UE ACLR is 45 dB (defined on 180 KHz bandwidth), the CDMA system average uplink capacity loss is around 5%. So, to satisfy the CDMA performance loss criteria that capacity loss is less or equal to 5%, the requirement for NB-IoT UE ACLR should be equal or larger than 45 dB for its own bandwidth.

- For Case 4, i.e. CDMA interfering NB-IoT in uplink, since the legacy CDMA UE ACLR is fixed to 34.2 dB (defined on 180 KHz bandwidth), and the input of NB-IoT BS ACS is 40/45/50 dB (defined on 180 KHz bandwidth), we can see that the major ACIR interference is from CDMA UE ACLR. The results show that even when the NB-IoT BS ACS is 40 dB (defined on 180 KHz bandwidth), it can well satisfy the NB-IoT performance loss criteria that average of SNR value loss is less or equal to 1 dB.

The above observations can be summarized as below:

Observation 1: To satisfy CDMA performance loss criteria, the requirement for NB-IoT UE ACLR should be equal or larger than 45 dB for its own bandwidth.

Observation 2: The NB-IoT performance loss criteria can be well satisfied even when NB-IoT BS ACS is 40 dB for its own bandwidth.

# Conclusion

In this contribution, we provide our updated results for the uplink scenarios based on the simulation assumptions and parameters provided in the WF [2]. Based on the simulation results, the following observations are given:

Observation 1: To satisfy CDMA performance loss criteria, the requirement for NB-IoT UE ACLR should be equal or larger than 45 dB for its own bandwidth.

Observation 2: The NB-IoT performance loss criteria can be well satisfied even when NB-IoT BS ACS is 40 dB for its own bandwidth.

# Reference

[1] 3GPP RP-161187, “WID: Study on NB-IoT RF requirement for coexistence with CDMA”, China Telecom.

[2] 3GPP R4-166994, “Way Forward on Simulation Assumption and Methodology for the coexistence study between CDMA and NB-IoT”, China Telecom, Samsung, Huawei, Qualcomm, Ericsson.

[3] 3GPP R4-165934, “Simulation Results on NB-IoT Coexistence Study with CDMA system for Uplink Scenarios”, Samsung.

[4] 3GPP R4-167363, “Simulation Results on NB-IoT Coexistence Study with CDMA system for Downlink Scenarios”, Samsung.

# Appendix

The detailed simulation parameters are provided as below:

Table 5.1 Simulation assumption for coexistence study between NB-IoT and CDMA2000 1x system

|  | NB-IoT standalone | CDMA2000 1x |
| --- | --- | --- |
| Carrier frequency in GHz | 0.85 | 0.85 |
| Size of each nominal channel BW in MHz | 0.2 | 1.23 |
| Transmission bandwidth in MHz | 0.18 | 1.23 |
| Environment | Urban macro | Urban macro |
| Network layout | 19-sites [57 sectors] with wrap-around | 19-sites [57 sectors] with wrap-around |
| Inter-site distance in meter | 750m (Same for aggressor and victim) | 750m (Same for aggressor and victim) |
| System loading and activity | Full buffer 100% | All users with 9.6kbps speech |
| Network location | Non co-located (at cell edge of legacy) | Non co-located with IoT |
| DL subcarrier spacing | 15kHz | n/a |
| UL | See RP-152284 | n/a |
| DL power control | No | TR25.942 (reuse UMTS PC) |
| UL power control | 36.942 section 5.1.1.6 (set 1) by bandwidth scale, target SNR at BS is 15 dB | TR25.942 (reuse UMTS PC) |
| Frequency reuse | 1 | 1 |
| Number of scheduled UE per cell (DL) | 1 | According to 95% users achieving target of (Eb/N0-0.5)dB; non orthogonality 0.2; target Eb/N0=5.5dB |
| Number of scheduled UE per cell (UL) | 3 for multi-tone (60kHz per UE), 12 for 15kHz single-tone, 48 for 3.75kHz single-tone | according to 5.5dB noise rise; target Eb/N0=4.0dB no MUD utilized, beta=0 |
| BS antenna height in meter | 30 | 30 |
| BS max TX power in dBm | 43/200kHz | 43 |
| BS Tx power of common channel in dBm* | n/a | 36.5 (R4-030558 22.4% Total Power) |
| BS antenna gain including feeder loss in dBi | 15 | 15 |
| BS antenna pattern | Horizontal (TR36.942) | Horizontal (65deg for -3dB) |
| BS antenna front-back ratio in dB | 20 | 20 |
| UE antenna height in meter | 1.5 | 1.5 |
| UE TX power in dBm | -40 to 23 | -52 to 23 |
| UE antenna gain in dBi | 0 | 0 |
| Building penetration loss | 45.820 Annex D.1 | n/a |
| Cell selection margin in dB | 3 | 3 |
| BS-MS min couple loss in dB | 70 | 70 |
| BS ACLR in dB | 40 to 60** | 37.2 (own channel measurement BW: 1.23MHz adjacent channel measurement BW: 180kHz) |
| BS ACS in dB | 40 to 50** | 71 (own channel measurement BW: 1.23MHz adjacent channel measurement BW: 180kHz) |
| UE ACLR in dB | 20 to 50** | 34.2 (own channel measurement BW: 1.23MHz adjacent channel measurement BW: 180kHz) |
| UE ACS in dB | 20 to 40** | 49.5 (own channel measurement BW: 1.23MHz adjacent channel measurement BW: 180kHz) |
| BS noise figure in dB | 5 | 5 |
| UE noise figure in dB | 9 | 9 |
| BS-UE path-loss model | TR36.942 macro urban | TR36.942 macro urban |
| Standard deviation of BS-UE log-normal shadow fading in dB | 10 | 10 |
| Shadowing correlation | Inter-cell 0.5 intra-cell 1 | Inter-cell 0.5 intra-cell 1 |
| Evaluation metrics | SINR vs ACS (as victim) | Capacity vs NB-IoT ACLR (as victim). For DL, capacity is the number of satisfied users. For UL, capacity is the number of users when 5.5dB noise rise is reached |
| Carrier separation*** | 1.1MHz to CDMA(central frequency separation) | 1.1MHz to NB-IoT(central frequency separation) |

*DL overhead channel resource consumption, including pilot, paging and sync overhead.

**Similar to coexistence study in TR36.802, NB-IoT ACLR and ACS are defined for its own bandwidth size.

*** Other carrier separation options are not excluded.