3GPP TSG-RAN WG4 RAN4 #80bis                                       R4-167365

Ljubljana, Slovenia, 10 - 14 Oct, 2016

Agenda item:			9.3.4

Source:			Samsung

Title:	Discussion on RF Requirement Impact Based on Co-existence Simulation Results

Document for:			Discussion

# 1	Introduction

The study item on NB-IoT RF requirement for coexistence with CDMA was approved at RAN #72. According to WID [1], the study item has set up objectives as follows:

- Identify operating bands for interference evaluation on NB-IoT coexistence with CDMA, such as bands around 800M (B5, B26 etc.) - For the NB-IoT stand-alone operation mode, evaluate coexistence requirement between NB-IoT and CDMA, and further evaluate whether R13 NB-IoT RF requirements could be reused or not. - Identify necessary additional RF requirements to ensure NB-IoT for co-existence with CDMA.

Based on the simulation assumptions and evaluation methodology for the coexistence study between CDMA and NB-IoT agreed in RAN4 #80 [2], we have provided our simulation results in accompanying contributions [3][4], for the four scenarios. Based on that, we will provide our analysis on whether or not additional NB-IoT RF requirements is needed to ensure NB-IoT for co-existence with CDMA.

# Discussion on RF Requirement Impact

As described in [2], there are total four coexistence simulation scenarios between NB-IoT and CDMA2000 1x system as below. In the following discussion, we will focus on the impact on RF requirement based on the simulation results of below scenarios respectively.

Table 2.1 Simulation Scenarios of coexistence study for NB-IoT with CDMA

| Cases | Aggressor system | Victim system | Link Direction | Deployment |
| --- | --- | --- | --- | --- |
| 1 | NB-IoT | CDMA 1x | DL | Uncoordinated |
| 2 | CDMA 1x | NB-IoT | DL | Uncoordinated |
| 3 | NB-IoT | CDMA 1x | UL | Uncoordinated |
| 4 | CDMA 1x | NB-IoT | UL | Uncoordinated |

## 2.1 NB-IoT BS ACLR Based on Scenario-1

Based on our simulation results for the coexistence scenario 1 [3], the CDMA performance loss criteria (equal or less than 5% capacity loss) can be well satisfied even when NB-IoT BS ACLR is 40 dB for its own bandwidth. By comparing with current Rel-13 NB-IoT requirement defined in TS 36.104, as listed in the below table, the current requirements (40dB for 1st neighboring channel and 50dB for 2nd one) are stringent enough to guarantee the protection for CDMA system, especially considering more guard band (1.1 MHz for central frequency separation) is reserved for the coexistence scenario for CDMA system.

Table 2.1.1 Base Station ACLR for standalone NB-IoT operation in paired spectrum in Rel-13

| Channel bandwidth of NB-IoT lowest/highest carrier transmitted BWChannel [kHz] | BS adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- |
| 200 | 300 kHz | Standalone NB-IoT | Square (180 kHz) | 40 dB |
|  | 500 kHz | Standalone NB-IoT | Square (180 kHz) | 50 dB |

Proposal 1: Rel-13 NB-IoT BS ACLR requirement is stringent enough to guarantee the protection for CDMA system, and no additional requirement is required.

## 2.2 NB-IoT UE ACS Based on Scenario-2

Based on our simulation results for the coexistence scenario 2 [3], to satisfy NB-IoT performance loss criteria (equal or less than 1dB SINR loss), the requirement for NB-IoT UE ACS should be equal or larger than 25 dB for its own bandwidth. By comparing with current Rel-13 NB-IoT requirement analyzed in TR 36.802, as listed in the below table, the current requirements (30dB for neighboring GSM signal and 35dB for neighboring LTE signal) are stringent enough to guarantee the protection for CDMA system. Similar to our analysis on NB-IoT BS ACLR requirement, the guard band assumption here (1.1 MHz for central frequency separation) also provide enough gap to guarantee that the protection requirement for CDMA system can be meet easily.

Table 2.2.1 NB-IoT ACS requirement parameters analysed in TR 36.802

| ACS1 test Parameters |  |  |
| --- | --- | --- |
| Interferer | GSM (GMSK) | E-UTRA |
| NB-IoT signal power (Pwanted ) / dBm | REFSENS + 14 dB |  |
| interferer signal power (PInterferer ) / dBm | REFSENS + 44 dB – SNR | REFSENS + 49 dB – SNR |
| Interferer bandwidth | 200 kHz | 5 MHz |
| Interferer offset from NB-IoT channel edge | ±200 kHz | ±2.5 MHz |
| ACS2 test Parameters |  |  |
| Interferer | GSM (GMSK) | E-UTRA |
| NB-IoT signal power (Pwanted ) / dBm | -25 dBm -30 dB + SNR | -25 dBm -35 dB + SNR |
| interferer signal power (PInterferer ) / dBm | -25 dBm |  |
| Interferer bandwidth | 200 kHz | 5 MHz |
| Interferer offset from NB-IoT chanel edge | ±200 kHz | ±2.5 MHz |

Proposal 2: Rel-13 NB-IoT UE ACS requirement is stringent enough to guarantee the coexistence with neighboring CDMA system, and no additional requirement is required.

## 2.3 NB-IoT UE ACLR Based on Scenario-3

Based on our updated simulation results for the coexistence scenario 3 [4], to satisfy the CDMA performance loss criteria that capacity loss is less or equal to 5%, the requirement for NB-IoT UE ACLR should be equal or larger than 45 dB for its own bandwidth. By revisiting current Rel-13 NB-IoT requirement defined in TS 36.101, as listed in the below table, the current ACLR requirements is 20dB over adjacent GSM channel and 37dB over adjacent UTRA channel.

Table 2.3.1 NB-IoT UE ACLR Requirement in TS 36.101

| Victim system | GSM | UTRA |
| --- | --- | --- |
| ACLR | 20 dB | 37 dB |
| Adjacent channel measurement bandwidth | 180 kHz | 3.84 MHz |
| Measurement filter | Rectangular | RRC-filter α=0.22 |
| Adjacent channel center frequency offset from NB-IoT Channel edge | ±200 kHz | ±2.5 MHz |

Since UTRA and CDMA system has similar the channel bandwidth and channel separation, their corresponding ACLR requirements can be comparable. By scaling the ACLR requirement for UTRA (i.e. 37dB over 3.84MHz measurement bandwidth) to 180kHz, we can reach that around 50dB is required over 180kHz measurement bandwidth, which is more stringent than the requirement for CDMA coexistence, i.e. 45dB. However, on the other hand, the difference between ACLR requirements for legacy system and CDMA is relatively limited, therefore we propose that this NB-IoT UE ACLR requirement could be further analyzed and discussed:

Proposal 3: Based on simulation results, Rel-13 NB-IoT UE ACLR requirement can guarantee certain protection for CDMA system. Whether or not additional requirement is needed could be further analyzed and discussed.

## 2.4 NB-IoT BS ACS Based on Scenario-4

Based on our simulation results for the coexistence scenario 4 [4], the NB-IoT performance loss criteria (equal or less than 1dB SINR loss) can be well satisfied even when NB-IoT BS ACS is 40 dB for its own bandwidth. By comparing with current Rel-13 NB-IoT requirement defined in TS 36.104, the current requirements (-49dBm neighboring interfering signal and PREFSENS + 12 dB for wanted signal) are stringent enough to guarantee the protection for CDMA system. Similarly, the guard band assumption here (1.1 MHz for central frequency separation) also provide enough margin to guarantee the protection requirement for CDMA system can be meet easily.

Proposal 4: Rel-13 NB-IoT BS ACS requirement is stringent enough to guarantee the coexistence with neighboring CDMA system, and no additional requirement is required.

# Conclusion

In this contribution, we provide our analysis on whether or not additional NB-IoT RF requirements is needed to ensure NB-IoT for co-existence with CDMA. Based on our simulation results provided in accompanying papers, the following observations are given:

Proposal 1: Rel-13 NB-IoT BS ACLR requirement is stringent enough to guarantee the protection for CDMA system, and no additional requirement is required.

Proposal 2: Rel-13 NB-IoT UE ACS requirement is stringent enough to guarantee the coexistence with neighboring CDMA system, and no additional requirement is required.

Proposal 3: Based on simulation results, Rel-13 NB-IoT UE ACLR requirement can guarantee certain protection for CDMA system. Whether or not additional requirement is needed could be further analyzed and discussed.

Proposal 4: Rel-13 NB-IoT BS ACS requirement is stringent enough to guarantee the coexistence with neighboring CDMA system, and no additional requirement is required.

# Reference

[1] 3GPP RP-161187, “WID: Study on NB-IoT RF requirement for coexistence with CDMA”, China Telecom.

[2] 3GPP R4-166994, “Way Forward on Simulation Assumption and Methodology for the coexistence study between CDMA and NB-IoT”, China Telecom, Samsung, Huawei, Qualcomm, Ericsson.

[3] 3GPP R4-167363, “Simulation Results on NB-IoT Coexistence Study with CDMA system for Downlink Scenarios”, Samsung.

[3] 3GPP R4-167364, “Updated Simulation Results on NB-IoT Coexistence Study with CDMA system for Uplink Scenarios”, Samsung.