3GPP TSG-RAN WG4 Meeting #81                                                              R4-1609117

# Reno, Nevada, USA, 14 – 18 November, 2016

Agenda Item:	6.1.2.3

Source:	Intel

Title:	MPDSCH simulation results for discussion

Document for:	Discussion

# 1	Introduction

RAN4#80bis agreed with WF[1] and required companies to provide the simulation results. In this contribution, we update our simulation results and proposal.

# 2	Discussion

## 2.1	TM 6 && TM2 CE Mode A Simulation results

The TM6 simulation parameter are in [2]. Table 1 is the simulation parameter list.  Frequency offset = 50Hz is considered. Fig.1 is the simulation results for TM6 with fix PMI=0 and following PMI with PUCCH-1 as well as TM2. There is no repetition.

Figure 1: eMTC MPDSCH TM6 demodulation performance in CE Mode A EPA5

When 2dB impairment is considered, the following SNR is required:

Table 1 summary of TM6 and TM2 simulation results of eMTC UE (FDD, with 2 dB impairment)

| Test number | PMI feedback | Bandwidth and MCS | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE DL category |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| TM6 | PUCCH 1-1 | 10 MHz 16QAM 1/2 | EPA5 | 2x1 Low | 70 | 7.2 | M1 |
| TM2 | N/A | 10 MHz 16QAM 1/2 | EPA5 | 2x1 Low | 70 | 9.2 | M1 |

As there is no big difference between FDD and TDD, we propose:

Proposal 1: For MPDSCH Mode A TM6, SNR= 7.2dB is suggest for FDD and TDD for achieving 70% of maximum throughput.

Proposal 2: For MPDSCH Mode A TM2, SNR= 9.2dB is suggest for FDD and TDD for achieving 70% of maximum throughput.

## 2.2	TM 9 Mode A Simulation results

For PDSCH TM9 test, the simulation results for 504bits with repetition number = 8 is shown in figure 2.

Figure 2: eMTC MPDSCH TM9 TB size 504 bits demodulation performance in CE Mode A EPA5

When 2dB impairment is considered, the following SNR is required:

Table 2 TM9 simulation results of eMTC (FDD, with 2dB impairment)

| Test number | Bandwidth and MCS | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  |  | UE DL category |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Fraction of Maximum Throughput (%) | Repetition numbers | SNR (dB) |  |
|  | 10 MHz QPSK 1/3 (payload=504bit) | EPA5 | 2x1 Low | 70 | 8 | -1.3 | M1 |

Proposal 3: For TBS=504bits and 8 repetitions for PDSCH TM9 Mode A test,  SNR= -1.3dB is suggest for FDD and TDD for achieving 70% of maximum throughput.

## 2.3	TM 2 Mode B Simulation results

Fig.4 is TM2 Mode B simulation results for ETU1 channel.

Figure 4: eMTC MPDSCH TM2 demodulation performance in CE Mode B ETU1

Simulation results is given in Table 3.

Table 3 simulation results of eMTC TM2 (FDD, with 2dB impairment)

| Duplex mode | Bandwidth and MCS | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  |  | UE DL category |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Fraction of Maximum Throughput (%) | Repetition numbers | SNR (dB) |  |
| FDD | 10 MHz QPSK 1/3 | ETU1 | 2x1 Low | 70 | 64 | -15.1 | M1 |

Proposal 4: Set 16 repetitions for PDSCH TM2 Mode B test.  SNR= -15.1dB is suggest for FDD and TDD for achieving 70% of maximum throughput.

# 3	Conclusion

Proposal 1: For MPDSCH Mode A TM6, SNR= 7.2dB is suggest for FDD and TDD for achieving 70% of maximum throughput.

Proposal 2: For MPDSCH Mode A TM2, SNR= 9.2dB is suggest for FDD and TDD for achieving 70% of maximum throughput.

Proposal 3: For TBS=504bits and 8 repetitions for PDSCH TM9 Mode A test,  SNR= -1.3dB is suggest for FDD and TDD for achieving 70% of maximum throughput.

Proposal 4: Set 16 repetitions for PDSCH TM2 Mode B test.  SNR= -15.1dB is suggest for FDD and TDD for achieving 70% of maximum throughput.

# 4	Reference

R4-168697, “WF on eMTC UE demodulation  requirements,” Ericsson, Huawei, HiSilicon, 3GPP RAN4 #80bis, October 2016

# 5	Appendix

Table: MPDSCH simulation parameter

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
| Parameters | CE Mode A | CE Mode A | CE Mode B |
| Target SNR [dB] | 10dB | Option 1: 0dB Option 2: -6dB Option3: [-3dB] | -15dB |
| System bandwidth | 10MHz | 10MHz | 10MHz |
| PDSCH transmission mode | TM6 | TM9 single layer | TM2 |
| MCS | [16QAM 1/2] | [QPSK 1/3]/[ QPSK 1/10] | [QPSK 1/3] |
| Allocated resource blocks | 3 (Note 2) | 6 | 6 |
| TBS | 744 | 152 | 152 |
| OCNG pattern | FFS | FFS | FFS |
| Propagation condition | EPA5 | EPA5 | Option 1: EPA1 Option 2: AWGN Option 3: ETU1 |
| Antenna configuration and correlation matrix | 2x1 low | 2x1 low | 2x1 low |
| OFDM starting symbol (startSymbolLC) | 2 | 2 | 2 |
| Maximum numbers of repetitions | 1 | FFS (Note 1) | FFS (Note 1) |
| Frequency hopping | OFF | ON | ON |
| Frequency hopping offset | N/A | [1] | [1] |
| Number of narrowbands for frequency hopping | N/A | [2] | [4] |
| Frequency hopping interval | N/A | [8] | [16] |
| PMI delay [ms] | FFS | N/A | N/A |
| Reporting interval [ms] | FFS | N/A | N/A |
| Reporting mode | PUCCH 1-1 | N/A | N/A |
| DL Scheduling assumption | See Figure 1 for information | See Figure 2 for information | See Figure 2 for information |
| UE Frequency error [Hz] | [50] (Note 4) | [50] | [100] |
| Note |  | Reuse TS36.101 Table 8.9.1.1.3.1-1 for FDD Reuse TS36.101 Table 8.9.1.2.3.1-1 for TDD |  |
| Note 1) The repetition is decided to satisfy BLER=30% with the target SNR. Note 2) Assume 2 PRBs are allocated for M-PDCCH transmission Note 3) Assume M-PDCCH and PDSCH are scheduled in different narrowbands for tests 2 and 3. Note 4) Companies are encouraged to investigate the degradation with 50Hz UE frequency error compared with no UE frequency error. |  |  |  |