3GPP TSG-RAN WG4 Meeting #81	R4-1609130

Reno, NV, US, 14 – 18 November, 2016

Source:	SoftBank Corp., APT, MTI, Sharp

Title:	TP for TR36.714-02-01: the support of CA_28A-41A BCS1

Agenda item:	8.2.2

Document for:	Approval

1.  Introduction

B28+B41 BCS1 was approved in 2DL/1UL basket WID in the second last RAN-P [1] but left untouched for one RAN-P cycle. This paper is to evaluate IM/harmonics, relaxation required to provide TP for the completion of this combination.

Since the intention to introduce BCS1 is to add 15MHz and 20MHz BW support in Band 28, most of the essential figures remain unchanged.

2.  Reference

[1]      RP-161202   LTE Advanced inter-band CA Rel-14 for 2DL/1UL, Qualcomm (rapporteur).

Annex: TP for TR36.714-02-01 (v0.3.1)

[Unchanged portions skipped]

6.A	CA_28A-41A_BCS1

6.A.1	Operating bands for CA

Table 6.A.1-1: Inter-band CA

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) band |  |  |  | Downlink (DL) band |  |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | Channel BW (MHz) | BS transmit / UE receive |  |  | Channel BW (MHz) |  |
|  |  | FUL_low – FUL_high |  |  |  | FDL_low – FDL_high |  |  |  |  |
| CA_28-41 | 28 | 703 MHz | – | 748 MHz | 3, 5, 10, 15, 20 | 758 MHz | – | 803 MHz | 3, 5, 10, 15, 20 | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 5, 10, 15, 20 | 2496 MHz | – | 2690 MHz | 5, 10, 15, 20 | TDD |

6.A.2		Channel bandwidths per operating band for CA

Table 6.A.2-1: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| CA operating / channel bandwidth |  |  |  |  |  |  |  | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |
| CA_28A-41A | 28 |  |  | Yes | Yes | Yes | Yes | 1 |
|  | 41 |  |  | Yes | Yes | Yes | Yes |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in table 5.6.1-1 of TS 36.101 [6] when operating in single carrier mode.

6.A.3	Co-existence studies

The harmonic relations of Band 28 and Band 41 are calculated as Table 6.A.3-1.  It can be concluded that no harmful fall down toward DL bands of interest is observed.

Table 6.A.3-1: Impact of UL Harmonic Interference

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 28 | 703 | 748 | 758 | 803 | 1406 | 1496 | 2109 | 2244 | 1516 | 1606 | 2274 | 2409 |
| 41 | 2496 | 2690 | 2496 | 2690 | 4992 | 5380 | 7488 | 8070 | 4992 | 5380 | 7488 | 8070 |

Table 6.A.3-2: Band 28 and Band 41 DL harmonics and IMD products

| BS DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 758 | 803 | 2496 | 2690 |
| 2nd order harmonics frequency range (MHz) | 1516 | 1606 | 4992 | 5380 |
| 3rd order harmonics frequency range (MHz) | 2274 | 2409 | 7488 | 8070 |
| 2nd order IMD products | ¦f2_low – f1_high¦ | ¦f2_high – f1_low¦ | ¦f2_low + f1_low¦ | ¦f2_high + f1_high¦ |
| IMD frequency limits (MHz) | 1693 | 1932 | 3254 | 3493 |
| 3rd order IMD products | ¦f2_high – 2*f1_low¦ | ¦f2_low – 2*f1_high¦ | ¦2*f2_low – f1_high¦ | ¦2*f2_high – f1_low¦ |
| IMD frequency limits (MHz) | 890 | 1174 | 4189 | 4622 |
| 3rd order IMD products | ¦2*f1_low + f2_low¦ | ¦2*f1_high + f2_high¦ | ¦2*f2_low + f1_low¦ | ¦2*f2_high + f1_high¦ |
| IMD frequency limits (MHz) | 4012 | 4296 | 5750 | 6183 |
| 3rd order IMD products | ¦f1_low – f2_high + f2_low¦ | ¦f1_high + f2_high – f2_low¦ | ¦f2_low – f1_high + f1_low¦ | ¦f2_high + f1_high – f1_low¦ |
| IMD frequency limits (MHz) | 564 | 997 | 2451 | 2735 |
| 3rd order IMD products (with maximum channel bandwidth) | (f1_low – f2_BWmax) | (f1_high + f2_BWmax) | (f2_low – f1_BWmax) | (f2_high + f1_BWmax) |
| IMD frequency limits (MHz) | 738 | 823 | 2476 | 2710 |

It can be seen from Table 6.A.3-2 that:

- the 2nd harmonics of BS transmitting in Bands 28 and 41 may fall into the BS receive band of Band 46,

- the 3rd harmonics may fall into BS receive band of Bands 30 and 40,

- the 2nd IMD products may fall into BS receive band of Bands 1, 2, 3, 4, 9, 10, 22, 25, 33, 35, 36, 37, 39, 42, 65, 66 and 70,

- the 3rd IMD products may fall into the BS receive band of Bands 5, 6, 7, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 38, 41, 44, 46 and 68,

assuming (except the last row) the whole 45MHz DL frequency of Band 28 and the whole 194 MHz DL frequency of Band 41.

If the BS is only transmitting up to 20 MHz DL in Band 28 and 20MHz DL in Band 41 as stated in the WID, then the 3rd IMD products will not fall into the BS receive band of Band 5, 6, 12, 17, 19, 20 and 68 as shown in the last row in Table 6.A.3-2. In addition, at the time of writing, B46 is not assumed to support UL transmission.

It should be noted that, at the time of writing, Bands 2, 4, 10, 13, 14, 22, 25, 27, 20, 65, 66 and 70 are not intended for use in the same geographical area as Bands 28 and 41. In TDD mode, IMD or harmonics caused by its own Tx does not affect its Rx so Band 41 can be excluded. Band 44 is unlikely to use with Band 28 as Band 44 is TDD relevant of Band 28. Therefore, the focus here will be on the harmonics and IMD falling into Bands 1, 3, 7, 8, 9, 18, 26, 28, 33, 35, 36, 39, 40 and 42.

With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the IMD interference generated within the Band 1, 3, 7, 8, 9, 18, 26, 28, 33, 35, 36, 39, 40 or 42 receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Bands 28 and 41 BS transmitters do not share the same antenna with Band 1, 3, 7, 8, 9, 18, 26, 28, 33, 35, 36, 39, 40 or 42 BS receiver.

6.A.4	ΔTIB and ΔRIB values

For two simultaneous DL and only one UL, the TIB,c and RIB values are given in the tables below. The values are the same as 28A-41A BCS0 in [5].

Table 6.A.4-1: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_28A-41A | 28 | 0.3 |
|  | 41 | 0.3 |

Table 6.A.4-2: ΔRIB

| Inter-band CA Configuration | E-UTRA Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_28A-41A | 28 | 0 |
|  | 41 | 0 |

[Unchanged portions skipped]

form change history:

2013-12-06 v1.14.1 modified §11 to read: <FamilyName>, <GivenName>, (If the person is new to 3GPP work, give full contact coordinates, in particular, email address.)

2013-10-03 v1.14.0 removal of embedded help text

v1.13.2: adds tdoc header

v1.13.1: minor changes resulting from discussions at CT#41 & SA#41

v1.13.0: mods to enforce linkage amongst stages 1, 2, 3

draft mods Scarrone-Meredith 2008-07 ff

v1.12.1: removes revision marks following approval at SP-29
v1.12.0: includes provision for Study Items (SP-29)

v1.11.0: includes those changes from v1.8.0 agreed at SP-25.

v1.10.0: full circle

v1.9.0: a clean sheet

v1.8.0: includes comments from SA#24

v1.7.0: includes comments from RAN, CN and T #24; also includes “early implementation” data

v1.6.0: includes comments made during review period prior to TSGs#24

v1.5.0: includes comments made at TSGs#23 (Phoenix)

v1.4.0: offered to SA#23 for approval

v1.3.0: offered to CN#23, RAN#23 and T#23 for comments

DRAFT4 v1.3.0: 2004-03-09: Incorporation of comments from Leaders list

DRAFT3 v1.3.0: 2004-02-19: Incorporation of comments from MCC members

DRAFT2 v1.3.0: 2004-01-29: Complete redraft:

v1.2.0: 2002-07-04: "USIM" box changed to "UICC apps"

2003-05-28: spelling of “rapporteur” corrected

2002-07-04: "USIM" box changed to "UICC apps"