3GPP TSG-RAN WG4 Meeting #81                                                                                         R4-1609342

Reno, NV, USA, 14 – 18 Nov, 2016

Agenda Item:	6.3.4.2.2

Source: 	Samsung

Title: 	Simulation results for NB-IoT NPUCCH format1

Document for:	Discussion

Introduction

In this contribution, we give both alignment and impairment results for NB-IoT PUSCH format 1.

Discussion

Evaluation cases were summarized in table below.

| Test Number | Subcarrier Spacing | Number of Tones | Number of RU | Number of Repetition | Propagation Condition | Antenna Configuration | MCS | Modulation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 3.75kHz | 1 | 2 | 1, 16, 64 | ETU1 | 1 X 2 | 0 | BPSK |
| 2 | 15kHz | 1 | 2 | 1, 16, 64 | ETU1 | 1 X 2 | 0 | BPSK |
| 3 | 15kHz | 12 | 1 | 2, 16, 64 | ETU1 | 1 X 2 | 9 | QPSK |
| 4 | 15kHz | 6 | 1 | 2, 16, 64 | ETU1 | 1 X 2 | 7 | QPSK |
| 5 | 15kHz | 3 | 1 | 2, 16, 64 | ETU1 | 1 X 2 | 3 | QPSK |

| * Maximum number of transmission is 4 |
| --- |
| * Channel estimation lengths are 16ms for 3.75kHz and 4ms for 15kHz |  |  |
| * Timing/frequency offset estimation is performed |  |

Alignment results: Reference SNR for 70% relative throughput was given in table 1 below. And table 2 gives impairment results.

Table 1: Alignment Results

| Test Number | Number of Repetition | SNR [dB] @ 70% Maximum Throughput |
| --- | --- | --- |
| 1 | 1 | -4.0 |
|  | 16 | -11.2 |
|  | 64 | -13.9 |
| 2 | 1 | -4.6 |
|  | 16 | -12.0 |
|  | 64 | -15.7 |
| 3 | 2 | -2.3 |
|  | 16 | -10.7 |
|  | 64 | -14.7 |
| 4 | 2 | -3.4 |
|  | 16 | -11.3 |
|  | 64 | -15.2 |
| 5 | 2 | -5.8 |
|  | 16 | -12.6 |
|  | 64 | -16.5 |

Table 2: Impairment Results

| Test Number | Number of Repetition | SNR [dB] @ 70% Maximum Throughput |
| --- | --- | --- |
| 1 | 1 | -2.5 |
|  | 16 | -9.2 |
|  | 64 | -11.9 |
| 2 | 1 | -3.1 |
|  | 16 | -10.0 |
|  | 64 | -13.7 |
| 3 | 2 | -0.8 |
|  | 16 | -8.7 |
|  | 64 | -12.7 |
| 4 | 2 | -1.9 |
|  | 16 | -9.3 |
|  | 64 | -13.2 |
| 5 | 2 | -4.3 |
|  | 16 | -10.6 |
|  | 64 | -14.5 |

Conclusion

In this contribution, we give simulation results for NB-IoT PUSCH format 1.