3GPP TSG-RAN WG4 Meeting #81	R4-1609895

Reno, NV, USA, November 14-18, 2016

Agenda Item:	5.2.3.2

Source: 		Ericsson

Title: 	Simulation results for TM9 8.3.1.1 and 8.3.2.1A  test 2 with 4Rx

Document for:	Discussion

# Introduction

In the WF [1],  it was agreed to investigate the performance of the testcases of 8.3.1.1(FDD) #2 and 8.3.2.1A(TDD) #2. in the next RAN4 meetings since there were indications that there is a problem to use the 2Rx duplication in this case.

# Discussions and Results

Below are simulation results for the section 8.3.1.1, Test#2 with 2Rx, 4Rx with duplicated paths (Type 2 UE test) and 4Rx.

Figure 1: Simulation results of FDD testcase in 8.3.1.1 test#2 with 4Rx using eIRC

Below is also the results from the corresponding TDD testcase, 8.3.2.1A, Test2.

Figure 2  Simulation results of TDD testcase in 8.3.2.1A test#2 with 4Rx using eIRC

The collected results of these simulations at 70% of the maximum throughput for each of these testcases are shown in Table 1.

Table 1 Simulated SNR at 70% of the Max throughput for 8.3.1.1 and 8.3.2.1A with 2Rx, 4Rx with duplication and 4Rx

|  | 2Rx | 4Rx with duplication | 4Rx |
| --- | --- | --- | --- |
| 8.3.1.1#2 | 17,9 | 14,9 | 11,2 |
| 8.3.2.1A#2 | 17,8 | 14,4 | 11,1 |

Observation 1: The simulated results indicats that for both FDD and TDD the 4Rx with duplicated paths has a performance 3 dB better than the receiver using 2Rx. Thereby it seems that the testing of the legacy requirement with duplicated paths works when applicability rule in 8.1.2.6.1 are applied.

# Conclusions

Observation 1: The simulated results indicats that for both FDD and TDD the 4Rx with duplicated paths has a performance 3 dB better than the receiver using 2Rx. Thereby it seems that the testing of the legacy requirement with duplicated paths works when applicability rule in 8.1.2.6.1 are applied.

# References

- R4-168630 Way forward on TM9 Test 8.3.1.1 #2 with 4Rx, Intel, Qualcomm, ZTE, Ericsson, Samsung

- R4-167248 Remaining issue of 4 RX AP Performance Requirement, Intel